3GPP TSG-RAN WG4 Meeting #117                                            	  R4-2520268

Dallas, TX, USA, November 17-21, 2025

Title:	Views on 6G coexistence studies

Source:	Qualcomm Incorporated

Agenda item:	8.4.3

Document for:	Approval

# Introduction

In [1], the study item on 6G was approved in RAN#108 and the first 6G discussion starts in RAN4#116bis meeting. In [2], a WF in RAN4#116bis meeting was approved for the BS RF track, entailing agreements related to 6G coexistence studies, which is captured below.

List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

In this contribution, we share our views on the above agreements regarding 6G adjacent channel coexistence studies during the 6G study item.

# Coexistence studies

Historically, adjacent channel coexistence studies have been carried out in RAN4 to derive some of the Tx/Rx RF requirements (e.g., ACLR/ACS, etc.).

## Coexistence for sub 6GHz bands

The goal in this section is to identify the main RF characteristics of 6G which would eventually require additional coexistence studies compared to the ones already performed for NR. (i.e., TR 38.803, TR 38.921, and TR 38.922). It is important first to note that the main RF requirements that are byproduct of the coexistence study in NR were based on coexistence work done in LTE. In other words, no additional coexistence work for sub 6GHz was carried out in NR study item and subsequent different work items.

Observation 1: No additional coexistence study was done for sub 6GHz when studying and standardizing NR in RAN4, as the outcome of the LTE coexistence study, documented in TR 36.942, was sufficient.

Consequently, the RF requirements from the 6G coexistence study are expected to be aligned with NR sub 6 GHz RF requirements. In Table 1, we summarize the main simulation assumptions for the RAN4 adjacent channel coexistence study. The table compares NR parameters, as given in TR 36.942 and TR 38.922 and what we believe are the settings to be adopted for possible 6G sub 6GHz operation. It can be observed that most of the parameters are comparable, and it is not expected to have strong impact in terms of outcome of the adjacent channel coexistence. For example, if the number of antenna elements in 6G SI is higher than what was studied in NR, it is improbable that this will lead to tighter requirements. In other words, existing requirements would still be sufficient to ensure feasible coexistence in the adjacent channel.

Table 1. Summary of simulation parameters coexistence study below 6GHz.

|  | NR Sub 6GHz (TR 36.942 and 4GHz in TR 38.922) | 6G sub 6 GHz |
| --- | --- | --- |
| Methodology | Static Monte-Carlo simulation |  |
| Deployment | Indoor and Hexagonal grid, 9 tri-sector cell with wrap around. |  |
| Victim system | NR, LTE, UMTS, GSM depending on the carrier frequency. | 6G, NR, LTE, UMTS, and GSM depending on the carrier frequency. |
| PathLoss Model | Different models given in TR 38.901 (UMa, Rma, InH, etc.) |  |
| Frequency ranges | 900MHz, 2GHz, and C-band (4 GHz) | C-band and other already existing NR frequency ranges |
| Channel Bandwidth | 100 MHz per carrier component | Still under discussion. Typical value could be greater than 100 MHz per carrier component |
| UL power control | Same as TR 36.942 and TR 38.803 (power control configurations, e.g., bandwidth, noise figure, Clxile, depending on the frequency range) |  |
| DL power control | No power control |  |
| UE antenna | Omni with 0 dBi antenna gain |  |
| UE Tx Power | Up to 31 dBm depending on the specific band. In addition, RAN4 captured typical maximum output power for C-band in TR 38.922. | Maximum output power is still under discussion. However, for coexistence studies, NR assumptions could be reused (depending on the band). |
| BS antenna | AAS parameters for sub6GHz is given in TR 38.922. | RAN4 have documented in TR 38.922 BS AAS parameters for C-band, 7GHz, and 15GHz. |
| SINR to th_put mapping | Scaled Shannon's formula as given in TR 38.922 |  |
| Duplexing | TDD, FDD, and SBFD (TR 38.858). | Still under discussion, with expectation to reuse SBFD coexistence work from Rel-18 to Rel.20. |
| Traffic | Full buffer |  |
| ACI leakage model | ACIR = 1/(1/ACS+1/ACLR) |  |
| Performance metric | Throughput degradation compared to single operator case, i.e. no ACI. Both mean and 5%-tile are considered. |  |

Observation 2: No additional coexistence work is needed for sub 6GHz as existing requirements should suffice to ensure protection from adjacent channel operation for 6G SI considerations.

## Coexistence for FR2 bands

One of the main characteristics of NR coexistence work was the consideration of FR2 adjacent channel coexistence. The coexistence framework, assumptions, results, and conclusions were documented in TR 38.803 for 30 GHz, 45 GHz, and 70 GHz. In addition, RAN4 conducted coexistence study for FR2-2 during Rel-17 and the outcomes were aligned with what was given in TR 38.803, leading, to adopting BS and UE ACLR/ACS for FR2-2 similar to FR2-1.

In addition, latest RAN4 coexistence work in NR was targeting 15 GHz and was documented in TR 38.922. Comparing the 15 GHz parameters and the ones in TR 38.803, one could identify some differences, mainly, regarding BS AAS parameters. It is thus viable to discuss within RAN4 if it is needed to revisit the coexistence work for FR2-1 (i.e., 30 GHz) given that the BS AAS parameters are different between extrapolated values from TR 38.922 and TR 38.803. However, it is important to note that with increasing number of antenna elements at the BS side, the emissions are more directed towards their intended directions and a more feasible coexistence in terms of adjacent channel operation could be achieved. Thus, existing FR2-1 RF requirements would be sufficient even with adjusting the BS AAS parameters for FR2-1 scenarios.

Observation 3: Existing RF requirements from NR are sufficient for 6G FR2-1 scenarios even if BS AAS parameters are updated to reflect increasing number of antenna elements.

## Coexistence for WRC-27 frequency ranges

For 6G-related spectrum, RAN4 studied the different RF parameters and the BS AAS parameters for three different frequency ranges (4 GHz, 8GHz, and 15 GHz) to provide a response to WP5D regarding WRC-27 sharing studies in ITU-R in [2]. In [2], adjacent channel coexistence studies were only conducted for the 15GHz since it represents the only frequency range, where previous RAN4 coexistence work could not be reused/ extended (i.e., 4 GHz parameters were based on defined band n79, while 8GHz parameters were based on n104 adjacent channel coexistence studies that were done in Rel-17 study item, documented in TR 38.921). With the start of the 6G SI [1], our view that it is not needed to redo the coexistence study for the WRC-27 IMT bands, since ITU is already conducting the sharing study based on RAN4 response during Rel-19. If identified strong need to redo the coexistence work, it is recommended to not do it specifically for those frequency ranges.

Observation 4: No additional coexistence work is needed for the frequency ranges 4.4-4.8 GHz, 7.125-8.4 GHz, and 14.8-15.35 GHz, since RAN4 identified the main RF parameters and captured them in TR 38.922.

In the following section, we will highlight our views on the different aspects that could be further investigated to enhance the RAN4 adjacent channel coexistence methodology.

# 3      Potential simulation methodology improvements

## BS modelling

Since the initial NR study item, the AAS model described in Section 7.7 of TR 38.922 has been used for numerous studies in RAN4, including AAS, NR WI, IAB, etc. The model has been adopted in other forums outside RAN4 and is also described in RAN1 and in ITU-R in recommendation M.2101. It is noteworthy that the baseline model given in M.2101 has been extended to account for subarray architectures. Beside the model, one of the main AAS parameters is base station horizontal and vertical coverage/ steering range. The horizontal steering range is typically set to +/- 60 degrees to address the typical tri-sectorization of the cell site. For the vertical steering range, it typically allows 10-20 degrees of vertical for macro urban and suburban deployments, while larger steering range is allowed for indoor wall-mounted ceiling deployments.

While utilizing the abovementioned AAS model (i.e., extended M.2101 with subarrays), the beamforming weights are generated in each snapshot typically depending on the vertical and horizontal steering angles, derived based on the location of the transmitter and receiver. Perfect location of the Tx and Rx is typically assumed (i.e., full CSI knowledge), and beamforming gain is computed according to the model. It is probable that depending on the location of either the Tx or Rx, the generated beam might be outside of the maximum steering range. Based on the adopted model, further clarification is needed within RAN4 on how to address the cases where the generated beam might be outside of the steering range. We believe that such modeling detail should be further studied and clarified in 6G coexistence studies to ensure that the generated beams from the adopted beamforming model mimics what is happening in realistic scenarios.

Observation 5: If additional coexistence studies are agreed in RAN4, RAN4 could study how to harmonize in the coexistence framework the incorporation of the parameterized BS AAS steering limits in 6G coexistence studies.

With the introduction of AAS, the topic of how to model the AAS radiation pattern in the out-of-band adjacent domain has been investigated in different study items in RAN4. In the NR study item, documented in TR 38.803, RAN4 has reached the following conclusion in section 5.2.6:

it is assumed that both ACLR (or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model is expressed as function of ACLR and ACS (assuming ACLR, ACS and ACIR to be linear)

The above conclusion leads to the assumption within the coexistence framework that the total amount of adjacent channel power radiated that matters in the coexistence performance will be fully beamformed similar to the in-band power. Thus, modelling the OOB AAS radiation pattern is typically assumed to be fully correlated as the in-band AAS radiation pattern. 

In TR 38.922, similar discussion was initiated as WP 5D asked RAN4 about how the AAS performs in adjacent bands i.e. does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation. In this regard, the composite AAS radiation gain pattern for pure LOS channel, which is based on the the correlation factor defined in the interval , was further studied, and a stepwise model to quantify that correlation factor was introduced in Section 7.3.2.1.2 in TR 38.922. For that model to be utilized in coexistence studies, the quantification of different parameters, i.e.,  still needs to be determined.

Observation 6: If additional coexistence studies are agreed in RAN4, RAN4 could study the characterization of the underlying parameters of the out-of-band AAS radiation pattern model in TR 38.922.

## UE modelling

During NR study item, as documented in TR 38.803, a UE antenna element model was proposed in Section 5.2.3.3. Such model emulates the presence of two panels with a 180 shift in horizontal orientation while the UE orientation is uniformly distributed in the azimuth domain between -180 and 180 degrees. Since there are two panels assumed, panel selection is assumed between the panels. This UE beamforming model has been used in several RAN4 coexistence studies (e.g., NR study item, ATG, SBFD, and Rel19 IMT-2030 parameters). The UE RF architecture and the beamforming model are an important aspect for discussion for both FR1 and FR3 future coexistence studies. For FR1, the focus should be on how to account for beamforming capability with increasing number of Txs (e.g., 4 or 2 Txs).

On the other hand, for FR3, it will be important to discuss within RAN4 what modelling to be used, if 6G adjacent channel coexistence studies will be carried out, for the case of the beamforming at the UE side and if advancements should be made on the existing model given in TR 38.803.

Observation 7: If additional coexistence studies are agreed in RAN4, proper modeling of UE beamforming gain could be discussed. Both the impact of increasing the number of TXs in FR1 (e.g., 4 or 2 TXs) and the extensions to UE beamforming model (as described in Section 5.2.3.3 in TR 38.803) in FR3 could be addressed.

# 4      Conclusion

In this contribution, we provided our views on future 6G adjacent channel coexistence studies within RAN4 with the following observations and proposals:

Observation 1: No additional coexistence study was done for sub 6GHz when studying and standardizing NR in RAN4, as the outcome of the LTE coexistence study, documented in TR 36.942, was sufficient.

Observation 2: No additional coexistence work is needed for sub 6GHz as existing requirements should suffice to ensure protection from adjacent channel operation for 6G SI considerations.

Observation 3: Existing RF requirements from NR are sufficient for 6G FR2-1 scenarios even if BS AAS parameters are updated to reflect increasing number of antenna elements.

Observation 4: No additional coexistence work is needed for the frequency ranges 4.4-4.8 GHz, 7.125-8.4 GHz, and 14.8-15.35 GHz, since RAN4 identified the main RF parameters and captured them in TR 38.922.

Observation 5: If additional coexistence studies are agreed in RAN4, RAN4 could study how to harmonize in the coexistence framework the incorporation of the parameterized BS AAS steering limits in 6G coexistence studies.

Observation 6: If additional coexistence studies are agreed in RAN4, RAN4 could study the characterization of the underlying parameters of the out-of-band AAS radiation pattern model in TR 38.922.

Observation 7: If additional coexistence studies are agreed in RAN4, proper modeling of UE beamforming gain could be discussed. Both the impact of increasing the number of TXs in FR1 (e.g., 4 or 2 TXs) and the extensions to UE beamforming model (as described in Section 5.2.3.3 in TR 38.803) in FR3 could be addressed.

# References

- RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), RAN Meeting #108, Prague, Czech Republic.

- R4-2514638, “WF on 6G BS RF and co-existence” Feature Lead (Ericsson), RAN4#116bis, Prague, Czech Republic.