3GPP TSG-RAN WG4 Meeting #117										                    R4-2520291

Dallas, USA, November 17th – 21st, 2025

Source:	Murata Manufacturing Co.

Title:	4.9GHz AMPR Requirement Impact

Agenda item:	5.11.4

Document for:	Approval

# 1.	Introduction

In this contribution, we will present views for the UE RF requirements and the impact on AMPR for the NR TDD 4.9GHz band.

# 2.	Discussion

The following agreements were made in the last WF [1] regarding the emission mask requirements. We present our views on some clarifications of the agreement considering FCC regulations. The first 3 agreements will help shape the new 3GPP AMPR emissions mask and hence the new requirement for the new 4.9GHz band. As per the last agreement of the WF, we will assess the potential AMPR impact of the new emission requirement by looking at some preliminary measurements.

<Agreement>: Resolution bandwidth

- Adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth.

  - Spec wording is FFS.

<Agreement>: Zero dB reference for applicable FCC Emission mask M

- The zero dB reference for FCC Emission mask M is calculated based on UE power class maximum output power and the configured UE channel bandwidth.

  - The zero dB reference is used for mask location and for A-MPR analysis. .

<Agreement>: New NS, Impacted frequency range for new NS, Other requirements for new NS

- Specify a new NS for the US 4.9GHz band with FCC part 90 including:

  - The Mask M requirement (note this covers the 5% channel bandwidth edge (inside), the SEM domain and the spurious domain).

  - Maximum output power limitation in Sub-topic 4-1.

<Agreement>:

For the next meeting, companies are invited to provide the simulation assumption to have a unified understanding of assumption when the simulation campaign starts.

<Note>:

It is encouraged companies to study the impact of FCC part 90 for A-MPR simulation.

## AMPR Requirements and Emission Mask

Looking at only FCC part 90 section 90.210 [2], and based on the agreed WF [1], we present a mask like the one that was presented in [5]. Unlike the SEM mask in the general requirement, which remains fixed versus power class, the 4.9GHz band additional emission requirement scales versus power class according to our interpretation of the regulatory requirement. The scaling of the mask versus power class could allow simplification of the back-off requirements for single TX operation for example and slight adjustments to the back-off could be made for multi-TX operation to account for antenna interaction effects.  Figure 2.1-1 shows the 20MHz mask. The emission masks of all other supported BWs are shown in section 5.1 of the appendix. As also previously reported, the mask will be tighter than the 3GPP SEM mask when comparing it to a common measurement bandwidth. Not only is the level requirement tighter in the ΔFOOB and spurious regions, but there is also a mask within the channel BW that overlaps the guard-band as well as allocated RBs, which undoubtedly requires excessive back-off when there is full spectral utilization of the allocated RBs.

From the FCC regulation [2], our interpretation is that the zero dB reference is relative to the highest average power rather than the instantaneous power. Having the power referenced to the highest average power would lead to an emissions type spec rather than an ACLR type spec if the instantaneous power is used. Coexistence among other users is paramount, especially for low LCRB allocations, so it seems logical to fix the reference according to power class. The 0dB reference for the FCC Mask with MBW of 1% of channel BW, would then be as follows:

0dBrdBm/MBW=PowerClassdBm+10log10MBWCBW

The reference shows the mask scaling versus the power class unlike the general SEM mask. The 0dB reference does not include the power class tolerance because PPowerClass in TS38.101-1 is defined as nominal power without tolerance. This simplifies the back-off simulation and measurement procedure. The FCC backoff will include the effect of tolerances as will be discussed later in section 2.2, but the emission related backoff, as with other NS emission requirements, will be independent of power class tolerance.

Figure 2.1-1: 20MHz Emissions mask example

Observation 1: The 0dB reference, 0dBr, and the emission mask scales with the power class, do not include tolerances, and is independent of the actual transmission bandwidth of the allocated waveform.

The example of the general A-MPR table and the mapping of the network signalling label are shown in tables 2.1-1 and 2.1-2. The actual proposed emission mask is shown in Table 2.1-3.

Table 2.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_XX | TBD | nXXX | 10, 15, 20, 25, 30, 40, 50 | Table 5.3.2-1 | Table 2.1-2 |

Table 2.1-2: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| nYYY | NS_01 | NS_XX |  |  |  |  |  |  |

Proposal 1: The  zero dB reference can be taken as follows. The emission limits shall be taken relative to 0dBr, and the power back-off shall be taken relative to 0dB MPR:

0dBrdBm/MBW=PPowerClass+10log10MBWBWchannel

Proposal 2: Define a the 4.9GHz emissions mask in TS38.101 as shown in Table 2.1-3.

Table 2.1-3: Additional requirements for 4.9GHz band "NS_XX"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBr) | Measurement bandwidth, MBW |
| --- | --- | --- |
|  | 10, 15, 20, 25, 30, 40, 50 MHz |  |
| Fc ± 0.45*BWchannel ≤ f < Fc ± 0.5*BWchannel | -568*log10( ¦f-Fc¦ / (.45*BWchannel) ) | 1 % of Channel BW |
| Fc ± 0.5*BWchannel ≤ f < Fc ± 0.55*BWchannel | -26-145*log10(¦f-Fc¦ / (.50*BWchannel) ) |  |
| Fc ± 0.55*BWchannel ≤ f < Fc ± BWchannel | -32-31*log10(¦f-Fc¦ / (.55*BWchannel) ) |  |
| Fc ± BWchannel ≤ f < Fc ± 1.5*BWchannel | -40-57*log10(¦f-Fc¦ / (BWchannel) ) |  |
| F<Fc-1.5*BWchannel, f >Fc+1.5*BWchannel | Max (-50, 25+ PPowerClass) |  |
| Note 1: 0dBr = PPowerClass + 10*log10(MBW/BWchannel); The spectrum emission limit is the attenuation from 0dBr Note 2: The frequency range applies to either the higher side or the lower side of Fc Note 3: 4940MHz + BWchannel / 2 ≤ Fc ≤ 4990MHz - BWchannel / 2 |  |  |

## AMPR due to Maximum Power Limitations with tolerances

The following is the excerpt from the WF [1] regarding power limitations. FCC addresses the conducted power and power spectral density (PSD) limitations in FCC part 90 section 90.1215 [3].

<Agreement>: Maximum output power

- Only FCC “high power device” is considered for 3GPP UE, and two requirements below need to be satisfied.

  - Peak power spectral density: 21 dBm per one MHz.

  - High power maximum conducted output power:

| Channel bandwidth (MHz) | High power maximum conducted output power (dBm) |
| --- | --- |
| 10 | 30.0 |
| 15 | 31.8 |
| 20 | 33.0 |
| 25 | 34.0 |
| 30 | 34.8 |
| 40 | 36.0 |
| 50 | 37.0 |

<Way forward>: Maximum output power tolerance

- FFS impact of maximum output power tolerance on maximum output power requirements above.

There will be 2 types of backoff for the 4.9GHz band. There is back-off to meet the emission requirement discussed in section 2.1 and the back-off to meet the FCC maximum power limitation. Of the FCC maximum power limitation, there are 2 sub-categories for back-off. There is the conducted power back-off as well as the PSD back-off. The FCC backoff was also discussed in previous contributions [6] [7]. The FCC related back-off will include the effect of the power class tolerances unlike the emission related back-off.

Observation 2: To meet the FCC maximum power limitation, the FCC related back-off must include the effect of power class tolerance.

Using the FCC PSD limit of 21dBm/MHz, the FCC PSD Backoff for any RB allocation and any supported channel bandwidth is as follows:

FCC PSD BackoffdB=-min⁡21dBm/MHz-AssignedPSD1MHzBW,0

AssignedPSD1MHzBW=PowerClassdBm+toldB-10log10max1MHz,TransBWMHz

TransBWMHz=12∙SCS∙LCRB

Figure 2.2-1 shows the FCC PSD back-off required Vs SCS Vs Power Class Vs LCRB for any supported channel BW assuming 2dB tolerance. Table 2.2-1 shows the LCRB threshold for {Power Class, SCS} for no FCC PSD limitation with/without tolerances. If the NRB for the given channel BW exceeds the LCRB threshold in table 2.2-1, then those channel BWs for the given SCS will have FCC back-off regardless of RB allocation. This happens for 10MHz BW, PC1.5 and PC1 and for 15MHz PC1 for the +2dB tolerance. These lower bandwidths are still useful at the higher power classes if the backoff is less than 3dB for the higher LCRB allocation.

Observation 3: 10MHz BW, PC1.5 and PC1 and for 15MHz PC1 will always have backoff due to FCC PSD limitation regardless of the RB allocation.

Figure 2.2-1: FCC PSD back-off

Table 2.2-1: LCRB threshold for FCC PSD back-off

After the FCC PSD limit of 21dBm/MHz has been reached, the FCC Conducted power back-off, using the limits from the FCC table in [3] at the minimum supported channel bandwidth for the given allocated LCRB, is as follows:

FCC Conducted BackoffdB=-min⁡FCClimitminCBW-ConductedPowerdBm,0

ConductedPowerdBm=FCC limited PSDdBm/MHz+10log10max1MHz,TransBWMHz

FCC limited PSDdBm/MHz =AssignedPSD1MHz-FCC PSD BackoffdB

The channel BWs Vs power class showing the breach of the FCC conducted power limit is shown in Figure 2.2-2. The LCRB thresholds of the channel bandwidths having the conducted power limit problem is shown in Table 2.2-2. It is shown that the conducted power back-off required, after the PSD back-off is reached, ranges from 0.8dB max at SCS=15KHz and 0.4dB max at SCS=60KHz. Tables showing the FCC PSD and conducted power breach for various LCRB allocation, minimum supported BW, and power class are shown in sections 5.2 and 5.3 in the appendix.

Figure 2.2-2: Channel BWs and power class requiring FCC conducted power back-off when LCRB > threshold in Table 2.2-2

Table 2.2-2: LCRB threshold for FCC Conducted power back-off

The Emission-related backoff, AMPRemissions in section 2.1 can be first measured or simulated without accounting for tolerances and without applying FCC backoff in section 2.1. The final AMPR using both the emission related backoff and the FCC related backoff (with tolerance) can then be expressed in proposal 3.

Proposal 3: The total backoff for the 4.9GHz band can be expressed as follows:

Total Backoff=max⁡(MPR, AMPR)

AMPR=max⁡(AMPRemissions ,FCC BackoffdB)

FCC BackoffdB=FCC Conducted BackoffdB+FCC PSD BackoffdB

The key takeaway is that the FCC related back-off is predictable for any LCRB and channel bandwidth, and thus no restriction should be placed on the supported channel bandwidths. There maybe large backoff associated with the FCC emission and power limitation, but appropriate resources could be configured to minimize the back-off, so that the configured power at the higher power class with backoff does not fall below the power level capability at the next lower power class.

Proposal 4: All channel bandwidths specified in the WID [4] shall be supported for all power classes regardless of power limitations.

## AMPR Preliminary Study

The following sections discuss a preliminary AMPR study pending the final agreement of the AMPR requirements. The following areas of interest are analyzed.

- Emission backoff due to FCC regulation inside the channel bandwidth.

- Emission backoff due to low LCRB without FCC power limitation backoff.

- LCRB extension ratio for no emission back-off or 0dB AMPR.

- Expected AMPR regions.

Emission backoff due to FCC regulation inside the channel bandwidth:

As mentioned in section 2.1, the FCC mask specifies emission requirements within the channel bandwidth outside of the 90% spectral utilization region. Since the fully allocated NR signal has potentially 93.8% to 97.2% spectral utilization, significantly large back-off would be required for allocations involving the outer RBs since the mask attenuation region overlaps the guard-band. Lower LCRB at the guard-band offset would get the largest back-off. The results are shown in Table 2.3-1. The amount of back-off depends on the guard-band of the channel bandwidth and the actual measurement bin offset from the 90% utilization point. For Fully allocated waveforms, the back-off ranges from 7.5dB to 16.5dB. For single RB waveforms, the back-off ranges from 27.5dB to 36.5dB due to the higher PSD for the given power class. As expected, the more the utilization, the more RB shift is required to have no back-off due to utilization.

Table 2.3-1: PC3 Estimated/Measured back-off of outer RBs due to full spectral utilization for 15KHz SCS with fully allocated waveforms. For single RB waveforms, the backoff is much larger.

Observation 4: Large back-off is expected for allocations outside of the 90% spectral utilization region. For fully allocated waveforms, the back-off ranges from 7.5dB to 16.5dB. For single RB waveforms, the back-off ranges from 27.5dB to 36.5dB due to the higher PSD for the given power class.

Emission backoff due to low LCRB without FCC power limitation backoff.

Preliminary measurements were made for the TX+LO, TX+Image,TX+CIMD3 intermodulation products. If the emission related backoff is less than the FCC-related backoff, then from the discussion in section 2.2, the maximum of the 2 values can be taken. It is not inconceivable for UE implementations to pre-program the FCC related backoff with tolerance for a specific RB allocation. Table 2.3-2 shows the emission back-off for low LCRB allocation. The required FCC backoff of 4dB to meet the PSD limit of 21dBm/MHz from section 2.2 is insufficient to meet the emission requirement. The TX Image IMD product requires 7-8dB backoff. Also, note that the figure above the table shows that 0dBr is NOT equal to 0dB MPR. The back-off is always taken relative to 0dB MPR, and the emission limits are taken relative to 0dBr as pointed out in proposal 1.

Table 2.3-2: PC3 1RB back-off for single RB at the 90% spectral utilization point

Observation 5: The FCC-related power back-off is insufficient to meet the emission requirements for low LCRB allocations less than the thresholds indicated in Table 2.2-1 and when the allocation is within the 90% spectral utilization region. For PC3 about 7-8dB back-off is required to meet the FCC mask.

LCRB margin ratio for no emission back-off or 0dB AMPR waveforms.

As shown in the power domain enhancement work item, there will be a specific margin ratio for which no power back-off is necessary for which ACLR1 emission will fall under the FCC mask as illustrated in Figure 2.3-1. Table 2.3-3 shows the LCRB margin required on either side of the allocated RBs. It was shown that a 62% margin (ratio of symmetrical RBshift/LCRB) was required. The table only shows PC3 and could possibly be applied to PC2.

For the higher power classes like PC1.5 or PC1, the FCC PSD related backoff exceeds the emission related backoff for the 10, 15MHz bandwidth cases, and consequently, there will be no 0dB AMPR waveforms. But, there could be < 3dB AMPR waveforms to at least allow a higher power than the previous power class.

Table 2.3-3: PC3/PC2 Estimated RB shifts on either side of the LCRB allocation for no AMPR

Figure 2.3-1: Spectral plot showing LCRB margin for no AMPR. 0dB AMPR waveform

Observation 6: No back-off is required with about a 62% LCRB margin on each side of the allocation for the BW cases provided there is no FCC conducted power limitation.

Figure 2.3-2: Back off for fully allocated waveform without SU backoff

Observation 7: At the maximum LCRB allocation without exceeding 90% spectral utilization and when there is no FCC conducted backoff, about 3-3.5dB emission-related back-off is observed.

Expected AMPR regions

With all previous points considered and requirements are finalized with a common understanding, we can expect the following AMPR regions and verify after embarking on a detailed simulation plan.

Figure 2.3-3: Expected AMPR regions

In this sub-section, we discussed the backoff due to emissions mask within the channel bandwidth, and in the figure, this region is denoted as SU backoff. Referring to Figure 2.3-3, for low LCRB, there is excessive backoff and this will dominate and trump the backoff due to FCC power limitation, but for larger LCRB exceeding 90% SU, the SU backoff should dominate. The number of outer RBs affected depends on the channel bandwidth.

The low LCRB backoff due to FCC limitation was discussed in subsection 2.2. The backoff in this section will vary with SCS and reduce as the LCRB increases. There will be FCC backoff due to large LCRB, but the backoff due to emissions in the SU region as well as emission related backoff of 3-3.5dB will trump the FCC power backoff.

There is also low LCRB emission due to TX image IMD where the allocation falls within the 90% utilization region for which the FCC PSD limitation backoff is also not enough. About 7-8dB back off is required. Full RB backoff is about 3-3.5dB without overlapping the SU region.

Finally, there will be RB allocation with RBshift_L and RBshift_H, where no backoff will be necessary or 0dB AMPR. This will be like an inner AMPR RB allocation specific to the 4.9GHz band.

Actual remaining region definitions and backoff versus power class are FFS awaiting final agreement for the AMPR requirements.

# Conclusion

Observation 1: The 0dB reference, 0dBr, and the emission mask scales with the power class, do not include tolerances, and is independent of the actual transmission bandwidth of the allocated waveform.

Proposal 1: The  zero dB reference can be taken as follows. The emission limits shall be taken relative to 0dBr, and the power back-off shall be taken relative to 0dB MPR :

0dBrdBm/MBW=PPowerClass+10log10MBWBWchannel

Proposal 2: Define a the 4.9GHz emissions mask in TS38.101 as shown in Table 2.1-3.

Table 2.1-3: Additional requirements for 4.9GHz band "NS_XX"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBr) | Measurement bandwidth, MBW |
| --- | --- | --- |
|  | 10, 15, 20, 25, 30, 40, 50 MHz |  |
| Fc ± 0.45*BWchannel ≤ f < Fc ± 0.5*BWchannel | -568*log10( ¦f-Fc¦ / (.45*BWchannel) ) | 1 % of Channel BW |
| Fc ± 0.5*BWchannel ≤ f < Fc ± 0.55*BWchannel | -26-145*log10(¦f-Fc¦ / (.50*BWchannel) ) |  |
| Fc ± 0.55*BWchannel ≤ f < Fc ± BWchannel | -32-31*log10(¦f-Fc¦ / (.55*BWchannel) ) |  |
| Fc ± BWchannel ≤ f < Fc ± 1.5*BWchannel | -40-57*log10(¦f-Fc¦ / (BWchannel) ) |  |
| F<Fc-1.5*BWchannel, f >Fc+1.5*BWchannel | Max (-50, 25+ PPowerClass) |  |
| Note 1: 0dBr = PPowerClass + 10*log10(MBW/BWchannel); The spectrum emission limit is the attenuation from 0dBr Note 2: The frequency range applies to either the higher side or the lower side of Fc Note 3: 4940MHz + BWchannel / 2 ≤ Fc ≤ 4990MHz - BWchannel / 2 |  |  |

Observation 2: To meet the FCC maximum power limitation, the FCC related back-off must include the effect of power class tolerance.

Proposal 3: The total backoff for the 4.9GHz band can be expressed as follows:

Total Backoff=max⁡(MPR, AMPR)

AMPR=max⁡(AMPRemissions ,FCC BackoffdB)

FCC BackoffdB=FCC Conducted BackoffdB+FCC PSD BackoffdB

Observation 3: 10MHz BW, PC1.5 and PC1 and for 15MHz PC1 will always have backoff due to FCC PSD limitation regardless of the RB allocation.

Proposal 4: All channel bandwidths specified in the WID [4] shall be supported for all power classes regardless of power limitations.

Observation 4: Large back-off is expected for allocations outside of the 90% spectral utilization region. For fully allocated waveforms, the back-off ranges from 7.5dB to 16.5dB. For single RB waveforms, the back-off ranges from 27.5dB to 36.5dB due to the higher PSD for the given power class.

Observation 5: The FCC-related power back-off is insufficient to meet the emission requirements for low LCRB allocations less than the thresholds indicated in Table 2.2-1 and when the allocation is within the 90% spectral utilization region. For PC3 about 7-8dB back-off is required to meet the FCC mask.

Observation 6: No back-off is required with about a 62% LCRB margin on each side of the allocation for the BW cases provided there is no FCC conducted power limitation.

Observation 7: At the maximum LCRB allocation without exceeding 90% spectral utilization and when there is no FCC conducted backoff, about 3-3.5dB emission-related back-off is observed.

# 4.  References:

- R4-2515103, “Way Forward for [116bis][322] NR_TDD_4.9GHz_band”, Fujitsu, RAN4#116bis, Prague, Czech Republic, June 9-13, 2025.

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I/section-90.210

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y/section-90.1215

- RP-251887, “New WID on Introduction of NR TDD 4.9GHz Band for US Operation”, Fujitsu, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

- R4-2514419, “On US 4.9 GHz UE RF requirements”, Qualcomm, RAN4#116bis, Prague, Czech Republic, June 9-13, 2025

- R4-2514502, “Handling of Power and PSD Limits for the NR 4940-4990 MHz Band for US Public Safety”, Lenovo, Motorola Mobility, RAN4#116bis, Prague, Czech Republic, June 9-13, 2025

- R4-2514330, “TBD”, Skyworks Solutions Inc., RAN4#116bis, Prague, Czech Republic, June 9-13, 2025

# Appendix

## FCC mask M for 10, 15, 25, 30, 40, 50MHz channel BWs

## FCC Power Limitation (Full Allocation)

FCC power breached at Power Class 1.5 and Power class 1 at 15KHz SCS with full allocated waveforms at 10MHz, 15MHz channel BWs. Tolerance of +2dB is applied.

## 5.3 FCC Power Limitation (LCRB at minimum supported BW)