3GPP RAN WG4 Meeting #117																	 R4-2520293

Dallas, US, 17th – 21st November 2025

Agenda item:	8.11

Source:	Apple Inc

Title:	Initial views on 6G OTA Testability

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

In RAN#108, the Rel-20 SID was approved that aimed at enabling the 6G radio access technology for the next-generation radio access system capable of addressing the full spectrum of future use cases, operational constraints, and deployment realities [1]. One of the SID objectives was to define and introduce the testability methodology framework into the 3GPP specifications.

This paper shares a number of views on the OTA testability of potential RF requirements for devices capable of the 6G radio access technology specification.

# 2	Discussion

### 2.1		Improved Testability of Conducted Requirements

The way forward from RAN4#116-bis indicated a sub-topic related to evaluating whether any conducted requirements can be verified via OTA approach.

For regulatory-facing requirements (e.g. emissions) there can be a benefit for the UE to be verified as TRP, since the additional losses through the antenna can be taken into account during device calibration and can result in smaller power backoffs to meet regulatory limits.  This can be an overall advantage to the network, as the UE can deliver more power to the link while complying with regulatory emission limits.  The draw-backs of this approach include: additional test time, implementation complexity to derive the calibrations based on the finished device, and an unclear process to set the requirement.  On the latter aspect, it is not clear whether RAN4 would need to assume a standard antenna efficiency or if it is feasible to consider a simulation or measurement campaign to derive this parameter.

- Observation 1: Verifying regulatory-facing requirements via the OTA approach can yield some benefits to the network, but the trade-off in additional test time, implementation complexity associated with device calibration, and unclear process to specify the requirement should be taken into consideration.

Because there is impact on UE RF architecture as well as the definition of the core UE RF requirement, RAN4 should first discuss whether RF requirements specified in the conducted domain should shift to the radiated domain in the UE RF discussion thread.  Once confirmed, the test methodology enhancement thread can pick up the topic.

Proposal 1: Whether to introduce the OTA approach to requirements currently specified in the conducted domain should be discussed in the UE RF discussion thread for the 6G SI.

The example listed in the FR1 conducted tests which are not verified via radiated approach includes reference to MSD. MSD scenarios for OTA testing. MSD requirements are very complicated to test even in conducted domain as even replicating a subset of the interferer configurations from TS 38.521-1 would likely be highly complex in radiated environment. The focus for the 6G study for this sub-topic should be limited to topics such as Tx/Rx spurious emissions or ACLR/SEM which are also mapped to regulatory requirements. This will help in harmonizing FR1 radiated testing between regulatory and 3GPP test specifications.

Proposal 2: For evaluation of conducted requirements to be verified via OTA approach, do not include MSD scenarios for evaluation due to the complexity of setting up configurations.

### 2.2		New Test Methodologies for New 6G Frequencies

#### 2.2.1	Views on UE Beamforming on new frequency range

In RAN4#116-bis meeting[2], there was an agreement to further study the OTA testing methodology for new frequency range, i.e.:

In 5G networks, beam management including analog beamforming has been one of the key building blocks to operate in higher frequency range, i.e. FR2. Beam management L1/L2 procedures are used to acquire and maintain a set of TRxP(s) and/or UE beams that can be used for DL and UL transmission/reception.

To address the testability aspect of the potential UE test requirements, it might be beneficial to categorize the potential 6GR UE test requirements based on their purpose. Subsequently, we can identify the essential high-level requirements for the corresponding test setup. In NR, baseline testing methodology for frequencies above 24GHz was tested over the air [4]and below a certain frequency threshold, such as [6 GHz], the UE testing setup can be reused from NR-FR1[5].

As captured in TR 38.803, Study on new radio access technology[3], UE beamforming performance results are obtained at 15GHz for UE with 4 and 8 antennas arrays. Similar to NR, 6GR can use baseline over the air test methodology for frequencies more than 15GHz.

For the assessment of UE beamforming performance, the following beamforming methods were evaluated in these simulations: MRT (Maximum Ratio Transmission), PO (Power Only beamforming), AS (or Antenna Selection), Isotropic (Ideal omni-directional radiation), and PMI (Precoding Matrix Indicator).

- Observation 2: UE beamforming performance gain was captured in TR 38.803 at 15 GHz for 5GNR.

Proposal 3: It is feasible to consider over-the-air test methodologies for frequencies > 15GHz.

#### 2.2.2	Views on Temperature Conditions

With the Rel-17 study item on FR2 measurement methodology enhancements concluding that the objective related to ETC testability has been met, test equipment is now capable of performing OTA testing of FR2 devices under Extreme Temperature Conditions (ETC). RAN4 will study and quantify the deltas in the OTA testing core requirements between NTC and ETC for the 6GR new frequency range between FR-1 and FR-2[6].

- Observation 2: 6G frequency range overlaps with existing 5G NR frequency range FR-1 and FR-2.

Proposal 4: Reuse the same temperature condition for 6GR overlapping frequencies with NR.

Proposal 5: FFS how to study and quantify the temperature condition for 6GR new frequency range for OTA testing including recommendations for MU.

#### 2.2.3	Reference Test Methodology for SISO OTA

The definition of frequency ranges is being studied under the 6G Spectrum topic. The WF from RAN4#116-bis indicated multiple criteria to gather company views. It was also agreed that currently the reference should only be to defined frequency ranges such as FR1 (up to 7.125 GHz), range between FR1 and FR2-1 (including around ~7 GHz) and FR2-1 (24.25 GHz-52.6 GHz)

While any new test methodologies for the new frequency range is contingent upon the definition of the details of this range, a study can begin on the test methodologies for the known FR1 (up to 7.125 GHz) and if the higher end of the support of the existing test methodologies can extend above the current upper boundary (of 7.125 GHz).

As of Rel-19, the Anechoic Chamber is explicitly designated as the reference method for SISO OTA testing involving the measurement of Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) measurements.

Observation 4: AC method is explicitly defined as reference method for SISO OTA testing involving measurement of TRP/TRS performance for smartphones.

Since this reference method provides the most direct and traceable characterization of a DUT's radiated performance (free from statistical field assumptions) it can remain the starting point or reference for studying improved test methods for SISO OTA.

Proposal 6: Consider the Anechoic Chamber (AC) defined up to FR1 as the starting point for study of 6G SISO OTA testing

Based on a review of commercially available SISO anechoic test chambers, several of these extend the TRP and TRS measurement operation up till at least 10 GHz. While further enhancements in these chambers cannot be precluded closer to Rel-21, the 6G OTA study can begin by studying the possibility of extending existing AC reference methodology beyond the current 5G NR FR1. This would provide an initial view of the extension of test methodology

Proposal 7: Study the feasibility of extending existing Anechoic Chamber (AC) methodology to frequency range beyond FR1 for 6G SISO testing (for e.g. 10 GHz, Actual higher end being FFS).

2.3		OTA Test Methods for Multi-Tx and CA

2.3.1			Views on single carrier with multi-Tx

In RAN4#116-bis meeting [2], there was an agreement to further study the testing methodology for UE with multi-Tx/Rx antennas, i.e.:

On OTA testing with Multi-Tx Antenna:

In general, the UE with multi-Tx antenna configuration can be categorized into two test cases, i.e., TxD (Transmit Diversity) and UL-MIMO. UE multi-Tx mode allows UEs to produce the UL signal from more than one antenna connector or polarization simultaneously.

There has been some work related to creation of test methods for non-coherent UL-MIMO and coherent UL-MIMO supporting UEs in Release 18 and Release 19. The test metrics and test methods have already been defined in TR 38.870. The same can be used as baseline for 6G Uplink MIMO and Transmit Diversity studies.

Proposal 8: Definition of multi-Tx schemes (such as UL-MIMO) shall be studied in the '6G General RF and UE RF' topic.

On OTA Testing with Multi-Rx Antenna:

As captured in TR 38.834, 'Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1' [7] devices with multiple Rx antennas requires enabling all Rx antennas during OTA testing. A similar approach may be applied for 6GR FR-1 OTA testing.

Observation 5: Test methodology for UE with multi-Rx antenna is captured in TR 38.834 for OTA TRS test.

Proposal 9: It is feasible to consider multi-Rx antennas enablement for 6GR FR-1 OTA testing.

### 2.4		AI/ML OTA Testability

The below way forward was agreed with respect to AI/ML OTA testability wherein the methodologies defined in 5G will be the starting point.

The 5G AI/ML OTA baseline is based on the requirements that are being discussed as part of the 5G-advanced Rel-19 (and upcoming Rel-20) work items. These are primarily based on Demodulation and Performance requirement scenarios. At the same time, as part of the 6G AI/ML study some RF requirements are also included in the study scope at the RAN4#116-bis.

With several new areas for OTA testing that will be explored as part of the sub-topics in sections 2.1, 2.2, 2.6 and 2.7, it would be useful to explore any new test cases that verify AI/ML driven enhancements to TRP/TRS testing.

Proposal 10: RAN4 to study on any potential new OTA use cases related to AI/ML assisted features that could potentially lead to TRP/TRS performance enhancement and associated OTA test feasibility.

### 2.5		Harmonized Testing for TN and NTN

#### 2.6.1 	Harmonized test framework for TN and NTN in FR1.

The WF from RAN4#116-bis indicated an initial focus on frequency ranges based on TN deployment as part of the 6G Spectrum study.

At the same time, it is also agreed that the starting point for the definition of the frequency criteria would include " Unification of TN and NTN operation" as a criteria.

Within this 6G OTA study, the WF from RAN4#116-bis indicates an aligned approach vis-a-vis a harmonized test system/test method for TN and NTN devices for FR1.

The slight staggering of timelines between when 5G OTA test methods for TN (Rel-17) and NTN (Rel-19) devices were defined does not take away from the fact that the general test methodology has stayed the same for TN and NTN devices (with some new partial sphere TRP/TRS metrics defined for NTN UEs in Rel-19).

Observation 6: The anechoic chamber-based reference method that also applies to NTN UEs in the SISO OTA test specification TS 38.161 and can be the baseline for such a harmonized TN/NTN test system for FR1.

Proposal 11: Consider the Anechoic chamber (AC) reference method as the starting point for the harmonized TN/NTN test framework specific to FR1.

Several of the existing assumptions made specific to NTN testing can be retained and used in any 6G NTN test study. This is including but not limited to -

- Zero doppler settings

- Max Power (via TPC up) i.e. consistent TRP across the sphere

- Transparent payload (no feeder link emulation)

- Need for UE frequency pre-compensation

- Full or Partial sphere TRP/TRS measurement over an angular orientation depending on the NTN UE usage scenarios

Some new aspects that might have to be considered for 6G NTN UEs are as follows -

- Coordination of multiple nodes (TN and NTN in same setup). More critical in scenarios where link handovers might be needed

- Factor in TN-NTN MRSS

- Connectivity across multiple types of orbits (LEO+MEO+GEO etc.).

- New NTN usage scenarios

Proposal 12: RAN4 to consider below factors specific to NTN (other factors not precluded) when evaluating the Harmonized TN/NTN OTA FR1 test aspects for 6G

- New NTN Usage scenarios beyond what has been defined in Rel-19

- Coordination of multiple nodes (TN and NTN in same setup).

- TN-NTN MRSS

- Connectivity across multiple types of orbits (LEO+MEO+GEO etc.).

2.6		Improved Test Methods for SISO OTA

In RAN4#116-bis meeting [2], there was an agreement to further study the testing methodology for 6GR OTA UEs with antenna switching capability.

The OTA testing methodology and configurations for UE with Tx antenna switch is captured in TR 38.834, 'Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1’[7]. Following factors are indicated which may trigger the UE antenna switching and Downlink Rx signal is the one of the most effective factors.

Observation 7: Test methodology and configurations for UE with Tx antenna switch is captured in TR 38.834.

Proposal 13: RAN4 to further evaluate the similar approach for 6GR FR-1 OTA testing.

2.7		Improved Test Methods for DL MIMO OTA

In RAN4#116-bis meeting, there was an agreement to further study the implementation of NR Dynamic Channel Models for 6GR OTA, i.e.:

Preliminary work related to the definitions of FR1 Dynamic Channel Model with link adaptation had been done in Rel. 19 TR 38.762 [6]. Despite differences on environmental conditions, the definitions captured on [8] leverage important aspects commonized in other SDOs which adopt similar Dynamic Channel Models.

Observation 8: 6GR should build on FR1 Dynamic Channel Model definitions from [6] including the conclusion of FoM metric respective weights if applicable.

Proposal 14: RAN4 to continue the definition of FR1 2D Dynamic Channel Model with Link Adaptation started on [6] for 6GR FR1 MIMO OTA testing.

The study initiated on Rel. 19 FR1 Dynamic Channel Model could not progress further due the lack of MIMO OTA integrated systems capable to emulate the Dynamic Channel Model. Therefore, no Free Space or Phantom measurement results are available.

Observation 9: There are no commercially available MIMO OTA integrated systems capable to emulate FR1 Dynamic Channel Model with Link Adaptation

Proposal 15:	RAN4 to prioritize FR1 MIMO OTA with Dynamic Channel Model in Free Space, defining lab alignment framework and measurement campaigns on 6GR, prior to develop phantom testing.

There are technical limitations based on anechoic chamber reflection and probe antennas bandwidth to expand the capabilities of FR1 MIMO OTA systems to frequencies beyond 10GHz.

Technical implementation details need to be considered for the adoption of FR2 3D MIMO OTA systems for frequencies between 7.125-24.25GHz, such as probe antennas bandwidth, amplifiers linearity over such wide BW, potentially need of down-converters, multiple system calibrations, new channel model validation targets, etc.

Observation 10:	The further development of FR2 and new frequency ranges MIMO OTA will reduce the resources to proper complete the ongoing work done in FR1 MIMO OTA with Dynamic Channel Model with link adaptation.

Proposal 16: RAN4 to prioritize the completion of Free Space FR1 MIMO OTA with Dynamic Channel Model test methodology and FoM metric on 6GR, deprioritizing FR2 and new frequency ranges.

# 3	Conclusions

In this discussion paper, we have presented our initial views on 6G OTA testability.

Observation 1: Verifying regulatory-facing requirements via the OTA approach can yield some benefits to the network, but the trade-off in additional test time, implementation complexity associated with device calibration, and unclear process to specify the requirement should be taken into consideration.

Proposal 1: Whether to introduce the OTA approach to requirements currently specified in the conducted domain should be discussed in the UE RF discussion thread for the 6G SI.

Proposal 2: For evaluation of conducted requirements to be verified via OTA approach, do not include MSD scenarios for evaluation due to the complexity of setting up configurations.

- Observation 2: UE beamforming performance gain was captured in TR 38.803 at 15 GHz for 5GNR.

Proposal 3: It is feasible to consider over-the-air test methodologies for frequencies > 15GHz.

- Observation 3: 6G frequency range overlaps with existing 5G NR frequency range FR-1 and FR-2.

Proposal 4: Reuse the same temperature condition for 6GR overlapping frequencies with NR.

Proposal 5: FFS how to study and quantify the temperature condition for 6GR new frequency range for OTA testing including recommendations for MU.

Observation 4: AC method is explicitly defined as reference method for SISO OTA testing involving measurement of TRP/TRS performance for smartphones.

Proposal 6: Consider the Anechoic Chamber (AC) defined up to FR1 as the starting point for study of 6G SISO OTA testing

Proposal 7: Study the feasibility of extending existing Anechoic Chamber (AC) methodology to frequency range beyond FR1 for 6G SISO testing (for e.g. 10 GHz, Actual higher end being FFS).

Proposal 8: Definition of multi-Tx schemes (such as UL-MIMO) is pending in the '6G General RF and UE RF' topic.

Observation 5: Test methodology for UE with multi-Rx antenna is captured in TR 38.834 for OTA TRS test.

Proposal 9: It is feasible to consider multi-Rx antennas enablement for 6GR FR-1 OTA testing.

Proposal 10: RAN4 to study on any potential new OTA use cases related to AI/ML assisted features that could potentially lead to TRP/TRS performance enhancement and associated OTA test feasibility.

Observation 6: The anechoic chamber-based reference method that also applies to NTN UEs in the SISO OTA test specification TS 38.161 and can be the baseline for such a harmonized TN/NTN test system for FR1.

Proposal 11: Consider the Anechoic chamber (AC) reference method as the starting point for the harmonized TN/NTN test framework specific to FR1.

Proposal 12: RAN4 to consider below factors specific to NTN (other factors not precluded) when evaluating the Harmonized TN/NTN OTA FR1 test aspects for 6G

- New NTN Usage scenarios beyond what has been defined in Rel-19

- Coordination of multiple nodes (TN and NTN in same setup).

- TN-NTN MRSS

- Connectivity across multiple types of orbits (LEO+MEO+GEO etc.).

Observation 7: Test methodology and configurations for UE with Tx antenna switch is captured in TR 38.834.

Proposal 13: RAN4 to further evaluate the similar approach for 6GR FR-1 OTA testing.

Observation 8: 6GR should build on FR1 Dynamic Channel Model definitions from [6] including the conclusion of FoM metric respective weights if applicable.

Proposal 14:	RAN4 to continue the definition of FR1 2D Dynamic Channel Model with Link Adaptation started on [6] for 6GR FR1 MIMO OTA testing.

Observation 9: There are no commercially available MIMO OTA integrated systems capable to emulate FR1 Dynamic Channel Model with Link Adaptation

Proposal 15: RAN4 to prioritize FR1 MIMO OTA with Dynamic Channel Model in Free Space, defining lab alignment framework and measurement campaigns on 6GR, prior to develop phantom testing.

Observation 10:	The further development of FR2 and new frequency ranges MIMO OTA will reduce the resources to proper complete the ongoing work done in FR1 MIMO OTA with Dynamic Channel Model with link adaptation.

Proposal 16: RAN4 to prioritize the completion of Free Space FR1 MIMO OTA with Dynamic Channel Model test methodology and FoM metric on 6GR, deprioritizing FR2 and new frequency ranges.

# 4	References

RP-251881, "New SID: Study on 6G Radio", NTT DOCOMO, June 9-13, 2025

R4-2514647, WF for [116bis][110] 6G testability and OTA, 13th – 17th October 2025

TR 38.803, "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects"

R4-168786, Way Forward on UE RF Requirements for mm-Wave, Qualcomm, RAN4#80bis, 10-14 October 2016

R4-167126, Way Forward on Testability, Intel, RAN4#80, 22-26 August 2016

R4-2203702 ,ETC applicability for FR2 requirements, Apple, RAN4#102-e, February 21 - March 3, 2022

TR 38.834, 'Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1'

TR 38.762, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs