3GPP TSG-RAN WG4 Meeting #117	R4-2520308

Dallas, USA, Nov. 17-21, 2025

Source: 	KDDI Corporation

Title: 	Discussion on spectrum sharing for 6GR

Agenda Item:	8.9

Document for:	Discussion

- 1. Introduction

In RAN4#116-bis, RAN4 began the discussion on spectrum sharing for 6GR and agreed the WF of general aspects including interference handling as follows[1].

- Sub-topic 1-2: Aspects to facilitate MRSS Issue 1-2-0: General consideration Agreement - To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study - System parameters - RF requirements - RRM requirements - Interference handling - Taking following table as starting point to discuss which system parameters are impacted by MRSS. System parameters; Whether impacted by MRSS. Numerology; Maybe impacted. Channel raster; Maybe impacted. Sync raster; Maybe impacted. Waveform; FFS. Modulation; FFS. Channel bandwidth; FFS.

Additionally, RAN4 also agreed the WF on the numerology as follows[1].

- Issue 1-2-5: Numerology - Proposals from companies - P1 (CMCC): the numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD. - P2 (OPPO): For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel - P3 (ZTE): for the MRSS BS, the commercialized NR subcarrier spacing (e.g. FR1 FDD with 15kHz, FR1 TDD with 30kHz and FR2-1 with 120kHz SCS) should be the baseline for 6G BS - P4 (Ericsson): During the discussion on system parameters, take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS - P5 (Xiaomi): On 5G migration spectrum/bands, harmonize the numerology between 5G and 6G by taking existing NR commercialization deployment choice into account. - Way forward - Further discuss numerology impacted by MRSS

In this contribution, we focus on 5G and 6G spectrum sharing and propose some issues especially for interference handling and numerology.

- 2. Discussion

  - Lessons and learn from DSS between LTE and 5G NR

Rel-17 Minimum requirements for CRS-IM

In Rel-17, RAN4 standardized CRS-IM feature as “optional” and it works for mitigating NR throughput degradation in PCI Mod-3 unmatched case.

DSS interference issue between LTE and 5G NR

In Rel-17, RAN4 standardized CRS-IM feature as “optional” and it works for mitigating NR throughput degradation in PCI Mod-3 unmatched case.

With some UEs, NR throughput degradation was critical in Mod-3 unmatched case. The inability to mitigate NR throughput degradation caused by LTE CRS interference from adjacent cells is hindering the deployment of DSS infrastructure.

Observation 1: Rel-17 CRS-IM feature works for mitigating 5G NR throughput degradation in Mod-3 unmatched case.

Proposal to mandate Rel-17 CRS-IM for UE

In RAN#105, we proposed to mandate Rel-17 CRS-IM feature “crs-IM-DSS-15kHzSCS-r17” for UE and some operators supported it. However, the proposal was not agreed eventually.

Observation 2: Proposal to mandate Rel-17 CRS-IM feature for UE was not agreed in 5G NR.

  - MRSS between 5G NR and 6GR

    - Interference handling

When it comes to 5G NR, its flexibility of control channels increase compared to 4G. Actually, there are not specific fixed reference signals like LTE CRS. Therefore, regarding the study of MRSS, firstly RAN4 needs to know and understand whether there are technically any interference issues or not, based on outcomes and progress of 6GR control channels’ design in RAN1.

Proposal 1: RAN4 needs to know and understand whether there are technically any interference issues or not, based on outcomes and progress of 6GR control channels’ design in RAN1.

And then, if RAN4 identify possibilities on any interference issues based on RAN1’s outcomes and progress, RAN4 need to study candidate solutions for the issues and expect to mandate related features for 6GR UE in order to avoid any performance degradation due to interference issues between 5G NR and 6GR. This is because even if 6GR use PCI similar to LTE and 5G NR, generally speaking operators can’t ensure to construct PCI Mod-3 matched network.

Proposal 2: If RAN4 identify possibilities on any interference issues based on RAN1’s outcomes and progress, RAN4 need to study candidate solutions for the issues and expect to mandate related features for 6GR UE.

    - Numerology

6G will be deployed widely in spectrum currently used for other RATs, in particular 4G and 5G. Since operators provide a service to existing users with legacy RATs, we will need to continue to allocate large amounts of this spectrum to legacy RATs well into the lifetime of 3GPP not to degrade the user experience of existing users. At the same time, when deploying 6G, we will need to provide sufficient spectrum that the 6G user experience is satisfying. It will be necessary for users to use the same spectrum for 5G and 6G.

The details of MRSS depends on the design of synchronization signals, reference signals and broadcast channel that are still open. For sharing time and frequency resources though, it is fairly obvious that the time/frequency grid needs to be compatible between 5G and 6G. In principle, the numerology, symbol length etc. should be at least compatible and in principle the same.

Proposal 3: Take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS.

Given the reality, 5G de fact standard of commercial products for FDD is obviously 15kHz SCS. To align the numerology between 5G and 6G, only 15kHz is a candidate of the numerology.

Proposal 4: The numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD.

- 3. Conclusion

Observation 1: Rel-17 CRS-IM feature works for mitigating 5G NR throughput degradation in Mod-3 unmatched case.

Observation 2: Proposal to mandate Rel-17 CRS-IM feature for UE was not agreed in 5G NR.

Proposal 1: RAN4 needs to know and understand whether there are technically any interference issues or not, based on outcomes and progress of 6GR control channels’ design in RAN1.

Proposal 2: If RAN4 identify possibilities on any interference issues based on RAN1’s outcomes and progress, RAN4 need to study candidate solutions for the issues and expect to mandate related features for 6GR UE.

Proposal 3: Take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS.

Proposal 4: The numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD.

- 4. Reference

[1] R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC, RAN4#116bis