3GPP TSG-RAN WG4 Meeting #117											R4-2520317

Dallas, US, Nov. 17 – 21, 2025

Agenda item:	5.11.3

Source: 	Fujitsu

Title: 	Discussion on BS-RF requirements for US 4.9GHz new band

Document for:	Approval

# 1. Introduction

<Sub-topic 3-1> BS RF requirements

<Way forward>: BS max output power

- Encourage companies to investigate if/which maximum output power limit(s) should be specified.

At the RAN4#116-bis meeting, the following WF for BS-RF requirements was approved [1].

In this paper, we would like to provide our view on the above WF for BS max output power.

# 2. Discussion

Regarding the BS maximum output power limits for US 4.9GHz, there are two requirements according to the FCC §90.1215. One is for base, mobile, and temporary fixed operations. The transmission power requirements for base, mobile, and temporary fixed operations are defined in the FCC§90.1215 (a) – (e). The other is for the permanent fixed point-to-point and point-to-multipoint stations. The requirements for the permanent fixed point-to-point and point-to-multipoint stations are defined in the FCC §90.1215 (f).

Observation 1: Regarding the BS maximum output power limits, there are two requirements according to the FCC §90.1215.

According to the FCC §90.1215 (a) – (e), the maximum EIRP value can be calculated as 63 dBm, considering the maximum conducted output power when the channel bandwidth is 50 MHz and the directional gain of Tx antennas. Our understanding is that it is necessary to specify the above BS maximum output power requirements because its target device types and deployment conditions covers the typical scenario for cellular network. Considering it, our proposal is below.

Proposal 1: RAN4 to specify the transmission power requirements for BS defined in §90.1215 (a) – (e).

According to the FCC §90.1215 (f), the maximum EIRP value for the permanent fixed point-to-point and point-to-multipoint stations is 85 dBm. One important thing is that its deployment is different from that for base, mobile, and temporary fixed operations. The FCC §90.1207 (d) states the licensing information for the permanent fixed point-to-point and point-to-multipoint stations as follows.

§ 90.1207 Licensing.

…

(d) Permanent fixed point-to-point and point-to-multipoint stations in the 4940-4990 MHz band must be licensed individually on a site-by-site basis. Such fixed stations are accorded primary status. Permanent fixed point-to-point and point-to-multipoint stations must use directional antennas with gains greater than 9 dBi.

According to the above, the permanent fixed point-to-point and point-to-multipoint stations in the 4940-4990 MHz band must be licensed individually on a site-by-site basis. It means that the maximum output power will be determined considering the deployment conditions, such as the positional relationships between the permanent fixed stations, and the neighboring stations causing interference. Considering it, we think further discussion in RAN4 is necessary on how to treat the transmission power requirements for the permanent fixed point-to-point and point-to-multipoint stations.

Proposal 2: RAN4 to further discuss how to treat the transmission power requirements for the permanent fixed point-to-point and point-to-multipoint stations.

# 3. Conclusions

In this contribution, we provided our view on the requirements for BS max output power.

Observation 1: Regarding the BS maximum output power limits, there are two requirements according to the FCC §90.1215.

Proposal 1: RAN4 to specify the transmission power requirements for BS defined in §90.1215 (a) – (e).

Proposal 2: RAN4 to further discuss how to treat the transmission power requirements for the permanent fixed point-to-point and point-to-multipoint stations.

# 4. Reference

- R4-2515103, Way Forward for [116bis][322] NR_TDD_4.9GHz_band, Fujitsu