3GPP TSG RAN4 Meeting #117						    	        R4-2520324

Dallas, USA, Nov 17-21, 2025

Agenda item:	8.2.5

Source:	Huawei, HiSilicon

Title:	On system parameters for 6G —— Device type

Document for:	Approval

Background

In RAN4#116bis, there was intensive discussion on the device type related issues and following items are included in the WF [1] for further discussion. This document provides our opinions on the topic.

The following aspects could be further studied Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features Other aspects are not precluded Collaborate on the framework with other WGs: Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

Discussion

RAN4 discussion scope

It’s noticed that there were concerns raised on the parallel discussion between RAN and RAN4. We think a clear scope of RAN4 discussion would help RAN4 make progress without forked view from RAN conclusion.

At the moment there are still questions to be answered by RAN, such as how many device types will be defined, and which UE capabilities/parameters should be supported by each device type, such as the channel bandwidth, #Tx/#Rx, power class, etc. While from RAN4 point of view, no matter how the device types are to be defined, we are always in the position of deriving the RF/RRM/Demod performance requirements for each UE capability/parameter.

Usually the RAN4 requirements are closely tied to different implementations, such as form factor, PA configuration, Tx/Rx reference architecture, etc. In our view, RAN4 should focus on the study of 6G requirements for each UE capability/parameter based on all kinds of practical implementations. In addition, it’s possible that a single device type defined by RAN covers different UE implementations. To avoid specifying fragmented requirements for a single device type, RAN4 could study the feasibility of specifying unified requirements for all the implementations.

As a summary, we think the RAN4 study areas could consist of:

- Identify implementation restrictions for each UE capability/parameter.

- Communicate the implementation restrictions with RAN to assist the discussion of device types.

- Study the 6G requirements enhancement with regard to the implementation restrictions.

- Study the feasibility of specifying a single set of requirements for all the implementations of each device type if defined by RAN.

In the end of the 6G study, RAN4 should be ready to have a framework to provide the full set of requirements for each commercialized device with understanding of the relevant implementation restrictions. Meanwhile the headache of how to name the device could be left for RAN.

Below Figure 1 shows an example of work split between RAN and RAN4 for the topic of #Tx/#Rx. For 6G study, we could study the performance enhancement for possible implementations, e.g. 2Tx/3Tx/4Tx requirements of different PA configurations, while how to categorize the devices is left for RAN decision.

Figure 1: Work split between RAN and RAN4 for the topic of #Tx/#Rx

Proposal 1: RAN4’s discussion scope is studying the RF/RRM/Demod performance requirements for each UE capability/parameter based on all kinds of practical implementations. Whether and how to map any UE capability/parameter/implementation to certain device type is left for RAN decision. For more concrete RAN4 discussion scope, following items are proposed:

- Identify implementation restrictions for each UE capability/parameter.

- Communicate the implementation restrictions with RAN to assist the discussion of device types.

- Study the 6G requirements enhancement with regard to the implementation restrictions.

- Study the feasibility of specifying a single set of requirements for each device type if defined by RAN.

Collaborate with RAN

In RAN#109, the agreement on diverse device types could be found below. It could be seen many UE capabilities/parameters are being considered for defining the device types.

Proposal 4: In terms of diverse device types, study further: Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type The value(s) for the identified parameters for a device type

From RAN4 point of view, we already have some knowledge on how these UE capabilities/parameters are restricted by UE implementations from 5G’s experience. The information could be provided to RAN to assist the discussion on device types.

An initial list of 5G UE implementation types in RAN4 spec with different requirement and/or capabilies is provided as Table 1. We use the terminology of ‘UE implementation type’ here for the convenient of RAN4 discussion, but it has no direct link with the 6G device type being discussed in RAN. We propose RAN4 to refine the list and communicate the information with RAN.

Proposal 2: RAN4 produces a list of 5G UE implementation types specified in RAN4 spec with different requirements and/or capabilities, and send this information to RAN to assist the device type discussion. The Table 1 could be used as a starting point.

Table 1: Summary of supported 5G UE implementation types in RAN4 spec with different requirement and/or capabilities

|  | Power class | #Tx | #Rx | Maximum Channel bandwidth | MIMO layers | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| Handheld UE | PC3, PC2, PC1.5 | 1Tx, 2Tx, 3Tx | < 1GHz: 2Rx(mandatory), 4Rx | Up to 100MHz (Band specific) | Up to 3-layer UL Up to 4-layer DL (NOTE 2) | TDD, FDD |
|  |  |  | > 1GHz: 2Rx, 4Rx, 6Rx(>2.5GHz) 4Rx is mandatory for bands n7, n38, n41, n48, n77, n78, n79, n104 (>=2.6GHz); 2Rx is mandatory for other bands (NOTE 1) |  |  |  |
| FWA/CPE | PC3, PC2, PC1.5, PC1 | 1Tx, 2Tx, 3Tx, 4Tx (NOTE 3) | < 1GHz: 2Rx(mandatory), 4Rx | Up to 100MHz (Band specific) | Up to 4-layer UL Up to 8-layer DL (including 6Rx/6-layer, 8Rx/8-layer) | TDD, FDD |
|  |  |  | > 1GHz: 2Rx, 4Rx, 6Rx(>2.5GHz), 8Rx 4Rx is mandatory for bands n7, n38, n41, n48, n77, n78, n79, n104 (>=2.6GHz); 2Rx is mandatory for other bands |  |  |  |
| RedCap | PC3, PC2 | 1Tx | 1Rx, 2Rx | - Up to 20MHz RF channel bandwidth (Band specific) - eRedCap is allowed to support up to 5MHz BB channel bandwidth | 1-layer UL Up to 2-layer DL | TDD, FDD, HD-FDD |
| XR/Vehicular UE | - | - | 2Rx is allowed on 4Rx mandatory bands | - | - | - |
| ATG | Rated Power class with 23~40dBm | 1Tx, 2Tx Omni-directional antenna or antenna array | 2Rx, 4Rx Omni-directional antenna or antenna array | Up to 100MHz (Band specific) | Up to 2-layer UL Up to 4-layer DL | TDD, FDD |
| UAV | PC3 | 1Tx | - | Up to 100MHz (Band specific) | 1-layer UL | TDD, FDD |
| V2X | PC2, PC3, PC1 | 1Tx, 2Tx | - | Up to 40MHz (Band specific) | Up to 2-layer SL | HD-FDD |
| NOTE 1: 8Rx was once proposed for Handheld UEs in Rel-20. NOTE 2: 6-layer DL was proposed and discussed for Handheld UEs in Rel-19. NOTE 3: 8Tx was once proposed for FWA UEs in Rel-19 and Rel-20 by vendors and operators respectively. |  |  |  |  |  |  |

Formfactor in the specification

A well understood physical restriction on RF requirements is formfactor. In 5G specifications, many requirements are indicated applicable for certain formfactors, such as handheld UE, FWA, XR, Vehicular UE, etc. This helps with distinguishing different sets of requirements derived from various assumptions, but it has some limitations and could be improved in 6G.

Limitation 1: Formfactor decides the feasibility of implementing different #Tx/#Rx, but listing a finite number of formfactors creates requirements hole. Below are some observed problems:

- Currently the 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices considering the implementational difficulty of handheld UEs. However, this decision doesn’t consider the wide variety of handheld UE design, such as the increasingly large foldable UEs, as a result prevents those UEs from providing better performance by implementing 8Rx.

- The 4Rx REFSENS requirements are specified differently for ‘FWA’ and ‘handheld UE’. The ATG and UAV requirements are referring to legacy UEs, but not mentioning about the formfactor. It’s unclear which set of requirements should be used for ATG and UAV UEs.

- There are many other devices not listed in the specification, such as tablets, laptops. It’s not clear whether these devices are allowed to support 8Rx.

To address the observed problems, it’s worth studying a way of specifying requirements without mentioning specific formfactors.

Limitation 2: Formfactor implies different physical restrictions for different requirements.

The PC1.5 MPR requirements are different for handheld UE and FWA, assuming different antenna isolation when deriving the requirements. Meanwhile the 4Rx REFSENS requirements are derived assuming different antenna imbalance for handheld UE and FWA/CPE/industrial UEs. PC1 is indicated as not supported by smart phone due to the power consumption. We could see the formfactor actually represents different physical restrictions in different requirements. In practice the physical restrictions are not strictly related to the formfactor, instead they might vary largely due to implementation even for the same formfactor. Not mention there are more form factors not listed in the specification.

To achieve a more scalable specification and support the diverse device types, it’s proposed we specify the RF requirements based on the actual physical restrictions, rather than based on finite number of formfactors or by specific implementation types. Below Table 2 is an example of some physical restrictions to be used for defining requirements.

Table 2: Example of physical restrictions used for define requirements

| Physical restriction | Possible values | Impacted requirements |
| --- | --- | --- |
| Tx antenna isolation | Low, High with certain assumptions | MPR requirements |
| Rx chain imbalance | Low, High with certain assumptions | REFSENS requirement |

Limitation 3: The difference in requirements across formfactors is very small but leaves the specification fragmented.

The REFSENS requirements for 4Rx and 6Rx are specified for FWA and handheld UE respectively with only 0.3dB difference. The minor difference has limited impact to the network performance but leaves the specification fragmented. This might be improved by merging the requirements into a single set covering all the possible formfactors.

As a summary, we have following proposal 3.

Proposal 3: To achieve a more scalable specification and support the diverse device types, following principles for specifying RF requirements are considered:

- Avoid mentioning formfactors explicitly in the specification.

- The RF requirements are specified based on the actual physical restrictions, rather than based on finite number of formfactors.

- Specify a single set of requirements if the difference caused by physical restrictions is negligible.

Simplification of feature level requirement

In 5G specification, RAN4 specified many sections for feature level requirements, such as MIMO, TxD, UAV, etc. For the features, usually only a few requirements need special consideration, while most of the requirements just refer to the basic requirements. The readability is not good enough since reader has to be navigated by the references from one section to another to find the real requirements. It also increases the difficulty of maintenance, sometimes even causes misalignment across the specification. In 6G we need to strive for certain level of simplification of the feature level requirements. For example, we could keep a single set of basic requirements, and only create a branch of feature specific requirement when necessary. If the branch of feature specific requirement could be captured in separate tables, then no new section needs to be introduced. With this approach, the section 6.2D/6/2G/6.2K in TS 38.101-1 could be merged into 6.2, when the MPR/AMPR requirements are captured in separate tables for 1Tx/2Tx/4Tx MIMO/TxD.

Furthermore, having feature level requirements in the specification also causes some redundancy in verification. For example, the transmit power requirements for MIMO/TxD/ULFPTx are all based on the transmission from 2 or 4 antenna connectors. The major difference is whether the data bits running on the antennas are from 1-layer or 2-layer. Considering RAN4’s concern is mainly towards the hardware performance, we could try to avoid specifying/verifying multiple sets of requirements for different physical layer configurations but the same hardware behaviour.

Proposal 4: To simplify the specification with regards to feature level requirements, following options are considered for RAN4 study:

- Keep a single set of basic requirements, and only create a branch of feature specific requirement when necessary. If the branch of feature specific requirement could be captured in separate tables, then no new section needs to be introduced.

- Try to avoid specifying/verifying multiple sets of requirements for different physical layer configurations but the same hardware behaviour, such as MIMO/TxD/ULFPTx.

Summary

This document provides our opinions on the topic of device type. Following proposals are for RAN4 group to discuss and decide:

Proposal 1: RAN4’s discussion scope is studying the RF/RRM/Demod performance requirements for each UE capability/parameter based on all kinds of practical implementations. Whether and how to map any UE capability/parameter/implementation to certain device type is left for RAN decision. For more concrete RAN4 discussion scope, following items are proposed:

- Identify implementation restrictions for each UE capability/parameter.

- Communicate the implementation restrictions with RAN to assist the discussion of device types.

- Study the 6G requirements enhancement with regard to the implementation restrictions.

- Study the feasibility of specifying a single set of requirements for each device type if defined by RAN.

Proposal 2: RAN4 produces a list of 5G UE implementation types specified in RAN4 spec with different requirements and/or capabilities, and send this information to RAN to assist the device type discussion. The Table 1 could be used as a starting point.

Proposal 3: To achieve a more scalable specification and support the diverse device types, following principles for specifying RF requirements are considered:

- Avoid mentioning formfactors explicitly in the specification.

- The RF requirements are specified based on the actual physical restrictions, rather than based on finite number of formfactors.

- Specify a single set of requirements if the difference caused by physical restrictions is negligible.

Proposal 4: To simplify the specification with regards to feature level requirements, following options are considered for RAN4 study:

- Keep a single set of basic requirements, and only create a branch of feature specific requirement when necessary. If the branch of feature specific requirement could be captured in separate tables, then no new section needs to be introduced.

- Try to avoid specifying/verifying multiple sets of requirements for different physical layer configurations but the same hardware behaviour, such as MIMO/TxD/ULFPTx.

Reference

[1]	R4-2514650, WF on 6GR system parameter, Feature lead (Huawei, HiSilicon), RAN4#116bis