3GPP TSG-RAN WG4 Meeting #117                                           R4-2520326

Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on BS RF study for 6G

Agenda Item:	8.4.1

Document for:	Approval

Introduction

6G BS RF and co-existence have been discussed extensively in last RAN4 meeting [1], and WF was agreed in [2].

This contribution provides our further consideration on 6G BS RF for 6G.

Discussion

BS RF requirements

Requirements to be re-assessed

RAN4 agreement on the issue is excerpted as below.

Way Forward: For next meeting, companies are encouraged to evaluate how 5G BS RF requirements should be considered for 6G and provide their view using the table in Annex A as example. Note: The goal is to identify: - 5G requirements that might also be applicable to 6G. - 5G requirements which will need to be re-evaluated based on 6G system parameters but would not need a further study. - 5G requirements which should be further studied in the scope of the 6G SI, a prioritization might be made in next meeting.

At last meeting, RAN4 agreed to consider 5G BS RF requirement as starting point and companies provides proposals on the requirements to be re-assessed. Due to the limited time, it has not been discussed in depth requirement-by-requirement at the last meeting. In the following text we focus on the potential optimization we identified in [3].

For FR1 FDD bands, in order to prevent the receivers of the BSs being desensitized by emissions from a BS transmitter, requirements called "protection of the BS receiver of own or different BS" was defined as below.

TS 38.104 Table 6.6.5.2.2-1: BS spurious emissions basic limits for protection of the BS receiver

| BS class | Frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| Wide Area BS | FUL,low – FUL,high | -96 dBm | 100 kHz |  |
| Medium Range BS | FUL,low – FUL,high | -91 dBm | 100 kHz |  |
| Local Area BS | FUL,low – FUL,high | -88 dBm | 100 kHz |  |
| NOTE 1: For BS operating in band n104, the basic limit is increased by 1dB. NOTE 2: For BS operating in regions where a band is only partially allocated for NR operations (e.g. band n28), this requirement only applies in the UL frequency range of the partial allocation. NOTE 3: For BS capable of multi-band operation, Table 6.6.5.2.2-1 assumes that the supported operating bands, where the corresponding BS transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special protection requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

The requirement assumes 30 dB coupling loss between BS transmitter to its own BS receiver. To meet the requirements, a large volume cavity filter with very high suppression is needed. This presents a number of obstacles to the innovation in energy-efficient and minimalist deployments. Meanwhile all RX requirements are tested with TX on. Protection of the BS own receiver is already covered by RX requirements, e.g. REFSENS. Hence the requirement becomes not needed.

Proposal 1: It is proposed to study the possibility to remove protection requirement of own BS receiver for FDD bands.

For NR RE power control dynamic range requirement is reused from LTE. RE power down requirement is limited by the EVM requirement as power de-boosting on some EPREs will lead to reduced Tx SNR which effectively requires a lower EVM requirement. When the requirement was set, 64 QAM was used as reference. Meanwhile considering that FR1 BS support of 256 QAM or 1024 QAM modulation is already the baseline, gNBs with existing capability can support larger power reduction than those in the current TS 38.104 specification. For example, if considering 256 QAM as baseline, there would be about 10 dB potential extension achievable. The RE power down extension would benefit the network power saving and reduction of co-channel interference. The example proposal for 6G can be found in Table 2.1-4.

Table 2.1-2: Existing RE power control dynamic range for NR/LTE

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

10dB

Table 2.1-3: EVM requirements for different modulation scheme as in 38.104

| Modulation scheme for PDSCH | EVM Requirements (%) | SNR = -20*log(EVM) |
| --- | --- | --- |
| QPSK | 17.5% | 15.1 |
| 16QAM | 12.5% | 18.1 |
| 64QAM | 8% | 21.9 |
| 256QAM | 3.5% | 29.1 |
| 1024QAM | 2.5% | 32.0 |

Table 2.1-4: Example proposal for RE power control dynamic range (new values highlighted)

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -16 | +4 |
| QPSK (PDSCH) | -16 | +3 |
| 16QAM (PDSCH) | -13 | +3 |
| 64QAM (PDSCH) | -10 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

Proposal 2: It is proposed to study the feasibility to extend BS RE power down dynamic range for FR1 bands.

One of the SI objective is to support wider channel bandwidth (at least 200MHz) for 6G deployments at least above 2 GHz, around 7 GHz. It is expected that major workload would be on the common system parameters such as spectrum utilization and sync raster design. For BS RF requirements, there would be a few impacts due to the introduction of wider FR1 channel bandwidth (200 MHz or wider). One point needs to be further discussed is operating band unwanted emission, considering that the PSD may be lower compared to small channel bandwidths.

Proposal 3: It is proposed to study the corresponding BS RF requirements for support wide channel bandwidth.

Based on the discussion above, the assessment for BS RF requirements can be found in following table for conducted requirements for example.

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Base station output power | X |  |  |
| Output power dynamics | n/a | n/a | n/a |
| RE power control dynamic range |  |  | X 1.gNBs with existing capability can support larger power reduction than those in the current 38.104 specification. 2.The RE power down extension would benefit the network power saving and reduction of co-channel interference. |
| Total power dynamic range |  | X related to system parameter SU | X To study whether the requirement is needed. |
| Transmit ON/OFF | X |  |  |
| Transmitted signal quality | n/a | n/a | n/a |
| Frequency error | X |  |  |
| Modulation quality |  |  | On hold, If new modulation scheme is introduced |
| Time alignment error | X |  |  |
| Unwanted emissions | n/a | n/a | n/a |
| Occupied bandwidth | X |  |  |
| ACLR |  | X related to system parameter new channel bandwidth | X it is to be clarified the absolute basic limit should be measured per MHz or per channel |
| Operating Band Unwanted Emissions |  | X related to system parameter new channel bandwidth |  |
| Transmitter spurious emission | X |  |  |
| General requirement |  |  |  |
| Protection of BS receiver of own or different BS |  |  | X It is already covered by RX requirements with TX on. |
| Additional spurious (coex) |  |  | X On hold, pending on Rel-19/ Rel-20 conclusions |
| Co-location |  |  | On hold, pending on Rel-20 conclusions |
| Transmitter intermodulation |  |  | On hold, pending on Rel-20 conclusions |
| Receiver sensitivity level |  | X related to system parameter new channel bandwidth |  |
| Dynamic range |  | X related to system parameter new channel bandwidth |  |
| In-band selectivity and blocking |  |  |  |
| ACS |  | X related to system parameter new channel bandwidth |  |
| In-band blocking |  | X related to system parameter new channel bandwidth |  |
| Out of band blocking |  | X related to system parameter new channel bandwidth |  |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation |  | X related to system parameter new channel bandwidth |  |
| In channel selectivity |  | X related to system parameter new channel bandwidth |  |

Proposal 4: It is proposed to take above assessment for BS RF requirements into account for the discussion on BS RF for scope of the 6G SI.

Requirements for cmWave bands

RAN1 reached the following agreement regarding max channel bandwidth.

RAN1 Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

For wider channel bandwidth support, there will some work on the common system parameters such as spectrum utilization and sync raster design. And for BS RF, as evaluated in below table, it is proposed to study whether existing ACLR/OBUE for transmitter and RX ACS/in-band blocking requirements are still applicable for 200/400 MHz CBW.

Table 2.1.2-1: Initial assessment for support wide channel bandwidth

| Subject | Requirement | Initial assessment |
| --- | --- | --- |
| Transmitter | General | no change |
|  | Base station output power | no change |
|  | RE Power control dynamic range | no change |
|  | Total power dynamic range | minor change, a new row should be added for a new CBW |
|  | Transmit ON/OFF power | no change |
|  | Transmitted signal quality | no change |
|  | Occupied bandwidth | no change |
|  | ACLR | TBD, new CBW should be added |
|  | Operating band unwanted emissions | To discuss whether existing OBUE is still suitable |
|  | Transmitter spurious emissions | no change |
|  | Transmitter intermodulation | no change |
| Receiver | General | no change |
|  | Reference sensitivity level | minor change, new CBW should be added |
|  | Dynamic range | minor change, new CBW should be added |
|  | In-channel selectivity | minor change, new CBW should be added |
|  | Adjacent Channel Selectivity (ACS) and in-band blocking | TBD, new CBW should be added |
|  | Out of band blocking | no change |
|  | Receiver spurious emissions | no change |
|  | Receiver intermodulation | minor change. The location of CW interfering signal shall be adjusted due to the new CBW. |

Proposal 5: It is proposed to study whether existing TX ACLR/OBUE and RX ACS/in-band blocking requirements for FR1 are still applicable for 200/400 MHz CBW.

RAN4 agreement on RX requirements for cmWave band is excerpted as below.

Agreement: For above 7 GHz, further study how BS Rx requirements should be specified, using FR2 approach or not, including if blocking should be specified based on FR2 methodology.

FR1 receiver requirements maintain links to the existing conducted requirements as such 2 sensitivity requirements were necessary to defined (Minimum sensitivity and reference sensitivity). FR2 has no equivalent conducted set of requirements so receiver requirements could be defined based on a single reference sensitivity. With the introduction of new high bands that will be based on AAS there is no need to maintain an equivalence to conducted requirements and a single OTA sensitivity can be used (based on minimum RX NF and antenna gain range assumptions). Interference requirements can then be derived based on this sensitivity declaration in a similar fashion to FR2.

Proposal 6: Since there are no existing non-AAS systems to maintain any equivalence with, it is proposed to study the possibility to remove the link to the existing conducted requirements. A similar approach as FR2 can be adopted, i.e. single declared sensitivity.

Conducted FR1 blocking specifies only the aggressor level without considering the victim system wanted signal level, and does so at very low probability (99.99%). In AAS system this leads to a difference in level depending on the assumptions of the receiver design i.e. if the interference is coherent or non-coherent, this makes deriving a requirement in this fashion without knowing the receiver architecture very difficult. By instead basing the requirement on the difference between the wanted signal and the blocking signal (at the victim) the correlation level does not affect the result and a single requirement regardless of system architecture and implementation) can be derived. Using a single sensitivity requirement and a wanted to interferer level blocking definition is more suitable for AAS systems in new bands.

Proposal 7: It is proposed to use the same methodology used for FR2 to derive blocking requirement, i.e. blocker vs wanted signal analysis.

BS type 1-H enhancement

RAN4 agreement on the issue is excerpted as below.

Agreement: BS Hybrid beamforming type of architecture shall be studied.

The topic additional hybrid requirement sets for BS type 1-H/large antenna array has been discussed in Rel-20 BS RF package [3]. The justification is to add more OTA options to reduce the test effort and complexity, and to proceed the verification on the overall radiated performance, e.g. transmitted signal quality, co-location requirement, and transmitter intermodulation [4]. Meanwhile the situation is becoming true only for large antenna array in the future, and for the legacy bands there is no need to update since the whole ecosystem has been matured. Another open issue is that whether a new BS type is needed for hybrid beamforming with active phase shift in the radio distribution network (RDN). In our view, since the requirements will keep the same, no new BS type 1-H is needed. Meanwhile the definition of transceiver array boundary may need to be updated to cover such an HBF implementation.

Figure 2.1.3-1: Radiated and conducted reference points for BS type 1-H

Proposal 8: the enhancement of BS type 1-H is focusing on large antenna array case for cmWave bands.

Proposal 9: new BS type 1-H is not needed for BS type 1-H enhancement.

Consideration of frequency range 7 – 24 GHz

In relation to the discussion on the Frequency Ranges definition for TN, there is a need to decide on the requirement sets (i.e., conducted, hybrid, or OTA) for any possible future bands in the range of 7 – 24 GHz. This is to set frequency boundary among the FR1-based approach (where conducted, hybrid, and OTA requirement sets were defined in the past for BS) among FR2-base approach (OTA only). Initial discussion towards this decision has taken please in Rel-16 in the SI on 7 - 24 GHz, as documented in TR 38.820, There are multiple aspects to be considered in such discussion, which include but are not limited to BS architecture, availability and practicality of connectors, etc. The work can focus on the 7 GHz firstly.

Proposal 10: Investigate frequency boundary limit among FR1 and FR2 ranges for the OTA-only requirements applicability for BS RF.

Consideration of environmental conditions

Recently, discussion on related environmental profiles has popped up in multiple ETSI-related technical foras. An ETSI environmental profile is a series of standards (ETSI EN 300 019 series) that define and classify environmental conditions and their severities for telecommunications equipment. These profiles are used to specify tests that ensure equipment can withstand various conditions like temperature, humidity, and mechanical stress throughout its lifecycle, from storage and transport to in-use environments. The results of these tests demonstrate compliance with the specified environmental class.

The latest OCG RED-EMCD report on ETSI Harmonised European Standards skeleton work has noted that there is expectation to receive updated feedback from European Commission on the environmental profile. Similar message was reported in the recent ETSI ERM report on validity of the EG 203 336 “Guide for the selection of technical parameters for the production of Harmonised Standards covering article 3.1(b) and article 3.2 of Directive 2014/53/EU”, as it may require revisions depending on the input from the European Commission with regard to environmental profiles in Harmonised Standards.

Considering potential developments above, it would be desirable that those are considered in future RAN4 work on the BS RF test requirements, to avoid potential risks of ETSI and 3GPP RAN4 frameworks diverging, leading to unnecessary work increase (ETSI to RAN4, or RAN4 to ETSI).

Proposal 11: Discussion on test requirements framework for 6G BS RF shall consider outcomes of the EC/ETSI discussion on the environmental profiles.

Conclusion

This contribution provides our consideration on BS RF and coexistence study for 6G.

Requirements to be re-assessed

Proposal 1: It is proposed to study the possibility to remove protection requirement of own BS receiver for FDD bands.

Proposal 2: It is proposed to study the feasibility to extend BS RE power down dynamic range for FR1 bands.

Proposal 3: It is proposed to study the corresponding BS RF requirements for support wide channel bandwidth.

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Base station output power | X |  |  |
| Output power dynamics | n/a | n/a | n/a |
| RE power control dynamic range |  |  | X 1.gNBs with existing capability can support larger power reduction than those in the current 38.104 specification. 2.The RE power down extension would benefit the network power saving and reduction of co-channel interference. |
| Total power dynamic range |  | X related to system parameter SU | X To study whether the requirement is needed. |
| Transmit ON/OFF | X |  |  |
| Transmitted signal quality | n/a | n/a | n/a |
| Frequency error | X |  |  |
| Modulation quality |  |  | On hold, If new modulation scheme is introduced |
| Time alignment error | X |  |  |
| Unwanted emissions | n/a | n/a | n/a |
| Occupied bandwidth | X |  |  |
| ACLR |  | X related to system parameter new channel bandwidth | X it is to be clarified the absolute basic limit should be measured per MHz or per channel |
| Operating Band Unwanted Emissions |  | X related to system parameter new channel bandwidth |  |
| Transmitter spurious emission | X |  |  |
| General requirement |  |  |  |
| Protection of BS receiver of own or different BS |  |  | X It is already covered by RX requirements with TX on. |
| Additional spurious (coex) |  |  | X On hold, pending on Rel-19/ Rel-20 conclusions |
| Co-location |  |  | On hold, pending on Rel-20 conclusions |
| Transmitter intermodulation |  |  | On hold, pending on Rel-20 conclusions |
| Receiver sensitivity level |  | X related to system parameter new channel bandwidth |  |
| Dynamic range |  | X related to system parameter new channel bandwidth |  |
| In-band selectivity and blocking |  |  |  |
| ACS |  | X related to system parameter new channel bandwidth |  |
| In-band blocking |  | X related to system parameter new channel bandwidth |  |
| Out of band blocking |  | X related to system parameter new channel bandwidth |  |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation |  | X related to system parameter new channel bandwidth |  |
| In channel selectivity |  | X related to system parameter new channel bandwidth |  |

Proposal 4: It is proposed to take above assessment for BS RF requirements into account for the discussion on BS RF for scope of the 6G SI.

Requirements for cmWave bands

Proposal 5: It is proposed to study whether existing TX ACLR/OBUE and RX ACS/in-band blocking requirements for FR1 are still applicable for 200/400 MHz CBW.

Proposal 6: Since there are no existing non-AAS systems to maintain any equivalence with, it is proposed to study the possibility to remove the link to the existing conducted requirements. A similar approach as FR2 can be adopted, i.e. single declared sensitivity.

Proposal 7: It is proposed to use the same methodology used for FR2 to derive blocking requirement, i.e. blocker vs wanted signal analysis.

Proposal 8: the enhancement of BS type 1-H is focusing on large antenna array case for cmWave bands.

Proposal 9: new BS type 1-H is not needed for BS type 1-H enhancement.

Consideration of frequency range 7 – 24 GHz

Proposal 10: Investigate frequency boundary limit among FR1 and FR2 ranges for the OTA-only requirements applicability for BS RF.

Consideration of environmental conditions

Proposal 11: Discussion on test requirements framework for 6G BS RF shall consider outcomes of the EC/ETSI discussion on the environmental profiles.

- References

[1] R4-2514510, Topic summary for [116bis][103] BS RF and coexistence

[2] R4-2514638, WF on 6G BS RF and co-existence

[3] RP-252904, Moderator's summary for RAN4 REL-20 5G-Adv: BS RF topics, Huawei

[4] RP-250284, Views on Rel-20 RAN4-led topics, Huawei, HiSilicon