3GPP TSG RAN WG4 Meeting #117	R4-2520338

Dallas, 17th – 21st November 2025

Source:	Rohde & Schwarz (R&S)

Title:	ISAC Test: Object Emulation

Agenda item:	8.10

Document for:	Discussion

# Introduction

While at RAN4#116-bis (Prague), it was decided that we should identify RAN4 centric ISAC aspects (which are not dependent on RAN1) [1]. Testability is a domain driven by RAN4. R&S would like to propose the usage of object emulation as an acceptable and 3GPP approved method for testing ISAC. R&S has been actively engaged in the recent past [5], and we look forward to continuing collaboration in technical studies to ratify the use of object emulation for ISAC tests.

# Discussion

Observing ISAC discussions at 3GPP, the test emphasis is heading towards four main objectives viz.

- Positional Accuracy: ISAC sensor reports on the target’s location vs how far the target really is

- Velocity Accuracy: Difference between the system’s estimated speed and the target’s real speed

- Missed Detection Probability: Chances of missing a target

- False Alarm Probability: Chances of falsely detecting a target

Observation 1: ISAC testing is expected to focus on Positional Accuracy, Velocity Accuracy, Missed Detection Probability, and False Detection Probability.

ISAC validation in the field using physical objects should be a valid alternative test method. However, this increases test time and cost due in part to the careful planning required to ensure traceability. A lab-based approach based on test instrumentation would generally cover a broader parameter range (range/speed) while ensuring reliable and repeatable testing.

Observation 2: Verification and validation can be made more efficient if we pursue a repeatable lab-based procedure to evaluate the conformance and performance of ISAC sensors.

Commercial solutions exist which enable object emulation based on point targets [4]. Emulated objects can be varied in distance and velocity.

Observation 3: There are commercial solutions that offer variation in the position and velocity of emulated target objects for lab-based testing of ISAC.

# Summary

Observation 1: ISAC testing is expected to focus on Positional Accuracy, Velocity Accuracy, Missed Detection Probability, and False Detection Probability.

Observation 2: Verification and validation can be made more efficient if we pursue a repeatable lab-based procedure to evaluate the conformance and performance of ISAC sensors.

Observation 3: There are commercial solutions that offer variation in the position and velocity of emulated target objects for lab-based testing of ISAC.

Proposal 1: RAN4 agrees to include object emulation as an approved method to verify and validate ISAC in the (to be defined) test procedures.

Proposal 2: ISAC 3GPP tests could be subdivided into two formats viz Conformance and Performance. The Conformance section could focus on the functional aspects of ISAC where we check position and velocity metrics under ideal conditions. The Performance section could be more centric to test the missed detection and false alarm probabilities under fading conditions [3].

# References

[1]	RAN4 #116-bis Prague R4-2514592

[3]	Sec 7.9, 38.901, Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz

[4]	 AREG800A Technical Specifications - https://scdn.rohde-schwarz.com/ur/pws/dl_downloads/pdm/cl_brochures_and_datasheets/specifications/3609_8015_22/AREG800A_specs_en_3609-8015-22_v0900.pdf

[5]	      Experimental Demonstration of Multi-Object tracking in ISAC http://arxiv.org/abs/2510.22180