3GPP TSG-RAN WG4 Meeting #117				                 R4-2520342

DALLAS, TEXAS, 17th – 21st November, 2025

Agenda Item: 	8.3

Source: 	MediaTek Inc.

Title: 	Views on 6G general RF and UE RF

Document for: 	Approval

1	Introduction

Following the 6G SI [1] approved in RAN#108, the 6G discussion starts in RAN4 meetings. In this paper, we provide our views on the 6G general RF and UE RF with the focus on the following aspects from the agreed WF [2]:

- Topic#3: Single carrier output power

- Topic#5: Tx requirements

- Topic#6: Rx requirements

- Topic#7: Frequency range between FR1 and FR2-1

- Topic#8: Spectrum aggregation

- Topic#9: Joint UE and BS RF

2	Topic #3: Single carrier output power requirements

RAN4#116Bis WF - Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR

2.1	Default power class for around 7GHz

Also RAN plenary has agreed the following objectives for 6GR in [1].

- Enhanced overall coverage, focus on cell-edge performance and UL coverage.

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

The default power class for 5G NR is PC3 (23dBm). We believe it is feasible to support PC2 (26dBm) as default power class at least for handheld devices operating in spectrum around 7GHz. The benefit of this would be that operators can rely on such Tx power as a baseline for uplink coverage in around 7GHz.

Applicability of PC2 as the default power class for other frequency bands can be studied further.

Proposal 1: For eMBB service in 6GR, RAN4 can consider default power class with PC2 (26dBm) for 6G new candidate spectrum at around 7GHz.

2.2	MPR/A-MPR aspects

2.2.1	General

In last RAN1 meeting, RAN1 agreed to take 5G NR CP-OFDM and DFT-s-OFDM as 6GR waveform as baseline and they will further discuss to enhance low PAPR for uplink transmission on those CP-OFDM and DFT-s-OFDM to further reduce the PAPR of the uplink signal. Also, they do not preclude other new waveform in 6G to reduce signal envelope fluctuations and prevent non-linear distortion in power amplifiers compared to 5G. The RAN1 scope will be affected to RAN4 work to reduce MPR/A-MPR in 6GR.

Also, in 6G, the UE default power class with 26dBm and a wider CBW would be considered, then the required MPR/A-MPR level will be increased compared to 5G NR system.

Observation1: The large MPR under a default power class of PC2: 26dBm and 200MHz channel bandwidth could shrink the uplink coverage compared to 5G NR in 3.5GHz.

To enhance MPR requirements in 6GR, RAN4 can consider the following different candidate solutions described below

- MPR relaxation approaches in 6

- Consider solutions that may reduce the level of RF impairments / leakage outside of the signal bandwidth

- Relaxed unnecessary emission limits (ACLR, SEM, Spurious Emission, Tx EVM)

2.2.2	Reducing Tx RF impairments to improve A-MPR and possibly MPR

Flexible/fast LO retuning

In 5G NR, RAN4 supported cross-band retuning. 2Tx/3Tx switching between a CA feature to UL-MIMO feature vice versa and enhance the T-put performance to consider the channel quality between a NR CA operating band and the other CA operating band. Also, LBCA via switching was supported between FDD low band and SDL band in Rel-19 with multiple switching period {35,70 and 140us}. However, there are still required a suitable LO retuning time to change the center frequency between CCs. However, within a UE channel bandwidth, MPR and A-MPR requirements typically assume the case where the LO is located at the centre of the channel. Where SEM or spurious emission requirements levels are the driving factor for MPR or A-MPR requirements, the ability to shift the Tx LO location according to the location of the transmitted RBs could enable to reduce the level of emissions and allow for tighter A-MPR requirements and potentially tighter MPR requirements and improving the UE’s ability to transmit higher power.

We suggest RAN4 to study this approach further, and also consider the feasible retuning time for relocating the LO within the channel.

Proposal 1: Consider whether it would be possible to study the feasibility and benefits of lo retuning to improve A-MPR, and potential MPR.

The fast LO retuning scheme can improve MPR level and enhance coverage enhancement by shifting fast LO retuning. Especially, when 6G UE LO was moved from edge to center of carrier, then the benefit of relaxed MPR level is larger since the high PSD allocations was leaded higher MPR level due to the presence of images. For 6GR, RAN4 could consider whether increased flexibility in the LO location may be feasible and useful to enable relaxed MPR/A-MPR.

2.2.3	Requirements relaxation to allow reduced MPR

Scenario-specific MPR reduction

In Rel-19, Single Carrier MPR improvement was specified whereby the UE can be configured to consider “outer RB allocations” as “inner RB allocations” for QPSK and 16QAM modulations when the spectrum block is sufficiently larger than the UE CBW (e.g. > 100MHz for general eMBB UE and > 20MHz for RedCap UE. The feature can apply 1dB+ tightening of MPR compared to previous Releases for UEs in such a scenario, translating into enhanced coverage and cell edge performance.

For 6GR, it is anticipated that there will still be scenarios where OOB emission requirements are limiting the maximum output power achievable by the UE, and where the spectrum block may be larger than the applicable UE channel bandwidth. Therefore, we recommend considering such a concept in 6GR from the start.

Observation 2: Rel-19 MPR improvement feature can enable to de-restrict maximum output power by up to 1dB by converting “outer RB allocations” to “inner RB allocations”.

Proposal 2: Consider re-application of the Rel-19 single carrier MPR improvement concept for 6GR.

To further extend applicability of the above concept, ACLR requirements in 3GPP were originally defined considering two Operators A and B in adjacent spectrum, each with its own site deployment independent of each other, and even deployed in the worst type of uncoordinated scenario. Those ACLR requirements put restrictions on available transmission power for low modulation orders with medium large RB allocations (in terms of MPR).

Over the last decade, site sharing and even leasing of sites by multiple operators has become an increasingly attractive trend to enable deployment cost reduction, which means the assumptions on site deployments always being “uncoordinated” may no longer always apply. However, this was not really covered by the Rel-19 feature.

We have performed some coexistence evaluations considering the difference between coordinated and uncoordinated deployments in terms of the ACLR between 2 adjacent operators, and the results are provided below (it considers FR1 macro deployment at 2GHz). If we take UE ACLR to be dominating ACIR, the results suggest that if 2 adjacent operators are sharing the same sites within an area, then the relaxation of at least 5dB could be considered, which could enable a 1dB+ increase in Tx output power vs the reference without the relaxation.

For the full gains to be enabled, the following would also need to be considered:

- Potential relaxation of the 3GPP SEM requirement in the same scenario. Whether the SEM is relaxed or not in a geographical area in such a deployment scenario may then be left to regulatory decision.

- A mechanism for the network to inform the UE that the emissions may be relaxed (similarly as for the Rel-19 “Single carrier MPR improvement” feature). The involved operators would be responsible to mutually agree with each other on the conditions where to configure the relaxation, possible with local regulatory endorsement.

Observation 3: Extending the concept of relaxed OOB emissions to de-restrict Tx power can be extended to broader scenarios such as adjacent MNO site sharing within an area.

Proposal 3: Consider extension of the Rel-19 single carrier MPR improvement general concept to other scenarios including adjacent channel MNO site sharing for 6GR.

EVM relaxations

The benefits of EVM relaxations to enable increased UE output power are discussed in Topic#9.

2.2.4	Other MPR aspects

Also, RAN4 need to consider how to specify the simple MPR requirements for 6G UE compared to 5G without changing 5G core requirements and inducing NBC issue. So, the following single and scalable MPR requirements can be defined in 6GR.

Proposal 4: To simplify MPR specification, RAN4 can consider whether single or scalable requirements to consider the multiple Tx antenna configuration, simple antenna isolation level.

Proposal 5: To simplify MPR specification, RAN4 can consider whether scalable requirements to consider the multiple power classes.

2.3	SAR/MPE for 6GR

SAR/MPE compliance needs to be ensured when considering higher maximum Tx power for 6GR devices. In 5G-A, RAN4 specified P-MPR method and Duty-cycle mechanism to satisfy the SAR/MPE regulations as follows:

- FR1 SAR/MPE solution

  - Clause 6.2.1 inTS38.101-1

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

  - FR1 clause 6.2.4 in TS38.101-1

6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

****  Omitted some parts  ******

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

- FR2 SAR/MPE solution

  - clause 6.2.4 in TS38.101-2

PPowerclass + DPIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

****    Omitted some parts  ******

maxUplinkDutyCycle-FR2, as defined in TS 38.306 [14], is a UE capability to facilitate electromagnetic power density exposure requirements. This UE capability is applicable to all FR2 power classes.

If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted including any PRACH transmission within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c.

If the field of UE capability maxUplinkDutyCycle-FR2 is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means.

P-MPRf,c is the power management maximum output power reduction. The UE shall apply P-MPRf,c for carrier f of serving cell c only for the cases described below. For UE conformance testing P-MPRf,c shall be 0 dB ,  except for the testing of UL gap for Tx power management, where P-MPRf,c may be non-zero dB.

a)	ensuring compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic power density exposure requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

NOTE 1:	P-MPRf,c  was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRf,c and maxUplinkDutyCycle-FR2 may impact the maximum uplink performance for the selected UL transmission path.

NOTE 3:	MPE P-MPR Reporting capability tdd-MPE-P-MPR-Reporting-r16, as defined in TS 38.306 [14], is used to report P-MPRf,c when the reporting conditions configured by gNB are met. This UE capability is applicable to all FR2 power classes.

The uplink duty cycle restriction and P-MPR mechanism were introduced in 5G NR to address SAR/MPE solution. In real field in FR1, most NW settled to restrict the uplink duty-cycle ratio with less than 50% for high power 5G UE in TDD bands, which aligns well for a PC2 operation. Also, the PC1.5 UE will be deployed in real field in near future. However, the duty-cycle mechanism is quite complicated and difficult how to apply the duty-cycle solution for SAR/MPE compliance. Furthermore, the duty cycle restriction for TDD band is cleared compared to FDD band since RAN4 still not supported the detail UE behaviour and solution by duty-cycle mechanism for high power FDD UE.

However, in 6GR, the duty-cycle mechanism can be useful not only for TX power boosting (to meet RFX compliance requirements) but also for thermal control. As a result, if 6G can provide signaling framework (e.g., UAI or other periodic/aperiodic signalling) for the UE to dynamically report its preferred duty cycle, then the UE could maximise UL performance while also ensuring an acceptable end user experience.

NR reuses the P-MPR approach from LTE to solve the electromagnetic energy absorption requirements and some de-sense problem to coexist with non-3GPP features. But, P-MPR usage for complying with SAR/MPE solution has drawbacks in that there is a lack of network awareness of P-MPR application by the UE and this may impact system performance. However, maybe 6G UE still needs this mechanism to control TX power in a proprietary way to meet regulatory requirements or thermal requirements. Therefore, we are also thinking that the PMPR adjustment might need a standard test from the TX power boost perspective.

Observation 1: The duty-cycle restriction was already considered in NW configuration for high power UE with less than 50% for TDD bands.

Observation 2: The existing duty-cycle solution is based on UE capability, leading to scheduling inflexibility that hinders further improvement in UL performance.

Proposal 1: RAN4 need to study how to define a flexible/efficient duty-cycle control mechanism for 6GR.

Proposal 2: RAN4 can consider both P-MPR and duty-cycle control mechanism to satisfy the SAR/MPE requirements in 6GR.

3	Topic #5: Tx requirements

RAN4#116Bis WF - Agreements - PA modelling is discussed in system parameter thread - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear - Spurious emissions requirements, SEM and ACLR will be defined for 6GR - FFS for - OOB boundary - potential accommodations for different frequency sub-ranges - Encourage inputs on regulatory survey being conducted in spectrum agenda - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5) - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

3.1	ACLR/ACS/SEM

In 6G BS session, RAN4 already discussed and had consensus on the re-evaluation of the adjacent carrier coexistence system level simulations to derive ALCR and ACS requirements according to operating frequency ranges including 5G-refarming bands and 6G new candidate spectrum.

Based on the coexistence evaluation results in all candidate operating spectrums, RAN4 can determine the exact ACLR/ACS levels to protect legacy 5G system based on the enhanced coexistence evaluation assumptions. The expected different points are supporting a wider CBW (e.g. 200MHz at around 7GHz), enhanced gNB antenna elements and default power class with 26dBm at around 7GHz.

Observation 1: In BS RF session, RAN4 already agreed to evaluation the coexistence study to consider the different points compared to 5G coexistence parameters. The expected different points are supporting a wider CBW, enhanced gNB antenna elements and defaults power class as PC2 in around 7GHz.

On top of coexistence evaluation results, RAN4 can determine the band specific ACLR/ACS requirements compared to 5G NR. Specially in Rel-19, n104 ACLR requirements was relaxed from 30dBc to 26dBc based on the coexistence evaluation results. So, RAN4 can consider the band specific way for ACLR/ACS requirements in 6GR.

Proposal 1: Based on the coexistence evaluation results, RAN4 can update ACLR and ACS requirements according to frequency ranges (e.g, less 1GHz frequency, 2GHz, 3.5GHz, 4.5GHz and around 7GHz). It means different ACLR requirements can be considered and supported in 6G.

Relaxations of the UE ACLR/SEM requirements in specific operating scenarios

As shown under Topic #3, and as also determined in the Rel-19 EN-DC phase 4 work item for single carrier MPR reduction, there are potential benefits to maximising the Tx output power level by enabling an operator-controlled relaxation in ACLR/SEM requirements for modulations up to 16QAM. For 6GR, propose to consider such approaches from the start.

We also show in Topic#3 that the concept could be extended for multi-operator site-sharing scenarios, to obtain the same type of Tx power improvement benefits.

Proposal 2: Consider specifying an operator-controlled relaxation in UE ACLR/SEM for 6GR from first Release, and further study the extended benefits of this for multi-operator site sharing scenarios.

3.2	OOB boundary for spurious emissions

In 5G NR, RAN4 define the OOB boundary for spurious emissions with BW +5MHz from the CBW edge. However, the 3GPP OOB boundary is not align with the recommendation with 250% rule of Regulation body ITU-R.

Observation 1: NR’s OOB boundary for spurious emissions has been defined as “BW+5MHz from the CBW edge in 5G compared to the recommendation of ITU-R SM.1539 and SM.329 with 250% of necessary channel bandwidth rule.

So, RAN4 can consider the ITU-R SM 1539-2 and SM 1541 as baseline to determine the OOB boundary between out of band emission and spurious emission domains in 6GR.

Proposal 1: RAN4 can consider the SM.1539-2 and SM.1541 recommendation to determine the boundary between the out of band emission and spurious domains for 6G study.

The recommendation is from the ITU-R SM 1541 as follows:

Recommendation ITU-R SM.1541 and recommends 2.3 of Recommendation ITU-R SM.329, the boundary between the OoB and spurious domains, which is generally specified as being separated from the centre frequency by 250% of the necessary bandwidth, needs to be modified for narrow-band and wideband (including multicarrier) systems, and certain other situations. This Annex provides:

–	a set of guidelines on values of bandwidths across the spectrum at which the general definition needs to be varied; and

–	a set of known situations in which additional guidelines are required.

| Type of emission | Necessary bandwidth (BN) | Frequency separation between the centre frequency and the spurious boundary |
| --- | --- | --- |
| Narrow-band case | < BL | 2.5 BL |
| Normal case | BL to BU | 2.5 BN |
| Wideband case | > BU | BU + 1.5 BN |

Guideline values for frequency separation between the centre frequency 
and the boundary of the spurious domain

| Frequency range | Narrow-band case (BN < BL) |  | Normal separation | Wideband case (BN > BU) |  |
| --- | --- | --- | --- | --- | --- |
|  | BL | Separation |  | BU | Separation |
| 9 kHz < fc < 150 kHz | 250 Hz | 625 Hz | 2.5 BN | 10 kHz | 1.5 BN + 10 kHz |
| 150 kHz < fc < 30 MHz | 4 kHz | 10 kHz | 2.5 BN | 100 kHz | 1.5 BN + 100 kHz |
| 30 MHz < fc < 1 GHz | 25 kHz | 62.5 kHz | 2.5 BN | 10 MHz | 1.5 BN + 10 MHz |
| 1 GHz < fc < 3 GHz | 100 kHz | 250 kHz | 2.5 BN | 50 MHz | 1.5 BN + 50 MHz |
| 3 GHz < fc < 10 GHz | 100 kHz | 250 kHz | 2.5 BN | 100 MHz | 1.5 BN + 100 MHz |
| 10 GHz < fc < 15 GHz | 300 kHz | 750 kHz | 2.5 BN | 250 MHz | 1.5 BN + 250 MHz |
| 15 GHz < fC < 26 GHz | 500 kHz | 1.25 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| fc > 26 GHz | 1 MHz | 2.5 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |

4	Topic #6: Rx requirements

RAN4#116Bis WF - Agreements - Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s - Study concept for RX requirements for new frequencies, especially those above 7.125GHz - Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases - Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well - Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference - Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements - Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable

Observation 1: In 5G, the TDD band RX REFSENS for wider CBWs can be derived based on Table and equation below. This 5G TDD band REFSENS equation can be considered for 6G as staring point.

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411, n901 | 15 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 50 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n54 | 15 | 5 | -100 | TDD |
| n677 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20,25,30,40,50 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -95.3 + 10log10(NRB/52) | SDL |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| n1041,10 | 15 | 20, 30, 40, 50 | -90.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -90.8 + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -91.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. NOTE 11: Applies for DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz SCS and > 30 MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |

Regarding wider CBW in RX path (e.g., 200MHz within -100MHz ~ +100MHz covered by analog/digital circuits), it is necessary to study whether there would be performance impact in +50MHz ~ +100MHz and -50MHz ~ -100MHz frequency ranges. If there is performance impact after study, it may be needed to add delta values associated with wider CBW to TDD-band REFSENS equation.

Proposal 1: As for deriving 6G TDD band wider CBW (e.g., >100MHz) REFSENS, discuss whether the 5G TDD band REFSENS equation could be a starting point with modification (e.g., consider performance impact due to wider CBW).

Proposal 2: The 5G ΔRIB,c usage shall be kept for 6G CA. For a 6G UE supporting CA, the minimum requirement for single-carrier reference sensitivity shall be increased by the amount given by ΔRIB,c specified in 6G.

Proposal 3: Regarding HPUE self-interference REFSENS RSD issue in FDD bands, discuss whether it would be proper to study RSD mitigation.

5	Topic #7: Frequency range between FR1 and FR2-1

RAN4#116Bis WF - Agreements - Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz.

In this section, we provided some different points between 7Ghz RF-FE design and 15GHz RF-FE design to compensate each channel characteristics based on the TR38.901 and TR38.820 for 7-24GHz study.

Figure 1: Channel loss characteristics according to operating frequency at 3.5GHz, 7GHz and 15GHz

From the Figure 1, the worst DL 50% CDF RSRP delta levels (channel loss terms) in Dense Urban deployment scenario is about 7.4dB for 7GHz from 3.5GHz and 14.8dB for 15 GHz from 3.5GHz. Also, the following penetration loss term should be considered to maintain the same coverage as target Downlink/uplink coverage in 3.5GHz.

  - 7G total loss term: 12 dB = Frequency loss of 7dB + Penetration loss of 5dB.

  - 15G total loss term: 22 dB = Frequency loss of 15dB + Penetration loss of 7dB.

Based on the above observation, the required base station antenna elements, UE Tx/Rx antennas, wide CBW and high order modulation could be improved to maintain the same coverage at 3.5GHz as follow

- 3.5G system configuration

  - Base station: 8x16x2 (128 AEs)

  - UE: 4Rx DL, 2Tx UL

- 7G possible system configuration:

  - Base station: 16x24x2 (768 AEs)

  - UE: 4/6/8Rx DL, 4Tx UL

- 15G possible system configuration:

  - Base station: 24x64x2 (3072 AEs)

  - UE: 8Rx DL and 4Tx UL

In our internal system level simulation results, we can observe the FR1 panel architecture outperforms compared to both FR1-conv. Architecture and mmWave architectures in terms of both cell-edge and cell-average T-put. Also, the coherent transceiver design could improve UL performance by enabling UL coherent MIMO TPMI precoding, and the use of the 3GPP Rel-18 type-1 codebook.

- FR1-panel architecture: (TS38.901 v19.1.0)

  - Directional antenna system with digital beamforming

  - Apply Conductive 3GPP conformance test methodology

  - External FE module: PAM and FE filter

- FR1-conv. architecture: (5G FR1 assumption)

  - Omni antenna system with digital beamforming

  - Apply Conductive 3GPP conformance test methodology

  - External FE module: PAM and FE filter

- Radiated mmWave architecture: (5G mmWave assumption)

  - Directional antenna system with analog beamforming

  - Apply OTA 3GPP conformance test methodology

  - Integrated module: no FE filter, min post-PA loss

Observation 1: To achieve same coverage between 3.5GHz and new 6G candidate spectrum in 7GHz and 15GHz, RAN4 need to compensate channel loss and penetration loss about 12 dB in 7GHz frequency range and compensate 22dB in 15GHz frequency range.

Proposal 1: At least new 7GHz candidate spectrum, FR1-panel architecture with directional antenna system (as refer TR38.901 v19.1.0) can be considered as baseline in 6GR.

Proposal 2: On top of with UE/gNB enhanced antenna elements with enhanced system configuration, RAN4 shall consider additional enhancements skills such as coherent UL-MIMO/beam forming, power boosting with relaxed ACLR and Tx EVM, default power class with 26dBm and SBFD in both gNB/UE and other candidate schemes.

6	Topic #8: Spectrum aggregation

RAN4#116Bis WF - Agreements - Below spectrum aggregation topics can be discussed as starting point. - RF switch-time performance - Need for intra-band support of non-contiguous UL CA and RB allocations - Study benefits-drawbacks of using CA vs. using wider single carrier BW - Study how to account for simultaneous TX/RX between the bands properly from the beginning - Discuss band combination simplification in spectrum agenda

6.1	RF switching time performance:

The interruption defined in RRM requirements is associated with various procedures, such as Bandwidth Part (BWP) switching, and consists of both RF switching time and baseband (BB) processing time. This results in an overall interruption requirement of approximately 1 ms. Based on existing NR requirements, RF switching time is defined as 0.5 ms for FR1 and 0.25 ms for FR2, while BB processing time is assumed to be 0.5 ms. However, recent advancements in UE implementation suggest that RF switching time can be performed much faster than the original value of 0.5 ms from NR Rel-15. In addition, in NR, some WIs already introduced faster RF retuning time for Rx or Tx switching (e.g., 35 us or 140 us). Therefore, it is recommended that RAN4 further investigate ways to reduce interruption times and define requirements that more accurately reflect the capabilities of the latest UE technologies.

One aspect is that the legacy interruption requirements were based on slot-level granularity, and hence RAN4 shall strive to define requirements based on symbol-level granularity. In addition, it is commonly understood that interrupting DCI would cause more serious issue in network scheduling. Compared to interrupting PDSCH. PDCCH requires better protections because it may carry more important indications, such as UL grant, BWP switch, TCI-state switch.

Furthermore, for some data traffic, the UE is not fully scheduled all the time and hence a UE might be able to place interruption in symbols without impacting control and data packets. Therefore, RAN4 selects best possible location for the interruption to minimize PDCCH, HARQ ACK and data loss.

Observation 1: The requirements of interruption including the RF switching time in previous mobile generation has not been optimised since NR Rel-15.

Observation 2: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels.

Proposal 1: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

7	Topic #9: Joint UE and BS RF

RAN4#116Bis WF - Agreements - Encourage companies to contribute whether there are concrete study points for low power receiver. - Coexistence study including need for coex study will be discussed in agenda for BS RF and co-existence. - Prioritize work for the baseline single carrier output power requirement framework before detailed studies on ACLR and SEM relaxation. - Discuss PAPR reduction topics in system parameter agenda - For Tx EVM relaxation - Discuss non-AI based UL demod in UE RF thread - The impact on BS receiver could be considered. - Do not discuss AI based solution(s) in UE RF thread, except the potential common part. - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible. - Strive to study TX non-linearity sources, including PA, in this thread - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G. - Strive to avoid overlapping work between different threads - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g. - Which aspects need to be considered to conclude on feasibility - Need for network control

For Tx EVM, the main dominant factor is PA non-linearity with the almost saturation power level to support high order modulation schemes in 6G. So, RAN4 need to study how much level will be compensated at either Tx part with pre-distortion mechanism or Rx part with equalization. And RAN4 agreed to focus on non-AI based UL in UE Tx part.

Based on our MPR simulation assumption and results in 5G NR, the current MPR requirements for inner RB allocation by CP-OFDM waveform, there are some room to enhance MPR levels by dominant factor of Tx EVM level. So, we think RAN4 can focus on the inner RB allocation first with CP-OFDM waveform.

Observation 1: For the target UL enhancement scenario i.e. Inner RB region with 64QAM and 256QAM in CP-OFDM waveform, Tx EVM is the bottleneck for meeting MPR requirements of high-order modulations.

To evaluate Tx EVM analysis, RAN4 can reuse the agreed 5G-A WI scope as follow

Proposal 1: RAN4 can consider the inner RB allocation first based on the 5G-A Tx EVM relaxation scope as starting points.

  - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM

  - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements

    - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance.

    - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged.

  - The study is performed based for FR1 single CC operation.

    - Example bands as n41, n77/n78

    - PC3 with 1Tx, PC2 with both 1Tx and 2Tx

  - Waveform: CP-OFDM

Also, the evaluation metric is net gain can be aligned with RAN1 as following.

Proposal 2: The net gain is calculated as achieved MPR gain – loss SNR level i.e., (original MPR – relaxed MPR) - (updated SNR – original SNR).

8	Summary

In this paper, the observations and proposals for the 6G general RF and UE RF are provided.

< Single carrier output power requirements >

Default power class for around 7GHz

Proposal 1: For eMBB service in 6GR, RAN4 can consider default power class with PC2 (26dBm) for 6G new candidate spectrum at around 7GHz.

MPR/A-MPR aspects

General:

Observation1: The large MPR under a default power class of PC2: 26dBm and 200MHz channel bandwidth could shrink the uplink coverage compared to 5G NR in 3.5GHz.

Reducing Tx RF impairments to improve A-MPR and possibly MPR: Flexible/fast LO retuning

Proposal 1: Consider whether it would be possible to study the feasibility and benefits of lo retuning to improve A-MPR, and potential MPR.

Requirements relaxation to allow reduced MPR: Scenario-specific MPR reduction

Observation 2: Rel-19 MPR improvement feature can enable to de-restrict maximum output power by up to 1dB by converting “outer RB allocations” to “inner RB allocations”.

Proposal 2: Consider re-application of the Rel-19 single carrier MPR improvement concept for 6GR.

Observation 3: Extending the concept of relaxed OOB emissions to de-restrict Tx power can be extended to broader scenarios such as adjacent MNO site sharing within an area.

Proposal 3: Consider extension of the Rel-19 single carrier MPR improvement general concept to other scenarios including adjacent channel MNO site sharing for 6GR.

Other MPR aspects

Proposal 4: To simplify MPR specification, RAN4 can consider whether single or scalable requirements to consider the multiple Tx antenna configuration, simple antenna isolation level.

Proposal 5: To simplify MPR specification, RAN4 can consider whether scalable requirements to consider the multiple power classes.

SAR/MPE for 6GR

Observation 1: The duty-cycle restriction was already considered in NW configuration for high power UE with less than 50% for TDD bands.

Observation 2: The existing duty-cycle solution is based on UE capability, leading to scheduling inflexibility that hinders further improvement in UL performance.

Proposal 1: RAN4 need to study how to define a flexible/efficient duty-cycle control mechanism for 6GR.

Proposal 2: RAN4 can consider both P-MPR and duty-cycle control mechanism to satisfy the SAR/MPE requirements in 6GR.

< Tx requirements >

ACLR/ACS/SEM

Observation 1: In BS RF session, RAN4 already agreed to evaluation the coexistence study to consider the different points compared to 5G coexistence parameters. The expected different points are supporting a wider CBW, enhanced gNB antenna elements and defaults power class as PC2 in around 7GHz.

Proposal 1: Based on the coexistence evaluation results, RAN4 can update ACLR and ACS requirements according to frequency ranges (e.g, less 1GHz frequency, 2GHz, 3.5GHz, 4.5GHz and around 7GHz). It means different ACLR requirements can be considered and supported in 6G.

Proposal 2: Consider specifying an operator-controlled relaxation in UE ACLR/SEM for 6GR from first Release, and further study the extended benefits of this for multi-operator site sharing scenarios.

OOB boundary for spurious emissions

Observation 1: NR’s OOB boundary for spurious emissions has been defined as “BW+5MHz from the CBW edge in 5G compared to the recommendation of ITU-R SM.1539 and SM.329 with 250% of necessary channel bandwidth rule.

Proposal 1: RAN4 can consider the SM.1539-2 and SM.1541 recommendation to determine the boundary between the out of band emission and spurious domains for 6G study.

< Rx requirements >

Observation 1: In 5G, the TDD band RX REFSENS for wider CBWs can be derived based on Table and equation below. This 5G TDD band REFSENS equation can be considered for 6G as staring point.

Proposal 1: As for deriving 6G TDD band wider CBW (e.g., >100MHz) REFSENS, discuss whether the 5G TDD band REFSENS equation could be a starting point with modification (e.g., consider performance impact due to wider CBW).

Proposal 2: The 5G ΔRIB,c usage shall be kept for 6G CA. For a 6G UE supporting CA, the minimum requirement for single-carrier reference sensitivity shall be increased by the amount given by ΔRIB,c specified in 6G.

Proposal 3: Regarding HPUE self-interference REFSENS RSD issue in FDD bands, discuss whether it would be proper to study RSD mitigation.

< Frequency range between FR1 and FR2-1 >

Observation 1: To achieve same coverage between 3.5GHz and new 6G candidate spectrum in 7GHz and 15GHz, RAN4 need to compensate channel loss and penetration loss about 12 dB in 7GHz frequency range and compensate 22dB in 15GHz frequency range.

Proposal 1: At least new 7GHz candidate spectrum, FR1-panel architecture with directional antenna system (as refer TR38.901 v19.1.0) can be considered as baseline in 6GR.

Proposal 2: On top of with UE/gNB enhanced antenna elements with enhanced system configuration, RAN4 shall consider additional enhancements skills such as coherent UL-MIMO/beam forming, power boosting with relaxed ACLR and Tx EVM, default power class with 26dBm and SBFD in both gNB/UE and other candidate schemes.

< Spectrum aggregation >

RF switching time performance:

Observation 1: The requirements of interruption including the RF switching time in previous mobile generation has not been optimised since NR Rel-15.

Observation 2: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels.

Proposal 1: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

< Joint UE and BS RF >

Observation 1: For the target UL enhancement scenario i.e. Inner RB region with 64QAM and 256QAM in CP-OFDM waveform, Tx EVM is the bottleneck for meeting MPR requirements of high-order modulations.

Proposal 1: RAN4 can consider the inner RB allocation first based on the 5G-A Tx EVM relaxation scope as starting points.

  - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM

  - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements

    - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance.

    - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged.

  - The study is performed based for FR1 single CC operation.

    - Example bands as n41, n77/n78

    - PC3 with 1Tx, PC2 with both 1Tx and 2Tx

  - Waveform: CP-OFDM

Proposal 2: The net gain is calculated as achieved MPR gain – loss SNR level i.e., (original MPR – relaxed MPR) - (updated SNR – original SNR).

9	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Qualcomm (Feature Leader/Moderator)