3GPP TSG-RAN WG4 Meeting # 117                                                            R4-2520348

Dallas, USA, 17th – 21st Nov., 2025,

Agenda item:	7.12.3

Source:	MediaTek Inc.

Title:	Discussion on RRM measurement prediction

Document for:	Discussion

# Introduction

RAN#109 plenary has agreed the updated version of the WID with the following RAN4 objectives [1]:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In this contribution, we would like to discuss RRM measurement prediction.

# Discussion

As discussed in our another contribution R4-2520347, for legacy L3 measurement, RAN4 only defined test cases for event triggered L3 measurement report. As AI/ML model is power consuming, for the sake of power saving, we hope UE not to be mandatory to predict the future all the time, which is also not necessary. For temporal prediction, it should be up to UE implementation when to do the prediction as long as UE can meet the requirements. So measurement event prediction should be the focus.

Proposal 1: For temporal prediction, focus on measurement event prediction.

In the next, we would like to focus on inter-frequency measurement prediction. As discussed in R4-2520347, it is not clear to us whether the case that the co-located cells have different horizontal coverage is in scope or not. So we focus on the case that the co-located cells have the same horizontal coverage range. If the horizontal coverage ranges of the two frequency layers are the same, low band is usually deployed for coverage, and higher band is deployed for capacity. The measurement on another band is mainly for load balance purpose HO or SCell addition. In this scenario, RSRP of low band will be surely higher than high band. If current serving cell is on high band, then the collocated low band cell is surely of good quality. In this case, we think prediction on low band is redundant. In the next we focus on predicting RSRP of high band through the measurement results of low band. Consider a UE moving from Area 1 to Area 3 as shown in Fig.1. Cell 1 and Cell 2 are on the low band. Cell 1’ and Cell 2’ are on the high band. For load balance purpose HO and SCell addition, the measurement on high band is mainly to make sure the target cell is not too bad. In Area 1, even without any measurement or prediction on high band, both NW and UE can know that the co-located Cell 1’ is of good quality as RSRP of Cell 1 is high enough. So in Area 1, UE may not need to measure or monitor RSRP of the high band. In Area 2, UE may need to predict RSRP of Cell 1’ as well as monitor the prediction accuracy. If there are some other inter-frequencies to measure. RAN4 needs to discuss how to share the gap among all the inter-frequency layers. We think UE is intelligent enough to know which inter-frequency layer should be measured/monitored more frequently and which can be measured/monitored more frequently.  When defining requirements, RAN4 should allow UE to measure/monitor smartly.

Fig. 1

Proposal 2: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss how to update inter-frequency measurement delay requirements for monitoring. RAN4 should investigate how to accommodate UE adapting inter-frequency measurements smartly.

# Conclusion

In this contribution, we provide our views for measurement event prediction. Observations and proposals are summarized as follows.

Proposal 1: For temporal prediction, focus on measurement event prediction.

Proposal 2: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss how to update inter-frequency measurement delay requirements for monitoring. RAN4 should investigate how to accommodate UE adapting inter-frequency measurements smartly.

# Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.