3GPP TSG-RAN WG4 Meeting # 117                                         R4-2520350

Dallas, USA, 17th – 21st Nov., 2025

Agenda Item: 6.17.2.3

Source: MediaTek Inc.

Title: Discussion on test cases for conditional LTM

Document for: Discussion

# Introduction

In this contribution, we would like to discuss the remaining issues of performance part for conditional LTM.

# Discussion

RAN4# 116bis reached the following agreement on the test cases for conditional LTM. In the next, we would like to discuss the applicability rules.

Issue 1-1: Test case list for CLTM < Agreement > - Define the following test cases for FR1: - #1: RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 - #2: RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 - #3: RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 - #4: RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1 UE to pass one test case for FR1 based on the supported UE capability. FFS the detailed applicability rule. - Define the following test cases for FR2: - #1: RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2. - #2: RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2. UE to pass one test case for FR2 based on the supported UE capability. FFS the detailed applicability rule.

The related UE capabilities are copied for information.

cltm-EarlyTA-Indication-r19 Indicates whether the UE supports early TA MAC CE reception for CLTM by indicating the maximum number of TA values that the UE can store. A UE supporting this feature shall also indicate support of at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19 and support of rach-EarlyTA-Measurement-r18 for at least one band.

cltm-ExecutionConditionL1-r19 Indicates whether the UE supports the evaluation of LTM conditions evaluation based on L1 measurements. The UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 for at least one band.

cltm-ExecutionConditionL3-r19 Indicates whether the UE supports the evaluation of LTM conditions evaluation based on L3 measurements, by indicating the maximum number of trigger events for the same execution condition. A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 for at least one band.

intraFreqL1-MeasConfig-r18 Indicates whether UE supports intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxIntraFreqCellsConfig-r18 indicates the maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement; - supportedMaxIntraFreqCellsPerReport-r18 indicates the maximum number of candidate cells in one report where a SSBRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement; - supportedMaxReportBeamsPerReportedCell-r18 indicates the maximum number of candidate beams per candidate cell in one report where a SSBRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement; - supportedMaxReportBeamsReports-r18 indicates the maximum number of candidate beams in total across all cells in one report where a SSBRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement; - supportedMaxAperiodic-LTM-CSI-ReportConfig-r18 indicates maximum number of aperiodic LTM-CSI-ReportConfig; - supportedMaxPeriodic-LTM-CSI-ReportConfig-r18 indicates maximum number of periodic LTM-CSI-ReportConfig; - supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r18 indicates maximum number of semi-persistant LTM-CSI-ReportConfig; UE supporting this feature shall also indicate support of periodicBeamReport or aperiodicBeamReport or sp-BeamReportPUCCH or sp-BeamReportPUSCH.

ltm-InterFreq-r18 Indicates UE supports inter-frequency MCG LTM on all the bands where the UE indicates support of ltm-MCG-IntraFreq-r18 or inter-frequency SCG LTM on all the bands where the UE indicates support of ltm-SCG-IntraFreq-r18 respectively. A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18.

ltm-InterFreqMeasGap-r18 Indicates whether the UE supports SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM. A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18.

For the test cases to be defined, the required capabilities are as following:

- FR1:

  - #1: RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

    - Requires UE supporting intra-f L1 measurement, i.e., intraFreqL1-MeasConfig-r18

  - #2: RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

    - Requires UE supporting inter-frequency L1-RSRP measurements with measurement gaps, i.e., ltm-InterFreqMeasGap-r18

  - #3: RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

    - Requires UE supporting TA value reception before cell switch and intra-f L1 measurement, i.e., cltm-EarlyTA-Indication-r19 and intraFreqL1-MeasConfig-r18.

  - #4: RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1

    - Requires UE supporting TA value reception before cell switch, i.e., cltm-EarlyTA-Indication-r19

- FR2:

  - #1: RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

    - Requires UE supporting intra-f L1 measurement, i.e., intraFreqL1-MeasConfig-r18

  - #2: RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

    - Requires UE supporting TA value reception before cell switch and intra-f L1 measurement, i.e., cltm-EarlyTA-Indication-r19 and intraFreqL1-MeasConfig-r18.

Based on the above, we have the following proposal:

Proposal 1: For the test cases for CLTM, the applicability rules are

- For UE supporting “cltm-EarlyTA-Indication-r19” and not supporting “intraFreqL1-MeasConfig-r18”, UE is required to pass the test case:

  - RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1

- For UE supporting “intraFreqL1-MeasConfig-r18” and not supporting “cltm-EarlyTA-Indication-r19” & “ltm-InterFreqMeasGap-r18”, UE is required to pass the test case:

  - RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

  - RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2

- For UE supporting “cltm-EarlyTA-Indication-r19” & “intraFreqL1-MeasConfig-r18”, UE is required to pass the test case:

  - RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

  - RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

- For UE not supporting “cltm-EarlyTA-Indication-r19” and supporting “ltm-InterFreqMeasGap-r18”, UE is required to pass the test case:

  - RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

For subsequent CLTM, the only difference from CLTM is applying the configurations for the next CLTM after finishing the previous CLTM. It is a basic function that UE applies the configurations at a command or meeting some condition. We don’t think it is necessary to test this. Therefore, we propose not to define test cases for subsequent CLTM.

Proposal 2: Not to define test cases for subsequent CLTM.

# Summary

In this paper, we provide some view on performance requirements of R19 mobility. We have the following proposals:

Proposal 1: For the test cases for CLTM, the applicability rules are

- For UE supporting “cltm-EarlyTA-Indication-r19” and not supporting “intraFreqL1-MeasConfig-r18”, UE is required to pass the test case:

  - RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1

- For UE supporting “intraFreqL1-MeasConfig-r18” and not supporting “cltm-EarlyTA-Indication-r19” & “ltm-InterFreqMeasGap-r18”, UE is required to pass the test case:

  - RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

  - RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2

- For UE supporting “cltm-EarlyTA-Indication-r19” & “intraFreqL1-MeasConfig-r18”, UE is required to pass the test case:

  - RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

  - RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

- For UE not supporting “cltm-EarlyTA-Indication-r19” and supporting “ltm-InterFreqMeasGap-r18”, UE is required to pass the test case:

  - RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

Proposal 2: Not to define test cases for subsequent CLTM.

# 4 Reference

RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.