3GPP TSG-RAN WG4 Meeting # 117                                      R4-2520351

Dallas, USA, 17th – 21st Nov., 2025,

Agenda Item: 7.10.3

Source: MediaTek Inc.

Title: Discussion on RRM impact for NR MIMO Phase 6

Document for: Discussion

# Introduction

RAN#109 agreed on the revised WID “NR MIMO Phase 6” [1]. According to WID, RAN4 is supposed to define requirements for the following enhancements:

- On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - On phase continuity, RAN4 to study and conclude by RAN#111 whether the requirements from the legacy RAN4 spec for DMRS bundling can be retained. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

In this contribution, we would like to discuss the potential RRM core impact.

# Discussion

## 2.1 Objective#1: Enhancing UL capacity and coverage

For Objective#1, there are two enhancements. Both of the enhancements are on SRS transmission. In this contribution, we focus on cross-slot SRS. For SRS transmission, RAN4 defined the interruption requirements due to SRS carrier switching and antenna switching. The legacy interruption requirements are defined for SRS resources with no more than 6 symbols. The enhancement is targeting for SRS pattern larger than 1 symbol. For the SRS resources with 2-6 symbols, the legacy interruption requirements are defined as slot level. Even cross-slot SRS is supported, the legacy requirements are also applicable. We also observed that there is proposal in RAN1 to introduce SRS resources with more than 14 symbols. If it is agreed, RAN4 may need to further check whether the legacy requirements are still applicable.

Proposal 1: For the enhancement “Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”, wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching and antenna switching are still applicable.

2.2 Objective#2: Enhancing DL CSI acquisition

According to the WID, there are three scenarios to enhance. In the next, we would like to discuss the RRM impact one by one.

### 2.2.1 UE transitioning from IDLE/INACTIVE to CONNECTED mode

One scenario is for UE transitioning from IDLE/INACTIVE to CONNECTED mode. Last meeting, there was some discussion on the timeline. We understand that the same issue is discussed in RAN1 too. But RAN4 is more specialized on the requirements defined for BWP switch. We prefer to discuss this issue in RAN4.

Issue 3-1-1: Timeline of early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode < Way forward > FFS the following proposal: - Proposal 1 (Ericsson): - RAN4 shall first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, then RAN4 should clarify to RAN1 through LS that, as per the existing procedure, UE cannot transmit/receive from the RRCSetup reception until RRCSetup processing delay is complete. RRCSetup processing delay as per TS 38.331 is 10ms. - RAN4 shall first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, then RAN4 should clarify to RAN1 that, as per the existing procedure, UE cannot transmit/receive from the RRCResume reception until RRCResume processing delay is complete. RRCResume processing delay as per TS 38.331 is 6 or 10ms based on different scenarios.

According to RAN1 agreement, early SRS-AS/CSI-RS/CSI are triggered at Msg4. Usually, UE sends RRCsetupRequest in Msg3 and NW sends RRCsetup to UE in Msg4. In the RRCsetup message, first active BWP is usually configured. According to the processing delay requirements defined in 38.331, in case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3. According to 38.133, the BWP switching delay requirements are also applicable when there is any parameter change of the active BWP. During the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC, UE is not required to receive or transmit on the cell where RRC-based BWP switch occurs.

38.331 38.133 ……

According to 38.331, at RRC Setup Configuration, NW is mandatory to indicate the ID of the first active BWP.

38.331

If the first active BWP is not BWP#0, RRC Setup will surely trigger BWP switch. If the first active BWP is BWP#0, according to RAN2 spec, there can be two options as shown below. No matter BWP#0 is configured only in ServingCellConfigCommon or both in ServingCellConfigCommon and ServingCellConfig, BWP#0 should be configured. As there are huge of BWP related parameters, we don’t think UE is required to compare whether the first active BWP shares the same parameters as the initial BWP. Therefore, as long as first active BWP is configured in RRCsetup message, UE is not required to receive or transmit during by TRRCprocessingDelay+TBWPswitchDelayRRC.  The above analysis is applicable to RRC resume.

38.331 # B.2 Description of BWP configuration options There are two possible ways to configure BWP#0 (i.e. the initial BWP) for a UE: 1) Configure BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon, but do not configure dedicated configurations in BWP-DownlinkDedicated or BWP-UplinkDedicated in ServingCellConfig. 2) Configure both BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon and configure dedicated configurations in at least one of BWP-DownlinkDedicated or BWP-UplinkDedicated in ServingCellConfig. The same way of configuration is used for UL BWP#0 and DL BWP#0 if both are configured. With the first option (illustrated by figure B2-1 below), the BWP#0 is not considered to be an RRC-configured BWP, i.e., UE only supporting one BWP can still be configured with BWP#1 in addition to BWP#0 when using this configuration. The BWP#0 can still be used even if it does not have the dedicated configuration, albeit in a more limited manner since only the SIB1-defined configurations are available. For example, only DCI format 1_0 can be used with BWP#0 without dedicated configuration, so changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching. Figure B2-1: BWP#0 configuration without dedicated configuration With the second option (illustrated by figure B2-2 below), the BWP#0 is considered to be an RRC-configured BWP, i.e. UE only supporting one BWP cannot be configured with BWP#1 in addition to BWP#0 when using this configuration. However, UE supporting more than one BWP can still switch to and from BWP#0 e.g. via DCI normally, and there are no explicit limitations to using the BWP#0 (compared to the first option). Figure B2-2: BWP#0 configuration with dedicated configuration For BWP#0, the BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon should match the parameters configured by MIB and SIB1 (if provided) in the corresponding serving cell. If an RedCap-specific initial DL BWP is configured, for BWP switching, the BWP #0 always maps to the RedCap-specific initial DL BWP. If a RedCap-specific initial UL BWP is configured, for BWP switching on NUL, the BWP #0 always maps to the RedCap-specific initial UL BWP, for BWP switching on SUL, the BWP#0 always maps to the initial UL BWP.

Observation 1: At RRC setup/resume configuration, first active BWP shall be configured.

Proposal 2: When UE transitions from IDLE/INACTIVE to CONNECTED mode, UE is not required to transmit or receive during TRRCprocessingDelay+TBWPswitchDelayRRC after receiving RRC setup/resume configuration.

### 2.2.2 SCell activation

Another scenario is for SCell activation. Last meeting, there were proposals on the applicable scenarios of early SRS/CSI/CSI-RS triggering, the corresponding timeline and the impact on RRM spec. In the following, we would like to discuss those issues.

Applicable scenarios:

From R15 to R19, there are many SCell activation scenarios discussed, we summarize them as

- Non-PUCCH SCell activation – single SCell (R15)

  - Known cases

  - Unknown cases

    - when TCI state indication is provided

    - when TCI state indication is not provided

- PUCCH SCell (8.3.12)

  - when the UE has a valid TA for transmitting on the SCell

  - when the UE does not have a valid TA for transmitting on the SCell

- Fast SCell activation (8.3.16)

- SCell activation based on measurement in IDLE/INACTIVE mode (8.3.2A)

- OD-SSB based SCell Activation

- SCell Activation with the L3 reporting during activation (8.3.17)

- SCell activation with Multiple SCells

For the following two scenarios, due to lack of measurement results, NW does not know which TCI state should be used after SCell activation before UE reports the measurement results. In the two scenarios, TCI state is indicated either after UE reports L1 measurement results (R15) or L3 measurement results (SCell Activation with the L3 reporting during activation). In the enhanced DL CSI acquisition feature, NW is supposed to trigger SRS/CSI/CSI-RS at sending SCell activation command, similarly, due to lack of measurement results, NW would not know which SRS/CSI-RS to trigger in the following two scenarios. Therefore, we can conclude that the enhanced feature is not applicable to the following two scenarios.

- Non-PUCCH SCell activation – single SCell (R15)

  - Unknown cases

    - when TCI state indication is not provided

- SCell Activation with the L3 reporting during activation (8.3.17)

For PUCCH SCell, if UE does not have a valid TA for transmitting on the SCell, then UE can only transmit SRS/CSI on the to-be-activated SCell after the RACH procedure. Usually, there are many uncertainties in the RACH procedure, e.g., when UE transmits PRACH, when NW sends Msg2. Due to these uncertainties, it is difficult for NW to determine the occasion of the to-be-triggered AP-SRS. Therefore, we can conclude that early AP-SRS/CSI triggering at Scell activation is not applicable for “PUCCH SCell activation when UE does not have a valid TA for transmitting on the SCell”.

Proposal 3: Early SRS/CSI/CSI-RS triggering at SCell activation is not applicable for the following three scenarios:

- Non-PUCCH SCell activation – single SCell (R15)

  - Unknown cases

    - when TCI state indication is not provided

- SCell Activation with the L3 reporting during activation (8.3.17)

- PUCCH SCell (8.3.12)

  - when the UE does not have a valid TA for transmitting on the SCell

We understand this issue is also discussed in RAN1. As RAN4 are much clear about the requirements for different scenarios, it is better to have discussion in RAN4.

Timeline of Early SRS/CSI/CSI-RS triggering for SCell activation:

If not to consider the potential gain, we think the enhanced feature “early SRS/CSI/CSI-RS triggering at SCell activation” are applicable for the other scenarios. The timeline will be different for non-PUCCH SCell and PUCCH SCell.

For non-PUCCH SCell, no matter to meet Te requirement for SRS transmission or to guarantee the accuracy of CSI measurement, UE is not supposed to measure CSI or transmit SRS before finishing T/F tracking.

For PUCCH SCell when the UE has a valid TA for transmitting on the SCell, UE can start to measure CSI after finishing T/F tracking. But for SRS/CSI transmission, UE needs to finish PL-RS measurement at first.

Proposal 4: For non-PUCCH SCell, UE is not required to measure CSI or transmit SRS before finishing T/F tracking.

Proposal 5: For PUCCH SCell when the UE has a valid TA for transmitting on the SCell, UE is not required to measure CSI before finishing T/F tracking, and UE is not required to transmit SRS/CSI on the to-be-activated SCell before finishing PL-RS measurement.

Whether to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation:

For early CSI/CSI-RS triggering, checking the legacy SCell activation delay requirements, the scenario that CSI-RS triggered at the same time as SCell activation command had also been considered. The ending point of legacy SCell activation requirements is that UE reports the first valid CSI report for the SCell. So RAN4 does not need to define any new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

Proposal 6: No need to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

For early SRS triggering, legacy SCell activation requirements include three parts: THARQ, Tactivation_time and TCSI_reporting. Even early SRS is triggered for SCell activation, the first two parts, i.e., THARQ, Tactivation_time, should be the same. The only issue is whether to update TCSI_reporting to the time to transmit SRS or something others. From UE’s perspective, for non-PUCCH SCell, after Tactivation_time, UE is ready to receive or transmit on the SCell. For PUCCH SCell when the UE has a valid TA for transmitting on the SCell, UE is ready to transmit on the SCell after PL-RS measurement. The ending point is just the point to notify NW that SCell has been activated. We are open to use either the first valid CSI report or the first SRS transmission as the ending point.

Proposal 7: When early SRS is triggered for SCell activation, the two parts THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

### 2.2.3 SCell dormancy

The third scenario is switching out of SCell dormancy. RAN4 defined the switching delay and interruption requirements for SCell dormancy. After the switching delay, UE will be ready to transmit SRS or receive CSI-RS for CSI acquisition. Switching delay and interruption on other carriers are caused by UE BB/RF processing. Early SRS/CSI/CSI-RS triggering has no impact on UE BB/RF processing for BWP switch, so there is also no impact on the switching delay and interruption requirements of SCell dormancy.

Proposal 8: For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable.

# Summary

In this paper, we provide some views on the potential RRM core impact of NR MIMO Phase 6. We have the following proposals:

Proposal 1: For the enhancement “Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”, wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching and antenna switching are still applicable.

Observation 1: At RRC setup/resume configuration, first active BWP shall be configured.

Proposal 2: When UE transitions from IDLE/INACTIVE to CONNECTED mode, UE is not required to transmit or receive during TRRCprocessingDelay+TBWPswitchDelayRRC after receiving RRC setup/resume configuration.

Proposal 3: Early SRS/CSI/CSI-RS triggering at SCell activation is not applicable for the following three scenarios:

- Non-PUCCH SCell activation – single SCell (R15)

  - Unknown cases

    - when TCI state indication is not provided

- SCell Activation with the L3 reporting during activation (8.3.17)

- PUCCH SCell (8.3.12)

  - when the UE does not have a valid TA for transmitting on the SCell

Proposal 4: For non-PUCCH SCell, UE is not required to measure CSI or transmit SRS before finishing T/F tracking.

Proposal 5: For PUCCH SCell when the UE has a valid TA for transmitting on the SCell, UE is not required to measure CSI before finishing T/F tracking, and UE is not required to transmit SRS/CSI on the to-be-activated SCell before finishing PL-RS measurement.

Proposal 6: No need to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

Proposal 7: When early SRS is triggered for SCell activation, the two parts THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

Proposal 8: For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable.

# 4 Reference

RP-252510, Revised Work Item: NR MIMO Phase 6.