3GPP TSG-RAN WG4 Meeting # 117	R4-2520358

# Dallas, USA, 17th – 21th, Nov 2026

Source: 	Samsung

Title: 	Discussion on general RF and UE RF for 6GR

Agenda Item:	8.3

Document for:	Approval

- Introduction

This paper organizes the discussion of 10 aspects following WF [1] guidance and issues identified.

- Apply the same order of topics as topic summary in R4-2514509

- General aspects

- Single carrier output power

- CA output power

- Tx requirements

- Rx requirements

- Frequency range between FR1 and FR2-1

- Spectrum aggregation

- Joint UE and BS RF

- Testing and verification

- Others

- 2. Discussion

2.1 General (TN and NTN integration)

In RAN1#122, consensus was reached that studies focus on identifying technical aspects impacted by NTN characteristics, with attention to lessons learned from NR and IoT NTN. Harmonization in the context of 6G should be interpreted as the creation of a single and efficient framework, where TN serve as the baseline foundation. TN will continue to be the primary deployment scenario and the main driver of performance and scalability. For this reason, the baseline framework must be optimized around TN operation, ensuring that performance and simplicity are not compromised. NTN can then be introduced as scalable extensions that interoperate with the TN baseline, rather than imposing structural changes on the fundamental design from the outset. Harmonization, therefore, is not the act of embedding every NTN-specific requirement into the baseline but rather the establishment of an adaptable architecture. Such an approach allows NTN to be incorporated in a flexible, consistent manner while safeguarding the stability, efficiency, and integrity of TN operation.

From a feature perspective, the starting point should be a compact TN baseline feature set, rather than an extensive and pre-defined catalog of NTN requirements. This approach avoids unnecessary complexity and ensures that the core framework remains streamlined. Any potential NTN features should be subject to clear and strict criteria: they must be implementable as minimal add-ons that do not degrade TN performance, do not increase operational complexity, and do not introduce unintended limitations. NTN support in this model is envisioned as general, lightweight, and non-intrusive, so that integration can occur without undermining TN deployment. At the same time, certain fundamental aspects must be acknowledged. In particular, considerations for time and frequency compensation represent essential elements, as these are inherent to NTN operation and unavoidable in a harmonized framework. However, beyond such foundational elements, feature-specific optimizations targeting NTN should be postponed at this stage, to prevent the premature inclusion of niche-driven requirements that risk overcomplicating the baseline.

Proposal #2.1-1: Consider a TN-centric 6G baseline, with NTN supported only through minimal, scalable extensions. The focus should remain on preserving TN performance and simplicity, while limiting NTN features to essential aspects such as time/frequency compensation.

2.2 Single carrier output power

Before diving into any detailed discussion on other Tx requirements, the first thing to address is that whether to retain the power class concept, and if retained whether to maintain a 3dB between power classes. The pros and cons (lesson and learn from 5G) are summarized as following.

- Pros:

- Easy for requirements building (i.e., all requirements are built upon PC concept) and conformance test

- Cons:

- Inefficient PA utilization: Some UE may not fully utilize their PA capacity. Futures like “Power boosting” and “increasing higher power limit”, introduced in 5G for higher power transmission, have led to fragmented UE implementation choices, reducing the likelihood for implementation

Observation #2.2-1: Pros and cons for Power class concept.

- Pros:

- Easy for requirements building (i.e., all requirements are built upon PC concept) and conformance test

- Cons:

- Inefficient PA utilization: Some UE may not fully utilize their PA capacity. Futures like “Power boosting” and “increasing higher power limit”, introduced in 5G for higher power transmission, have led to fragmented UE implementation choices, reducing the likelihood for real implementation

Observation #2.2-2: The first thing to address is whether to retain the power class concept, and if retained whether to maintain a 3dB between power classes.

By considering above pros and cons, it is advisable to keep the power class concept with a 3dB step and its reporting, consistent as 5G, to ease the requirements development and conformance testing. We are open to exploit the full usage of power capacity of PA in the meanwhile avoiding excessive UE fragmentation which might be potentially caused by various power boosting features, as seen in 5G. One potential alternative could be to remove PCMAX_H, and only retain p-max to account for regulation limitation of transmission power as [3][6] suggested.

Proposal #2.2-1: It is advisable to keep the power class concept with a 3dB step and its reporting.

Proposal #2.2-2: Further exploit the full usage of PA capacity in the meanwhile avoiding excessive fragmented implementation choices.

Next hot topic is the default power class, we believe the default power class should still be PC3 (except that PC5 is the default for NR-U band) to ensure implementation possibility for low-end UEs. Nothing precludes advanced UE to implement higher power class provided the UE is capable enough to meet the requirements. This allows for scalability and performance optimization without affecting low-end UEs. For about 7GHz, to achieve comparable coverage as C-band with inter-site deployment, BS can implement more antenna elements.

Observation #2.2-3: For about 7GHz, to achieve comparable coverage as C-band with same assumption of inter-site deployment, BS can implement more antenna elements.

Proposal #2.2-3: PC3 should still be the default power class. This allows for scalability and performance optimization without affecting low-end UEs.

As [6] well summarized, and quoted here, “ΔTIB/ΔRIB are rather tedious CA requirements which have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests”. In RAN4 permanent reference document (PRD) clause 5.4 developed in the late Rel-19 stage, much more simplified rules have been captured in an effort to replace the case-by-case study/specification for ΔTIB/ΔRIB requirements or at least to provide aligned rules. However, these simplified rules are currently only documented in PRD, while the extensive pages for ΔTIB/ΔRIB requirements for CA are still remained in specification at least in Rel-19 and most likely also in Rel-20. For 6G, it is right time to review whether the justification of ΔTIB/ΔRIB still stands for CA. They were initially motivated by the consideration of the needed additional RF components, such as multi-plexer, to support CA. However, it is important to note that single carrier actually passes through the same RF components as a carrier within CA. As such, the original motivation no longer holds.

For ΔTIB, it should be removed from CA requirements but kept it in single carrier PCMAX_L equation as it is. Further, simplification can be made using the contents in RAN4 permanent reference document (PRD) clause 5.4 as starting point. Note that this kind of simplification discussion should be taken place in spectrum thread.

Proposal #2.2-4: Remove ΔTIB from CA requirements, while retaining it in the single carrier PCMAX_L equation as it is.

Observation #2.2-4: Simplification kind of discussion for ΔTIB should take place in Spectrum thread.

For single carrier, for UE equipped with multiple PAs, there were proposals on asymmetric TxD/MIMO in 5G, which is to advocate the sum of each PA capability according to the proponents. While UEs can achieve higher conductive power, the gain from the OTA perspective remains questionable. Further, this approach would further complicate CA implementation. We are open to discussing it further but would like to see more justification.

| Single CC | Proposal | Details | Pros and Cons |
| --- | --- | --- | --- |
| Single CC | Asymmetric TxD Asymmetric UL MIMO | Like 26dBm+23dBm Like 26+23+23+23 | Pros: Fully utilize PA capability, higher power for conductive Cons: Questionable gain for OTA; Even complex for 4Tx-TxD; Further complicate CA implementation; Need to consider whether to define req. for non-integer PC (i.e., duty cycle for SAR, MPR/AMPR, ACLR etc) and if new conformance test rule needed |

Proposal #2.2-5: More justification should be provided for asymmetric TxD for single carrier, if it is still of interest.

In 5G, there are primarily two approaches to address SAR issue, i.e., P-MPR (Not specified in 3GPP) and duty cycle solution (specified in 3GPP). The duty cycle solution, though specified for both single CC and CA, has never been implemented by any UE vendors due to the clear drawbacks. These includes the big 3 dB step-size power fallback, and the unclear estimated evaluation window, which have hindered its implementation. For 6G, it is advisable not to specify the duty cycle solution anymore. The P-MPR solution remains viable, though it is not specified in 3GPP.

Proposal #2.2-6: Do not specify duty cycle solution for SAR issue.

2.3 CA output power

Deprioritized in this meeting per FL’s guidance. This part is skipped in RAN4#117.

2.4 Tx requirements

It was agreed that 6G PA modelling will be discussed in the system parameter thread, which is mainly for RAN1 waveform evaluation purposes, and early feedback to RAN1 is required. RAN4 may further evolve this Power Amplifier (PA) model for RAN4-centric studies such as MPR evaluation. In addition to PA models, Tx impairment assumptions, including carrier leakage and image rejection, are also critical factors that should be determined early. As noted by companies in [4][5], the -28 dB value for both parameters has been in use for over 20 years, and it is time to revisit this, especially to support the higher-order modulation scheme 1024QAM. To accommodate different UE types (in particular low-tier and low-cost UEs), we propose adopting -35 dB for both carrier leakage and image rejection for frequencies up to around 7GHz. For Maximum Power Radiation (MPR) evaluation, for the sake of simplicity, the 5G assumption is reused, i.e., the LO is always placed in the middle of the CBW. In addition, same as 5G, the UE Digital Pre-Distortion (DPD), which is highly implementation relevant, is not considered in the transmission modelling for MPR study.

Proposal #2.4-1: For frequencies up to around 7GHz, adopt -35 dB for carrier leakage and -35 dB for image rejection.

Proposal #2.4-2: For MPR evaluation, reuse the 5G assumption, i.e.,

- The LO is placed in the middle of the CBW

- UE DPD is not considered in the transmission modelling

Next important requirement to discuss is MPR. To date, there are over 20 MPR tables in TS 38.101-1, considering the following factors.

- Different power classes (PC3, PC2, PC1.5, PC1, [PC1bis])

- Different UE architectures (TxD, dualPA, 1LO1PA),

- Different modes (single carrier, UL MIMO, UL NC CA, UL C CA with contiguous RB, UL C CA with non-contiguous RB),

- Different Tx Chain numbers (1~4 Tx)

- Different form factor (For PC1.5 and above, there are separate MPR requirements assuming different antenna isolation for handheld and FWA)

- Different use case (e.g.,V2X)

MPR evaluation is widely recognized as time-consuming for both measurement and simulation approaches, especially in case of multiple Tx chains. Given that reverse-IMD and counter-IMD cannot be accurately modeled, measurements are considered a more reliable method than simulations, albeit more time consuming.

Regarding different RF architectures for MPR evaluation, for instance, for PC2 UL contiguous CA with contiguous RB allocation, MPRs were evaluated across 3 different RF architectures (not just the widely implemented one), with MPR differences of 0.5–1 dB among these architectures. While for PC1.5 UL contiguous CA with contiguous RB allocation, only one architecture was studied as the referenced architecture due to time constraints; requirements were defined based on this reference architecture, while other implementations are not precluded provided they can meet the specified requirements.

Regarding the different Tx Chain number for single carrier operation with the same power class, during discussion on 3Tx UL MIMO, a brief evaluation was conducted. Ultimately, it was agreed to reuse the PC2 2Tx MPR requirements for PC2 3Tx. For FWA PC1.5 2Tx and 4Tx, the MPR difference is approximately 1–1.5 dB for edge RB allocation, and 0.5–1 dB for outer and inner RB allocations.

Above is kind of lesson and learn from 5G. We therefore propose streamlining MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors.

Proposal #2.4-3: Strive to streamline MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors.

- For instance, for one device type, with same power class and mode (i.e., single carrier, or C/NC CA), strive to specify only one set of MPR requirements. This doesnot preclude other architecture implementations (i.e., those not selected as reference architecture for deriving requirements), provided the specified requirements are met.

2.5 Rx requirements

It was agreed in [2] that RAN4 shall reuse the prefix concept from NR for 6G. It is expected that many 5G bands will be refarmed for 6G. It was agreed in [1] to study two aspects. First, whether the Rx requirements remain relevant for respective 6G bands up to 100 MHz. Second, how to define relevant Rx requirements for CBWs greater than 100 MHz.

It was proposed in [3] to evaluate if the narrowband blocking requirements, introduced to protect 5G/4G from GSM, is still needed. From our knowledge, GSM is still in use in some European countries, and it remains unclear whether all operators will completely decommission GSM before the first 6G specifications are finalized. Therefore, narrowband blocking requirements should be retained.

Observation #2.5-1: It remains unclear whether the GSM network will be fully decommissioned prior to the finalization of the first 6G specification.

In our view, other Rx requirements remain relevant for the respective bands with a CBW of up to 100 MHz. The same requirements framework can be extended to 200 MHz for the existing FR1, as well as to the band around 7 GHz with a CBW of up to 200 MHz.

Proposal #2.5-1: All existing Rx requirements remain relevant for respective bands with a CBW of up to 100MHz. Same requirements framework/concept can be applicable to frequencies up to around 7GHz with a CBW of up to 200MHz.

The most essential requirement for Rx is Reference Sensitivity(REFSENS), as many other Rx requirements are built on top of it. For 5G, REFSENS is calculated by the formula: REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv. For 6G, we think this equation should still be used for REFSENS calculation, at least for frequencies up to around 7GHz with a CBW of up to 200MHz. This is because for up to around 7GHz, UE is still expected to have a conducted interface, assuming an isotropic radiation pattern antenna and without beamforming. Parameters such as Noise figure (NF) and Implementation margin (IM) need to be discussed further. The Signal-to-Noise Ratio(SNR) very likely would remain at -1dB, but this will need to be confirmed through further evaluation by RAN1.

Proposal #2.5-2: For the derivation of REFSENS in the frequency range up to around 7GHz with a CBP of up to 200MHz, the formula "REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv" shall still be adopted.

Regarding how to specify 6G REFSENS in a scalable manner in terms of different Rx Chain count, in our view, the 5G approach, where REFSENS is defined for all bands for 2Rx, with delta values added for 4Rx, 6Rx,8Rx, and 1Rx based on 2Rx, is already scalable. The values are not per band but per frequency range. Thus, the key discussion point here is likely whether 2Rx should remain the "baseline" for REFSENS requirements, with delta values added for other Rx counts. We suggest retaining 2Rx REFSENS as the baseline, given that it is not practical to mandate all kinds of UEs across all bands to support 4Rx or higher.

Proposal #2.5-3: The 2Rx REFSENS shall remain the baseline, with delta values applied to it to derive REFSENS for other Rx count such as 4Rx, 6Rx, 8Rx and 1Rx.

Regarding different power classes, they have no impact on TDD REFSENS but FDD REFSENS. In 5G, the HPUE FDD reference value is derived from PC3 by adding delta values, with consideration of different numbers of UL Tx chains (1Tx or 2Tx). For 6G, we still believe PC3 should serve as the basis for FDD REFSENS, as it is expected to be the default power class for FDD bands. For simplicity, it is preferable to have a single set of requirements regardless of the number of Tx chains.

A pain point noted by company[6] regarding FDD REFSENS/UL configurations is that the rules for selecting UL configurations (i.e., RB allocation) are not transparent, making it difficult for those not closely following technical discussions to understand how UL configurations/RBs are selected. It is proposed to discuss the possibility of adopting aligned rules, such as using the worst-case scenario (i.e., full RB allocation), similar to the methodology for MSD, which is also based on worst-case configurations.

Proposal #2.5-4: For FDD REFSENS, PC3 is the basis, and delta values to higher power class can be added to derive HPUE REFSENS.

- For the same power class, only one set of requirements is defined, regardless of the number of Tx chains.

Proposal #2.5-5: For FDD UL configuration for REFSENS, discuss the feasibility to adopt full RB allocation.

As [6] well summarized, and quoted here, “ΔTIB/ΔRIB are rather tedious CA requirements which have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests”. In RAN4 permanent reference document (PRD) clause 5.4 developed in the late Rel-19 stage, much more simplified rules have been captured in an effort to replace the case-by-case study/specification for ΔTIB/ΔRIB requirements or at least to provide aligned rules. However, these simplified rules are currently only documented in PRD, while the extensive pages for ΔTIB/ΔRIB requirements for CA are still remained in specification at least in Rel-19 and most likely also in Rel-20. For 6G, it is right time to review whether the justification of ΔTIB/ΔRIB still stands for CA. They were initially motivated by the consideration of the needed additional RF components, such as multi-plexer, to support CA. However, it is important to note that single carrier actually passes through the same RF components as a carrier within CA. As such, the original motivation no longer holds. In our view, ΔRIB can be removed from CA requirements, consequently the equivalent effect as ΔRIB should be taken into account in single carrier REFSENS. For simplicity, we propose to incorporate it into implementation margin(IM) for single carrier REFSENS.

Proposal #2.5-6: Remove ΔRIB from CA requirement, and incorporate equivalent effect as ΔRIB into implementation margin (IM) of single carrier REFSENS.

For band combination simplification, several Maximum sensitivity degradation (MSD) simplification ideas have been proposed in the “Spectrum” thread [7]. OTA testing for MSD has also been raised up again in “OTA” thread [8]. In our opinion, before starting the requirements simplification kind of discussion such as leveraging LUT for all MSD cases, and OTA testing, it is essential to first establish the 6G MSD framework in “General RF and UE RF” session. Several directions are identified and we propose to thoroughly check the pros and cons for each one.

Proposal #2.5-7: Discuss and establish the 6G MSD framework, before starting MSD OTA testing discussion.

- Alt1: The MSD calculation and specification manner remain unchanged, i.e., accounting for both the conductive part and OTA part, which are then combined

- Alt2: Conductive-only MSD calculation and specification, i.e., the MSD calculation and specification only consider the conductive part. The terminology must be changed (e.g., MSDConductive) to differentiate with 5G.

- Alt3: Separate conductive MSD and OTA MSD specification. E.g., Define MSDConductive and MSDOTA

- Alt4: UE Dynamic MSD reporting [3][6] → Whether the proposal is UE to report the actual MSD for any configurations requested by NW needs further clarification

- Alt5: UE Semi-static MSD reporting, i.e., UE can report the actual MSD for certain specific configurations for each combo if NW requests such configurations

2.6 Frequency range between FR1 and FR2-1

The WF [10] guides the group to study the RF characteristics for the frequency range between FR1 and FR2-1. However, it remains unclear to us what else to study within this UE RF thread beyond what has already been studied and documented in TR 38.922. Our views remain unchanged, for around 7GHz, UE is expected to have a conducted interface, assuming an isotropic radiation pattern antenna and without beamforming as the antenna array size poses a concern for supporting beamforming. For around 15GHz, the size of antenna is still the concern, Figure 6.5.2-2[12] below presents several example cases with different antenna arrangements accounting for the typical spacing between elements of the array antenna of half wavelength.

Figure 2.6-1: Estimated antenna size at 15GHz

FL summary [11] includes several captured proposals including some features seems are not specific for this frequency range. During last meeting, it has been figured out that the frequency range definition should be discussed in Spectrum thread. Additionally, in our view, the default power class, number of Tx/Rx Chains, and maximum CBW should be discussed in System parameter thread under the umbrella of device type discussion to get a full picture. Thermal issue is not supposed to be addressed by RAN4 spec. Noise figure is something can be further discussed based on the various values documented in different TRs. Overall, it would needed to figure out first what is the scope for this topic.

Proposal #2.6-1: Discuss and clarify what else to study beyond that has already been studied and documented in 5G TR 38.922 and covered by other 6G threads (such as System parameter and Spectrum).

2.7 Spectrum aggregation

Several switching features have been introduced from Rel-16 to Rel-19 including 1Tx-2Tx UL switching and 2Tx-2Tx UL switching for up to 4 bands, L-L CA via switching between FDD and SDL band which is band switching between 2 bands.

For LO switching time which is one factor considered of the switching time, there is misalignment between RRM assumption and RF assumptions. 500 us is assumed for FR1 from LTE period for RRM and never changed. While in latest RF discussion, 210us is the maximum value for Tx switching adopted for Rel-16 to Rel-18 Tx switching features, while 140us is the maximum value adopted for L-L CA via switching feature. To leave some implementation margin, 210 us is proposed for 6G, for both FR1 RF and RRM requirements evaluation.

Proposal #2.7-1: For FR1, 210 us as the assumption for LO switching time for switching relevant requirements.

Intra-band non-contiguous ULCA was introduced from Rel-17 but has never been implemented in the filed from what we heard. One of the primary drawbacks is the significantly large MPR and A-MPR, another is the big REFSENS degradation (ΔRIBNC) for certain FDD intra-band UL CA such as n25(2A) and n71(2A). Given these disadvantages, it remains unclear whether this configuration will remain attractive in the context of 6G. It would be beneficial to gather operators' views on this matter, taking into account the identified drawbacks and potential technology advancements.

Proposal #2.7-2: Check if non-contiguous ULCA still remains attractive in the context of 6G, taking into account the identified drawbacks in 5G and potential technology advancements in 6G.

In 4G/5G, intra-band contiguous CA employs the concept of CA bandwidth classes. For 6G, it is advisable to retain this framework, while the specific details require further checking. For TDD bands, the advantages of bandwidth class B (20 MHz ≤ BWChannel_CA ≤ 100 MHz) over a single carrier are not clear. Further, if 6G introduces 200MHz for certain TDD bands, the advantages of bandwidth class C may also become ambiguous.

Proposal #2.7-3: For intra-band contiguous CA, the CA bandwidth class concept is kept. For TDD bands,

- Further check if class B is still needed, by considering the operators’ demand and if there if benefit over single carrier

- If 200MHz CBW is introduced for certain bands, further check if class C is still needed, by considering the operators’ demand and if there is benefit over single carrier

Following are RAN1-relevant, ok to postpone the discussion if time is limited in RAN4#117

UL-DL decoupling:

NR supports a pairing of a DL carrier and an UL carrier which belong to a same band for FDD/TDD cells and thus, if a DL carrier for a cell is selected by a UE, then an UL carrier associated with the DL carrier is automatically determined. This approach may be difficult to optimize various KPIs because DL and UL carrier to optimize KPIs such as UL coverage and DL throughput may be on the different bands. Note that NR has introduced an additional UL carrier (SUL) on a separate band on top of legacy DL-UL paring in order to address UL coverage issue. Consequently, 6GR should study DL-UL paring with a DL carrier and a UL carrier across different bands to optimize various KPIs. For example, for UL coverage which is one of the most crucial aspects for 6GR design, the higher band DL spectrum can be paired with the low-band UL spectrum which is similar to an SUL carrier in NR.

However, this approach should be thoroughly examined within the RAN4 domain, as achieving complete flexibility is not practically feasible. The UL-DL pairing between two different bands (e.g., UL in band X and DL in band Y) must be conditioned upon UE RF hardware already supporting CA between these two bands (i.e., CA_X-Y). This ensures that the necessary RF front-end components are in place to achieve such pairing.

Proposal #2.7-4: The UL-DL pairing between two different bands (e.g., UL in band X and DL in band Y) must be conditioned upon UE RF hardware already supporting CA between these two bands (i.e., CA_X-Y). This ensures that the necessary RF front-end components are in place to achieve such pairing.

N-Carrier Single cell

N-carrier Single Cell (NSC) can be a candidate for 6GR carrier aggregation framework to improve spectrum utilization and reduce latency by aggregating fragmented spectrum resources into a single cell. Unlike conventional carrier aggregation in NR, NSC enables simplified multi-carrier operation within a single cell, allowing seamless integration of diverse fragmented spectrum. This framework supports cross-carrier functionalities such as scheduling, retransmission, measurement and reporting, fast activation/deactivation, and reducing overhead and improving efficiency and latency. By aggregating multiple carriers into a unified cell structure, NSC maximizes the value of fragmented spectrum, enhances coverage, and supports energy-efficient operations.

Observation #2.5-2: Potential scenarios for N-carrier single cell are identified as follows.

BWP adaption

RAN4 shall hold on and wait for RAN1 conclusion as RAN1 will discuss whether or not to still have BWP or similar concept in 6G.

2.8 Joint UE and BS RF – Tx EVM relaxation

2.8.1 Motivation

In general, the ever-growing data rate demands in mobile communications motivate the exploitation of higher order modulations (HOM) such as 256QAM and 1024QAM. At a given transmitter-receiver distance, the HOM requires larger transmission power than QPSK, 16QAM, and 64QAM due to its reduced distance between constellation points, which results in smaller coverage for HOM. A natural way to improve the HOM coverage would be to increase the transmission power level. However, the possible increase of transmission power level is limited by the maximum power that can be provided by the power amplifier (PA). To maximize the HOM coverage, it is expected that the PA output power should be as close to its saturation level as possible. This would, however, result in severe performance loss due to the nonlinear distortion caused by typical characteristics of radio frequency (RF) components such as PA. It is clear that such non-linear distortion needs to be compensated at either transmit side (TX) with pre-distortion mechanism or receive side (RX) with equalization to ensure block error rate (BLER) or coverage requirements for HOM.

To leverage HOM, it is essential to develop a more effective scheme to compensate the nonlinear distortion. Two viable alternatives are to embrace advanced BS receiver with artificial intelligence (AI) or Non-AI solutions, both of which have been recognized as promising approaches for UE coverage enhancement.

Non-AI algorithms are typically based on predefined models and hence have the well-known problem of “model deficit” in reality. AI solution is good choice to avoid the model deficit problem as it can be used for any types of PA having different non-linearity characteristics and different channel environments. With a EVM relaxation from 3.5% to 6% for 256QAM, AI solution outperforms Non-AI solution 2dB at BLER of 10%.

2.8.2 Proposed study scope for Tx EVM relaxation for 5G-A

The following Note, including a check point, was captured in Rel-20 WID (New WID: UE RF enhancements for NR FR1, Phase 5) [9] during RANP#109 as working guidance. Motivated by the potential commonality of non-linearity modeling of transmission signals and feasibility study steps, which are likely to be agnostic of AI-solution and Non-AI solutions, the group decided to combine the 5G and 6G discussion into one track to avoid the duplicated efforts and inconsistence. Specifically, it was emphasized that the 5G-A solution should be non-AI based and should not impact RAN1. Additionally, it is natural to assume all discussions for 5G-A Non-AI solution are based on 5G waveform and mechanism/assumption. While for 6G, new DMRS waveforms and/or AI-based solution can be considered for both AI solution and non-AI solution.

For the 5G-A non-AI based solution, according to the above Note, 2025 Q4 will primarily focus on the scope discussions. Once the scope is clearly defined, feasibility discussions will commence from 2026 Q1 and continue through Q3.

In alignment with our views expressed in RAN#109[3], for core part study, we propose the study scope as in Proposal# 2.8.1. In our view, the spirit of this study is to increase the possibility of low-end UE transmitting higher power for higher modulation schemes, with advanced compensation solutions implemented at BS side.

I/Q imbalance, which includes phase imbalance and amplitude imbalance, causes distortion of the constellation diagram, making it elliptical or rotated. This results in larger EVM. More specific, in the case of amplitude imbalance, the inconsistence of I and Q paths causes the points on the constellation diagram to deviate from their ideal positions. In the case of phase imbalance, a phase difference between I and Q signals deviating from 90 degree rotates the constellation diagram. Higher modulation such as 64QAM and 256QAM, having smaller spacing between constellation points, demanding higher signal quality.

Observation #2.8-1: The goal of the work is to enable low-end UE to transmit higher power, by leveraging the capabilities of advanced BS receiver.

Observation #2.8-2: The EVM of higher modulation is significantly affected by I/Q imbalance.

Therefore, to emulate the realistic UE behavior, we believe the non-linearity model(s) selected to evaluate BS compensation capabilities and UE MPR reduction limit should also account for other RFIC impairments including at least I/Q imbalance, other factors such as fractional timing error are not precluded. This is reflected in Proposal #2.8.1.

In addition, to simplify the discussion considering the time constrains, it is advisable to prioritize FR1 single CC operation, for PC3 with 1Tx and PC2 with both 1Tx and 2Tx. Further, selecting CP-OFDM as the waveform is recommended which is suitable for both Layer-1 and Layer-2.

For the performance study part, whether BS performance requirements are needed to verify the BS’s capability to compensate for the non-linearity of transmission signals with modulation orders of 64QAM and 256 QAM can be deferred to the WI phase (if any).

Proposal #2.8-1: Adopt the following study scope of 5G-A solution.

Reduction of UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged. - The study is performed based for FR1 single CC operation. - Example bands as n41, n77/n78 - PC3 with 1Tx, PC2 with both 1 TX and 2Tx - Waveform: CP-OFDM

2.8.3 General assumption for both AI and Non-AI solutions

To better organize the discussion between AI and Non-AI solutions, UE RF and demodulation, the following was agreed and captured in [1].

At the initial stage of 6G discussion, while the discussion on the new waveform and new PA modeling for 6G are going, it is advisable to prioritize the study of AI and non-AI solutions based on 5G waveforms and 5G PA models. Once 6G waveforms and 6G PA models are agreed, they can be incorporated into the study.

Proposal #2.8-2: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) and the working steps for feasibility confirmation, are determined in UE RF session, which are common for BS compensation with AI and Non-AI solutions.

To evaluate the effectiveness of proposed techniques related to nonlinearity, PA models in TR 38.803 can be considered for evaluation purpose, e.g., generalized memory-based polynomial (GMP) model, in conjunction with modelling of other non-ideal factors including I/Q imbalance, fractional timing error and so on.

Proposal #2.8-3：Non-linearity model(s) of transmission signals shall account for both PA non-linearity and other RFIC impairment including at least I/Q imbalance

- For evaluation purpose, adopt generalized memory-based polynomial (GMP) model in TR 38.803

As mentioned in clause 2.8.1, high order modulation scheme is sensitive to I/Q imbalance, it is suggested to consider an I/Q imbalance modelling as following. Let st=sIt+jsQt and sLO,It=cos2πf0t denote perfect baseband signal and perfect local oscillator in-phase output, respectively. Then perfect I/Q modulation

rIQ,idealt=sItcos2πf0t+sQtcos2πf0t+π2=sItcos2πf0t-sQtsin2πf0t

Let gI and gQ denote the amplitude impairment of in-phase reference signal and quadrature reference signal, respectively. Furthermore, define phase impairment of in-phase reference signal and quadrature reference signal as θI and θQ. With such distorted reference signals, I/Q modulation output is given by:

rIQt=gI∙sItcos2πf0t+θI-gQ∙sQtsin2πf0t+θQ

Corresponding baseband equivalent signal of the in-phase part and quadrature part of rIQt can be expressed as follows:

rI,bbt=LPFrIQtcos2πf0t=gI∙sItcosθI-gQ∙sQtsinθQ

rQ,bbt=LPF-rIQtsin2πf0t=gI∙sItsinθI+gQ∙sQtcosθQ

Then the baseband equivalent signal of rIQt is given by:

rIQ,bbt=rI,bbt+jrQ,bbt

=gI∙sItcosθI-gQ∙sQtsinθQ+jgI∙sItsinθI+gQ∙sQtcosθQ

=β∙st+γ∙s*t

where β=12gIejθI+gQejθQ and γ=12gIejθI-gQejθQ  represent the coefficients of st and its mirrored component s*t, respectively.

Imbalance between the in-phase and quadrature components of signal output from RF receivers can be cost-effectively compensated rather than improving the analog front-end RF hardware. If gI≈gQ and θI≈θQ after compensation, β≈1 and will be significantly greater than γ. In such case, mirrored component s*t can be ignored and I/Q imbalance can be approximated as a slight amplitude scaling and a slight phase rotation.

In general, it is enough to model I/Q imbalance with the above frequency-independent model. If imperfect RF frequency response in in-phase and quadrature brunches are further considered, the two multiplication operations in rIQ,bbt=β∙st+γ∙s*t should be replaced with convolutions:

rIQ,bbt=βt*st+γt*s*t

where βt and γt are the inverse Fourier transform of the imperfect frequency response in in-phase and quadrature brunches, respectively.

βt=IFT12HIf∙gIejθI+HQf∙gQejθQ

γt=IFT12HIf∙gIejθI-HQf∙gQejθQ

Proposal #2.8-4: For evaluation purpose, the above I/Q imbalance modelling can be considered.

- Wherein amplitude imbalance (gI,gQ) ~ [0, 0.5dB] unified distributed, phase imbalance(θI,θQ) ~ [-5, 5 degree] unified distributed.

Furthermore, considering that operators have a commercial interest in higher uplink modulation schemes such as 1024QAM for 6G, we recommend evaluating 1024QAM for the 6G Tx EVM relaxation study once 1024QAM is confirmed as one of the modulation schemes for 6G

Proposal #2.8-5: Uplink 1024 QAM can be evaluated as part of the 6G Tx EVM relaxation study once it is confirmed as one of the 6G modulation schemes.

- Downlink 1024 QAM MCS table can be reused

2.8.4 Working steps for feasibility study for both AI and Non-AI solutions

To efficient the complex discussion, it is essential to establish clear criteria for confirming feasibility. We propose to agree on the following steps listed in Proposal #2.8-6. Note part of the steps were already agreed in last meeting in AI session and captured in [10].

Proposal #2.8-6: Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

      - Note: UE DPD is not considered in modelling

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity model: SNR-a

    - Case 2: Non-AI based non-linearity (NC) receiver with including non-linearity model: SNR-b

    - Case 3: Conventional receiver without including any non-linearity model: SNR-c

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-b and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-b(c) should be feasible for network deployment

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-b derived based on Case 2 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation and evaluate the Net Gain:

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting/gain (i.e., MPR reduction) for different power class

    - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER)

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

2.8.5 Simulation Assumption for feasibility study for both AI and Non-AI solutions

In order to facilitate the feasibility study, we propose the following simulation assumption with considering the typical PUSCH demodulation assumption in RAN4 for performance evaluation as starting point based on above working steps.

Proposal #2.8-7: For performance evaluation, following simulation assumption with considering the typical PUSCH demodulation assumption in RAN4 can be considered as starting point.

| Parameters | Value |
| --- | --- |
| UE power classs | PC3 with 1Tx PC2 with both 1Tx and 2Tx |
| PA model | Generalized memory-based polynomial (GMP) model in TR 38.803 |
| I/Q imbalance modelling | amplitude imbalance (g_I,g_Q) ~ [0, 0.5dB], phase imbalance(θ_I,θ_Q) ~ [-5, 5 degree] |
| Carrier frequency | 3.5 GHz |
| # of TX layers/users | Both 1/2 can be considered |
| # of RX antennas | 4/8/64 |
| Waveform | CP-OFDM |
| # of PRB | 50 |
| # of symbol per slot | 12 |
| # of DMRS per slot | 2 |
| Modulation | 64 / 256 QAM |
| MCS index | [28] for 64QAM and [28] for 256QAM (Table 5.1.3.1-2 in TS 38.214) |
| Channel model | TDL-A 30ns |
| Subcarrier spacing | 30 kHz |
| Evaluation metric | BLER with 10% |
| Benchmark | Conventional receiver (without non-linearity compensation): e.g., MMSE-IRC Non-AI based non-linearity receiver: e.g., GMP based method |

2.9 Testing and verification

TxD(2Tx-TxD, 4Tx-TxD) and dualPA(introduced for intra-band CA) are signallings related to UE RF architecture. They are widely recognized as being introduced to indicate different RAN4 RF requirements compared to 1LO1PA architecture. However, these IEs could not really aid the NW in optimizing the scheduling, as NW does not consider the UE architecture or minor differences in UE requirements.

It has been argued that these IEs could facilitate conformance test. However, a more straightforward approach would be to rely on UE declarations, which are already utilized in many testing cases. In 6G development, the introduction of such IEs, designed solely to indicate minor requirement differences without impacting network scheduling, should be avoided.

Proposal #2.9-1: The introduction of such IEs, designed solely to indicate minor requirement differences without impacting network scheduling, should be avoided.

2.10 Others

This part is skipped in RAN4#117

- 3. Conclusion

General (TN and NTN integration)

Proposal #2.1-1: Consider a TN-centric 6G baseline, with NTN supported only through minimal, scalable extensions. The focus should remain on preserving TN performance and simplicity, while limiting NTN features to essential aspects such as time/frequency compensation.

Single carrier output power

Observation #2.2-1: Pros and cons for Power class concept.

- Pros:

- Easy for requirements building (i.e., all requirements are built upon PC concept) and conformance test

- Cons:

- Inefficient PA utilization: Some UE may not fully utilize their PA capacity. Futures like “Power boosting” and “increasing higher power limit”, introduced in 5G for higher power transmission, have led to fragmented UE implementation choices, reducing the likelihood for real implementation

Observation #2.2-2: The first thing to address is whether to retain the power class concept, and if retained whether to maintain a 3dB between power classes.

Proposal #2.2-1: It is advisable to keep the power class concept with a 3dB step and its reporting.

Proposal #2.2-2: Further exploit the full usage of PA capacity in the meanwhile avoiding excessive fragmented implementation choices.

Observation #2.2-3: For about 7GHz, to achieve comparable coverage as C-band with same assumption of inter-site deployment, BS can implement more antenna elements.

Proposal #2.2-3: PC3 should still be the default power class. This allows for scalability and performance optimization without affecting low-end UEs.

Proposal #2.2-4: Remove ΔTIB from CA requirements, while retaining it in the single carrier PCMAX_L equation as it is.

Observation #2.2-4: Simplification kind of discussion for ΔTIB should take place in Spectrum thread.

Proposal #2.2-5: More justification should be provided for asymmetric TxD for single carrier, if it is still of interest.

Proposal #2.2-6: Do not specify duty cycle solution for SAR issue.

CA output power

Deprioritized in this meeting per FL’s guidance. This part is skipped in RAN4#117.

Tx requirements

Proposal #2.4-1: For frequencies up to around 7GHz, adopt -35 dB for carrier leakage and -35 dB for image rejection.

Proposal #2.4-2: For MPR evaluation, reuse the 5G assumption, i.e.,

- The LO is placed in the middle of the CBW

- UE DPD is not considered in the transmission modelling

Proposal #2.4-3: Strive to streamline MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors.

- For instance, for one device type, with same power class and mode (i.e., single carrier, or C/NC CA), strive to specify only one set of MPR requirements. This doesnot preclude other architecture implementations (i.e., those not selected as reference architecture for deriving requirements), provided the specified requirements are met.

Rx requirements

Observation #2.5-1: It remains unclear whether the GSM network will be fully decommissioned prior to the finalization of the first 6G specification.

Proposal #2.5-1: All existing Rx requirements remain relevant for respective bands with a CBW of up to 100MHz. Same requirements framework/concept can be applicable to frequencies up to around 7GHz with a CBW of up to 200MHz.

Proposal #2.5-2: For the derivation of REFSENS in the frequency range up to around 7GHz with a CBP of up to 200MHz, the formula "REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv" shall still be adopted.

Proposal #2.5-3: The 2Rx REFSENS shall remain the baseline, with delta values applied to it to derive REFSENS for other Rx count such as 4Rx, 6Rx, 8Rx and 1Rx.

Proposal #2.5-4: For FDD REFSENS, PC3 is the basis, and delta values to higher power class can be added to derive HPUE REFSENS.

- For the same power class, only one set of requirements is defined, regardless of the number of Tx chains.

Proposal #2.5-5: For FDD UL configuration for REFSENS, discuss the feasibility to adopt full RB allocation.

Proposal #2.5-6: Remove ΔRIB from CA requirement, and incorporate equivalent effect as ΔRIB into implementation margin (IM) of single carrier REFSENS.

Proposal #2.5-7: Discuss and establish the 6G MSD framework, before starting MSD OTA testing discussion.

- Alt1: The MSD calculation and specification manner remain unchanged, i.e., accounting for both the conductive part and OTA part, which are then combined

- Alt2: Conductive-only MSD calculation and specification, i.e., the MSD calculation and specification only consider the conductive part. The terminology must be changed (e.g., MSDConductive) to differentiate with 5G.

- Alt3: Separate conductive MSD and OTA MSD specification. E.g., Define MSDConductive and MSDOTA

- Alt4: UE Dynamic MSD reporting [3][6] → Whether the proposal is UE to report the actual MSD for any configurations requested by NW needs further clarification

- Alt5: UE Semi-static MSD reporting, i.e., UE can report the actual MSD for certain specific configurations for each combo if NW requests such configurations

Frequency range between FR1 and FR2-1

Proposal #2.6-1: Discuss and clarify what else to study beyond that has already been studied and documented in 5G TR 38.922 and covered by other 6G threads (such as System parameter and Spectrum).

Spectrum aggregation

Proposal #2.7-1: For FR1, 210 us as the assumption for LO switching time for switching relevant requirements.

Proposal #2.7-2: Check if non-contiguous ULCA still remains attractive in the context of 6G, taking into account the identified drawbacks in 5G and potential technology advancements in 6G.

Proposal #2.7-3: For intra-band contiguous CA, the CA bandwidth class concept is kept. For TDD bands,

- Further check if class B is still needed, by considering the operators’ demand and if there if benefit over single carrier

- If 200MHz CBW is introduced for certain bands, further check if class C is still needed, by considering the operators’ demand and if there is benefit over single carrier

Proposal #2.7-4: The UL-DL pairing between two different bands (e.g., UL in band X and DL in band Y) must be conditioned upon UE RF hardware already supporting CA between these two bands (i.e., CA_X-Y). This ensures that the necessary RF front-end components are in place to achieve such pairing.

Observation #2.5-2: Potential scenarios for N-carrier single cell are identified as follows.

Joint UE and BS RF – Tx EVM relaxation

Observation #2.8-1: The goal of the work is to enable low-end UE to transmit higher power, by leveraging the capabilities of advanced BS receiver.

Observation #2.8-2: The EVM of higher modulation is significantly affected by I/Q imbalance.

Proposal #2.8-1: Adopt the following study scope of 5G-A solution.

Reduction of UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged. - The study is performed based for FR1 single CC operation. - Example bands as n41, n77/n78 - PC3 with 1Tx, PC2 with both 1 TX and 2Tx - Waveform: CP-OFDM

Proposal #2.8-2: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) and the working steps for feasibility confirmation, are determined in UE RF session, which are common for BS compensation with AI and Non-AI solutions.

Proposal #2.8-3: Non-linearity model(s) of transmission signals shall account for both PA non-linearity and other RFIC impairment including at least I/Q imbalance

- For evaluation purpose, adopt generalized memory-based polynomial (GMP) model in TR 38.803

Proposal #2.8-4: For evaluation purpose, the above I/Q imbalance modelling can be considered.

- Wherein amplitude imbalance (gI,gQ) ~ [0, 0.5dB] unified distribution, phase imbalance(θI,θQ) ~ [-5, 5 degree] unified distribution.

Proposal #2.8-5: Uplink 1024 QAM can be evaluated as part of the 6G Tx EVM relaxation study once it is confirmed as one of the 6G modulation schemes.

- Downlink 1024 QAM MCS table can be reused

Proposal #2.8-6: Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

      - Note: UE DPD is not considered in modelling

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity model: SNR-a

    - Case 2: Non-AI based non-linearity (NC) receiver with including non-linearity model: SNR-b

    - Case 3: Conventional receiver without including any non-linearity model: SNR-c

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-b and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-b(c) should be feasible for network deployment

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-b derived based on Case 2 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation and evaluate the Net Gain:

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting/gain (i.e., MPR reduction) for different power class

    - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER)

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

Proposal #2.8-7: For performance evaluation, following simulation assumption with considering the typical PUSCH demodulation assumption in RAN4 can be considered as starting point.

| Parameters | Value |
| --- | --- |
| UE power classs | PC3 with 1Tx PC2 with both 1Tx and 2Tx |
| PA model | Generalized memory-based polynomial (GMP) model in TR 38.803 |
| I/Q imbalance modelling | amplitude imbalance (g_I,g_Q) ~ [0, 0.5dB], phase imbalance(θ_I,θ_Q) ~ [-5, 5 degree] |
| Carrier frequency | 3.5 GHz |
| # of TX layers/users | Both 1/2 can be considered |
| # of RX antennas | 4/8/64 |
| Waveform | CP-OFDM |
| # of PRB | 50 |
| # of symbol per slot | 12 |
| # of DMRS per slot | 2 |
| Modulation | 64 / 256 QAM |
| MCS index | [28] for 64QAM and [28] for 256QAM (Table 5.1.3.1-2 in TS 38.214) |
| Channel model | TDL-A 30ns |
| Subcarrier spacing | 30 kHz |
| Evaluation metric | BLER with 10% |
| Benchmark | Conventional receiver (without non-linearity compensation): e.g., MMSE-IRC Non-AI based non-linearity receiver: e.g., GMP based method |

Testing and verification

Proposal #2.9-1: The introduction of such IEs, designed solely to indicate minor requirement differences without impacting network scheduling, should be avoided.

Others

This part is skipped in RAN4#117

- 4. Reference

[1] R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Feature lead (Qualcomm), RAN4#116bis

[2] R4-2514626, WF on 6G Spectrum, Feature lead (Nokia), RAN4#116bis

[3] R4-2513021, Qualcomm views on 6G General and UE RF, RAN4#116bis

[4] R4-2513145, 6G General RF and UE RF, Nokia, RAN4#116bis

[5] R4-2513117, General and UE RF considerations for 6G, Ericsson, RAN4#116bis

[6] R4-2513349, Initial views on 6G UE RF requirements framework, Apple, RAN4#116bis

[7] R4-2514511, [116bis][104] Feature Lead Summary for 6G Spectrum, Feature lead (Nokia), RAN4#116bis

[8] R4-2514517, Topic summary for [116bis][110] 6G testability and OTA, Feature lead (vivo), RAN4#116bis

[9] RP-252952, New WID: UE RF enhancements for NR FR1 Phase 5, RAN#109

[10] R4-2514637, WF on 6G AI, Feature lead (Samsung), RAN4#116bis

[11] R4-2514509, Topic Summary for [116bis][102] 6G general RF and UE RF, Feature lead (Qualcomm), RAN4#116bis