3GPP TSG-RAN WG4 Meeting #117                                                               R4-2520372

Dallas, USA, 17th – 21st November, 2025

Source:	Samsung

Title:	Discussion on AI/ML general aspects for 6GR

Agenda item:			8.8.1

Document for:	Approval

# 1. Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [1, RP-252912] in RAN#109. Specifically, AI/ML is the indispensable enabler for 6G RAN, for which at least AI/ML framework and new AI/ML use cases are highlighted as key aspects to be studied, with objectives provided as follows [1]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In RAN4#116bis, 6G AI/ML study scope and timeline has been approved [3], in which AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues) are identified as part of general aspects, and it is agreed that in Stage-1 (Q4’25-Q1’26), RAN4 will clarify the study scope for RAN4 AI/ML framework.

In this contribution, we would like to provide our viewpoints on AI/ML general aspects for 6GR including the RAN4 related issues for AI/ML framework.

# 2. RAN4 AI/ML framework

## 2.1 AI/ML framework for RAN4-driven use cases

Based on Rel-18/19 relevant study and work items, the NR AI/ML framework for the air-interface is a compromise between practicality and forward compatibility. The main aspects of the NR AI/ML framework can be summarized with the following points in Table 1:

Table 1 AI/ML framework review of 5G-Adv AI/ML for air interface

|  | Rel-18/19 Discussion Points | Experience learnt for 6G |
| --- | --- | --- |
| AI/ML Framework and Life Cycle Management (LCM) | Data Collection (UE-sided): - Enabled through UE measurement - NW provide configuration and reference signal transmissions - Data collection framework was not complete in Rel-18/19. Data Collection (NW-sided): - Extension of the MDT framework. | Data collection work is mainly RAN2/SA related. |
|  | Applicability Report (UE-sided model) - UE reports its ML capabilities via capability reporting. - NW configurations, based on the reported capabilities, are used to enable inference operations. - UE may additionally report applicability/inapplicability of AI/ML functionalities in response to the NW’s indication. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model management and model-ID-LCM (UE-sided model) - Model management is up to UE implementation. - NW controls the ML usage at the UE via functionality-based LCM. - Functionality-based LCM indirectly supports model selection, activation, deactivation, switching, and fallback Model management and model-ID-LCM (NW-sided model) is up to the NW implementation. | The necessity of model management depends on RAN4 discussion. Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Protection for NW-side implementation (UE-sided model) - NR framework leverages the concept of an Associated ID - Associated ID is to implicitly indicate NW-side additional conditions that may impact performance of a UE-side model. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Performance monitoring (UE-side model) - The monitoring outcome may assist NW decisions on configurations/functionalities switching including fallback to non-AI/ML approaches. - Two performance monitoring approaches: UE-assisted performance monitoring where performance monitoring metric is calculated at the UE and NW-side metric calculation considering ground-truth measurements report from UE as input. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model training: - In the NR offline model training is assumed. Thus, there was no explicit specification support for on-device (online) model training and fine-tuning. | Need to reconsider by taking the solutions for RAN4 use case into account Model retuning and/or online training should also be considered. |
|  | Inter-vendor training collaboration (two-sided model): - Rel-19 identified three inter-vendor training collaboration options, namely: i) fully specified reference model ii) reference encoder parameters sharing-based approach iii.) encoder-related training dataset sharing-based approach. | No need to consider this if only one-sided model is considered for RAN4 use case. If two-sided model is used, Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model Transfer: - In the NR model transfer across vendors, e.g., from NW to the UE, has not been supported. - It is assumed that the UE-part of the models are trained offline by UE-side vendors and delivered to the UEs in a transparent manner. | Work is mainly RAN2/SA related. Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Processing unit and timeline: - The 5G NR acknowledged AI/ML processing may be carried out by a separate hardware from non-AI/ML processes. - AI/ML processing unit (APU) is introduced to quantify the number of concurrently processed AI/ML-based CSI reports. - A separate CSI processing timeline is considered for AI/ML-based CSI reports. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Testing and Performance Requirements - Test cases, core and performance requirements, and generalization requirements are use-case specific - Requirements for LCM - Post-deployment testing (LCM based is prioritized). - Model level testing is not supported. | RAN4 centric work in 5G-A discussion. |

Based on Rel-18/19 experience, the LCM-relevant discussion went slowly, and it would be complicated and inefficiency to design LCM procedures for each studied use case. Therefore, to align with existing RAN1-led use cases (AI-BM, AI-CSI prediction and AI-positioning) and the new 6G use cases driven by different working groups, the unified LCM framework shall still be desired and targeted for.

By examining the Rel-18/19 outcome for LCM framework, RAN4 shall base its LCM study for AI/ML on the NR framework in TR 38.843, covering its terminology, UE/network data collection, applicability reporting, implicit-condition IDs, performance monitoring, APU timelines, and corresponding test/performance requirements.

- For RAN4 use case(s) to be identified in Rel-20, RAN4 should at least consider the following AI/ML framework aspects be applicable or not:

-	UE-side and NW-side data collection if needed

-	Applicability report

-	Associated ID to indicate additional conditions that may not be explicitly configured

-	Performance monitoring

-	Dedicated AI/ML processing unit (APU) and timeline

-	Testing and Performance Requirements

## 2.2 Detailed RAN4 specific issues

### 2.2.1 post-deployment enhancement

Post-deployment enhancement is considered as one RAN4 specific issue, and it is agreed to be considered as part of AI/ML framework in RAN4 discussion [3]:

Issue 2-2: Post-deployment enhancement Agreement: - RAN4 will discuss the potential post-deployment testing enhancements scope in 6G

As identified in Rel-18/19 AI/ML SI/WI, the importance of post-deployment enhancement has been identified if online training/finetuning are utilized, and there are two options identified for post-deployment model testing, i.e., Option 1 (Post-deployment pre-activation functionality/configuration update testing) and Option 2 (Post-deployment post-activation functionality testing based on performance monitoring). Between two options, the Option 2 is prioritized while it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases.

Based on the discussion points provided in [R4-2513072], we see the necessity to firstly identify the difference between the following ways to guarantee AI/ML performance.

There are different ways to guarantee AI/ML performance which has been identified in 5G-A AI/ML work items.

|  | Ways to guarantee AI/ML performance | Time | Where to perform | Comment |
| --- | --- | --- | --- | --- |
| 1 | Pre-deployment conformance test | Before cell-phone shipped into market | Testing lab for conformance testing | Same as existing conformance testing |
| 2 | Post-deployment pre-activation functionality test | After cell-phone shipped into market, but before new AI/ML functionality activated | UE vendors’ lab | Similar as product testing |
| 3 | Post-deployment post-activation functionality testing based on performance monitoring | After new AI/ML functionality activated | In-field practical network environment | No yet introduced in Rel-19. |

If post-deployment pre-activation functionality test is not part of conformance test, the potential specification impact and the necessity to be discussed in RAN4 should be clarified by the proponent firstly.

For the post-deployment post-activation functionality testing based on performance monitoring, it has not been introduced in 5G-A, but the feasibility can be open for discussion in 6G scope for the identified 6G use cases.

- Post-deployment post-activation functionality testing based on performance monitoring can be further discussed in 6G scope for the identified 6G use cases, as per-use case basis.

# 3. Conclusion

In this contribution, we have provided our viewpoints on the general aspects including RAN4-relevant issues of AI/ML framework:

- For RAN4 use case(s) to be identified in Rel-20, RAN4 should at least consider the following AI/ML framework aspects be applicable or not:

-	UE-side and NW-side data collection if needed

-	Applicability report

-	Associated ID to indicate additional conditions that may not be explicitly configured

-	Performance monitoring

-	Dedicated AI/ML processing unit (APU) and timeline

-	Testing and Performance Requirements

- There are different ways to guarantee AI/ML performance which has been identified in 5G-A AI/ML work items.

|  | Ways to guarantee AI/ML performance | Time | Where to perform | Comment |
| --- | --- | --- | --- | --- |
| 1 | Pre-deployment conformance test | Before cell-phone shipped into market | Testing lab for conformance testing | Same as existing conformance testing |
| 2 | Post-deployment pre-activation functionality test | After cell-phone shipped into market, but before new AI/ML functionality activated | UE vendors’ lab | Similar as product testing |
| 3 | Post-deployment post-activation functionality testing based on performance monitoring | After new AI/ML functionality activated | In-field practical network environment | No yet introduced in Rel-19. |

If post-deployment pre-activation functionality test is not part of conformance test, the potential specification impact and the necessity to be discussed in RAN4 should be clarified by the proponent firstly.

- Post-deployment post-activation functionality testing based on performance monitoring can be further discussed in 6G scope for the identified 6G use cases, as per-use case basis.

# 4. Reference

[1] RP-252912 (revision from RP-251881), “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

[2] R4-2514636,	“Topic Summary for [116bis][107] 6G AI”, Samsung, RAN4#116bis.

[3] R4-2514637,	“WF on 6G AI”, Samsung, RAN4#116bis.