3GPP TSG-RAN WG4 Meeting #117                                                               R4-2520375

Dallas, USA, Nov. 17-21, 2025

Source:	Samsung

Title:	Discussion on general aspects and workplan

Agenda item:			7.9.2

Document for:	Discussion

# 1. Introduction

Rel-20 new WID was approved in RAN#108 and was further updated in RAN#109 [1]. Among objectives of this work item, the following are RAN4 relevant work scope on inter-vendor training collaboration for two-sided AI/ML models, and RAN4 requirements of interoperability and RRM:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

The RAN check-point is scheduled in RAN#110 in December. Theoretically, RAN4 should take RAN1 scalability study outcome into account and respond to RAN plenary the feasibility of fully defined/specified reference model (“Direction C”) by repeating evaluation of the model alignment. However, it is impossible due to the limited time before the check-point. Therefore, in the last meeting, following agreements [2] were achieved, including the clarification on the relation between Option 3 models in RAN4 and Direction C models in RAN1 and the conclusion on how to respond to the RAN check-point.

Agreement: Definitions: Fully specified the model is defined as a specified structure with parameter in [TS]. Reference model means UE/NW is not mandated to implement it. Test decoder means TE vendor needs to implement it for testing purpose Direct C in RAN1: fully specified reference encoder and decoder Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder) Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE. Relation between RAN1 and RAN4: • The test decoder of option 3 is the same as the reference decoder of direction C - Test decoder of option 3 will be specified by RAN4 - If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed. • The reference encoder is the same for both option 3 and direction C • The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1. - Reference encoder of option 3 will be specified in RAN4. How to respond to the RAN checkpoint • Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. • Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. • FFS on detailed steps to develop the model • RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint. Agreement Which SI option to use for the WI Based on the current WID RP-251870, RAN4 only defines the requirements for the encoder. RAN4 will discuss option 3 only this WI. Test decoder and reference encoder backbone The test decoder and the reference encoder in Option 3 will have the same backbone. - It is FFS if a single test decoder will be specified. - The test decoder and the reference encoder can have different structures, e.g. # of layers Dataset for model evaluation RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations. - Details are FFS including the channel model(s). Scalability Unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1. Model backbone Transformer is assumed as the baseline Only Quantized parameters are specified Steps to develop the test decoder and reference encoder: - Step 1: Create dataset - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any - Step 1-3 Companies create datasets - Step 1-4 Identify which company datasets to include in the mixed dataset(s) - Step 1-5 Create new dataset(s) - Step 2: Align on the model backbone, model hyperparameters and the scalability solution. - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution. - This can be in parallel with step 1. - Transformer is agreed as baseline for backbone - Same backbone for test decoder and reference encoder - Step 2-2: Preliminary selection of structures, taking into account complexity - [Step 2-2a Align on training parameters] - Step 2-3: Agree on metrics and steps for evaluation - Step 2-4: Check on performance alignment -> see simulation results from contributing companies. - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models. - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4. - Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion. Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies - Step 3-1 Decide how to select from the trained model(s) from each company - Step 3-2: Freeze model(s) - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

In this contribution, we aim to discuss the general aspects and workplan for CSI feedback enhancement.

# 2. Discussion

## 2.1 RAN4 response for RAN#110 check-point

In Rel-19 RAN4 had fully studied and confirmed the feasibility for Option 3 and Option 4. Meanwhile, RAN1 had verified the feasibility of model scalability solutions. In Rel-20, RAN4 is to prove the feasibility of scalable reference encoder and scalable test decoder. From the aspect of AI model training, it is the combination of autoencoder model and some tricks for generalization on varying input sizes and output sizes. By following the same methodology developed by RAN4 in Rel-19 and reusing the same scalability solutions from RAN1, RAN4 can complete the evaluation for Option 3 with scalability taken into account.

The same evaluation process will be conducted to verify the feasibility, which includes determining the simulation assumptions, generating the mixed dataset, aligning the model hyperparameters and scalability solutions, and selecting and specifying the reference encoder and test decoder. However, before the check-point in December, insufficient time remains to proceed the whole verification process. As described in the above agreements, RAN4 will response that Option 3 is feasible for RAN4 to complete.

Proposal 1: RAN4 responds the conclusion that it is feasible to create scalable models for Option 3, and “FFS on detailed steps to develop the model” can be left for RAN4 discussion and no need to include in the LS to RAN.

## 2.2 Discussion on other general issues

It was agreed in the last meeting that only quantized parameters are specified. It should be noted that the quantized parameters, regardless of in INT8, BF16 or FP16 format, will bring in degradation of intermediate performance, compared to float32 format parameters. If quantized parameters are specified for consideration of practical deployment, the degradation must be considered in advance. In general, there are two types of model quantization methods to obtain quantized parameters, Quantization-Aware Training (QAT) and Post-Training Quantization (PTQ). Different companies may use different methods with different detailed configurations.

Observation 1: Quantized parameters, regardless of in INT8, BF16 or FP16 format, will bring in degradation of intermediate performance, compared to float32 format parameters.

Observation 2: There are two types of model quantization methods to obtain quantized parameters, Quantization-Aware Training (QAT) and Post-Training Quantization (PTQ). They both will influence the model alignment among companies.

Proposal 2: RAN4 should discuss the parameter (/model) quantization method, in parallel with discussing the model structure and scalability solutions.

Proposal 3: RAN4 should study the intermediate performance degradation from the parameter (/model) quantization method and determine the detailed parameter (/model) quantization method.

Proposal 4: RAN4 should conduct the alignment of models with quantized parameters.

In Rel-20, RAN4 not only works on aligning scalable models, but also performs evaluation on throughput gains with AI/ML models in link-level simulation. It is suggested to use one single set of link-level simulation parameters to both generate training/testing dataset and evaluate the throughput gains.

Observation 3: The specified reference encoder and test decoder will be embedded into LLS to define the performance requirement of the encoder.

Proposal 5: RAN4 takes the typical cases with acceptable final throughput gains into consideration when discussing the assumptions for the dataset generation.

# 3. Conclusion

In this contribution, we provided our viewpoints on the general aspects and workplan of CSI feedback enhancement, accordingly the following observations and proposals are obtained:

Proposal 1: RAN4 responds the conclusion that it is feasible to create scalable models for Option 3, and “FFS on detailed steps to develop the model” can be left for RAN4 discussion and no need to include in the LS to RAN.

Observation 1: Quantized parameters, regardless of in INT8, BF16 or FP16 format, will bring in degradation of intermediate performance, compared to float32 format parameters.

Observation 2: There are two types of model quantization methods to obtain quantized parameters, Quantization-Aware Training (QAT) and Post-Training Quantization (PTQ). They both will influence the model alignment among companies.

Proposal 2: RAN4 should discuss the parameter (/model) quantization method, in parallel with discussing the model structure and scalability solutions.

Proposal 3: RAN4 should study the intermediate performance degradation from the parameter (/model) quantization method and determine the detailed parameter (/model) quantization method.

Proposal 4: RAN4 should conduct the alignment of models with quantized parameters.

Observation 3: The specified reference encoder and test decoder will be embedded into LLS to define the performance requirement of the encoder.

Proposal 5: RAN4 takes the typical cases with acceptable final throughput gains into consideration when discussing the assumptions for the dataset generation.

# 4. Reference

[1] RP-252445 (revised from RP-251870), “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#109.

[2] R4-2514614, “WF on R20 NR_AIML_air_Ph2”, Ericsson, RAN4#116bis.