3GPP TSG RAN4 Meeting #117 							R4-2520379

Dallas, USA, Nov. 17th- 21st November 2025

Agenda Item:		5.10.3
Source: 			SES
Title: 			Ku Band Performance Part: Simulation Results of Downlink

and Uplink Channels

Document for: 		Discussion and Decision

# Introduction

To progress the Ku Band Performance work, as agreed in the Way forward […] in RAN4 #116-bis, we present simulation results of several Uplink and Downlink channels. The purpose of these simulations is to assess their demodulation performances.

The channels that have been simulated are as follow:

- PDSCH

- PDCCH

- PUSCH

- PUCCH

The PDSCH, PDCCH, PUSCH, and PUCCH simulations assume imperfect receiver synchronization including channel estimation, equalization, demodulation, and decoding. The results do not include hardware receiver implementation.

To progress the simulation work, two new Reference Channel Models were prepared for 15 and 30 kHz Sub-Carrier Spacing.

# Ku Band UE Demodulation

## PDSCH

For Ku Band , PDSCH minimum demodulation requirements are defined.  The key parameters specified in Table 1 are valid for all PDSCH demodulation simulation.

Table 1: Common Test Parameters for PDSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel Bandwidth | MHz | 50 |
| Subcarrier Spacing | kHz | 15 and 30 |
| Resource Block |  | 270 and 133 |
| Cyclic prefix |  | Normal |
| Modulation |  | QPSK |
| Coding Rate |  | 307/1024 ~ 0.3 |
| DMRS Type A Position |  | 2 |
| DMRS Length |  | 1 |
| DMRS Additional Position |  | 0 |
| DMRS Configuration Type |  | 1 |
| Num CDM Groups without Data |  | 1 |
| PTRS configuration |  | None |
| HARQ feedback |  | disabled |
| Propagation condition |  | NTN-TDLC5-1200 |
| Number of transmit antenna |  | 1 |
| Number of receive antenna |  | 1 |
| Throughput | Mbps | 25.104 |

Figure 1 illustrates throughput versus SNR for PDSCH with 15kHz and 30kHz subcarrier spacing (SCS).  At 70% of the maximum throughput the required Es/No (channel SNR) are about -1.8 dB for both 15kHz and 30kHz SCS.

Figure 1: Throughput versus SNR for PDSCH 15kHz and 30kHz SCS.

Table 2 summarizes the key assumptions and results for PDSCH demodulation requirements for 15kHz and 30kHz SCS.

Table 2: Minimum PDSCH Demodulation Requirements Reference

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | Es/No (dB) |
| 1-1 | 50 / 15 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | -1.82 |
| 1-2 | 50 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | -1.8 |
| Note1: The Maximum throughput is based upon HARQ feedback disabled. |  |  |  |  |  |  |

Observation 1: For the PDSCH, 70% throughputs were observed at -1.82 dB and -1.8 dB  Es/No (dB) for 15 and 30 SCS, respectively

## PDCCH

For Ku Band, PDCCH minimum demodulation requirements are defined.  The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).  The key parameters specified in Table 3 are valid for all PDCCH demodulation simulation.

Table 3: Common Test Parameters for PDCCH

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel Bandwidth | MHz | 50 |
| Subcarrier Spacing | kHz | 15 and 30 |
| Resource Block |  | 133 |
| Polar modulation |  | QPSK |
| Number DCI message bits |  | 48, 64, 140 |
| Polar coding rate |  | 48/864 (0.056), 64/864 (0.074), 140/864 (0.162) |
| Corset duration |  | 1 |
| Corset CCE REG Mapping |  | interleaved |
| Corset REG Bundle Size |  | 6 |
| Corset Interleaver Size |  | 2 |
| Aggregation Level |  | 8 |
| Propagation condition |  | NTN-TDLC5-1200 |
| Number of transmit antenna |  | 1 |
| Number of receive antenna |  | 1 |

Figure 2 illustrate PDCCH Probability of Miss Detection (P miss) versus SNR (Es/No) for DCI bit length of 48, 64, and 140 bits, and subcarrier spacing of 15 KHz respectively. For 1% probability of miss-detection of the Downlink Scheduling Grant, the required Es/No for DCI bits 48, 64, and 140 bits are 0.7, 1.6, 4.5 dB, respectively.

Figure 2:  Pmiss versus SNR for PDCCH for DCI bit of 48, 64, and 140 (15kHz SCS).

Figure 3 illustrate PDCCH Probability of Miss Detection (P miss) versus SNR (Es/No) for DCI bit length of 48, 64, and 140 bits, and subcarrier spacing of 30 KHz respectively. For 1% probability of miss-detection of the Downlink Scheduling Grant, the required Es/No for DCI bits 48, 64, and 140 bits are 0.5, 1.4, 4.2 dB, respectively.

Figure 3: Pmiss versus SNR for PDCCH for DCI bit of 48, 64, and 140 (30kHz SCS).

Table 4 summarizes the key assumptions and results for PDCCH demodulation requirements for various DCI bit length.

Table 4: Minimum PDCCH Demodulation Requirements Reference

| Test number | Bandwidth (MHz) | CORESET RB | DCI bits | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | Es/No (dB) 15kHz/30kHz |
| 1-1 | 50 | 48 | 48 | 1 | 8 | NTN-TDLC5-1200 | 1x1 | 1 | 0.7 / 0.5 |
| 1-2 | 50 | 48 | 64 | 1 | 8 | NTN-TDLC5-1200 | 1x1 | 1 | 1.6 / 1.4 |
| 1-3 | 50 | 48 | 140 | 1 | 8 | NTN-TDLC5-1200 | 1x1 | 1 | 4.5 / 4.2 |

Observation 2: For the PDCCH with 15kHz SCS, 1% miss-detection at Es/No of 0.7, 1.6 and 4.5 dB for DCI bits of 48, 64 and 140 bits, respectively.

Observation 3: For the PDCCH with 30kHz SCS, 1% miss-detection at Es/No of 0.5, 1.4 and 4.2 dB for DCI bits of 48, 64 and 140 bits, respectively.

# Ku SAN Demodulation

## PUSCH

For Ku Band , PUSCH demodulation requirements are defined.  The key parameters are specified in Table 5 for all PUSCH demodulation simulation.

Table 5: Common Test Parameters for PUSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel Bandwidth | MHz | 10 |
| Subcarrier Spacing | kHz | 15 and 30 |
| Resource Block |  | 52 and 24 |
| Cyclic prefix |  | Normal |
| Modulation |  | QPSK |
| PUSCH Mapping Type |  | A (slot-wise) |
| PUSCH transform precoding |  | False |
| PUSCH transmission scheme |  | nonCodebook |
| Coding Rate |  | 193/1024 ~ 0.19 |
| DMRS Type A Position |  | 2 |
| DMRS Length |  | 1 |
| DMRS Additional Position |  | 1 |
| DMRS Configuration Type |  | 1 |
| Num CDM Groups without Data |  | 2 |
| PTRS configuration |  | None |
| HARQ feedback |  | disabled |
| Propagation condition |  | NTN-TDLC5-1200 |
| Number of transmit antenna |  | 1 |
| Number of receive antenna |  | 1 |
| Throughput | Mbps | 2.856 and 2.64 |

4 illustrates PUSCH throughput versus Es/No for 15kHz and 30kHz SCS.  At 70% of the maximum throughput the required Es/No is about -3.7 and -3.6 dB for 15kHz and 30kHz SCS, respectively.

Figure 4: Throughput versus SNR for PUSCH 15kHz and 30kHz SCS.

Table 6 summarizes the key assumptions and results for PUSCH demodulation requirements for 15kHz and 30kHz SCS.

Table 6: Minimum PUSCH Demodulation Requirements Reference

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | Es/No (dB) |
| 1-1 | 10 / 15 | QPSK, 0.19 | NTN-TDLC5-1200 | 1x1 | 70 | -3.7 |
| 1-2 | 10 / 30 | QPSK, 0.19 | NTN-TDLC5-1200 | 1x1 | 70 | -3.6 |
| Note1: The Maximum throughput is based upon HARQ feedback disabled. |  |  |  |  |  |  |

Observation 4: For the PUSCH, 70% throughputs were observed at -3.7 dB and -3.8  dB  Es/No (dB) for 15 and 30 SCS, respectively.

## PUCCH

For Ku Band, PUCCH demodulation requirements are defined.  The performance is measured by the required SNR at UCI block error probability not exceeding 1%.  The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent.  The key assumptions are that PUCCH format 3 is assumed for use case of transmitting large UCI payloads over a longer duration, providing robustness and reliability.  The key parameters specified in Table 7 are valid for all PUCCH demodulation simulation.

Table 7: Common Test Parameters for PUCCH format 3

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel Bandwidth | MHz | 10 |
| Subcarrier Spacing | kHz | 15 and 30 |
| Resource Block |  | 52 |
| Polar modulation |  | QPSK |
| Cyclic prefix |  | Normal |
| Intra-slot Frequency Hopping |  | enabled |
| Group and sequence hopping |  | neither |
| Additional DMRS |  | 0 or 1 |
| Number UCI Bits |  | 16 |
| Coding Rate |  | 16/240 ~ 0.067 |
| Number of PRB |  | 1 |
| Number of symbols |  | 14 |
| First symbol |  | 0 |
| First PRB prior to frequency hopping |  | 0 |
| First PRB after frequency hopping |  | The largest PRB index – (Number of PRBs – 1) |
| UCI BLER probability |  | < 1% |
| Propagation condition |  | NTN-TDLC5-1200 |
| Number of transmit antenna |  | 1 |
| Number of receive antenna |  | 1 |

Figure 4 illustrates PUCCH BLER versus Es/No for UCI with and without additional DMRS for 15kHz SCS.  Assuming 1% UCI probability of miss-detection on the uplink, the PUCCH Es/No is 1.3 dB and 0.6 dB for no additional DMRS and additional DMRS, respectively.

Figure 4: PUCCH BLER versus SNR with and without Additional DMRS for 15kHz SCS.

Figure 5 illustrates PUCCH BLER versus Es/No for UCI with and without additional DMRS for 30kHz SCS.  Assuming 1% UCI probability of miss-detection on the uplink, the PUCCH Es/No is 1.3 dB and 0.9 dB for no additional DMRS and additional DMRS, respectively.

Figure 5: PUCCH BLER versus SNR with and without Additional DMRS for 30kHz SCS.

Table 8 summarizes the key assumptions and results for PUCCH demodulation requirements with and without additional DMRS.

Table 8: Required SNR for PUCCH Format 3

| Test number | Bandwidth (MHz) | Cyclic Prefix | Additional DMRS Configuration | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | UCI BLER (%) | Es/No (dB) 15kHz / 30kHz |
| 1-1 | 10 | Normal | No additional DMRS | NTN-TDLC5-1200 | 1x1 | 1 | 1.3 / 1.3 |
| 1-2 | 10 | Normal | Additional DMRS | NTN-TDLC5-1200 | 1x1 | 1 | 0.6 / 0.9 |

Observation 5: For the PUCCH, assuming 15kHz SCS 1% miss-detection at Es/No of 1.3 and 0.6 dB for no additional DRMS and additional DRMS, respectively

Observation 6: For the PUCCH, assuming 30kHz SCS 1% miss-detection at Es/No of 1.3 and 0.9 dB for no additional DRMS and additional DRMS, respectively

# Summary and Conclusion

From our simulations we have observed the following:

Observation 1: For the PDSCH, 70% throughputs were observed at -1.82 dB and -1.8 dB  Es/No (dB) for 15 and 30 SCS, respectively

Observation 2: For the PDCCH with 15kHz SCS, 1% miss-detection at Es/No of 0.7, 1.6 and 4.5 dB for DCI bits of 48, 64 and 140 bits, respectively.

Observation 3: For the PDCCH with 30kHz SCS, 1% miss-detection at Es/No of 0.5, 1.4 and 4.2 dB for DCI bits of 48, 64 and 140 bits, respectively.

Observation 3: For the PDCCH with 30kHz SCS, 1% miss-detection at Es/No of 0.5, 1.4 and 4.2 dB for DCI bits of 48, 64 and 140 bits, respectively.

Observation 4: For the PUSCH, 70% throughputs were observed at -3.7 dB and -3.8 dB  Es/No (dB) for 15 and 30 SCS, respectively.

Observation 5: For the PUCCH, assuming 15kHz SCS 1% miss-detection at Es/No of 1.3 and 0.6 dB for no additional DRMS and additional DRMS, respectively

Observation 6: For the PUCCH, assuming 30kHz SCS 1% miss-detection at Es/No of 1.3 and 0.9 dB for no additional DRMS and additional DRMS, respectively

Proposal: : For the Ku Band Performance part, we propose to proceed with the results presented in this contribution.

Please note:

- The presented results do not include hardware receiver implementation.

- The results include practical channel estimations

# References

- R42515160