3GPP TSG-RAN WG4 Meeting #117	R4-2520396

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	On remaining issues for performance part for LP-WUR

Agenda Item:	6.16.2

Document for:	Discussion

Introduction

In [1], the time budget for LP-WUR WI [2] performance part is included and the TU plan is copied below for reference.

LP-WUR performance part work has already started at RAN4 115 meeting. In this contribution, we provide our considerations on the LP-WUR performance part based on the topic summary and WF at previous RAN4 meeting [3],[4].

Discussion

At RAN4 116bis meeting, the following agreements were achieved:

Issue 3-1-1 General aspects

Ad-hoc meeting agreement:

RAN4 agree to define test cases for LP-WUR WI

Agreement:

Do not introduce a test mode from LP-WUR RRM performance test perspective

Issue 3-2-2: On OOK based LR and OFDB based LR for test cases where LR results are used

Agreement:

RAN4 defines separate test cases for LR measure PSS/SSS only and LR measure LP-SS only in the test case

FFS on use 320ms or 160ms for LP-SS periodicity

Issue 3-2-2-1: On OOK based LR for test cases where LR results are used

Ad-hoc meeting agreement:

For the OOK based LR,

Test cases should cover at least M = 1 and M > 1 scenarios

The test cases should not be duplicated or the test case number should not be increased for verifying different M value purpose.

Note: The {M,L} = {1,6} should not be used in test case for LR

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3

Ad hoc meeting agreement:

RAN4 to define at least test cases for exit conditions.

FFS on whether to define test cases for evaluation of entry conditions.

Issue 3-2-3-1: Detail on cases to be verified for entry/exit conditions for case 1/case 3

Agreement:

- Define test cases for evaluation of exit conditions for the following cases.

  - UE exit Case 1 to legacy

    - TC1: UE exit Case 1 to legacy where only LP-SS signal is used in the test case for OOK based LR

  - UE exit Case 3 to legacy

    - TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR

    - TC3: UE exit Case 3 to legacy where only LP-SS signal is used in the test case for OOK based LR

- The above agreement can be revisited if there no feasible way for test case design.

Issue 3-2-4: Test case design for RRM relaxation

Agreement:

- For RRM relaxation, define the following tests

  - intra-frequency cell re-selection

Issue 3-2-5: Test case design for offloading case

Agreement:

Cell reselection is triggered immediately at the timepoint X when exit from offloading (case 1) to legacy is triggered plus MR wake up period.

Cell reselection is triggered immediately at the timepoint X when exit from RRM relaxation (case 3) to legacy is triggered [plus MR wake up period].

Issue 3-2-9: On FR1/FR2 for test case

Agreement: Introduce one test case for FR2:

  - UE exit Case 3 to legacy

    - TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR

In this contribution, the following topics are further discussed based on [3].

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3

- Proposals

- Entry/exit condition evaluation verification

  - Option 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions (China Telecom Apple vivo Huawei QC)

  - Option 2: Define test case for entry/exit scenario (oppo ZTE Nokia)

  - Option 3: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).(MTK)

At previous meeting, it was agreed that “RAN4 to define at least test cases for exit conditions”, whether to define test case for entry conditions are FFS.

Regarding “entry” cases, when a UE satisfies the entry conditions for LP-WUS monitoring, offloading or RRM relaxation, that UE is not mandated to perform corresponding actions hence verifying UE behaviour through particular actions after “entry conditions” are satisfied is not feasible. At previous meeting there was recommendation during the meeting to verify a scenario where a UE should not enter a mode when corresponding conditions are not satisfied instead of defining test cast to verify entry conditions. In detail,

- Define a test case to verity a UE should not enter LP-WUS monitoring/offloading/RRM relaxation mode however enter into one of the LP-WUS monitoring, offloading and RRM relaxation mode

  - Note: the UE enters into one of the 3 modes should fail the test.

From performance perspective the former recommendation could be a possible way instead of defining test case for entry conditions.

Among the three modes, LP-WUS monitoring, offloading and RRM relaxation, it may be difficult to use LP-WUS monitoring since the following procedure, i.e., when a UE detects a paging and send feedback to a TE, does not have a precedent case. If using offloading or RRM relaxation mode in the test case, the test case design can utilize a cell reselection process similar to Issue 3-2-5. However, for such test cases to verify a UE does not enter into offloading or RRM relaxation mode when offloading or RRM relaxation conditions are not met, it could be argued that such functionality has already been verified by TC1, TC2/TC3. To our understanding, a test case to verify a UE at the legacy state does not enter into offloading/RRM relaxation mode when corresponding conditions are not met is still different comparing with a test case to verify a UE leaves offloading/RRM relaxation mode to legacy mode when corresponding conditions are not met. From the test case coverage point of view, such a test case is preferred to have. Between the offloading mode and RRM relaxation mode, offloading mode could be used since TC2 and TC3 are for RRM relaxation mode and one extra test case for offloading mode is reasonable.

Proposal 1: Define a test case to verity a UE does not enter into offloading mode when corresponding offloading conditions are not met. In the test case, both MR and LR threshold are set.

  - Note: the UE enters into offloading mode should fail the test.

Issue 3-2-3-1: Detail on cases to be verified for entry/exit conditions for case 1/case 3

The following test cases are agreed to be defined

  - TC1: UE exit Case 1 to legacy where only LP-SS signal is used in the test case for OOK based LR

  - TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR

  - TC3: UE exit Case 3 to legacy where only LP-SS signal is used in the test case for OOK based LR

It was also agreed that “Test cases should cover at least M = 1 and M > 1 scenarios and {M, L} = {1, 6} should not be used”. It is suggested for TC1, {M, L} ={1, 8} could be used and for TC3 {M, L} {4, 32} could be used.

Proposal 2: It is suggested for TC1, {M, L} ={1, 8} could be used and for TC3 {4, 32} could be used.

Issue 3-2-6: Test case design for LP-WUS monitoring

- Proposals

  - P1: RAN4 to define one test case to verify the MR wake up delay when UE receives LP-WUS (CATT vivo ZTE Huawei Nokia)

  - P1-2: No need to define test case for LP-WUR monitoring mode if the MR wake up delay verification can be covered by other test cases (vivo)

  - P2: Discuss whether to introduce a test-case to verify the MR wake-up delay after receiving the LP-WUS (QC)

  - P3: No need to discuss LP-WUS missed detection or false-alarm rate performance for RRM. (QC)

Regarding test case design for LP-WUS monitoring, one issue is how to verify the MR wake up delay in LP_WUS monitoring, as mentioned before, to our understanding it may be difficult to use LP-WUS monitoring since the following procedure, i.e., when a UE detects a paging and send feedback to a TE, does not have a precedent case. In addition the MR wake up delay could be covered by TC1 therefore the necessity is also not strong.

Proposal 3: For the test case to verify the MR wake up delay in LP_WUS monitoring, the necessity and the detail test case design should be discussed before any decision.

Issue 3-2-7: Test case design for higher priority frequency layer search

- Proposals

  - Option 1: Introduce test case for high priority frequency search in case 1. (oppo Apple ZTE Nokia Huawei)

    - Option 1-1: The legacy test procedure on high priority frequency measurement shall be reused as starting point(ZTE)

  - Option 2: Suggest do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode. (vivo)

Recommendations:

For the UE stays in case 1, the UE will have higher priority frequency layer search with the rate of 1 hour per frequency layer. Considering the measurement period is quite long, it is suggested not to introduce the higher priority frequency layer search test case when UE is in the offloading state. In addition, in the Rel-16, test cases had not been defined for case 3, i.e., when a UE fulfill both low mobility and not at the cell edge criteria, as the following conclusion in [5]:

- No test cases for Scenario 3 (both low mobility and not-at-cell-edge)

Hence we have the following proposal:

Proposal 4: Do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode.

Issue 3-2-x: Observations for test case design

In addition we have some observations for the test case design:

In Rel-16 UE power saving, for the not-at-cell-edge threshold setting, the following agreement are available [6]:

SSearchThresholdP  is configured to Srxlev – X (dB)

Agreement: X = 6dB for FR1, X = 7.5dB for FR2

Proposal 5：For the Rel-19 RRM relaxation and offloading case, X = 6dB for FR1, X = 7.5dB for FR2 in Rel-16 can be reused for test case for RRM relaxation and value used for test case for offloading case can be FFS.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: Define a test case to verity a UE does not enter into offloading mode when corresponding offloading conditions are not met. In the test case, both MR and LR threshold are set.

  - Note: the UE enters into offloading mode should fail the test.

Proposal 2: It is suggested for TC1, {M, L} ={1, 8} could be used and for TC3 {4, 32} could be used.

Proposal 3: For the test case to verify the MR wake up delay in LP_WUS monitoring, the necessity and the detail test case design should be discussed before any decision.

Proposal 4: Do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode.

Proposal 5：For the Rel-19 RRM relaxation and offloading case, X = 6dB for FR1, X = 7.5dB for FR2 in Rel-16 can be reused for test case for RRM relaxation and value used for test case for offloading case can be FFS.

- References

[1] RP-250821, TU budget after RP107, RAN#107

[2] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[3] R4-2514811, WF on NR_LPWUS_RRM, RAN4 116bis, vivo

[4] R4-2513516,	Topic summary for [116bis][210] NR_LPWUS_RRM, RAN4 116bis, vivo

[5] R4-2012124, WF on RRM test cases for power saving, CATT, RAN4 96e.

[6] RAN4 97e meeting report