3GPP TSG-RAN WG4 Meeting #117	R4-2520399

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on RRM for (e)RedCap UE enhancement

Agenda Item:	7.4.3.2

Document for:	Discussion

Introduction

In RAN Plenary #109 meeting, WI for “NR Radio Resource Management (RRM) Phase 6” was agreed [1].

The objectives of WI are duplicated as below:

- Specify the following requirements for (e)RedCap UEs [RAN4]

  - Relaxed RLM/BFD RRM requirements

    - Use R17 RLM/BFD relaxation for legacy UE as a baseline

  - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)

    - RF requirements, considering example bands: n100, n106, n26, n28 and n85

    - RRM requirements including relaxed RLM/BFD RRM requirements

    - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline

  - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

- For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4]

  - Note 3: No new PDSCH demodulation requirement shall be defined.

In this contribution considerations on the RRM part for this WI is provided.

Discussion

At RAN4 116bis meeting, the following agreements are available:

Issue 2-1-1: Scope for LT5 (e)RedCap UE

Agreement:

- For (e)RedCap UE supporting LT5:

  - CSI-RS based measurement is out of scope.

  - RAN4 only defines the requirements for PCell.

  - RAN4 only defines the requirements for NR FR1.

  - Introduce support for only 15 PRB transmission bandwidth configuration and 12 PRB SSB

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE

Agreement:

- The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes.

  - Handover

  - RLM/BFD

  - Intra-frequency/Inter-frequency measurement

- Further check for HD-FDD

Issue 2-2-1: Scope of relaxed RLM/BFD for (e)RedCap UE

Agreement:

RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms.

Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE

Agreement:

- Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

Sub-topic 2-3: Simulation assumption for LT5 1Rx (e)RedCap

Agreement:

Simulation Assumptions for SSB index reading and MIB reading Evaluation

| Parameter | Unit | Value |
| --- | --- | --- |
| - Number of PRBs for PBCH |  | 12 PRBs |
| - Carrier frequency | - GHz | - 900MHz |
| - Subcarrier spacing | - kHz | - 15 kHz |
| - Number of Tx antennas | - - | - 1 - 2 |
| - Number of Rx antennas | - - | - 1 |
| - DMRS | - - | - PBCH DMRS |
| - Other assumptions |  | - Tx BW and SSB puncturing are known at the Rx side |
| - CP Length | - - | - Normal |
| - Number of transmitted SS block within a SS burst set period (K) | - - | - 1 |
| - SS burst set periodicity | - ms | - 20 |
| - Frequency Offset relative to UE frequency reference | - Hz | - 0 |
| - PBCH symbols within the SS block |  | - PSS-PBCH-SSS-PBCH |
| - Data and Control Power offset with respect to PSS and SSS | - dB | - Baseline 0 |
| - PBCH power offset with respect to PBCH-DMRS | - dB | - 0 |
| - PBCH-DMRS power offset with respect to PSS and SSS | - dB | - 0 |
| - PSS and SSS sequences | - - | - No changes expected |
| - PBCH-DMRS sequences | - - | - No changes expected |
| - PBCH-DMRS RE positions within the PBCH resource | - - | - No changes expected except for puncturing impact |
| - PBCH Channel coding |  | - No changes expected to actual Channel coding - (Polar code with 512 length and 24bit CRC) |
| - PBCH Modulation | - - | - QPSK |
| - PBCH Payload (including the CRC) | - bits | - 56bit (CRC 24bit) |
| - PBCH SNR | - dB | - -10 : 0 dB, with 1 dB spacing |
| - Propagation Condition / Channel models | - - | - For 3 km/h UE speed - TDL-C 300ns - Additional scenarios can be considered |
| - Detection Method |  | - Baseline: One shot detection (i.e. no combination for different PBCHs) - FFS whether to remove Soft Combining or keep Soft Combining as Optional |
| - Metrics |  | - TBA |
| - NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |

Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE

Agreement:

For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

Issue 2-2-4: Entry and exit criteria, singling indication, applicability rules for 2Rx

Agreement:

Reuse R17 RLM/BFD Entry and exit criteria from legacy UE as a baseline

Sub-topic 2-1: RRM requirements for less than 5 MHz channel bandwidth(LT5)

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE

Agreement:

- The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes.

  - Handover

  - RLM/BFD

  - Intra-frequency/Inter-frequency measurement

- Further check for HD-FDD

For handover requirement for HD-FDD, the following specification is available:

For RedCap UE with HD-FDD, the handover requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

The above should not impacted by the bandwidth of Redcap and for HD-FDD, the legacy principle could be reused when specifying corresponding requirements.

Proposal 1: For HD-FDD for LT5 Redcap, the legacy principle of Redcap for HD-FDD could be reused when specifying corresponding requirements.

Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE

Agreement:

For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

To our understanding it is not usual to consider terminals with different capabilities, here is the number of antenna, to have different coverage. For Redcap UE with 1 antenna, the possible coverage issue due to the 3dB gain loss could be solved by various techniques, for example repetition. In addition, the bottleneck caused by 1Rx and 2Rx may be more at the uplink instead of downlink.

Proposal 2: Suggest to target on equivalence coverage between LT5 1Rx and 2Rx RedCap UEs.

Sub-topic 2-2: RRM requirements for relaxed RLM/BFD

Issue 2-2-3: Relaxed RLM/BFD for 1Rx (e)RedCap UE

- Proposals

  - Option 1: CATT, Ericsson, China Telecom, OPPO

    - RAN4 need to discuss the relaxation for 1Rx (e)RedCap UE.

      - For RLM, RAN4 to consider the relaxations for TEvaluate_out_SSB/CSI-RS

      - For BFD, RAN4 to consider the relaxations for TEvaluate_BFD_SSB/CSI-RS

    - Option 1a (CATT): For 1Rx (e)RedCap UE, the legacy relaxation due to reduction in the number of UE Rx branches need to be taken into consideration together to avoid relaxing too much for the above parameters.

  - Option 2: Qualcomm, CMCC, vivo, Samsung

    - RAN4 should discuss whether the relaxation with 2Rx can be reused.

  - Option 3: vivo

    - RAN4 to reuse the relaxation K1 and K3 in 2Rx for 1Rx directly.

  - Option 4: Huawei

    - The relaxation factor 2 as the start point.

  - Option 5: ZTE

    - For 1Rx (e)RedCap UE, the evaluation period should be extended based on legacy UE.

- Recommended WF

  - Further discussion.

For the Redcap UE with 2Rx, there is no difference compared with a normal UE and the relaxed requirements for both SSB based and CSI-RS based RLM can be reused for Redcap with 2Rx directly. Same principle can also apply for both SSB based and CSI-RS based Rel-17 BFD relaxation on Redcap UE with 2Rx. Hence, option 2 is supported.

And the corresponding requirements for Redcap UE with 1rx are:

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Compared with Redcap requirement with 2Rx as below when DRX cycle≤320 ms, the difference is the lower bound and evaluation are extended for Redcap UE with 1Rx.

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

To our understanding, the impact on requirements due to Redcap UE with 1Rx is fully addressed in the current specs. For the RLM/BFD RRM relaxation, at least one option is that the same value of K1 and K3 and corresponding configuration, can also be reused for Redcap UE with 1Rx.

Proposal 3: For the Redcap UE with 2Rx, reuse the current relaxed factor K3 and K1. For the Redcap UE with 1Rx, support to reuse the current relaxed factor K3 and K1.

Issue 2-2-4: Entry and exit criteria, singling indication, applicability rules

- Proposals

  - Option 1: Qualcomm, OPPO, vivo

    - Re-use the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for RedCap UEs.

    - No new signaling is needed.

  - Option 2: Ericsson

    - RAN4 to discuss whether to reuse the existing RLM/BFD relaxation requirements for non-RedCap UE.

  - Option 3: Huawei

    - RAN4 to consider the following aspects for the criteria

      - The length of DRX

      - Serving cell quality criteria

      - Mobility criteria

    - RAN4 to reuse R17 applicability rule.

      - The RLM/BFD relaxation is indicated by NW.

- Recommended WF

  - Further discussion.

For the relaxed criteria, the Rel-17 RLM/BFD is based on the low mobility and not at cell edge (good serving cell). RAN4’s corresponding requirements are defined based on these criteria. The same criteria will be reused in this WI.

Proposal 4: For RLM/BFD RRM requirement relaxation for (e)Redcap, criteria defined in Rel-17 RLM/BFD relaxation are reused and no new criteria will be considered.

Conclusion

In this contribution we further discuss potential issues regarding 6G spectrum sharing and have the following proposals:

Proposal 1: For HD-FDD for LT5 Redcap, the legacy principle of Redcap for HD-FDD could be reused when specifying corresponding requirements.

Proposal 2: Suggest to target on equivalence coverage between LT5 1Rx and 2Rx RedCap UEs.

Proposal 3: For the Redcap UE with 2Rx, reuse the current relaxed factor K3 and K1. For the Redcap UE with 1Rx, support to reuse the current relaxed factor K3 and K1.

Proposal 4: For RLM/BFD RRM requirement relaxation for (e)Redcap, criteria defined in Rel-17 RLM/BFD relaxation are reused and no new criteria will be considered.

- References

[1] RP-252955, New WID: NR RRM enh Phase 6, Apple (moderator), RAN 109

[2] RP-220966, Revised WID on support of reduced capability NR devices, Ericsson, RAN 95e

[3] R4-2206950, WF on RedCap RRM requirements, Ericsson, RAN4 102e