3GPP TSG-RAN WG4 Meeting #117                                           			R4-2520402

Dallas, Tx, USA. 17th – 21st  November 2025

Agenda item:	7.6.4.2

Source:		Eutelsat, THALES

Title:		HD-FDD RRM

Document for:		Discussion

# Introduction

At RAN#109 a new WID [1] with the following scope was agreed:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note : Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

# Overview

This contribution builds upon the discussion at RAN4#116bis and further considers the discussions and work needed to update TS 38.133 [2] due to the introduction of HD-FDD VSAT in Rel-20. The intention of this contribution is to provide comments on the WF [3] to identify topics that warrant further discussion and to progress the work.

# Discussion

## HD-FDD

RRM aspects of HD-FDD VSAT UE were discussed at RAN4#116bis. Based on the WF [3], no changes to the RRM specification [2] structure were identified and (e)RedCap NTN was seen as a sound basis for further work:

Observation 1: The difference between HD-FDD VSAT and RedCap/eRedCap UE is that the former employs a directional antenna, the latter an omnidirectional antenna.

Proposal 1: RAN4 to identify any changes from the baseline assumption of reusing RedCap/ eRedCap RRM requirements for VSAT UE (directional antennas).

VSAT type 6 (mobile VSAT served by LEO) requires both VSAT UE (unpredictable) motion and SAN (predictable) motion to be taken into account; this is the most dynamic scenario. Other VSAT UE types are either fixed served by LEO or GEO/GSO or mobile served by GEO/GSO.

Observation 2: As all VSAT types can benefit from the RRM work for VSAT Type 6 (mobile VSAT served by NGSO) there appears to be no need to explicitly consider other VSAT types.

Proposal 2: Initial RRM work in Phase 1 should first consider a HD-FDD VSAT UE Type 6 (mobile VSAT served by LEO). Other VSAT types are not precluded.

In [3] it was proposed that RAN4 consider adopting “as baseline the collision handling as designed by RAN1 for RedCap NTN. RAN4 to evaluate if more cases of collision handling need to be addressed”. This is in line with Proposal 1 and Proposal 2 of this contribution.

Additionally [4] made three further proposals:

Following investigation, “RAN4 to inform RAN1 about the findings, such that RAN1 can create a feature at their specification covering for such collision handling”.

A specific proposal regarding how HD-FDD scheduling might benefit by UE reporting Timing Advance: “RAN4 to discuss recommending to RAN1 that TA reporting should become a mandatory feature for Ku Bands supporting HD-FDD”.

The last proposal was: “RAN4 to study potential enhancements in intra-satellite measurements to support mobility measurements in HD-FDD mode”.

We agree with the intention of all these proposals. However, even if there are significant benefits, we would stop short of recommending TA reporting become a ‘mandatory feature’ for HD-FDD VSAT UE as this is likely to be controversial in other working groups (e.g., concerns about users revealing their exact location). Possibly TA Reporting could be considered if deemed conditional on the support of specific RRM features that would benefit from the knowledge of a more accurate TA..

Observation 3: The capabilities of the HD-FDD VSAT should be considered and, if beneficial, optional capabilities that can enhance HD-FDD VSAT link level and/ or overall system performance identified.

Proposal 3: In view of HD-FDD VSAT reduced transmission time, RAN4 should discuss and investigate the usefulness of TA reporting for HD-FDD mode VSAT UE and, if appropriate, make technical recommendations of how it can improve RRM performance (in line with [3] Issue 2-1-3 Option B and Issue 2-1-11/12)

In the course of the HD-FDD VSAT UE work, RAN4 should be mindful of the ability to verify any new HD-FDD requirements and identify the scenarios which stress these requirements. The test configurations may, or may not, be the same as for Full Duplex VSATs.

Proposal 4: RAN4 to discuss FD-FDD VSAT UE test configurations can be reused for HD-FDD VSAT; where there are differences (if any), identify new test configurations.

# Conclusions

An analysis of the impact of HD-FDD on the operation of VSAT UE above 10 GHz has been discussed and the following observations are made and proposals for future work are offered to progress the RAN4#116bis WF [2].

Observation 1: The difference between HD-FDD VSAT and RedCap/eRedCap UE is that the former employs a directional antenna, the latter an omnidirectional antenna.

Proposal 1: RAN4 to identify any changes from the baseline assumption of reusing RedCap/ eRedCap RRM requirements for VSAT UE (directional antennas).

Observation 2: As all VSAT types can benefit from the RRM work for VSAT Type 6 (mobile VSAT served by NGSO) there appears to be no need to explicitly consider other VSAT types.

Proposal 2: Initial RRM work in Phase 1 should first consider a HD-FDD VSAT UE Type 6 (mobile VSAT served by LEO). Other VSAT types are not precluded.

Observation 3: The capabilities of the HD-FDD VSAT should be considered and, if beneficial, optional capabilities that can enhance HD-FDD VSAT link level and/ or overall system performance identified.

Proposal 3: In view of HD-FDD VSAT reduced transmission time, RAN4 should discuss and investigate the usefulness of TA reporting for HD-FDD mode VSAT UE and, if appropriate, make technical recommendations of how it can improve RRM performance (in line with [3] Issue 2-1-3 Option B and Issue 2-1-11/12)

Proposal 4: RAN4 to discuss FD-FDD VSAT UE test configurations can be reused for HD-FDD VSAT; where there are differences (if any), identify new test configurations.

# References

[1]  RP-252962 New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN#109, Sept. 2025

[2] TS 38.133 NR; Requirements for support of radio resource management (release 19),  v19.2.0, Sept. 2025

[3] R4-2515082 Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek, October 2025

[4] R4-2514349 “On introducing HD-FDD on the Ku Band”, Nokia, RAN4#116bis, October 2025