3GPP TSG-RAN WG4 Meeting #117	R4-2520410

Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.194 | CR |  | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38194 on A-IoT BS RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is Foffset for transmitter requirements, but no Foffset for receiver requirements; Interfering signals for ACS, in-band blocking and narrowband intermodulation are 3 MHz E-UTRA/NR. Given that 3 MHz NR is not widely deployed, use a 5 MHz NR signal as the interfering source can be better align with real-world network scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note:Changes are based on the endorsed CR R4-2515143. - A-IoT receiver requirements apply with a frequency offset Foffset; - Change the interfering signal for ACS, in-band blocking, and narrowband intermodulation from 3 MHz E-UTRA/NR to 5 MHz NR. - For 200kHz D2R CBW, change the ACS interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-100kHz to +/-340kHz. - For 3.52M D2R CBW, change the ACS interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-100kHz to +/-2500kHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACS, in-band blocking and narrowband intermodulation requirements for A-IoT BS are not appropriately defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3, 7.3.1, 7.3.2, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.3.3	Foffset for A-IoT operation

For A-IoT operation, A-IoT BS requirements for transmitter and receiver shall apply with a frequency offset Foffset  as defined in Table 5.3.3-1 below where CBW is defined in 5.3.1 and 5.3.2.

Table 5.3.3-1: Foffset for A-IoT operation

| Carrier | Foffset |
| --- | --- |
| Ambient IoT | 100 kHz+CBW/2 |

< Non-changed part is omitted >

<<Next Change>>

7.3	In-band selectivity and blocking

7.3.1	Adjacent Channel Selectivity

#### 7.3.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 7.3.1.2	Minimum requirement for BS type 1-C

The BLER  shall be 10% of the reference measurement channel as specified in annex A.1.

The wanted and the interfering signal coupled to the BS type 1-C antenna connector are specified in table 7.3.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.3.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS D2R channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±340 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB, closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±2500 | 5 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±340 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB, closest to wanted signal |
| 3520 | ±2500 | 5MHz DFT-s-OFDM NR signal |

7.3.2	In-band blocking

#### 7.3.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.3.2.2	Minimum requirement for BS type 1-C

The BLER performance shall be 10% of the reference measurement channel as specified in annex A.1, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in tables 7.3.2.2-1 for general blocking. The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

The in-band blocking requirement shall apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. The ΔfOOB for BS type 1-C is defined in table 7.3.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

Table 7.3.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low ≤ 200 MHz | 20 |
| BS type 1-C | 200 MHz < FUL,high – FUL,low ≤ 900 MHz | 60 |

Table 7.3.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6 dB | -38 | ±7.5 | 5MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 |  |  |  |  |

< Non-changed part is omitted >

<<Next Change>>

7.6	Receiver intermodulation

7.6.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

7.6.2	Minimum requirement for BS type 1-C

The BLER performance shall be 10% of the reference measurement channel as specified in annex A.1, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS type 1-C antenna connector, with the conditions specified in tables 7.6.2-1 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in tables 7.2.2-1 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edg [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note 1) | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5 MHz NRsignal, 1 RB (Note 2) |
| 3520 | PREFSENS + 6dB (Note 1) | -53 | ±270 | CW |
|  |  | -53 | ±780 | 5 MHz NR signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

<<End of Change>>