3GPP TSG-RAN WG4 Meeting #117	R4-2520411

Dallas, US, 17th – 21st November, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on R19 A-IoT BS and CW conformance testing

Agenda Item:	6.14.2

Document for:	Approval

Introduction

According to the work item description, the objective is to specify RF requirements for Ambient-IoT BS, device 1, and CW [1]. The core part of WI on solutions for Ambient IoT in NR has been completed at RAN#109 and TS38.194 has been released during last RAN#109 meeting [2]. The objective of performance part WI includes specify the necessary conformance testing for Ambient-IoT BS.

Last meeting, a WF was agreed on conformance testing [3]. This contribution we discuss some issues for testing A-IoT including measurement uncertainties, manufacturer declaration, new test model and test configurations that would be needed to specify A-IoT requirements tests.

Discussion

Manufacturer declaration

Compared with NR, A-IoT only support single carrier both R2D and D2R, no CA, no multi-band operation, manufacturer declaration can be simplified base on NR manufacturer declaration.

Proposal 1: Use Table 1 as A-IoT BS manufacturer declarations.

Table 1 Manufacturer declarations for A-IoT BS test requirements

| Declaration identifier | Declaration | Description | BS type 1-C |
| --- | --- | --- | --- |
| D.1 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-C. | x |
| D.3 | Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Declarations shall be made per antenna connector for BS type 1-C. | x |
| D.4 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x |
| D.5 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. | x |
| D.6 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x |
| D.7 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x |
| D.11 | Maximum Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band for single-band operation. Declared per supported operating band, per antenna connector for BS type 1-C. | x |
| D.14 | A-IoT supported channel bandwidths and SCS | A-IoT supported SCS and channel bandwidths per supported SCS. Declared per supported operating band, per antenna connector for BS type 1-C. | x |
| D.21 | Rated carrier output power (Prated,c,AC) | Conducted rated carrier output power, per single band connector. Declared per supported operating band, per antenna connector for BS type 1-C. | x |
| D.22 | Rated total output power (Prated,t,AC) | Conducted total rated output power. Declared per supported operating band, per antenna connector for BS type 1-C. | x |
| D.32 | Equivalent connectors | List of antenna connectors of BS type 1-C, which have been declared equivalent. Equivalent connectors imply that the antenna connector of BS type 1-C, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector of BS type 1-C, are identical and the transmitter unit and/or receiver unit driving the antenna connector of BS type 1-C are of identical design. | x |
| D.35 | Connecting network loss range for BS testing with ancillary RF amplifiers | Declaration of the range of connecting network losses (in dB) for BS type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. | x |
| NOTE 5: This manufacturer may declare two values, one with a minimum of +6dB and the other with a minimum of +3dB. |  |  |  |

A-IoT test configuration

In RAN4#116bis meeting, below agreements about test configuration were agreed.

Copied from [2] Issue 2-5: A-IoT test configuration - Tentative Agreement: - Use the single carrier operation for A-IoT BS R2D conformance testing - FFS whether and how to test FDM operation for different devices with different R under same CW frequency operation case if needed - FFS whether use the single or multi carrier operation for A-IoT BS D2R conformance testing

Device 1 supports FDM operation. This can help to enhance system capacity and reduce CW phase noise impact on the A-IoT BS receiver sensitivity. This FDM is different with multi-carrier operation. From implementation perspective, the BS receiver's filter needs to fix the widest bandwidth. RF performance testing for narrower bandwidth under wider filter is not proper.

Proposal 2: No need to test for FDM operation.

Proposal 3: Use the single carrier operation for A-IoT BS D2R conformance testing.

Conclusion

This contribution we share our considerations regarding the forthcoming band combination specifications, and have the following proposals:

Proposal 1: Use Table 1 as manufacturer declarations.

Proposal 2: No need to test for FDM operation.

Proposal 3: Use the single carrier operation for A-IoT BS D2R conformance testing.

- References

[1] RP-252745, TS 38.194 v1.0.0 NR; Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception, Huawei, RAN#109, Sept., 2025

[2] R4-2515147, WF on [116bis][326] A-IoT_Maintenance