3GPP TSG-RAN WG4 Meeting # 117	R4- 2520425

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.6

Source: 	National Taiwan University

Title: 	Cross-Frequency Channel Correlation Modelling for Inter-Frequency Measurement Prediction

Document for:	Approval

Introduction

In this contribution, we share our view on the study of channel modelling for inter-frequency measurement predictions.

Discussion

Inter-Frequency Received Power Channel Modeling

In order to properly evaluate inter-frequency RSRP prediction to define the proper requirement that can reflect the field performance, we need a precise channel model to capture the correlations of channels across different frequencies. Reviewing the existing channel model in TR 38.901, we found that center frequency is taken into account in:

- Pathloss modeling, e.g.,

- Path distribution related parameters in Table 7.5-6, e.g.,

| Scenarios |  | UMi - Street Canyon |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | mlgDS | -0.24 log10(1+ fc) - 7.14 | -0.24 log10(1+ fc) - 6.83 | -6.62 |
|  | slgDS | 0.38 | 0.16 log10(1+ fc) + 0.28 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | mlgASD | -0.05 log10(1+ fc) + 1.21 | -0.23 log10(1+ fc) + 1.53 | 1.25 |
|  | slgASD | 0.41 | 0.11 log10(1+ fc) + 0.33 | 0.42 |

- Antenna array location/velocity related impact through λ0 in clause 7.5 in step 11, e.g.,

However, besides the above factors, the phase changes on the propagation through the channel (besides antenna array impact) are different across different central frequencies, and this can contribute to the correlation across channels on different frequencies. Such impact is missing from current channel modeling methodology in TR 38.901 for both system level simulations, and link level simulations. At least the following factors are contributing to the channel coefficient difference across the different center frequencies [1]:

- Path length: the phase change on the propagation path is 2πfcdc, where d is the length of the path, and c is the speed of light. Therefore, the different path length leads to different phase differences between channel coefficients of one path with different center frequencies.

- Reflection and scattering: depending on the materials, reflection and scattering may cause different signal attenuation and phase change when different center frequencies are considered.

Since FR1 channel models don’t consider antenna array, the inter-frequency correlations solely depend on the pathloss modeling, which is obviously oversimplified the challenges, and may lead to oversimplified model with unrealizable gains that are useless in practice. Therefore, if RAN4 decides to keep working on cross-frequency measurement predictions, we need to have a better channel model to accurately capture the channel correlations across different center frequencies.

Proposal 1: If RAN4 decides to study any cross-frequency measurement predictions, RAN4 needs to first study how to capture cross-frequency correlation of channel coefficients through refine channel modeling from at least the following perspectives:

- Impact of path length on cross-frequency channel correlations

- Impact of reflection and scattering on cross-frequency channel correlations

Conclusion

Proposal 1: If RAN4 decides to study any cross-frequency measurement predictions, RAN4 needs to first study how to capture cross-frequency correlation of channel coefficients through refine channel modeling from at least the following perspectives:

- Impact of path length on cross-frequency channel correlations

- Impact of reflection and scattering on cross-frequency channel correlations

Reference

[1] Fan-Hao Lin, Chi-Jui Sung, Chu-Hsiang Huang, Hui Chen, Chao-Kai Wen, Henk Wymeersch, “Cross-Band Channel Impulse Response Prediction: Leveraging 3.5 GHz Channels for Upper Mid-Band”, submitted to IEEE International Conference on Communications (ICC) 2026, manuscript available at http://arxiv.org/abs/2510.27349.