# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520429

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.3

Source:	CMCC

Title:	Discussion on 6GR channel bandwidth

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). in last meeting, WF[2] and LS[3] are approved which show the high level guideline for further discussion.

- In this contribution, we show our initial analysis on channel bandwidth related discussion.

Discussion

Max CBW

- Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

- Agreement - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

- For max CBW, it depends on following aspects:

- FFT size

- SCS

- Potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum

- Potential RF impacts

- For the legacy 5G NR bands, the max frequency range for all operation bands below 3GHz is less than 100MHz. For 3.5GHz, current n77 occupies max 900MHz. For 4.9GHz, current n79 occupies max 600MHz. For future 6-7GHz, China MIIT has allocated total 1.2GHz for IMT identification from lower 5925MHz frequency edge to higher 7125MHz frequency edge.

Observation 1: China MIIT has allocated total 1.2GHz for IMT identification from lower 5925MHz frequency edge to higher 7125MHz frequency edge.

The FFT is based on max CBW. For the around 7GHz new bands, 400MHz CBW is approved with 30kHz SCS, which can be achieved by following approaches:

- UE requirement: the following options can be considered for further studies

  - Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

  - Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

  - Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

- A 400MHz CBW with 30kHz SCS can be achieved for around 7GHz by following approaches:

  - Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

  - Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

  - Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

Min CBW

- Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

- Different from RAN1, following aspects will impact the minimum CBW definition in RAN4.

- Operators’ spectrum allocation

- Sync raster design

- SSB minimum bandwidth

- Device type

- In 5G, the minimum CBW is 3/5MHz for below 3GHz and 10MHz for other FR1 and 50MHz for FR2. besides, the For the legacy bands, current minimum CBW lead to much dense sync raster with longer search time. Furthermore, for certain NR key bands e.g. n41, n77, n78, n79, operator’s spectrum allocation is much larger than current minimum CBW definition. It is reasonable to enlarge minimum CBW definition for most legacy bands. For the existing bands that operators’ spectrum is fragment with smaller bandwidth, existing minimum CBW could be reused. For the new bands, larger minimum CBW is suggested since operators spectrum is not fragment in theory.

- RAN4 further discuss the possibility of band basis minimum CBW rather than frequency range basis. For the legacy NR bands, the same minimum CBW can be reused while for the new bands, larger minimum CBW is suggested.

Numerology

- Agreement - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session. - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

The SCS of SSB will impact sync raster design. 5G commercial network have approved the robust of the same SCS between SSB and data/signal channel. Besides, RAN1 approved to use the same SCS between sync signal and other channels/signals (except PRACH) .

- RAN4 assume the same SCS between sync signal and other channels/signals (except PRACH) for a given band.

Following list the comparison of different SCS values

- The less SCS, the longer symbol duration which contribute to the larger CP and larger SCS number, more suitable for larger cell coverage range with lower frequency range

- The larger SCS, the shorter symbol duration which contribute to better robust for Doppler shift and phase noise, more suitable for higher speed scenario and higher frequency range

As discussed above, it’s reasonable to only define one unified SCS for specific band/frequency range. Further more, for specific bands if operators have different view for SCS value, it’s also OK to consider multiple values based on operators request but should be carefully justified.

- RAN4 define unified SCS per band/frequency sub-range. Exception is allowed when different operators have different view on the SCS value.

Spectrum utilization

- Agreement - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

The minimum guardband for each 5G NR UE channel bandwidth and SCS is referred as below for information. In 5G, maximum transmission bandwidth is specified one by one rather than based on unified spectrum efficiency. Specially, the minimum guard band for less CBW is even larger than that of larger CBW, e.g. the 30MHz and 35MHz minimum guard band as below is even larger than that of 40MHz. Then access failure occurs for such CBW with larger minimum guard-band when BS CBW is larger than UE CBW, e.g. for 700MHz as proposed in 5G. some resolutions are proposed by shifting the transmission bandwidth by 1PRB.

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

- In 6G, maximum transmission bandwidth configuration should be designed to avoid above issues and minimum guard band is defined with the logic that less CBW, less guard band.

- In 6G, maximum transmission bandwidth configuration should be specified with following principle that less CBW, less minimum guard band.

Irregular channel bandwidth

- Agreement - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths - Other options not presented in this meeting are not precluded

In NR R15, only the CBW multiples of 5MHz have been introduced as mandatory. In the following WI, operators proposed CBW that does not conform to existing NR channel bandwidths, commonly referred to as "irregular channel bandwidth. In NR Release 18, a Study Item (SI) titled "FS_NR_eff_BW_util" was established to facilitate more efficient utilization of such CBW with the general conclusions (TR 38.844 ) that two promising methods was proposed, 1)overlapping channels from network perspective and 2) the next larger channel. Both methods have pros and cons. Finally, RAN4 defines native irregular channel BWs, e.g. 7MHz into spec.

For 6GR, it’s better to consider such irregular channel bandwidth at first version for forward compatible. Following lists candidate 3 suggestions for further discussion.

- Option 1: specify the minimum RF requirements for min and max CBW as baseline and add scaling factor on top of the baseline for other CBWs.

  - exception is allowed for certain RF requirements when it’s hard to simplify RF requirements by scaling factor for different CBW.

- Option 2: consider the possibility of defining RF requirements based on actual configured/activated bandwidth i.e. BWP-like basis rather than CBW basis

- Option 3: same method as NR, in the first version define RF requirements for all CBW that has defined for NR or proposed by operators and consider other CBW in future release

- RAN4 consider irregular CBW in the first version with following options:

  - Option 1: in the first version define RF requirements for all CBW that has been defined for NR or proposed by operators and consider other CBW in future release

  - Option 2: based on the two promising method as concluded from NR study phase

  - Option 3: specify the minimum RF requirements for min and max CBW as baseline and add scaling factor on top of the baseline for other CBWs.

    - exception is allowed for certain RF requirements when it’s hard to simplify requirements by scaling factor for different CBW.

  - Option 4: consider the possibility of defining all/part of RF requirements based on actual configured/activated bandwidth i.e. BWP-like basis rather than CBW basis.

    - One example, RAN4 define RF requirements per RB basis rather than CBW basis. There is translation from baseline RB configuration to other configurations.

Conclusion

This contribution makes the following observations and proposals.

Observation 1: China MIIT has allocated total 1.2GHz for IMT identification from lower 5925MHz frequency edge to higher 7125MHz frequency edge.

- A 400MHz CBW with 30kHz SCS can be achieved for around 7GHz by following approaches:

  - Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

  - Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

  - Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

- Proposal 2.  RAN4 further discuss the possibility of band basis minimum CBW rather than frequency range basis. For the legacy NR bands, the same minimum CBW can be reused while for the new bands, larger minimum CBW is suggested.

- Proposal 3.	RAN4 assume the same SCS between sync signal and other channels/signals (except PRACH) for a given band.

- Proposal 4.	RAN4 define unified SCS per band/frequency sub-range. Exception is allowed when different operators have different view on the SCS value.

- Proposal 5.	In 6G, maximum transmission bandwidth configuration should be specified with following principle that less CBW, less minimum guard band.

- Proposal 6.	RAN4 consider irregular CBW in the first version with following options:

  - Option 1: in the first version define RF requirements for all CBW that has been defined for NR or proposed by operators and consider other CBW in future release

  - Option 2: based on the two promising method as concluded from NR study phase

  - Option 3: specify the minimum RF requirements for min and max CBW as baseline and add scaling factor on top of the baseline for other CBWs.

    - exception is allowed for certain RF requirements when it’s hard to simplify requirements by scaling factor for different CBW.

  - Option 4: consider the possibility of defining all/part of RF requirements based on actual configured/activated bandwidth i.e. BWP-like basis rather than CBW basis.

    - One example, RAN4 define RF requirements per RB basis rather than CBW basis. There is translation from baseline RB configuration to other configurations.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514641, WF on system parameter, HW

- R4-2514643, LS on 6G parameter, HW

- R1-2508069, LS on PA models in 6GR waveform evaluations, HW