# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520432

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.3

Source:	CMCC

Title:	Discussion on General UE RF requirement

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). According to chairman’s suggestion, this scope includes non-spectrum and non-AI UE RF, spectrum aggregation framework, UE RF related coverage and energy efficiency, and other joint UE-BS RF issues.

In last meeting, a WF on UE RF has been approved [2].

- In this contribution, we analyze the power class frame work, spectrum aggregation framework and power saving architecture.

Discussion

  - Power class frame work

- Agreements - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study how to maximize the UL transmission power capability for CA - Prioritize discussion for single carrier in RAN4#117 and RAN4#118

In 3GPP, UE power class has been defined as the UE transmitter maximum output power capability. It is also an indirect indication of how high the UE power amplifier output power can be driven while meeting certain emission requirements. Higher power class will improve UE UL coverage directly and have been a central topic of 3GPP RAN4 within the whole 5G stage. RAN4 has specified 20dBm, 23dBm, 26dBm, 29dBm, 31dBm and even 37dBm (only for NTN UE) power class for single band, inter-band CA, intra-band CA, DC related band combination. The detailed progress before R20 is listed as below for information. Basically, now FR1 FDD and TDD bands support 26dBm/1UL before R19 and in R20 new study phase is setup to analyze the feasibility of 29dBm/1UL for TDD bands.

| UL configuration | band/band combination list |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SUL | TDD | FDD | intra-band CA | inter-band CA | SUL CA |
| 1UL band | no PC1.5 (no request) | no PC1.5 for 1Tx PC1.5 for 2Tx | PC2 no PC1.5 | TDD: PC2/PC1.5 FDD: PC2 | TDD: PC2/PC1.5 FDD: PC2 | SUL: PC2 |
| 2UL band |  |  |  | - | (PC1.5)FDD+TDD: PC2+PC2/PC1.5 (PC1.5)TDD+TDD: PC2+PC2/PC1.5 no (PC1.5) FDD+FDD: PC2+PC2 | TDD: PC1.5 |
| 3Tx | - | PC2 | - | - | (PC1.5)FDD+TDD: PC2+PC1.5 | - |

5G NR power class related framework has following drawbacks, part of have been resolved in R19. Nevertheless, a subset of issues remains unresolved :

- Band combination fallback related issues. RAN2 adopts the principle of capability inheritance. A fallback band combination shall not be reported if it has the same or lower capabilities as a parent band combination. Instead, the UE can only report fallback BCs having additional functionality. Single band UL or intra-band UL CA is regarded as the fallback of UL inter-band CA and there are total three power class capabilities without definitive guidance on applicability for these three power class. Following issues occurs

  - there are several power class possibilities for the single band UL/ intra-band UL based on different implementation which is not allowed to be reported since it has the same or lower capability as parent band combination. As a consequence, there is no definitive guidance on which PPowerClass parameter serves as the reference power class for the Pcmax derivation.

  - There are three power class capabilities, the ue-PowerClass, power class of the BC,  ue-PowerClassPerBandPerBC-r17. For DL CA with UL single band, no definitive guidance on which power class is applicable or which power class is mandatory to support especially when ue-PowerClassPerBandPerBC-r17 is not present.

- Power class fallback related issues for CA, duty cycle capability, p-Max, etc. Detailed are listed as below

  - Power class fallback could potentially cause misunderstanding on how the UE configured maximum output power can be applied, and that may result in double-counting the PCMAX power reduction and excessive allowance of MPR/A-MPR for HPUE.

  - For single configured UL CC with DL CA, it’s approved to allow UE report higher power class than the applicability note as in Clause 5 in RAN4. Nonetheless, if the corresponding higher power class MSD is not specified, reference sensitivity and exceptions for reference sensitivity of default power class shall be verified with output power limited to default power class

  - The power class fallback value ∆Ppower class is limited to 3 or 6dB without dynamic indication

- The UE power class capability is static and remains the same once reported to the network regardless of activated/deactivated serving cell(s) and BWP(s).

- For R17 before UE, highpowerlimit-r17 is not applicable, which can’t utilize the full potential of PA because the sum power from all UL carriers can’t exceed the power class of UL CA .

- Based on above 5G experience, following list the initial suggestions for 6G power class framework study.

- RAN4 is suggested to be involved early for RAN2 band combination related capability discussion and avoid the band combination fallback related issues

- Re-design the power class fall back mechanism and how to define RF requirements

- Further study the possibility to allow much dynamic power class capability report mechanism

- Allow highpowerlimit feature from Day 1

- Proposal 1: RAN4 is suggested to study the 6G power class framework from following aspects.

- RAN4 is suggested to be involved early for RAN2 band combination related capability discussion and avoid the band combination fallback related issues

- Further study the possibility to allow much dynamic power class capability report mechanism

- Allow highpowerlimit feature from Day 1

- For 6G, higher frequency band will be introduced, e.g. 7125-8425MHz. To reduce cost, these new frequency bands tend to still use current FR1 RF front end architecture with single antenna element/UL, i.e. no antenna array gain. Besides, according to the SID objective, these bands are target to have the same coverage as 3.5GHz. In theory, higher power class is required for such bands and the default power class may be enlarged to PC2. To reduce RF chain number, 29dBm/1UL feasibility should be further studied for higher candidate frequency bands.

- Proposal 2: the default power class for higher frequency band, e.g. >6GHz should be PC1.5

UE Tx impairment assumption

The PA model will impact MPR, A-MPR, etc. Last meeting WF suggest to list the RF impairment for MPR evaluation. Following list the assumption for 5G/LTE MPR evaluation which can be used as the baseline for further enhancements.

|  | NR | LTE |
| --- | --- | --- |
| IQ suppression | 28dBc | 25dBc |
| Carrier leakage | 28dBc | 25dBc |
| Single carrier CIM3 | 60dBc | 60dBc |

Based on above enhancement, further check whether IQ suppression, carrier leakage, CIM3 can be further enhanced, e.g. enhanced by 3dBc.

Proposal 3: further check whether the IQ suppression and carrier leakage can be enhanced from 28dBc to 31dBc.

Spectrum aggregation framework

Agreement Study and identify the lessons learned from NR spectrum utilization and aggregation framework. - DC is subject to RANP decision in June 2026. - Note: MRSS aspects are separate discussion.

Multi-carrier schemes are treated as efficient ways to increase user throughput and improve coverage, and have been introduce to LTE and NR. But current multi-carrier schemes have following drawbacks:

- Lack of efficient solutions for multi-carrier operation for idle/inactive mode UEs

- Cell management overhead of existing CA framework, especially for fragmented carriers.

- Complex cross carrier scheduling design

Therefore, 6G should support solution for more flexible and efficient multiple frequency resource operation.

Elastic cell with multiple carriers

To realize efficient and flexible IDLE/INACTIVE mode multiple carriers operation, and at the same time, to reduce cell management overhead of existing CA framework, different carriers, regardless of intra-band, or inter-band, can be configured in a cell, named frequency combination based elastic cell. The elastic cell has the following characteristics,

- The multiple carriers are deployed co-located, and can be intra-band or inter-band carriers.

- System information can be broadcast only on one of the multiple carriess, i.e., so called anchor carrier, and one or more non-anchor carriers can be visible and accessed by UE, both anchor and non-anchor carrier can be selected by UE for initial access and data transmission.

- SSB transmission on non-anchor carrier can be configured by anchor carrier, e.g. non-anchor frequency resource can be configured with SSB-less or long SSB periodicity to reduce SSB overhead, UE can access non-anchor carrier with less SSB overhead than initial default SSB periodicity.

- Idle/inactive UEs only need to monitor paging on anchor carrier.

- DL carrier and UL carrier are coupled based on network indication.

- For connected mode, flexible intra-cell multiple carriers switching is supported for connected mode. Similar but more flexible behavior than CA is supported, for example, fast switching than CA can be achieved, since UE has all the SI information about multiple carriers in one elastic cell, L1/L2 based multiple carriers activation and deactivation can be studied, based on R18 L1/L2 mobility enhancement.

- From RF point of view, it is not allowed to randomly couple UL and DL carrier because the reference degradation occurs due to harmonic mixing, IMD interference, cross band interference for certain UL and DL combinations. RAN4 is suggested to be evolved by RAN1 LS for at least the UL/DL couple feasibility analysis.

- Based on discussion above, the following proposals is proposed,

- Proposal 4: Elastic cell with multiple carriers is suggested to be studied for multi-carrier operation and RAN4 is suggested to be evolved based on RAN1 LS, with the following characteristics,

- Multiple carriers are co-located, either intra-band or inter-band.

- RAN4 input for the feasibility of DL carrier and UL carrier couple.

- 5G has discussed Tx switching related issue from R15 and then further enhanced to support more and more bands. Until R19, Tx switching has already been applicable for CA with three or four bands. for 6G, it’s suggested to support Tx switching for CA with three or four bands from day 1.

- Proposal 5: it’s suggested to use 5G-A Tx switching requirements as baseline for 6G.

Power saving architecture

5G LP-WUS specify two sets of requirements for type 1 LR and type 2 LR. The former is defined with much loose requirements and the later is defined with relatively strict requirements. Both type 1 and type 2 use the same reference channel with overlaid OFDM sequence with OOK-4, so in theory there is no limit of Rx architecture. Both OOK based envelope detection and OFDM based Rx architecture are supported for both LR types.

- Following list R18/19 LP-WUS/WUR drawbacks：

- The coverage of LP-WUS/WUR is relatively poor. Therefore, UE can only receive LP-WUS or perform serving cell measurement in a limited area.

- Due to the alignment of payload between OOK-based receiver and OFDM-based receiver, the information bit carries in LP-WUS is relatively limited.

Based on the lessons learned from NR and potential issue on 6GR, we suggest to further enhance LP-WUS/WUR design in 6GR, which includes the following aspects:

- Overlaid-wise signals are not necessary to avoid unnecessary design trade-offs.

- Better receiver detection method for 6G LP-WUR more than envelope detection, can be considered.

- The receiver accuracy, e.g., option 3 or 4 in TR38.869, 10-20ppm and power consumption (0.1-1mW) can be considered as start point with justified power saving gain.

- At the starting point, RAN4 is suggested to be evolved for LP-WUS/WUR architecture design study based on RAN1 signal design.

- Proposal 6: RAN4 is suggested to be evolved early for LP-WUR design study based on RAN1 signal design.

Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: RAN4 is suggested to study the 6G power class framework from following aspects.

- RAN4 is suggested to be involved early for RAN2 band combination related capability discussion and avoid the band combination fallback related issues

- Further study the possibility to allow much dynamic power class capability report mechanism

- Allow highpowerlimit feature from Day 1

- Proposal 2: the default power class for higher frequency band, e.g. >6GHz should be PC1.5

Proposal 3: further check whether the IQ suppression and carrier leakage can be enhanced from 28dBc to 31dBc.

- Proposal 4: Elastic cell with multiple carriers is suggested to be studied for multi-carrier operation and RAN4 is suggested to be evolved based on RAN1 LS, with the following characteristics,

- Multiple carriers are co-located, either intra-band or inter-band.

- RAN4 input for the feasibility of DL carrier and UL carrier couple.

- Proposal 5: it’s suggested to use 5G-A Tx switching requirements as baseline for 6G.

- Proposal 6: RAN4 is suggested to be evolved early for LP-WUR design study based on RAN1 signal design.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514642, WF_6G_UE_RF