# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520433

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.5

Source:	CMCC

Title:	Discussion on 6G spectrum

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). According to chairman’s suggestion, this scope includes band/band combination definition and simplification, the definition of frequency ranges, 6G spectrum related other aspects, including the regulatory status survey.

In last meeting, a WF[2] is approved.

- In this contribution, we show out view on 6G spectrum.

Discussion

  - The definition of frequency ranges

The RAN led study on 6G Scenarios and requirements has been approved at RAN#108 [1]. Among its objectives, it includes

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - …

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - …

- Last meeting RAN4 agreements are listed as below:

- WF - RAN4 acknowledge that a definition of frequency ranges depends on multiple criteria’s which may have different importance dependent on individual company views. As a starting point the following can be discussed, not precluding companies to also consider others: - Regulatory framework - UE RF front-end architecture (e.g. whether requirements can be verified conducted or not) - Commonality of UE RF/RRM requirements - Numerology dependency - Unification of TN and NTN operation - BS types and requirements - Companies are encouraged to share information on the criteria they are considering for their proposal on frequency range definition(s)

Following list the progress of around 7GHz in ITU.

6 425 – 7 125 MHz range has been identified as IMT spectrum regional at WRC-23. Despite its previous strong opposition, India has shifted its position and now offers support IMT identification for 6425-7125MHz. Besides, following list the candidate IMT spectrum under WRC-27 agenda item 1.7

- 4400-4800MHz: the major proponents is Russia. US tends to abandon 7125-8400MHz and shift its position to support these frequency range

- 7125-8400MHz: this frequency range was opposed by many countries and regional groups, including CEPT, due to strategic usages. These strategic usages are, and will remain, critical in all of this frequency band. Besides, this frequency range is intensively used by a large number of government satellite systems for down-links (i.e. 7 250-7 750 MHz) and up-links (i.e. 7 900-8400 MHz). The upper part of the band (i.e. 8 025-8 400 MHz) is also used for the downlink of Earth observation data by a large majority of Earth Observation satellites. On the other side, Japan and Korea tends to support such frequency range.

- 14.8-15.35GHz: now no countries/groups strongly support such bands

Observation 1: above list the regulatory status survey of 6425-7125MHz, 4400-4800MHz, 7125-8400MHz and 14.8-15.35GHz.

Based on above spectrum state survey, it seems 7125MHz is the most promising higher bands compared with 8400MHz and 15.35GHz. Furthermore, it seems pessimistic to support 15.35GHz as IMT identification in WRC27 since no countries/groups strongly support it.

Following show the analysis of 8GHz coverage. 6G BS working at 8GHz will experience higher propagation loss and penetration loss compared with 5G at 2.6GHz and 3.5GHz. And since almost 80% traffic happens in the indoor scenario and served by outdoor Macro cells, the O2I penetration loss is also considered for the coverage enhancements. Base on TR38.901, we compared the additional loss induced by the higher operation frequency as below.

Table Additional propagation loss at 8.4GHz compared with 2.6GHz and 3.5GHz

|  | Additional loss at 8.4GHz |  |
| --- | --- | --- |
| Scenarios | 2.6GHz | 3.5GHz |
| LOS/NLOS pathloss (dB) | 10.2 | 7.1 |
| pathloss+ penetration loss low loss model (dB) | 11.5 | 8.0 |
| pathloss+ penetration loss high loss model (dB) | 17.2 | 11.6 |

Larger number of the antenna elements can compensate parts of the additional loss, but which may not be enough. Both traffic channel and common control channels should be considered for the coverage evaluations. The additional gain provided by the larger antenna array is listed below.

Table Additional gain brought by higher antenna element number

|  |  | 2.6GHz | 3.5GHz | 8.4GHz | 8.4GHz |
| --- | --- | --- | --- | --- | --- |
| BS antenna elements |  | 192 | 192 | 1024 | 2048 |
| Antenna array gain for traffic channel |  | 22.83 | 22.83 | 30.10 | 33.11 |
| Additional gain from BS antenna array for traffic channel |  |  |  | 7.27 | 10.28 |
| Additional gain from BS antenna array for common channel | Option 1: 3dB loss compared with traffic channel |  |  | 4.27 | 7.28 |
|  | Option 2: 6dB loss compared with traffic channel |  |  | 1.27 | 4.28 |

Table Preliminary coverage gap for 8.4GHz with 2048 antenna elements

| 8.4GHz-2048 antenna element-coverage gap |  |  |  |
| --- | --- | --- | --- |
| Coverage gap compared with 3.5GHz |  |  |  |
|  | O2O | O2I-low loss | O2I-high loss |
| Traffic channel | -3 | -2.1 | 1.5 |
| Common control channel with option 1 | -0.2 | 0.9 | 4.5 |
| Common control channel with option 2 | 3 | 4 | 7.5 |

* No additional gain from UE side is considered including higher number of antenna element or higher transmit power.

For hand held UE, based on above initial analysis, it seems the coverage gap is less if assume larger antenna element number at BS side and can be compensated by certain UL coverage enhancement scheme especially when comparable with 3.5GHz rather than using better RF front architecture, e.g. re-transmission or lower PAPR with less MPR performance. It seems legacy FR1 RF front end can be used for 8.4GHz.

- Observation 2: with larger antenna element number at gNB side, the UL coverage of 8.4GHz is almost comparable with 3.5GHz at least for O2O scenario.

Band/band combination definition and simplification

- During 5G stage, RAN 4 specified 70+ single bands and 10000+ band combinations numbers for CA, EN-DC, NE-DC, NR-DC, SUL which introduce huge workload for RAN4 requirement definition and RAN5 testing. As a results, increasing the RF front end complexity for UE when it has to support as much band combination as possible.

- Following options are proposed to simplify RAN4 and RAN5 workload of band combinations:

- Option 1: use band group concept. For the bands belong to the same band group, switching instead CA is suggested. For the bands cross band group, the CA is suggested. This is something like what is done for the LBCA WID which use switching instead of CA for less than 1GHz spectrum. And RAN4 can further study the applicable frequency range, e.g. further check whether can extend the upper frequency to 2GHz. The main benefit is to reduce RF front end complexity and reduce insertion loss caused by duplexer to enlarge UL/DL coverage.

- Option 2:reuse current 5G RF requirements for 6G for the legacy band/band combinations and further study the method to reduce conformance testing workload.

- For option 1, the band group could be categorized by current UE RF front duplex applicable frequency range. If only switching is supported for bands within the group, then the number of RFRE and transceiver could be reduced. One issue for option 1 is whether it is applicable only for the 6GR new bands or both NR legacy bands and new bands. Based on the survey of global spectrum status, it seems the number of new bands is limited. If option 1 is applicable for both legacy bands and new spectrum. some exception, e.g. intra-group CA should be allowed especially for the bands that operators have strong demands.

- Proposal 1: RAN4 can further study the options to reduce band combination workload and reduce RF front end complexity based on following options:

- Option 1: use band group concept. For the bands belong to the same band group, switching instead CA is suggested. For the bands cross band group, the CA is suggested.

  - Exception should be allowed to allow intra-group CA especially for the bands that operators have strong demands.

- Option 2:reuse current 5G RF requirements for 6G for the legacy band/band combinations and further study the method to reduce conformance testing workload.

Conclusion

This contribution makes the following observations and proposals.

- 6 425 – 7 125 MHz range has been identified as IMT spectrum regional at WRC-23. Despite its previous strong opposition, India has shifted its position and now offers support IMT identification for 6425-7125MHz.

- 4400-4800MHz: the major proponents is Russia. US tends to abandon 7125-8400MHz and shift its position to support these frequency range

- 7125-8400MHz: this frequency range was opposed by many countries and regional groups, including CEPT, due to strategic usages. These strategic usages are, and will remain, critical in all of this frequency band. Besides, this frequency range is intensively used by a large number of government satellite systems for down-links (i.e. 7 250-7 750 MHz) and up-links (i.e. 7 900-8400 MHz). The upper part of the band (i.e. 8 025-8 400 MHz) is also used for the downlink of Earth observation data by a large majority of Earth Observation satellites. On the other side, Japan and Korea tends to support such frequency range.

- 14.8-15.35GHz: now no countries/groups strongly support such bands

Observation 1: above list the regulatory status survey of 6425-7125MHz, 4400-4800MHz, 7125-8400MHz and 14.8-15.35GHz.

- Observation 2: with larger antenna element number at gNB side, the UL coverage of 8.4GHz is almost comparable with 3.5GHz at least for O2O scenario.

- Proposal 1: RAN4 can further study the options to reduce band combination workload and reduce RF front end complexity based on following options:

- Option 1: use band group concept. For the bands belong to the same band group, switching instead CA is suggested. For the bands cross band group, the CA is suggested.

  - Exception should be allowed to allow intra-group CA especially for the bands that operators have strong demands.

- Option 2:reuse current 5G RF requirements for 6G for the legacy band/band combinations and further study the method to reduce conformance testing workload.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514626 WF on 6G Spectrum_v4, Feature lead (Nokia)