# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520435

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.12.2.1

Source:	CMCC

Title:	Discussion on 6G RF spec enhancement

Document for:	Discussion

- Introduction

In RAN#108 meeting, 3GPP requirements for 6G Radio for new services are approved to be developed in SID for 6G[1].

According to chairman’s guideline, the scope includes spec improvement and modernization in general, new tools for band/band combinations, CR operation improvement and other RAN4 operation related aspects. A WF is approved in last meeting [2] that in RAN4#117, a dedicated singe sub-agenda item will be introduced for the spec improvement. Interested companies are encouraged to structure their contributions such that the spec specific proposals and the general proposals for all RAN4 spec can be separately presented.

This contribution gives proposals on 6G UE RF and BS RF spec simplification with following aspects:

- Band combination simplification

- RF release independent issues

- UE RF suffix issues

- Nonalignment between BS core and performance requirements

- Co-existence assumption alignment

- Discussion

  - 2.1	RF release independent issues

Due to lack of unified design/role for release independent spec at the beginning of NR, MCC point out the release independent related issues in RP-240905 as below:

- Which releases of 307s are modified when new feature is introduced

- If only latest release is updated what to do with earlier releases

- Release pointers

- Other role discussion for 307, e.g.

  - Whether and how to capture into the WID: e.g. Add NOTE: When a REL-independent approach is considered, in the corresponding RAN4 WID decided by TSG RAN, the release independent objective should be mentioned and it should be clarified which release this release independent approach is used from.

  - Differentiate the core and performance part, e.g. if the REL-independent requirement is a Core part requirement, then the REL-independent spec is impacted by the Core part. If the REL-independent requirement is a Perf. part requirement, then the REL-independent spec is impacted by the Perf. part.

Finally RAN4 approved following agreements: “When a new release independent feature is introduced, only the latest release of release independent specification shall be updated. The latest release of release independent specification refers to "release N", i.e. the release in which a feature is introduced into TS 38.101 [2-5, 8] or TS 38.133 [6]. And the common UE requirements tables are also specified from “release N” in the annexes, starting from annex B.”

Learned from 5G experience, it’s better to identify release independent related rules at the beginning of 6G.

Last meeting, [3] propose to use RAN2 spec architecture for release independent related rules. RAN2 use annex C for the “early implementable features and corrections” rather than use a new separate spec. RAN2 annex C is categorized by CR of new features with limited numbers. But the CR number for band/band combinations in RAN4 are much huge without simplification. Using annex method may overload the spec and make it unreadable. One method is to list band related release independent features into one separate file/spec, while for other features listed them in the annex like what RAN2 has done.

Proposal 1: Spec 3x.307 is scoped to only cover band and band combinations related features, while other features may adopt the same methodology as RAN2. To improve clarity and readability, these other features can also be placed in a separate file inside the 3x.101.zip file.

- 2.2	band combination simplification

band combination simplification is one of the key issues for 6G accounting for RAN4 has introduced thousands of band combinations. One promising method is to group bands as group. Furthermore use CA for inter-group and use switching for intra-group. Now the band combination simplification will be discussed in spectrum email thread. It’s better to split the work between spectrum email thread and this spec enhancement spec.

It is not sure whether the “group” concept is only applicable for 6GR new bands or not. But at least the band concept will still be maintained and there will be band combination besides group combination accounting for that operators may have strong views to support intra-group CA, i.e. band combination, especially for the NR legacy mid bands.

Observation 1: 6GR may still need to define the band combination even when “inter-group CA and intra-group switching” method is accepted.

Following list our view only for the band combination spec enhancement rather than group combination spec enhancement. The database should be utilized from the beginning of 6GR which will capture most band combination related RF requirements and the exception should be carefully justified. Besides, the conclusion of MSD, TIB, RIB in UR RF email thread will be reflected into this database, e.g. the calculation equation and assumptions can be captured into the database. Besides, Considering most NR band combinations are release independent from the first version, the latest spec is the super sets of all previous supported band combination. The database will be continuously updated for every future release. Current CR procedure all to start the discussion of new BC only when all fallback combinations have been finished. So it’s better that the database can auto check whether all fallback requirements have been finished or not.

Proposal 2: the database should :

- Include all band combination specific RF requirements

- Continuously update for every future release

- Support the integration of specific calculation formulas to enable the automated derivation of RF requirements, e.g. MSD, TIB, RIB based on UE RF requirements conclusion

- Support the integration of auto checking tool, e.g. for the check of fallback combination

  - 2.3	UE RF suffix issues

- Suffix approach helps to enhance the readability of UE spec but due to lack of unified rule for detailed spec drafting, some issue happens as below:

- There are blank sub-clauses for some suffix without any detailed RF requirements. There is no explanation of what does this mean. It may have two explanations by the reader, one is no RF requirements, another explanation is that the RF requirements without suffix is applicable. It’s better to have some explanation at the beginning of the spec

- for some suffix, the same requirements is just copy past without any updates as UE requirements without suffix. It’s better to define unified rule whether such copy past is necessary or it can be replaced by one sentence that the same requirements as in sub-clause xx is applied.

- Proposal 3: suffix approach increase the spec readability but there are some in-alignments. A unified rule for different suffix is needed and it’s better to list the rule at the beginning of spec,

- There are blank sub-clauses for some suffix without any detailed RF requirements. There is no explanation of what does this mean. It may have two explanations by the reader, one is no RF requirements, another explanation is that the RF requirements without suffix is applicable. It’s better to have some explanation at the beginning of the spec

- for some suffix, the same requirements is just copy past without any updates as UE requirements without suffix. It’s better to define unified rule whether such copy past is necessary or it can be replaced by one sentence that the same requirements as in sub-clause xx is applied.

  - 2.4 BS spec enhancement

- Compared with UE RF and RRM spec, the BS spec is much clear and streamline. But there are still some space for better readability.

- Issue 1: RAN4 perform much co-existence simulation and some of them are much similar and the simulation assumptions can be aligned.

  - For example, for the coexistence between NR and other system, NR system related assumption can be aligned, e.g. the deployment scenario related, the antenna model related and propagation related

- Issue 2: mismatch between requirements and conformance spec

- For the issue 1, one method is to use one separate co-existence spec to capture all co-existence simulation related information and update it contiguously for future release. For detailed WID, the simulation assumption can be chosen from the spec to shorten co-existence simulation duration.

- Proposal 4: To reduce the co-existence simulation workload, we suggest to introduce a dedicated co-existence specification. This document is intended to consolidate all simulation-related information and will be updated on an ongoing basis for future releases.

- Conclusion

In this contribution, our proposals and observations are listed as below:

Proposal 1: Spec 3x.307 is scoped to only cover band and band combinations related features, while other features may adopt the same methodology as RAN2. To improve clarity and readability, these other features can also be placed in a separate file inside the 3x.101.zip file.

Observation 1: 6GR may still need to define the band combination even when “inter-group CA and intra-group switching” method is accepted.

Proposal 2: the database should :

- Include all band combination specific RF requirements

- Continuously update for every future release

- Support the integration of specific calculation formulas to enable the automated derivation of RF requirements, e.g. MSD, TIB, RIB based on UE RF requirements conclusion

- Support the integration of auto checking tool, e.g. for the check of fallback combination

- Proposal 3: suffix approach increase the spec readability but there are some unalignments. A unified rule for different suffix is needed and it’s better to list the rule at the beginning of spec,

- There are blank sub-clauses for some suffix without any detailed RF requirements. There is no explanation of what does this mean. It may have two explanations by the reader, one is no RF requirements, another explanation is that the RF requirements without suffix is applicable. It’s better to have some explanation at the beginning of the spec

- for some suffix, the same requirements is just copy past without any updates. It’s better to define unified rule whether such copy past is necessary or it can be replaced by one sentence that the same requirements as in sub-clause xx is applied.

Proposal 4: To reduce the co-existence simulation workload, we suggest to introduce a dedicated co-existence specification. This document is intended to consolidate all simulation-related information and will be updated on an ongoing basis for future releases.

- Reference

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #108Prague, Czech Republic, June 9-13, 2025.

- R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT

- R4-2513346, RAN4 operation efficiency, Ericsson