3GPP TSG-RAN WG4 Meeting #117			                        R4-2520437

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.4.1

Source: 	CMCC

Title: 	Discussion on 6G BS RF requirement

Document for:	Discussion

# 1. Introduction

The last meeting failed to reach a valid conclusion on the 6G BS RF requirements. From the perspectives of various companies, some 6G BS RF requirements need to be re-evaluated.

This document will analyze from the perspective of operators which RF requirements in 6G need to be re-evaluated.

- Discussion

### 2.1 General consideration on 6GR BS RF requirement

During last meeting, RAN4 has achieved the following agreement and WF:

BS RF requirements Issue 1-1-1: Starting point Agreement: 5G BS RF requirements should be considered as starting point when defining the 6G BS RF requirements. Issue 1-1-2: Requirements to be re-assessed Way Forward: For next meeting, companies are encouraged to evaluate how 5G BS RF requirements should be considered for 6G and provide their view using the table in Annex A as example.

The last meeting has reached a preliminary agreement on the RF requirements of 6GR BS. Set the legacy 5G BS RF requirements as starting points and further decide which requirements need to be re-evaluated.

From our opinion, most of the legacy BS RF requirements could be reused by 6GR BS. However, considering new usage scenarios, energy saving, cost reduction and other factors, some BS RF requirements still need to be re-evaluated in 6G.

In terms of conduction requirements, we first focus on co-located unwanted emission. The requirements are still being studied in the current R20 WI. In the research scope of the WID, the current focus is mainly on the OTA co-location requirements of BS type 1-O. However, the current co-located requirements still have some challenges, especially after the introduction of new frequency bands, the co-located requirements need to continue to be studied. Considering that R20 is currently conducting relevant OTA requirements research, 6GR can give priority to co-located unwanted emissions for conducted requirements.

Observation 1: R20 is conducting research on co-located requirements, but the main research scope of WID is OTA co-located requirements of BS type 1-O.

Proposal 1: Co-located unwanted emissions could be re-evaluated based on system parameters decision for 6G.

Apart from co-located requirements, we also think that EVM requirements deserve focused research. Users have high expectations for the transmission speed and quality of 6GR networks, making advanced modulation methods essential. Additionally, legacy EVM requirements pose certain challenges for BS implementation, the study of this requirement in 6GR should also be taken into account. Besides, the following conclusions were reached regarding modulation for 6GR at RAN1#122 and 122bis meetings.

RAN1#122 Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes RAN1#122bis Agreement - For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following: - PAPR/CM of the resulting waveform - EVM, MPR/A-MPR - For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation.

Observation 2: RAN1 held discussions on modulation and EVM requirements.

Proposal 2: Modulation quality requirements could be re-evaluated based on system parameters decision for 6G.

Proposal 3: BS RF requirements which need to be re-evaluated is listed in table 1.

Table 1. BS RF requirements analysis

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Transmitted signal quality |  |  |  |
| Modulation quality |  | X |  |
| Unwanted emissions |  |  |  |
| Co-location |  | X |  |

### 2.2 BS type 1-H enhancement

Last meeting has reached the following agreement.

Agreement: BS Hybrid beamforming type of architecture shall be studied.

The meeting has agreed to conduct research on the BS Hybrid beamforming architecture. From the perspective of the operators, the current NR 1-H BSs have basically achieved hybrid beamforming. Therefore, the legacy definition of BS class 1-H is sufficient to meet the requirements, and we think that there is no need to define a new BS class or type.

Proposal 3: No need to define new BS class or BS type for BS Hybrid beamforming type.

# 3. Conclusions

Observation 1: R20 is conducting research on co-located requirements, but the main research scope of WID is OTA co-located requirements of BS type 1-O.

Proposal 1: Co-located unwanted emissions could be re-evaluated based on system parameters decision for 6G.

Observation 2: Both RAN1 and RAN plenary meeting held discussions on EVM requirements.

Proposal 2: Modulation quality requirements could be re-evaluated based on system parameters decision for 6G.

Proposal 3: BS RF requirements which need to be re-evaluated is listed in table 1.

Table 1. BS RF requirements analysis

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Transmitted signal quality |  |  |  |
| Modulation quality |  | X |  |
| Unwanted emissions |  |  |  |
| Co-location |  | X |  |

Proposal 3: No need to define new BS class or BS type for BS Hybrid beamforming type.

# 4. Reference

- R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)