# 3GPP TSG-RAN WG4 Meeting #117                            R4-2520454

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.12.3

Source: 	CMCC

Title: 			 Discussion on RRM requirements for RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [1]. In RAN #109 meeting, the revised WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [2]. The objectives for RAN4 are duplicated as following.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any

In last meeting, there is initial discussion on Artificial Intelligence (AI)/Machine Learning (ML) for mobility, and a WF was approved [3]. This contribution provides discussion on RRM requirements for RRM measurement prediction.

- Discussion

Issue 2-1-1: What kind of core requirements to be defined Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the follow requirements need to be defined: - Measurement delay/period requirements (OPPO, CMCC, ZTE, Samsung, QC) - Measurement and scheduling restrictions in OW/PW (QC, Samsung) - Prediction delay requirements (OPPO) - Prediction reporting delay requirements (OPPO, HW, QC) - OW/PW length (HW, LGE, Samsung) - Data collection delay requirements (Ericsson) - SCell related requirements (Samsung, QC) - UE mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements (QC)

According to the WID, for RRM measurement prediction, temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases are the main target. These three use cases are also prioritized during the study phase.

In study phase, RAN2 agreed the KPIs about measurement reduction rate, which is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS).

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

In legacy, there is no RAN4 requirements on measurement reduction, whether to introduce related requirements for AI/ML based mobility can be further discussed.

Another consideration is that the measurement delay requirements may also need to be newly defined for some scenarios. Taking FR1 inter-frequency inter-cell prediction as an example, prediction from low-frequency cell to high-frequency cell is expected, which means that not all configured MOs need to be measured. The legacy measurement delay is scaled with the number of MOs, which need to be updated to only consider the actually measured frequency layers. As for the intra-frequency temporal domain prediction, e.g. case B, the legacy measurement delay also need to be updated. In legacy, SMTC or MGRP is in use in the measurement delay requirements, while for temporal domain prediction, UE only measure subset of SMTC/MGRP, and predict the results of other measurement occasions.

Proposal 1: for RRM measurement prediction, the measurement delay requirements need to be defined/updated

- For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

- For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measured, which means that only the actually measured frequency layers are considered in the measurement delay requirements

Issue 2-1-5: LCM related requirements - Proposals and Observations: - Proposal 1 (OPPO): - Do not define deactivation/switching/fallback delay requirements for AI functionality. - For AI functionality activation delay and performance monitoring, wait for RAN2 progress on LCM framework. - Proposal 2 (CATT): - For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. - RAN4 to discuss how to define LCM requirements for performance monitoring and fallback procedures after RAN2 have more concrete progress. - Proposal 3 (Tejas Networks): - RAN4 shall specify the handling of LCM for all agreed scenarios and/or sub-cases. - Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values. - RAN4 to specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference. - Proposal 4 (HW): - For RRM measurement prediction, whether and how to define activation delay can wait for more progress in R19 AI/ML air interface beam management. - Proposal 5 (vivo): - RAN4 further discusses whether related LCM requirements are needed, depending on RAN2's design. - For RRM measurement prediction, it is within RAN4's scope of discussion to define the relevant metrics for performance monitoring (if any). - Proposal 6 (Ericsson): - RAN4 to identify performance metric to be reported for performance monitoring at network side. - RAN4 to define delay requirement for performance metric reporting for performance monitoring of UE sided model at the network node. - Proposal 7 (Samsung): - RAN4 to specify requirements for model monitoring including monitoring related conditions (e.g., criterion and the corresponding threshold) definition

Some companies suggest to follow the progress in R19 AI/ML air interface beam management. In Rel-19 beam prediction, it was agreed to deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement. Furthermore, it was agreed not to define deactivation delay requirements. And no activation requirement in RAN4 spec for beam prediction will be introduced, with the common understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4.

We are not sure whether the progress in R19 AI/ML air interface beam management can be reused for AI for mobility. LCM related requirements is pending on RAN2 design, since RAN2 WI work has not started, it is preferred to wait for RAN2 design.

Proposal 2: LCM related requirements is pending on RAN2 design, it is proposed to have further discussion based on RAN2 progress.

Issue 2-2-1: Performance metrics definition - Proposals and Observations: - Proposal 1 (Xiaomi): - RAN4 shall discuss on the definition of prediction accuracy for multi-instance reports for temporal domain Case A. - Proposal 2 (HW): - For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss whether only RSRP prediction accuracy under certain channel correlation coefficient is to be specified. - Proposal 3 (Samsung): - RAN4 to discuss the accuracy metrics/KPIs for cell prediction requirements, the following two options of the accuracy KPI can be taken as the candidate - Cell prediction accuracy: Top-1(%), Top-K(%) - Maximum RSRP among Top-K predicted cells is larger than the RSRP of the strongest cell – x dB - Proposal 4 (Apple): - RAN4 is invited to adopt a clear time-indexed definition of relative RSRP accuracy for intra-frequency temporal prediction. - At time t, cell 1 predicts its Layer-3 RSRP at t + Δt1, while cell 2 provides a reported (measured or predicted) RSRP at t + Δt2. The metric compares the predicted power gap to the true power gap at those future times and is expressed as ( P_pred1(t + Δt1) - P_rep2(t + Δt2)) -( P_true1(t + Δt1) - P_true2(t + Δt2)) - RAN4 shall define relative inter-frequency accuracy through two complementary cases to ensure consistent evaluation of cross-frequency prediction. - Case A – Time-domain, cross-frequency: Cell 1 on f₁ predicts its L3-RSRP at t + Δt₁ while Cell 2 on f₂ (same FR) provides a reported value at t + Δt₂ (measured or predicted); the KPI compares predicted vs. true inter-cell power gaps to assess future-time signal differences. - Case B – Frequency-domain, cross-cell: Cell 1 on f₁ is predicted across frequency using measurements from other cells, while Cell 2 on f₂ provides a reported value (predicted or measured); the KPI compares predicted vs. true inter-cell power gaps for cross-frequency inference. - Explicitly state the mathematical KPI as (Pred f1 – Pred/Reported f2) – (GT f1 – GT f2) for both cases. Issue 2-2-2: Prediction accuracy requirements for measurement prediction - Proposals and Observations: - Proposal 1 (HW): - RAN4 to discuss whether the prediction accuracy has the same or different accuracy requirements that are defined for the non-AI/ML RRM measurement. - Proposal 2 (Apple): - RAN4 should base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts. - Proposal 3 (ZTE, Nokia): - Regarding temporal domain measurement prediction B, study the impact of measurement skipping pattern on prediction accuracy for the same MRRT. - Proposal 4 (Samsung): - For temporal domain case A, OW length and/or PW length should be specified in Rel-20 AI mobility, the necessity of setting threshold (s) for OW length and/or PW length can be discussed. - For temporal domain case B, MRRT can be considered as a side condition when defining prediction accuracy requirement.

The performance metric is related with model output. And the output may be different for different scenario. For time domain prediction,according to Rel-19 study, the model output is L3-RSRP. For frequency domain prediction, the model output at least include L3-RSRP, whether cell ID is needed or not, there is no clear consensus in RAN2. Based on above consideration, at least RSRP prediction accuracy can be considered as performance metric.

In Rel-19 study phase, the definition of absolute and relative accuracy of predicted L3-RSRP are discussed for intra-frequency scenario and inter-frequency scenarios.

The absolute accuracy of predicted L3-RSRP is defined as:

- Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP.

The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2),

  - cell 1 and cell 2 are on the same frequency

  - the reported L3-RSRP of cell 2 can be measured or predicted.

The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2),

  - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

  - the reported L3-RSRP of cell 2 can be measured or predicted.

As for whether to consider other metric, it is pending on whether other model output is introduced. In Rel-19 beam prediction, successful rate for the correct prediction is introduced as KPI for beam ID prediction for the case beam ID is reported. The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB. For AI for mobility, whether successful rate for the correct prediction is introduced as KPI is pending on whether cell ID is the model output and the predicted cell ID is reported. For now, there is no clear consensus in RAN2, prefer to have further discussion.s.

Proposal 3: for AI for mobility, at least RSRP prediction accuracy can be considered as performance metric. As for whether to consider other metric, e.g. successful rate for the correct prediction, it is pending on whether cell ID is agreed as model output.

For the target for RSRP prediction accuracy for AI/ML for mobility, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in previous meetings, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, it is not a good way to achieve overhead reduction by sacrificing accuracy. If the predicted results are worse than the legacy measured results, it will have impact on the already deployed network, which is not preferred.

Proposal 4: For RSRP prediction accuracy for AI/ML for mobility, it is proposed that AI/ML based performance requirement should be no worse than the legacy measurement accuracy requirements.

- Conclusion

This contribution provides discussion on RRM requirements for measurement event prediction. The proposals are:

Proposal 1: for RRM measurement prediction, the measurement delay requirements need to be defined/updated

- For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

- For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measured, which means that only the actually measured frequency layers are considered in the measurement delay requirements

Proposal 2: LCM related requirements is pending on RAN2 design, it is proposed to have further discussion based on RAN2 progress.

Proposal 3: for AI for mobility, at least RSRP prediction accuracy can be considered as performance metric. As for whether to consider other metric, e.g. successful rate for the correct prediction, it is pending on whether cell ID is agreed as model output.

Proposal 4: For RSRP prediction accuracy for AI/ML for mobility, it is proposed that AI/ML based performance requirement should be no worse than the legacy measurement accuracy requirements.

- Reference

- RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- R4-2514827, WF on NR_AIML_Mob_RRM, OPPO

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR