3GPP TSG-RAN WG4 Meeting #117	R4-2520475

# Dallas, USA, 17th-21st November, 2025

Source:	Nokia

Title:	On UEIBM test cases

Agenda item:	6.12.2.1

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the test case for UEIBM window based scenario.

Discussion

In the last meeting, it was left FFS whether to draft a test case for window based scenario for FR1 and FR2. Since RAN4 has defined requirements for the window based scenario with eventInstanceCount-r19 > 1, we propose that a corresponding test case should be defined as well.

As an example, we describe the test procedure for FR1 Event-2, Intra-cell SSB with window time based and eventInstanceCount-r19 > 1, which we have proposed in our companion CR.

The UE is configured with eventDetectionTimeWindowLength-r19 of 80 slots with pucch-Resource-r19: periodicityAndOffset as 20 slots for FR1, 15KHz SCS. The event instance count is set to 3 and the event threshold is set to 5dB.

Figure 1

As illustrated in Figure 1, the TE first transmits SSB#1 which is the current beam only during T1. At the start of T2, the TE transmits SSB#2 (new beam) in addition to SSB#1 whose RSRP level is 5 dB higher than that of SSB#1. Setting the SSB transmit power is done considering the right margin taking into account RSRP measurement accuracy requirements. The RSRP levels are kept constant throughout the duration of the test to reduce the test complexity. Since the eventInstanceCount is set to 3, the event reporting condition is met within the window and the UE shall send the indication not before the 3rd occurrence of the first PUCCH within the window and within the maximum event triggered reporting delay specified.

A similar test can be introduced for FR2 where the number of samples can be increased to account for the UE beam sweeping factor.

Introduce test cases for FR1 and FR2, Event-2, Intra-cell SSB with window time based and eventInstanceCount-r19 > 1.

Conclusion

In this paper, the following observations and proposals were made:

Proposal 1: Introduce test cases for FR1 and FR2, Event-2, Intra-cell SSB with window time based and eventInstanceCount-r19 > 1.

- References

- R4-2514803, WF on NR_MIMO_Ph5_RRM_Part1, RAN4#116bis

- R4-2520476, DraftCR TC for Intra-cell SSB-based UEIBM reporting in FR1, Nokia, RAN4#117