3GPP TSG-RAN WG4 Meeting #117						R4-2520478

# Dallas, USA, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on NR_MIMO_Ph6 RRM requirements

Agenda item:	7.10.3

Document for:	Discussion

Introduction

In this paper, we present Nokia’s views on the RRM requirements for early CSI/CSI-Rs/SRS triggering as well as cross-slot SRS.

Cross Slot SRS

Issue 2-1-1: Whether to introduce additional RRM requirements of SRS antenna port switching for cross-slot SRS

< Way forward > FFS the following options:

- Option 1 (QC, Huawei, Ericsson): No RRM impact.

- Option 2 (MTK, LG): Wait for further progress in RAN1.

  - MTK: wait for RAN1 conclusions and check whether the legacy interruption requirements are still applicable.

  - LG: Regarding cross-slot SRS, RAN4 to postpone the discussion until further progress in RAN1

- Option 3 (vivo):

  - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

  - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

- Option 4 (Samsung)

  - FFS on whether to extend the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot.

The following issues with respect to the cross-slot SRS were discussed in RAN4#116bis

On the topic of SRS antenna port switching, as it was pointed out by some companies in the last meeting, the current requirements are applicable only if the SRS resource(s) are configured within the last 6 symbols of the slot. Since the Rel-20 MIMO phase -6 introduces the possibility of a cross slot SRS between a special slot and an adjacent uplink slot within a single SRS resource set, the applicability of SRS antenna port switching requirements should be extended to cover this scenario as well.

Existing SRS antenna port switching requirements are applicable if the SRS resource(s) are configured within the last 6 symbols of the slot.

RAN4 should extend the applicability of SRS antenna port switching requirements to the cross slot SRS scenario.

RAN4 to wait for further progress in RAN1 before concluding about any further impact on legacy SRS antenna port switching requirements.

Issue 2-1-2: Whether to introduce additional RRM requirements of SRS carrier switching for cross-slot SRS

< Way forward > FFS the following options:

- Option 1 (QC, MTK, LG):

  - RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

- Option 2 (Huawei):

  - No RRM impact.

- Option 3 (vivo):

  - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

  - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

We believe that the current RAN1 agreements for cross-slot SRS do not necessitate introduction of additional RRM requirements for SRS based carrier switching. Hence, RAN4 should wait for further progress in RAN1 before discussing the need for additional RRM requirements for SRS based carrier switching.

Current RAN1 agreements do not require introduction of additional RRM requirements for SRS based carrier switching.

RAN4 to wait for further progress in RAN1 on cross-slot SRS before concluding about RRM impact for SRS based carrier switching.

Issue 3-1-1: Timeline of early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

< Way forward > FFS the following proposal:

- Proposal 1 (Ericsson):

- RAN4 shall first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, then RAN4 should clarify to RAN1 through LS that, as per the existing procedure, UE cannot transmit/receive from the RRCSetup reception until RRCSetup processing delay is complete. RRCSetup processing delay as per TS 38.331 is 10ms.

- RAN4 shall first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, then RAN4 should clarify to RAN1 that, as per the existing procedure, UE cannot transmit/receive from the RRCResume reception until RRCResume processing delay is complete. RRCResume processing delay as per TS 38.331 is 6 or 10ms based on different scenarios.

Early SRS/CSI/CSI-RS triggering during UE transition from IDLE to CONNECTED mode.

In the RAN 4#116bis meeting, companies commented that the UE may be unable to transmit on the UL or receive on the DL during RRC processing in case BWP switch is part of connection setup. In such a scenario, RAN4 has already defined BWP switching delay requirements and hence these requirements would apply in case the UE switches the BWP during transition from IDLE/INACTIVE to CONNECTED mode. Apart from this, we see no reason why UE cannot receive/transmit during RRC setup or RRC resume procedures.

Issue 3-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

< Way forward > FFS the following options:

- Option 1 (QC, Nokia):

  - RAN4 should wait for further progress in RAN1 for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode.

- Option 2 (MTK, Huawei)

  - Not to define any new RRM core requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode.

  - Option 2a (Apple):

    - RAN4 do not consider new timeline related requirements for early CSI triggering when UE transitions from IDLE to CONNECTED mode.

  - Option 2b (LGE):

    - Regarding CSI acquisition in RRC_IDLE to RRC_CONNECTED, RAN4 to postpone the discussion until any RRM impact is identified.

During transition from IDLE to CONNECTED mode, UE is able to transmit or receive during RRC processing. In case BWP is changed at connection setup, existing BWP switching requirements apply.

Discussions on the early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode are ongoing in RAN1. Up until now, it has been agreed in RAN1 that the MAC-CE of msg4 shall be used to trigger the early aperiodic CSI-RS/CSI/SRS while the PUSCH timelines for CSI reporting are yet to be discussed.

Currently, RAN4 has no requirements for transition between idle/inactive and connected mode. Hence, no existing requirement will be impacted by this objective. However, RAN4 should wait for more RAN1 agreements for this topic before discussing whether early triggering of CSI reporting or SRS transmission at UE transition from idle/inactive to connected mode has RAN4 impact.

There are currently no RAN4 requirements for UE transition from idle/inactive to connected mode or CSI reporting or SRS transmission related to this transition.

RAN4 shall conclude about any possible RRM impact due to CSI reporting and SRS transmission for UE transition from idle/inactive mode to connected mode, when RAN1 has made sufficient progress on this topic.

SCell transition from deactivation to activation

Currently, SCell activation delay requirements have been defined for various scenarios, of which the following may be impacted by early CSI-RS or SRS transmission:

- SCell Activation Delay Requirement for Deactivated SCell (8.3.2)

- SCell Activation Delay Requirement for Deactivated PUCCH SCell (8.3.12)

- SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells (8.3.7)

- SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells (8.3.13)

- SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode (8.3.2A)

- Fast SCell Activation Delay Requirement for Deactivated SCell (8.3.16)

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation (8.3.17)

- SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting (8.3.18)

- OD-SSB based SCell Activation Delay Requirement for Deactivated SCell (8.3.19)

- OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells (8.3.22)

- SCell Activation Delay Requirement for Deactivated PUCCH SCell based on OD-SSB (8.3.24)

- OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation (8.3.26)

Furthermore, there are features whose impact to these sections is captured within some existing section, such as shortMeasInterval-r18. RAN4 needs to eventually clarify the scope regarding all such features.

In the coming sections we present our views on the impacts to SCell activation delay requirements during early triggering of aperiodic CSI report and the associated aperiodic CSI-RS resources.

Early triggering of aperiodic CSI-RS and CSI report

As noted earlier, there are multiple scenarios for which SCell activation delay requirements have been defined. In the case of early triggering of aperiodic CSI report and CSI-RS, the discussion in our view can start from the following scenarios for SCell activation delay:

- SCell Activation Delay Requirement for Deactivated SCell (8.3.2)

- SCell Activation Delay Requirement for Deactivated PUCCH SCell (8.3.12)

Start discussing early triggering of CSI/CSI-RS based on:

- SCell Activation Delay Requirement for Deactivated SCell.

- SCell Activation Delay Requirement for Deactivated PUCCH SCell

The existing SCell activation delay has been defined for known and unknown SCell activation. In the case of early triggering of CSI/CSI-RS when SCell transitions from deactivated to activated, we propose that RAN4 first discuss requirements for known SCell and wait for RAN1 discussions on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI to progress before discussing whether and how to define requirements for unknown SCell.

RAN1 is yet to agree on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

RAN4 can start discussing impacts to known SCell activation delay and wait for RAN1 progress on CSI-RS triggering and reporting timeline before discussing unknown SCell activation.

The existing SCell delay requirements take into account the delay to acquire the CSI-RS and the UE processing time, when needed, and the time to the first available uplink resource to transmit the CSI report. These delay requirements are based on periodic and semi-persistent CSI-RS resources. CSI reporting delay is part of the delay regardless of if the SCell is known or unknown, and the delay is defined as an uncertainty parameter. The components related to CSI-RS triggering and measurement may be part of the delay if the SCell is unknown.

SCell activation delay requirements have been defined for periodic and semi-persistent CSI-RS resources.

The existing SCell activation delay requirements also factor in the time for the UE to transmit a CSI report.

In the case of early CSI reporting, RAN1 has agreed that the aperiodic CSI reporting is associated with at least aperiodic CSI-RS resources for CSI on the SCell.

The discussion about the timeline of the aperiodic CSI reporting and associated aperiodic CSI-RS resources is still open in RAN1, and it has been discussed that RAN4 can discuss and define the related requirements. It is however still unclear which part of the timeline will be up to RAN1 and what is RAN4 responsibility.

From RAN4 point of view, what can be said is that in the case of periodic and semi-persistent CSI-RS resources/CSI reporting, the UE has more than one opportunity to measure and report the CSI, which is not the case in aperiodic CSI reporting using aperiodic CSI-RS resources. Hence, even though the resources (CSI-RS and UL resources for aperiodic CSI reporting) are assigned by the network and are therefore to be considered as uncertainty factors on UE side, it is important that the transmission of the aperiodic CSI-RS resource and UL resources for the ensuing aperiodic CSI report are assigned such that the UE is actually in a position to measure and report. RAN4 can in our view discuss this timeline, and inform RAN1 about the conclusions.

UE has only one opportunity to measure and report when the CSI-RS resources and the CSI report are aperiodic in nature.

RAN4 to discuss in relation to SCell activation delay when the UE is ready to receive the aperiodic CSI-RS resources and transmit the aperiodic CSI report.

Early triggering of SRS-AS

For early triggering of SRS-AS, when SCell transitions from deactivated to activated state, the SCell activation delay requirements need to be studied to determine when the UE shall be ready for the aperiodic SRS transmissions. As the SRS is transmitted on the SCell, PUCCH SCell activation requirements shall be studied. RAN4 can start by discussing single PUCCH SCell activation delay requirement.

RAN4 to discuss the timeline when the UE shall be ready to transmit the aperiodic SRS in relation to PUCCH SCell activation delay.

SCell transition from dormant to non-dormant BWP

Issue 3-3-1: RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

< Way forward > FFS the following options:

- Option 1 (MTK):

  - For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable.

- Option 2 (CATT, Huawei, LGE, Apple, Samsung, Nokia):

  - Wait for more RAN1 progress.

Agreement:

For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field

- FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4)

Agreement:

For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting for the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- The aperiodic CSI reporting is associated with aperiodic CSI-RS for CSI on the non-dormant BWP

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field

- FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

In RAN1#112bis meeting the following was agreed for early CSI triggering and SRS transmission when SCell transitions from dormant to non-dormant BWP.

Since the UE shall transmit the aperiodic SRS/aperiodic CSI report and receive the aperiodic CSI-RS associated to the aperiodic CSI report on the non-dormant BWP, this naturally means that the reception/transmission happens after the ending point of BWP switching delay. This in our understanding means that BWP switching delay is not impacted, and hence with the current agreements, there would be no RAN4 impact. However, before concluding about this, we think RAN4 should follow up on any further RAN1 agreements about the timeline.

With current RAN1 agreements, we believe there are no impacts on RAN4 requirements.

RAN4 shall wait for RAN1 to conclude on the timelines of the aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP before concluding on RAN4 requirement impact.

Conclusion

In this paper, the following observations and proposals were made:

Observation 1: Existing SRS antenna port switching requirements are applicable if the SRS resource(s) are configured within the last 6 symbols of the slot.

Proposal 1: RAN4 should extend the applicability of SRS antenna port switching requirements to the cross slot SRS scenario.

Proposal 2: RAN4 to wait for further progress in RAN1 before concluding about any further impact on legacy SRS antenna port switching requirements.

Observation 2: Current RAN1 agreements do not require introduction of additional RRM requirements for SRS based carrier switching.

Proposal 3: RAN4 to wait for further progress in RAN1 on cross-slot SRS before concluding about RRM impact for SRS based carrier switching.

Proposal 4: During transition from IDLE to CONNECTED mode, UE is able to transmit or receive during RRC processing. In case BWP is changed at connection setup, existing BWP switching requirements apply.

Observation 3: There are currently no RAN4 requirements for UE transition from idle/inactive to connected mode or CSI reporting or SRS transmission related to this transition.

Proposal 5: RAN4 shall conclude about any possible RRM impact due to CSI reporting and SRS transmission for UE transition from idle/inactive mode to connected mode, when RAN1 has made sufficient progress on this topic.

Proposal 6: Start discussing early triggering of CSI/CSI-RS based on:

 SCell Activation Delay Requirement for Deactivated SCell.

 SCell Activation Delay Requirement for Deactivated PUCCH SCell

Observation 4: RAN1 is yet to agree on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

Proposal 7: RAN4 can start discussing impacts to known SCell activation delay and wait for RAN1 progress on CSI-RS triggering and reporting timeline before discussing unknown SCell activation.

Observation 5: SCell activation delay requirements have been defined for periodic and semi-persistent CSI-RS resources.

Observation 6: The existing SCell activation delay requirements also factor in the time for the UE to transmit a CSI report.

Observation 7: In the case of early CSI reporting, RAN1 has agreed that the aperiodic CSI reporting is associated with at least aperiodic CSI-RS resources for CSI on the SCell.

Observation 8: UE has only one opportunity to measure and report when the CSI-RS resources and the CSI report are aperiodic in nature.

Proposal 8: RAN4 to discuss in relation to SCell activation delay when the UE is ready to receive the aperiodic CSI-RS resources and transmit the aperiodic CSI report.

Proposal 9: RAN4 to discuss the timeline when the UE shall be ready to transmit the aperiodic SRS in relation to PUCCH SCell activation delay.

Observation 9: With current RAN1 agreements, we believe there are no impacts on RAN4 requirements.

Proposal 10: RAN4 shall wait for RAN1 to conclude on the timelines of the aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP before concluding on RAN4 requirement impact.

- References

- R4-2514825  	WF on NR_MIMO_Ph6_RRM