3GPP TSG-RAN WG4 Meeting 117	R4-2520490

Dallas, USA, Nov. 17-21, 2025

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility

Agenda Item:	7.12.4

Document for:	Discussion

# 1 Introduction

This contribution focuses on establishing a framework for the core requirements of AI-based RRM measurement and event prediction. We identify and discuss two critical, interlinked questions that must be resolved by RAN4 to enable subsequent normative work: the composition of the prediction report and the corresponding performance metric.

# 2 Discussion

Event triggered reporting delay

As agreed in TR, there are two methods to predict measurement event, namely indirect and direct measurement event prediction.

In legacy, RAN4 defined event triggered L3 reporting delay in core part and define corresponding test case to verify whether UE will correctly detect the event by L3 reporting.

Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part.

Similarly, for measurement event prediction, it may have impact on predicated event triggered L3 reporting delay. RAN4 also needs to verify UE behaviour whether event can be correctly predicted. It will include the components of measurement time for event prediction.

Proposal 1: RAN4 to define predicted event triggered L3 reporting delay.

Performance metric for report

RAN2 #131 Agreements 1 For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters) 2 For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) 3 For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

Based on the conclusions from the RAN2 #131 meeting, the event prediction report is defined to contain two key elements: RRM measurements and the predicted timing information of the event. Consequently, RAN4 must define requirements for both of these reported quantities.

For the first element – the RRM measurements – a fundamental question remains regarding whether these values consist entirely of predicted results or represent a hybrid of both measured (actual) and predicted values.

Fig.1 illustrates how the TTT (Time-to-Trigger) window for event prediction may span different combinations of the Observation Window (OW, containing actual measurements) and Prediction Window (PW, containing predicted values). The event triggering decision can be based on:

- Case 1: Data exclusively from OW (legacy-like behavior)

- Case 2: A hybrid combination of data from both OW and PW

- Case 3: Data exclusively from PW (full prediction scenario)

The composition of the reported RRM measurements depends on which portion of the TTT window triggers the event, highlighting the need for requirements that account for this hybrid nature of measurement data.

Figure 1: Composition of Measurement Data in Event Prediction Triggering

Observation 2: The event prediction report may inherently contain a hybrid of both measured (from OW) and predicted (from PW) RRM values, as the TTT window for event evaluation can span both observation and prediction periods.

Observation 3: The TTT parameter, which was not considered in legacy measurement requirements, becomes crucial in AI-based prediction as it directly determines the composition of measured versus predicted values used for event triggering decisions.

Proposal 2: The TTT configuration needs to be explicitly considered in defining requirements for event prediction.

Proposal 3: RAN4 to define requirement for RRM results in predicted event triggered reporting.

For the second element – the time-related information – a core requirement must be established for the timing accuracy of the predicted event. This entails defining a requirement for the maximum permissible time difference between the predicted time of event occurrence and the actual ground-truth time of its occurrence.

The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1:

Figure 5.3.1-1: illustration of ETD

As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

Observation 4: RAN2 has defined the Event Time Difference (ETD) as a key performance metric for evaluating the timing accuracy of event prediction. RAN4 needs to specify a requirement on the timing error.

Proposal 4: RAN4 to define performance metric for Event Time Difference (ETD).

- Conclusion

In this contribution, we make the following proposals:

Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part.

Proposal 1: RAN4 to define predicted event triggered L3 reporting delay.

Observation 2: The event prediction report may inherently contain a hybrid of both measured (from OW) and predicted (from PW) RRM values, as the TTT window for event evaluation can span both observation and prediction periods.

Observation 3: The TTT parameter, which was not considered in legacy measurement requirements, becomes crucial in AI-based prediction as it directly determines the composition of measured versus predicted values used for event triggering decisions.

Proposal 2: The TTT configuration needs to be explicitly considered in defining requirements for event prediction.

Proposal 3: RAN4 to define requirement for RRM results in predicted event triggered reporting.

Observation 4: RAN2 has defined the Event Time Difference (ETD) as a key performance metric for evaluating the timing accuracy of event prediction. RAN4 needs to specify a requirement on the timing error.

Proposal 4: RAN4 to define performance metric for Event Time Difference (ETD).