# 3GPP TSG-RAN WG4 Meeting #117	R4-2520504

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.2.5.1

Source: 	Xiaomi

Title: 	View on Channel arrangement

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In this contribution, our preliminary view on channel raster, sync raster and channel spacing is provided.

- Discussion

Channel raster

Channel raster defines RF reference frequencies to identify channel position for both DL and UL (which always located in the sub-carrier which closet to channel centre with half SCS offset to right side). A global frequency raster is defined between 0-100GHz as NR-ARFCN.

And band specific band channel raster is also defined as multiple times of basic global channel raster granularity.  Considering 5G co-existence with 4G, 100kHz channel raster specified for LTE refarming bands. This brought some limitation on spectrum usage especially on less than 10MHz CHBW and enhanced channel raster feature was introduced in later stage of NR.

Considering network deployment flexibility, and spectrum usage flexibility, we didn’t see the motivation to restrict channel raster for each band. Especially, for NR, the RB allocation and SSB placement was designed based on SSB placement not related to channel raster.

Observation 1: Channel raster concept majorly existed in RAN4 for channel position indication and conformance test

Observation 2: Global channel raster with basic granularity as per sub-frequency range basis and per band channel raster with multiple step-size was introduced in NR

Figure 1: Global channel raster + band specific step-size in NR

Observation 3: From RAN1/RAN2 perspective, RB mapping/allocation was designed based on SSB position and reference point A

Observation 4: The concept of per band wide channel raster especially large channel raster i.e., 100kHz channel raster bring restriction on efficient spectrum usage and channel placement especially for irregular BW supporting and enhanced 10kHz channel raster was introduced in later stage for specific bands which originally have 100kHz channel raster.

Observation 5: Supporting 100kHz channel raster on LTE refarming bands in NR bring sync raster shift issue with 3 candidates on each raster location (100 kHz offset)

Proposal 1: Support 5kHz channel raster in day 1 for below 3GHz

Proposal 2: RAN4 further study whether 100kHz channel raster still required for below 3GHz bands

Proposal 3: Remove per band channel raster concept with following value per sub-frequency range basis

| Frequency range | Channel raster |
| --- | --- |
| <3GHz | 5kHz |
| 3GHz ~ 24.25kHz | 30kHz |
| 24.25GHz ~ 52GHz | 120kHz |

Sync raster

On sync raster, generally sync raster location was decided based on following equation with principle that 1) minimize candidate SS raster locations for initial cell search complexity; 2) allow each BW, at least one SS entries can be fully contained:

SS raster > = Minimum BW (RB)- SSB BW (RB) + channel raster

A global sync raster is defined for all frequencies (GSCN) which can be mapped to the frequency location if SS block.

The Sync raster was used to indicate SS block location of initial cell search.  It’s work to simplify sync raster design by consideration of following principle: 1) facilitate UE initial cell search (complexity/initial search time/power consumption 2) flexibility of SSB placement for 6GR spectrum allocation

Those sync raster design is still pending on RAN1 initial cell search design on SSB and 6GR minimum CHBW definition.

Given minimum CHBW on each band can be varied, per band step-size still can be helpful to reduce number of raster and save UE power consumption and initial cell search time.

Observation 6: Global sync raster with basic granularity per sub-frequency range and band specific step-size was introduced NR.

Figure 2: Global sync raster + band specific step-size in NR

Proposal 4: Postpone sync raster discussion until sufficient progress reached in RAN1 on 6GR initial cell search design e.g., no early Q2’ 2026

Proposal 5: Further evaluate to simplify sync raster to facilitate UE initial cell search (complexity/initial search time/power consumption vs flexibility for SSB placement) with potential area

- Flexible step size per sub-frequency range/per band

- Scalable step-size pending on SSB periodicity

- SS raster design for MRSS

Channel spacing

RAN4 introduce channel spacing between carriers as “normal channel spacing” which was majorly used for RAN4 requirements and conformance test. There is no restriction on real network deployment as well the channel spacing is less than “normal channel spacing”.

In NR mixed numerologies cases for carriers in the same band bring complicated specification definition. Given in 6GR, single numerology target to be introduced for band/sub-frequency range. We suggest to simplify channel spacing without consideration of mixed numerology case in 6GR.

The nominal channel spacing was also used to discriminate intra-band contiguous CA and non-contiguous CA with different RF architecture assumption. In Rel-19 RAN4 also studied the possibility of using single RF chain for intra-band non-contiguous CA. RAN4 may also need to further study the usage of “nominal channel spacing” to discriminate intra-band contiguous CA and non-contiguous CA.

Observation 7: “Normal channel spacing” concept was introduced for RAN4 requirements and conformance test. There is no restriction on real network deployment.

Observation 8: Mixed numerologies case were considered in NR on the same band which bring specification over complicated.

Proposal 6:  Simplify Channel spacing definition without consideration of mixed numerology case in 6GR.

Proposal 7: Further study “normal channel spacing” definition and the relation-ship between intra-band contiguous CA and NC CA.

- Conclusion

Channel raster

Observation 1: Channel raster concept majorly existed in RAN4 for channel position indication and conformance test

Observation 2: Global channel raster (basic granularity as per sub-frequency range basis) plus per band channel raster with multiple step-size was introduced in NR

Figure 1: Global channel raster with Per band step-size for channel raster in NR

Observation 3: From RAN1/RAN2 perspective, RB mapping/allocation was designed based on SSB position and reference point A

Observation 4: The concept of per band wide channel raster especially large channel raster i.e., 100kHz channel raster bring restriction on efficient spectrum usage and channel placement especially for irregular BW supporting and enhanced 10kHz channel raster was introduced in later stage for specific bands which originally have 100kHz channel raster.

Observation 5: Supporting 100kHz channel raster on LTE refarming bands in NR which bring sync raster shift issue with 3 candidate locations

Proposal 1: Support 5kHz channel raster in day 1 for below 3GHz

Proposal 2: RAN4 further study whether 100kHz channel raster still required for below 3GHz bands

Proposal 3: Remove per band channel raster concept with following value per sub-frequency range basis

| Frequency range | Channel raster |
| --- | --- |
| <3GHz | 5kHz |
| 3GHz ~ 24.25kHz | 30kHz |
| 24.25GHz ~ 52GHz | 120kHz |

Sync raster

Observation 6: Global sync raster (basic granularity per sub-frequency range) plus band specific step-size was introduced NR.

Figure 2: Global sync raster + band specific step-size in NR

Proposal 4: Postpone sync raster discussion until sufficient progress reached in RAN1 on 6GR initial cell search design e.g., no early Q2’ 2026

Proposal 5: Further evaluate to simplify sync raster to facilitate UE initial cell search (complexity/initial search time/power consumption vs flexibility for SSB placement) with potential area

- Flexible step size per sub-frequency range/per band

- Scalable step-size pending on SSB periodicity

- SS raster design for MRSS

Channel spacing

Observation 7: “Normal channel spacing” concept was introduced for RAN4 requirements and conformance test. There is no restriction on real network deployment.

Observation 8: Mixed numerologies case were considered in NR on the same band which bring specification over complicated.

Proposal 6:  Simplify Channel spacing definition without consideration of mixed numerology case in 6GR.

Proposal 7: Further study “normal channel spacing” definition and the relation-ship between intra-band contiguous CA and NC CA.

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514650,“WF on 6GR system parameter”, Feature Lead (Huawei)