# 3GPP TSG-RAN WG4 Meeting #117	R4-2520505

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.2.5.1

Source: 	Xiaomi

Title: 	View on Device type and RF/BB capabilities

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In RAN4#116bis, following agreements were reached on device Type:

# Topic #6: Number of Tx and Rx - Agreement - Study the framework of spec impact with different number of Tx/Rx - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective. - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design - Co-ordinate with RAN and RAN1 for number of Tx/Rx # Topic #7: Device types - Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

In this contribution, our preliminary view on device type and RF/BB capabilities are provided.

- Discussion

2.1 Overall scope

In last RAN4 meeting, initial discussion on device types was triggered under 6G system parameter agenda. Companies have questions and debats on how to organize the discussion related to device type:

- Why RAN4 needs to discuss device type ?

- What’s the RAN4 scope on device type ?

- What’s the relationship between RAN, other WGs and RAN4 discussion ?

To address above questions, first of all we need to clarify what’s current objective in 6G SID and what’s the RAN-P decision/guidance related device type.

Refer to 6G SID, relavant objectives related to device type reproduced belowL

6G SID (RP-252912): - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. …… - 6GR core and performance requirements - UE RF requirement aspects including band and band combination [RAN4] - Study UE RF capabilities considering different device types and implementations

Observation 1: According to 6G SID, 6GR target to have ”scalable and forward compatible design for diverse device types” which involved both RAN4 and other WGs on this objective.

Observation 2: According to 6G SID, RAN4 also needs to study UE RF capabilities considering different device types and implementations.

Regarding device type and minimum BW discussion, RAN#109 made following agreements:

RAN#109 - In RP-252873, proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion) Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. FFS: whether RF and BB UE BW are same or different Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

Observation 3: The standalization and defintion of device types belong to RAN-P decision including possible parameters/factors.

After the clarification of existing 6G SID objective and RAN-P agreement, then we need to clarify the expected RAN4 work and how to organize RAN4 discussion properly with tight coordination with RAN-P and other WGs i.e. items with cross WG impact, items majorly related to RAN4.

In 5G design, the initial target was for eMBB UE which brought NBC issue when new device type with less Rx antenna capability e.g., XR, Redcap and Vehicular device types and some exception was required on these device types. Due to coverage issue, cell barring scheme was introduced in NR to avoid the impact to network performance.

In order to have harmoic design from system perspective, 6GR common design need to support diverse device type in the same cell.

Obseravtion 4: 6GR design target to support diverse device types in the same cell.

Figure 1: 6GR harmonic system design for diverse device types

We need to identify the fundemental parameters/factors related to 6GR design on supporting diverse device types and then the baseline RF/BB capabilities on identified key parameters for the low tier UE that 6GR system shall support. Another key point related to coverage evaluation, 6GR target to provide same coverage as 5G mid-band (~3.5 GHz) on 7GHz around.

From system design perspective, 6GR needs to clarify minimum CHBW for common channel design including cell search, random access, and DL/UL scheduling. The minimum CHBW related to low-tier UE capabilities and also related avaliable spectrum in real deployment. The CHBW issue was discussed in another agenda in our t-doc xxx.

And then, RAN4 also needs to evaluate RF/BB capabilities considering  implementation feasibility and constraints on the key parameters which related to whole system design e.g. mandatory CHBW, PC and number Tx/Rx related coverage.

Proposal 1: RAN4 focus on the evaulation of the mandatory RF/BB capabilities with different device assumption considering  implementation feasibility and constraints

- Target to identify fundemental parameters/factors related to 6GR common system design and coverage (Tight cooperation with RAN1)

- Provice necessary input to RAN-P to assist the discussion on device type defintion

- Urgent to be concluded in intial stage on identified fundemental parameters for overall system design

Another issue related to UE capability signaling design and structure, RAN4 develop basisc mandatory and optional capabilties for each individual features which related to RF/BB as per feature basis and different capability reporting granuruity type e.g. per UE, per band, per BC.  6GR may require joint effort from RAN-P/RAN2/RAN1 and RAN4 to optimize capability signaling structure to reduce UE capability signaling reporting overhead and conformance test cost.

RAN4 also can discuss capabilities sets assioated with different device assumption and dynamic UE capabilities to optimize UE capability reporting and scheduling.

Proposal 2: RAN4 also can further discuss UE RF/BB capability sets and dynamic UE capability

- This work have RAN-P and RAN2 dependency

- No urgency to discuss in intial stage

In NR, RAN4 also introduce several requirements and features which have form factor dependency e.g., some PCs only target for non-handheld UE, FWA or CPE, separate requirements are specified based on UE form factor and some feature only applicable for certain device types e.g., 6RX with 4 layer and 6-layer feature. RAN4 requirements for 6Rx capabilities can be summarized in below table:

Figure 2: 6Rx capabilities and applicable requirements for handheld UE and FWA

Proposal 3: RAN4 also needs to study how to handle and discriminate different device assumption/form factors  from RAN4 RF/BB requirements perspective

- Target to have scalable RF/RRM/Demod requirement framework to support diverse device assumption especailly on form factor dependency requirements

- Not necessary treated as device types/capabilities

- Majorly RAN4 internal study, less cross WG dependency

- Discuss under UE RF, RRM and demod agenda separately

With above analysis on RAN4 scope related device type, a workflow and relationship with RAN-P and RAN1/RAN2 are provided below as example.

Figure 3: Workflow on RAN4 device type related and relationship with RAN1/RAN2 and RAN-P

2.2 RF/BB capabilities

Fundemental parameters related to device assumption

Typically device types shall consider many factors related to communication capacity e.g. use case, form factor, cost, coverage, battery life and peak data rate. From RAN4 perspective, we shall transfer above key KPIs into fudemental parameters relared RF/BB capacities/capabilities.

As mentioned in above section, RAN4 needs to identify the fundemental RF/BB parameters/factors which is related to whole system design especially related to coverage considering limitations of form factor. Taking NR experience, key parameters related different device types as summarized in below table:

Table 1: Fundemental RF/BB parameters for NR

| Parameters | R15 NR eMBB (Handheld UE) (Mandatory value) | NR Redcap | NR eRedCap | eMTC |
| --- | --- | --- | --- | --- |
| MIMO Tx/Rx | 1Tx/2Rx 2Rx TDD FR2 | 1 or 2Rx FR1, 2Rx FR2 | 1 or 2Rx FR1 | 1 Tx/Rx |
| Max. BW | 100 MHz FR1 400 MHz FR2 | 20MHz FR1 100 MHz FR2 | RF 20 MHz + BB 5MHz FR1 Not support FR2 | 1.4 MHz |
| Duplex | Duplex FDD TDD | Type A HD-FDD TDD | Type A HD-FDD TDD | Type B HD-FDD |
| Max. MIMO layer | 2 layers 2 layers FR2 | FR1: 1 DL layer (if 1 Rx branch) 2 DL layers (if 2 Rx branches) FR2: 1 or 2 DL layers (2 Rx branches are always supported) | 1 DL layer (if 1 Rx branch) 2 DL layers (if 2 Rx branches) | 1 |
| Max. Modulation | UL 64QAM DL 256QAM | DL 64QAM | DL 256QAM (optional) FR1 | Not support 64QAM in UL |
| Peak data rate | ~1Gbps | ~150Mbps | ~10Mbps | ~1Mbps |
| RRM Mobility | Normal | Relaxed RRM measurement | Relaxed RRM measurement |  |
| Same network | YES Note1: Mechanism on access control for Redcap UE Note2: Physical enhanced schemes for coverage enhancement on Redcap /MTC UE |  |  |  |

Based on above table, we can identify PC, number of Tx/Rx can be key parameters related to coverage. Number of Tx/Rx (MIMO layers), CHBW, and modulation orders are related to peak data rate.  In additional to coverage, peak data rate; all these parameters also have dependency of use cases with form factor and cost consideration. Also taking NR experience, at least Redcap like device and MBB device (handheld and FWA) needs to consider separately from fundamental RF/BB capabilities perspective due to difference on form factor restriction and use case.

Proposal 4: RAN4 focus on the fundamental RF/BB parameters as following which considering use cases, form factor limitation.

- Number of Tx/Rx

- Maximum CHBW

- Power class

- Modulation orders

- Duplex mode

- RRM mobility

Proposal 5: From RAN4 mandatory capabilities on fundamental RF/BB parameters, at least to discriminate as IoT device (e.g., Redcap like) and MBB device (e.g., handheld UE and FWA)

- FFS whether need to consider FWA separately

- RAN4 target to develop mandatory values on the identified fundamental RF/BB parameters as per device assumption basis

  - Additional optional options on top of mandatory values can be discussed in later stage

Regarding CHBW, power class and modulation orders which will be discussed in separate agendas. On Duplex mode, at least FDD, TDD and HD-FDD will be considered in separate agenda.

On number of Tx/Rx antennas, there are operating frequency dependency for antenna size and isolation distance between antennas. Due to the limited size of handheld UE, the possible placement of antenna is quite limited with complexed transmitter chain and receiver chain. The antenna gain, antenna efficiency, and antenna isolation across different antenna group can be diverse which is also pending operating mode (free space, bottom hold, upper hold, talking mode (head hand mode) or browsing/game mode). Due to the antenna gain imbalance, antenna isolation, and phase mismatch across different antenna groups, increasing number of simultaneous operated Tx/Rx antennas can’t achieve expected ideal gain in reality and the performance is also pending on operating modes. To increase UL coverage, increasing power class is more promising method than increasing number of antennas.

Figure 4: Possible antenna placement on handheld UE

UE also may switch on/off antennas under different operating modes (power saving mode vs high performance mode /game mode) and different operating state for foldable UE with different antenna performance under fold state and open state. This also can bring impact on number of Tx/Rx capacities for same device. Dynamic UE capability reporting can be considered to achieve better performance under different states.

Proposal 6: For MBB UE, mandatory capabilities on fundamental RF/BB parameters can be further decided by per sub-frequency rang basis

| Parameter | <2GHz FDD bands | 2GHz ~5GHz TDD bands | ~ 7GHz (6.425~7.125GHz;7.125 GHz – 8.4 GHz) |
| --- | --- | --- | --- |
| Duplex Mode | FDD | TDD | TDD |
| SCS | 15kHz | 30kHz | 30kHz |
| CHBW 8K FFT baseline | Minimum: 5MHz/3MHz (below 1GHz only) Maximum: 100MHz/50MHz? | Minimum: 10MHz Maximum: 100MHz/200MHz | Minimum :20MHz Maximum: 200MHz |
| Modulation order | DL/UL: 256QAM/64QAM | Possibility of supporting DL 1024QAM | Possibility of supporting DL 1024QAM |
| Power class | PC3 | PC2 | PC2 |
| MIMO Tx & Rx | 1Tx/2Rx | 1Tx/4Rx as baseline 2Tx, 6Rx as optional | 2Tx/4Rx as baseline 3Tx/4Tx, 6Rx as optional |

Proposal 7: UE capability update can be further studied e.g.  operating state for foldable device, operating mode of MBB device (power saving mode, high performance mode)

- Cooperation between NW and UE required for UE capability update e.g., conditional BS mandatory feature set/configuration

Figure 4: UE RF/BB capability framework

- Conclusion

In this contribution, we provide preminary views on device type.

Overall RAN4 scope

Observation 1: According to 6G SID, 6GR target to have ”scalable and forward compatible design for diverse device types” which involved both RAN4 and other WGs on this objective.

Observation 2: According to 6G SID, RAN4 also needs to study UE RF capabilities considering different device types and implementations.

Observation 3: The standalization and defintion of device types belong to RAN-P decision including possible parameters/factors.

Obseravtion 4: 6GR design target to support diverse device types in the same cell.

Proposal 1: RAN4 focus on the evaulation of the mandatory RF/BB capabilities with different device assumption considering  implementation feasibility and constraints

- Target to identify fundemental parameters/factors related to 6GR overall system design and coverage (Tight cooperation with RAN1)

- Provice necessary input to RAN-P to assist the discussion on device type defintion

- Urgent to be concluded in intial stage for mandatory capabilities on identified fundemental parameters for overall system design

Proposal 2: RAN4 also can further discuss UE RF/BB capability sets and dynamic UE capability

- This work have RAN-P and RAN2 dependency

- No urgency to discuss in intial stage

Proposal 3: RAN4 also needs to study how to handle and discriminate different device assumption/form factors  from RAN4 RF/BB requirements perspective

- Target to have scalable RF/RRM/Demod requirement framework to support diverse device assumption especailly on form factor dependency requirements

- Not necessary treated as device types/capabilities

- Majorly RAN4 internal study, less cross WG dependency

- Discuss under UE RF, RRM and demod agenda separately

Figure 3: Workflow on RAN4 device type related scope and relationship with RAN1/RAN2 and RAN-P

RF/BB capabilities

Proposal 4: RAN4 focus on the fundamental RF/BB parameters as following:

- Number of Tx/Rx

- Maximum CHBW

- Modulation orders

- Duplex mode

- Power class

- RRM mobility

Proposal 5: From RAN4 mandatory capabilities on fundamental RF/BB parameters, at least to discriminate as IoT device (Redcap like) and MBB device which considering use cases, form factor limitation

- FFS whether need to consider FWA separately

- RAN4 target to develop mandatory values on the identified fundamental RF/BB parameters as per device assumption basis

  - Additional optional options on top of mandatory values can be discussed in later stage

Proposal 6: For MBB UE, mandatory capabilities on fundamental RF/BB parameters can be further decided by per sub-frequency rang basis

Table 2: Mandatory capabilities on fundamental RF/BB parameters for MBB UE

| Parameter | <2GHz FDD bands | 2GHz ~5GHz TDD bands | ~ 7GHz (6.425~7.125GHz; 7.125 GHz – 8.4 GHz) |
| --- | --- | --- | --- |
| Duplex Mode | FDD | TDD | TDD |
| CHBW 8K FFT baseline | Minimum: 5MHz/3MHz (below 1GHz only) Maximum: 100MHz/50MHz? 15kHz SCS | Minimum: 10MHz Maximum: 100MHz/200MHz 30kHz SCS | Minimum :20MHz Maximum: 200MHz 30kHz SCS |
| Modulation order | DL/UL: 256QAM/64QAM | Possibility of supporting DL 1024QAM | Possibility of supporting DL 1024QAM |
| Power class | PC3 | PC2 | PC2 |
| MIMO Tx & Rx | 1Tx/2Rx | 1Tx/4Rx as baseline 2Tx, 6Rx as optional | 2Tx/4Rx as baseline 3Tx/4Tx, 6Rx as optional |

Proposal 7: UE capability update can be further studied e.g.  operating state for foldable device, operating mode of MBB device (power saving mode, high performance mode)

- Cooperation between NW and UE required for UE capability update e.g., conditional BS mandatory feature set

Figure 4: UE RF/BB capability framework

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514650,“WF on 6GR system parameter”, Feature Lead (Huawei)