# 3GPP TSG-RAN WG4 Meeting #117	R4-2520508

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.6

Source: 	Xiaomi

Title: 	Overview for 6GR spectrum

Document for:	Discussion

- Introduction

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. Spectrum is one of the area with following candidate scope:

According to the 6G SID, 6GR target frequency includes FR1, FR2-1 and frequency range between 7GHz ~ 24GHz:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Compared to 5G system, the brand new clean spectrum for 6GR day 1 deployment is rare. Following spectrum sharing/co-existence scenarios are observed:

- Spectrum sharing/co-existence with legacy system (4G and 5G) on existing reframing frequency in FR1 and FR2

- 6G system co-existence with incumbent services in FR3 e.g. radar, military, and so on

In this contribution, our preliminary view on 6G spectrum is provided on following areas:

- WRC-27/regulation situaion

- Band and CA simplification

- Frequency range definiton

- Discussion

  - Spectrum related regulatory survey

2.1.1 WRC-27 agenda item 1.7: Spectrum Identification for International Mobile Telecommunications (IMT)

Agenda Item 1.7 for the 2027 World Radiocommunication Conference (WRC-27) focuses on studying the potential identification of additional frequency bands for International Mobile Telecommunications (IMT). This includes current and future generations of mobile networks such as 5G, 6G, and IMT-2030.

Frequency Bands Under Study:

- 4 400–4 800 MHz

- 7 125–8 400 MHz

- 14.8–15.35 GHz

These bands are being considered to support the growing demand for mobile broadband, especially for high-capacity, low-latency applications expected in 6G and beyond.

Key Technical and Regulatory Considerations:

- Spectrum requirements for future IMT systems

- Compatibility and sharing studies with existing services (e.g., satellite, fixed, and scientific services)

- Feasibility of regulatory measures to ensure coexistence

- Global or regional harmonization to support economies of scale and cross-border coordination

As mobile data traffic continues to surge, new mid-band spectrum is essential to balance coverage and capacity. AI 1.7 is critical for preparing the regulatory framework for 6G, ensuring that sufficient spectrum is available to meet future connectivity demands.

Observation 1-1: The ITU-R is working on searching and identification new spectrum for IMT 6G, the potential frequency bands are 4 400–4 800 MHz, 7 125–8 400 MHz and 14.8–15.35 GHz.

During the meeting of WP5D#49, discussions of AI1.7 were held within the Working Parties (DGs) and Study Working Groups (SWG) to update the sharing and compatibility working document. The updates integrated studies presented during the session, accompanied by explanatory notes to clarify assumptions, methodologies, and key points raised. While all documents were introduced, some studies and associated notes were not fully reviewed or finalized, with conclusions pending further deliberation at the next WP 5D meeting (as indicated in Editor’s Notes of each attachment). Notably, WP 5D decided to create a separate attachment in Chapter 4 of its Chair’s Report to address potential interference from incumbent systems into IMT (International Mobile Telecommunications), ensuring focused treatment of this critical issue. Other works during WP5D#49 AI1.7:

- ITU-R P.2108 Model Application: In-depth discussion on adapting the Clutter Loss Model within WP 5D research frameworks.

- Document Structure Optimization: Development of dedicated working documents for each of AI1.7’s three frequency bands, with individual attachments recording input studies per scenario.

- CPM Text Development: Draft Conference Preparatory Meeting (CPM) Text established to evaluate frequency allocation feasibility, outlining multiple options for WRC (World Radiocommunication Conference) deliberations to narrow down scope.

- Coexistence Methodologies Enhancement: Continued refinement of technical approaches for IMT-incumbent system coexistence studies.

Sub-working group IMT characteristics mainly focusing on drafting the working document. There were two unresolved issues in the working document on “Characteristics of the terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”: technical characteristics and operational parameters for IMT UE (wall-mounted CPEs) and the protection criteria for IMT. Unfortunately, although these issues were discussed until Closing Plenary, no agreements were reached. Consequently, WP 5D decided to retain this working document within WP 5D and not send it out to other Working Parties responsible for WRC-27 agenda items.

Sub-working group IMT-MSS was working on drafting a possible liaison statement to WP4C with request for more specific information on technical matters from WP 4C. However, different views were expressed at the WG session about the LS is needed for WP4C and what the contents and questions should be involved in the LS. No agreement and the WP5D decided not to send this further LS as this time. Protection criteria of terrestrial IMT from DC-MSS-IMT systems is anther important topic of this SWG. The feasibility and impact of IMT protection criteria related to throughput loss in single-cell and multi-cell scenarios were discussed. A new drafting group “DG AI1.13 Technical Consideration” was established. This DG discussed potential approaches to developing protection measures. Though agreement for protection of IMT was not achieved, it was agreed that further discussions on possible approaches would continue once more detailed technical information was received from WP 4C.

Regarding compatibility studies for Agenda item 1.7, many countries and organizations have submitted sharing and compatibility study results. Due to varying assumptions and parameters from different parties, the findings in these proposals demonstrate divergent tendencies. During the WP5D discussions, intense debates have taken place among stakeholders over the differing assumptions, parameters, and outcomes, making consensus currently elusive. In these discussions, some nations have proposed conducting reverse compatibility studies (specifically examining interference from existing services to IMT systems) to achieve a comprehensive analysis of all interference scenarios and possibilities. However, this approach compounds the uncertainties surrounding AI1.7 and complicates its identification process.

The three frequency bands under study all present complex scenarios involving multiple interference situations. Not all countries show interest or hold a positive stance toward the new AI1.7 frequency band. Many nations consider the 6425-7125MHz band to be the primary frequency range for 6G. Decisions regarding whether each band will be identified for IMT, in which regions such identification will apply, and the restrictions on IMT usage for each band must be determined based on sharing and compatibility study results over the next two years. Final conclusions will be reached during the WRC-27 to be held in Shanghai in November 2027.

Observation 1-2: The sharing and compatibility studies of WRC-27 agenda item 1.7 in under study, the identification methods, requirements, and regulations for the new bands for IMT are also under discussion in ITU-R.

Observation 1-3: 6425-7125 MHz is regarded by multiple nations as the primary frequency band for 6G.

2.1.2 WRC-27 Agenda Item 1.13 – Mobile-Satellite Service (MSS) in the IMT Range

Study possible primary allocations to the Mobile-Satellite Service (MSS) in the 694 MHz – 2.7 GHz range so that MSS systems—especially Direct-Connectivity MSS (DC-MSS) that link ordinary handsets to NGSO or GSO satellites—can operate in the same bands already harmonized for terrestrial IMT.

Key tasks & expected outcome of WRC-27 Agenda Item 1.13 can be summarized as: set interference limits that let MSS share IMT bands without harming 5G, and deliver a sharing report plus draft treaty text for WRC-27.

Scope of studies:

- Bands already identified for IMT in ITU-R M.1036: 694-960 MHz, 1 710-1 880 MHz, 1 885-2 025 MHz, 2 110-2 200 MHz, 2 300-2 400 MHz, 2 496-2 690 MHz.

- Both GSO and NGSO MSS constellations; uplink and downlink directions; voice, data, IoT and broadcast-style services.

- Worst-case terrestrial densities (urban macro, indoor hotspot, high-speed rail) and MSS beam-edge geometries to be modelled in the compatibility analysis.

Observation 1-4: The ITU-R is working on the in-band sharing and compatibility study for searching and identification new spectrum for DC-MSS-IMT service, the potential frequency bands including the IMT bands: 694-960 MHz, 1 710-1 880 MHz, 1 885-2 025 MHz, 2 110-2 200 MHz, 2 300-2 400 MHz, 2 496-2 690 MHz.

Progress on Agenda Item 1.13 within ITU-R WP4C is significantly lagging that of Agenda Item 1.7. The sub-working group remains primarily focused on debating parameters and assumptions for each scenario, with very few contributions addressing actual sharing and compatibility studies. A key contributing factor is that protection criteria for IMT systems are still under deliberation in WP5D. Crucially, WP5D has yet to reach consensus on the liaison statement (LS) to WP4C regarding the IMT-MSS compatibility study.

Multiple methodologies for establishing IMT protection criteria—including PDF, EPFD, I/N, and throughput loss—remain under consideration. WP5D discussions continue regarding both the measurement approaches and determination frameworks for these criteria. Consequently, Agenda Item 1.13 warrants ongoing monitoring due to these unresolved foundational issues.

Within ITU-R WP4C, the DC-MSS-IMT systems presents compatibility study scenarios of heightened complexity. As the Figure 1 below shows a generalized representation of operations multiple DC-MSS-IMT systems in international borders of countries operating in adjacent geographical areas in the same frequency (channel). In this case, IMT stations in country may potentially receive interference from visible satellites of such DC-MSS-IMT systems from neighboring countries.

Figure 1

Terrestrial IMT in country C receives interference from operation of DC-MSS-IMT
 for neighbouring countries A and B

Consequently, this situation has raised significant concerns among numerous administrations regarding whether DC-MSS-IMT can adequately protect incumbent IMT services. For many nations, satellite infrastructure may prove economically unfeasible, whereas IMT services—as critical telecommunications infrastructure—require priority protection. This regulatory dilemma directly contributes to the persistent deadlock in reaching consensus on IMT protection methodologies within ITU-R WP5D.

Observation 1-5: The sharing and compatibility studies for DC-MSS-IMT, including the in-band scenario study with the terrestrial IMT to identify new spectrum to the DC-MSS-IMT system in the scenario of cross-border.

Observation 1-6: For the DC-MSS-IMT system, the criterial and mechanism for  protecting IMT systems operating is still on discussion

  - Band definition and CA simplification

Band definition

Pain points in 5G for band definition:

In 5G beginning, RAN4 introduced both 4G migrated bands and new bands for above 3GHz and FR2. These cumulative bands brought lots of work and number of bands for 5G in initial stage and many of them not deployed/commercialized in 5G day1.

And in 5G, NTN and TN bands are specified separately with different band numbers even the spectrum is partially overlapped or partially overlapped.

Observation 2-1:  Following pain points observed for 5G band definition:

- In 5G beginning, RAN4 introduced both 4G migrated bands and new bands for above 3GHz and FR2. These cumulative bands brought lots of work and number of bands for 5G in initial stage and many of them not deployed/commercialized in 5G day1.

- NTN and TN bands are specified separately with different band numbers even the spectrum is partially overlapped or partially overlapped.

Proposal 2-2: RAN4 shall further evaluate how to efficient manage NR refarming bands into 6GR as case by case manner

Proposal 2-3: RAN4 needs to further study how to handle 4G/5G migration spectrum/bands to 6G with potential research area

- Harmonize TN and NTN bands in same frequency range

- Harmonize spectrum/band definition with partial overlapping or fully overlapped operating frequency considering regulation difference

CA/Band combination

Pain points in 5G for CA/band combination:

In NR, band combination specific requirements are specified including RSD, MSD, delta_RIB/TIB. These band combination specific requirements are related to several factors including power class, number of Tx/Rx, device type, BCS, band and band combination.

And RAN4 develop a procedure with basket WI approach per operators’ request once generic requirements are introduced.  Accumulative band combinations increase specification work, product development complexity and conformance test complexity (<10% combos really deployed).

Observation 2-2: Following pain points observed for 5G band combination handling:

- Accumulative band combinations increase specification work, product development complexity and conformance test complexity (<10% combos really deployed).

New requirement framework shall be considered to simplify existing band combination specific requirement structure:

- Per band requirements still as basis

- Simplify BC specific requirements: “band group” concept

  - Remove BC specific requirements cap for “easy BC” e.g. Cross BG CA with BG basic requirements

    - Conformance test only select one band combination for same type of Cross BG CA

    - UE only report BG CA type, and corresponding bands in each band group; without indication band combination as one example provided in below table:

  - Difficult BC e.g. CA within BG

    - CA via switching -> Per band requirements basis/Per switching type basis requirements

    - Normal CA approach as per BC basis (2nd priority)

Benefits of these new BG based CA requirement’s structure:

- Simplify RAN4 specification work on BC specific requirements for most of cases

  - As show in table below, for cross BG CA, only Delta_TIB only specified based BG combination instead of detailed band combination

ΔTIB for Inter-band CA across two band groups (two bands)

| Inter-bandgroup CA combination CA_BGA-BGB | ΔTIB, for bands (dB) |  |
| --- | --- | --- |
|  | Component band in Group A | Component band in Group B |
| CA_BG1-BG2 |  |  |
| CA_BG1-BG3 |  |  |
| CA_BG2-BG3 |  |  |

  - Similar simplification can be introduced with band group-based combination requirements including MSD, delta_TIB, delta_RIB and RSD

- Save UE capability reporting

  - As show in below table UE indicate supported BGs, Cross -BG CA type, and supported bands in each band group instead of each band combinations across these band groups

| Band group | Operating frequency | Supported bands |
| --- | --- | --- |
| FR1-BG1 | <1GHz | Band 8, 20,28 |
| FR1-BG2 | 1GHz ~ 2.3GHz | Band 1, 3, 39 |
| FR1-BG3 | 2.3 GHz ~3GHz | Band 7, 40, 41 |
| FR1-BG4 | 3GHz ~5GHz | Band 77, 79 |

- Save conformance test effort

  - By introducing band group-based requirements, new test rule can be introduced with limited test cases representing all band combinations belongs same band group CA type/

    - Random selected one/multiple band combination from same BG CA type or other band combination selection rule can be considered

    - Per band requirements (single carrier) still needs to be verified as band specific requirements

Proposal 2-2: RAN4 needs to study how to simplify band combination including below direction:

- Data base tool

- New CA framework for requirement definition e.g., BG basis CA requirements for cross -BG CA

(Following contents provided as initial exemplary explanation for band group CA concept)

  - Per band requirements still as basis

  - Simplify BC specific requirements: “band group” concept

    - Remove BC specific requirements cap for “easy BC” e.g. Cross BG CA with BG basic requirements

      - Conformance test only select one band combination for same type of Cross BG CA

      - UE only report BG CA type, and corresponding bands in each band group; without indication band combination as one example provided in below table:

    - Difficult BC e.g. CA within BG

      - CA via switching -> Per band requirements basis/Per switching type basis requirements

      - Normal CA approach as per BC basis (2nd priority)

  - Frequency range definition

According to the SID, 6GR target on FR1 (up to 7.125GHz) , FR2-1 and frequency range between 7GHz and 24GHz. The frequency range definition will be specified and decided in RAN4.

NR design principle for frequency range definition: Numerologies and UE RF front-end architecture (conductive vs radiated requirements)

- FR1 with antenna connector (omnidirectional antenna) ->Conductive Requirements + SISO/MIMO OTA requirements

- FR2 integrated RF front-end (directional antenna) -> Radiated requirements only

NR initially split into FR1 and FR2 based on candidate numerologies and UE RF front-end architecture. And later, NTN Ku bands (which around 10~15GHz) require both FR1 and FR2 numerologies which is pending on operator deployment. RAN4 agreed to further extend FR1-NTN and FR2-NTN with overlapping ranges, by using different band numbers to discriminate FR1-NTN Ku bands and FR2-NTN Ku bands. Regarding requirement applicable rule, original NTN requirements (conductive or radiated requirements) were specified based on FR1-NTN or FR2-NTN and it’s further clarified that for NTN above 10GHz bands, radiated requirements are applied given VSAT for Ku bands with directional antennas.

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN | 410 MHz – 145007125 MHz |
| FR2-NTN | 1730010700 MHz – 30000 MHz |

Table 4.2-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | FR1-NTN bands |  | FR2-NTN bands |
|  | Below 10 GHz with antenna connector | Above 10 GHz |  |
| Conducted transmitter characteristics | Clause 6 | NA | NA |
| Conducted receiver characteristics | Clause 7 | NA | NA |
| Conducted performance requirements | Clause 8 | NA | NA |
| Radiated transmitter characteristics | NA | Clause 9 | Clause 9 |
| Radiated receiver characteristics | NA | Clause 10 | Clause 10 |
| Demodulation performance requirements (Radiated requirements) | NA | Clause 11 | Clause 11 |

Observation 3-1: NR divided frequency range majorly based on applicable numerologies sets and UE RF front-end architecture (conductive vs radiated requirements)

Proposal 3-1: RAN4 consider UE RF front-end architecture (e.g. whether requirements can be verified conducted or not) as major principle for FR definition.

Observation 3-2: On 7-24 GHz, depending on frequency with initial survey on UE RF front-end architecture:

- For below10GHz, FR1 like is feasible with antenna connector

- For above 15GHz, FR2 like is most likely with integrated RF front end

- 10GHz ~ 15GHz, unclear mixed UE architectures probably exist for same frequency

Observation 3-3: on conductive and radiated requirement applicable rule, RAN4 can further discuss following different UE requirement types:

- UE type C: only digital BF with omnidirectional antenna performance and antenna connector

- UE type H: Hybrid, conductive requirements can be applied on RIB and OTA requirements plus

- UE type O: OTA requirements only with analog BF, FR2 like requirements

Proposal 3-2: two alternative approaches can be considered for frequency range definition and extension:

- Alt 1: Extend FR1 and FR2 to cover 7-24GHz with overlapping range

- Alt 2: Introduce dedicated FR3 frequency with hybrid requirements pending on device type

Proposal 3-3: At least FR1 can be further extended to 8.4GHz, dedicated sub-frequency range can be introduced for 7.125~8.4GHz i.e. FR1-1.

Proposal 3-4: Other frequency between 7-24GHz can be kept open until regulation on spectrum arrangement is clear.

- Conclusion

In this contribution, our preliminary view on 6G spectrum is provided on following areas:

WRC-27/regulation situaion

Observation 1-1: The ITU-R is working on searching and identification new spectrum for IMT 6G, the potential frequency bands are 4 400–4 800 MHz, 7 125–8 400 MHz and 14.8–15.35 GHz.

Observation 1-2: The sharing and compatibility studies of WRC-27 agenda item 1.7 in under study, the identification methods, requirements, and regulations for the new bands for IMT are also under discussion in ITU-R.

Observation 1-3: 6425-7125 MHz is regarded by multiple nations as the primary frequency band for 6G.

Observation 1-4: The ITU-R is working on the in-band sharing and compatibility study for searching and identification new spectrum for DC-MSS-IMT service, the potential frequency bands including the IMT bands: 694-960 MHz, 1 710-1 880 MHz, 1 885-2 025 MHz, 2 110-2 200 MHz, 2 300-2 400 MHz, 2 496-2 690 MHz.

Observation 1-5: The sharing and compatibility studies for DC-MSS-IMT, including the in-band scenario study with the terrestrial IMT to identify new spectrum to the DC-MSS-IMT system in the scenario of cross-border.

Observation 1-6: For the DC-MSS-IMT system, the criterial and mechanism for protecting IMT systems operating is still on discussion.

Band and CA simplification

Observation 2-1:  Following pain points observed for 5G band definition:

- In 5G beginning, RAN4 introduced both 4G migrated bands and new bands for above 3GHz and FR2. These cumulative bands brought lots of work and number of bands for 5G in initial stage and many of them not deployed/commercialized in 5G day1.

- NTN and TN bands are specified separately with different band numbers even the spectrum is partially overlapped or partially overlapped.

Proposal 2-2: RAN4 shall further evaluate how to efficient manage NR refarming bands into 6GR as case-by-case manner instead of package treatment

Proposal 2-3: RAN4 needs to further study how to handle 4G/5G migration spectrum/bands to 6G with potential research area

- Harmonize TN and NTN bands in same frequency range

- Harmonize spectrum/band definition with partial overlapping or fully overlapped operating frequency considering regulation difference

Observation 2-2: Following pain points observed for 5G band combination handling:

- Accumulative band combinations increase specification work, product development complexity and conformance test complexity (<10% combos really deployed).

Proposal 2-2: RAN4 needs to study how to simplify band combination including below direction:

- Data base tool

- New CA framework for requirement definition e.g., BG basis CA requirements for cross -BG CA

(Following contents provided as initial exemplary explanation for band group CA concept)

  - Per band requirements still as basis

  - Simplify BC specific requirements: “band group” concept

    - Remove BC specific requirements cap for “easy BC” e.g. Cross BG CA with BG basic requirements

      - Conformance test only select one band combination for same type of Cross BG CA

      - UE only report BG CA type, and corresponding bands in each band group; without indication band combination

    - Difficult BC e.g. CA within BG

      - CA via switching -> Per band requirements basis/Per switching type basis requirements

      - Normal CA approach as per BC basis (2nd priority)

Frequency range definiton

Observation 3-1: NR divided frequency range majorly based on applicable numerologies sets and UE RF front-end architecture (conductive vs radiated requirements)

Proposal 3-1: RAN4 consider UE RF front-end architecture (e.g., whether requirements can be verified conducted or not) as major principle for FR definition.

Observation 3-2: On 7-24 GHz, depending on frequency with initial survey on UE RF front-end architecture:

- For below10GHz, FR1 like is feasible with antenna connector

- For above 15GHz, FR2 like is most likely with integrated RF front end

- 10GHz ~ 15GHz, unclear mixed UE architectures probably exist for same frequency

Observation 3-3: on conductive and radiated requirement applicable rule, RAN4 can further discuss following different UE requirement types:

- UE type C: only digital BF with omnidirectional antenna performance and antenna connector

- UE type H: Hybrid, conductive requirements can be applied on RIB and OTA requirements plus

- UE type O: OTA requirements only with analog BF, FR2 like requirements

Proposal 3-2: two alternative approaches can be considered for frequency range definition and extension:

- Alt 1: Extend FR1 and FR2 to cover 7-24GHz with overlapping range

- Alt 2: Introduce dedicated FR3 frequency with hybrid requirements pending on device type

Proposal 3-3: At least FR1 can be further extended to 8.4GHz, dedicated sub-frequency range can be introduced for 7.125~8.4GHz i.e., “FR1-1”.

Proposal 3-4: Other frequency between 7-24GHz can be kept open until regulation on spectrum arrangement is clear.

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] RAN1#122 meeting report

[3] R4-2514626, WF on 6G Spectrum, Feature lead (Nokia)