3GPP TSG-RAN WG4 Meeting #117                      R4-2520513

Dallas, USA, 17th – 21st, Nov, 2025

Agenda item:			8.4.3

Source:	Xiaomi

Title:	View on 6GR Co-existence

Document for:	Discussion

Background

In the RAN#108 meeting, the new SID study on 6G Radio was approval by the meeting. intra-3GPP co-existence studies is one of major objective for RAN4 6G study.

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. BS RF including co-existence study is one of the major area.

In NR, from Rel-14, there are several coexistence studies finshed by 3GPP RAN4 to determine the ACLR, ACS, blocking and spurious for FR1 and FR2 base stations and UEs including FR2 co-existence study in Rel-14, NTN co-existence study on FR1/FR2 NTN bands in Rel-17/18/19, co-existence study for SBFD operation and co-existence study for IMT parameters on 4GHz, 8GHz and 15GHz . In the 6G Radio study period, coexistence study is still very important for the determine the core requirment of 6G Radio.

In this document, initial analysis and views on potenail 6G co-existence study are provided.

Disucussion

List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

- In the first RAN4 6GR meeting, companies shown their interests for the 7GHz. There are two main frequency range around 7GHz. 7025-7125MHz is the globally identified frequency band for the IMT industry, and 6425-7025MHz have been identified in the Region-1 and some other countries out of Region-1 added in this identification by notes and ongoing study for 6425-7125MHz coordination in the ITU-R WP5D. 3GPP also identified bands n104 for the band 6425-7125MHz, as well as parameters in the TR 38.921. However, in the RAN plenary meeting#109, there are several agreements captured in the the RP-252914 agreements:

- “Around 7 GHz: Up to 2304 Tx and Rx antenna elements”, it is 48*48 elements antenna configuration for the base station, which is a significate difference with the TR 38.921 parameters.

- Observation 1: Antenna element configuration for 6425-7125MHz in TR38.921 and the 6GR agreements have significate difference.

- In the LS from 3GPP to ITU-R about the IMT parameters, 7125-8400MHz shared the same parameters (captured in the TR38.922) with 6425-7125MHz. Therefore, 7125-8400MHz difference between 5G and 6GR agreements also in this frequency band as well. However, this bands still under study in the ITU-R and still need two more years to know this final identification and regulatory requirements for the bands.

- Observation 2: Antenna element configuration for 7125-8400MHz in TR38.922 and the 6GR agreements have significate difference.

- Meanwhile, basic on the discussion of last meeting, many companies shown their interests about the around 7GHz for 6GR. But more detailed are still needed to have a complete picture about the 6GR parameter to support this study.

- Proposal 1: Coexistence study for around 7GHz could be the beginning of 6GR.

- Proposal 2: RAN4 can discuss the parameters, assumptions and necessary propagation models for the around 7GHz frequency band for the coexistence study.

- Proposal 3: RAN4 can consider Macro Urban, Macro Suburban, Macro Rural, Micro urban, Indoor hotspot and Indoor factory as the scenarios of around 7GHz.

- Proposal 4: RAN4 should not precluded other possible scenarios for the 6GR during the coexistence study.

- From the agreement of RAN-P#109, around 700MHz, 2GHz and 15GHz have similar antenna element configuration compare with 5G configuration. Antenna element configuration of 4GHz and 30GHz have significant differences from antenna element configuration with 5G.

- Observation 3: Coexistence study for around 4GHz and 30GHz have significant differences from antenna element configuration with 5G.

- Proposal 5: RAN4 can discuss the parameters and coexistence study necessity of frequency band around 4GHz and 30GHz.

- Proposal 6: RAN4 can wait more information about the 700MHz, 2GHz and 15GHz from other working group and discuss from RAN4’s perspective whether there are significate differences for coexistence study for these bands.

- NTN Way Forward: Additional input from satellite companies is expected for RAN4#117: - List of requirements to be further studied (using the same format proposed in 1.1.2) - List of coexistence studies to be considered (using the format proposed in 1.3.1).

- For the NTN coexistence study, in the 5G period, there are 2GHz, Ka-band, Ku-band and high-power UE coexistence studies finalized by RAN4. The studies basic on the 5G base station and UE parameters and assumptions. Based on our previous discussion, frequency bands around 15GHz and 30GHz could have significate difference from antenna element configuration. Since the different parameters, it is not clear that which could be the worst cases to determine the ACLR, ACS, etc. It is possible to determine new isolation distance, new ACLR and ACS for the 6GR TN system by redo the coexistence study between 5G NTN and 6GR TN.

- Meanwhile, 14.75-15.35GHz is the potential new frequency bands for IMT in WRC-27, and the NTN Ku-band uplink is the frequency range of 14GHz – 14.5GHz. 250 MHz gap between two service of TN and NTN, consider the OOBE might be an interference issue.

- Proposal 7: RAN4 can analysis whether the coexistence study is necessary between 6GR TN 14.75-15.35GHz and NTN Ku-band 14GHz – 14.5GHz uplink.

- Proposal 8: RAN4 should consider the necessity of redo the coexistence study between 5G NTN and 6G TN in the bands around 30GHz.

- To consider the limited spectrum resources of NTN system defined in 3GPP. It is valuable to consider the MRSS between 5G NTN and 6G NTN in the existing frequency bands of NTN system. It would be very helpful for the NTN operators who have limited spectrum at the beginning of 6GR to reuse the spectrum resources of what they have in the 5G. From the ITU-R WP4C studies to search new frequency bands for the DC-MSS-IMT service (including 3GPP NTN tech), it is still unclear to say how many spectrums could be identified for the DC-MSS-IMT service, as well as the unknown regulatory requirements. Therefore, since the limited spectrum source for the NTN system:

- Proposal 9: RAN4 can consider the necessity of the coexistence study between 5G NTN and 6G NTN in the existing bands of 3GPP.

Conclusion

- Observation 1: Antenna element configuration for 6425-7125MHz in TR38.921 and the 6GR agreements have significate difference.

- Observation 2: Antenna element configuration for 7125-8400MHz in TR38.922 and the 6GR agreements have significate difference.

- Proposal 1: Coexistence study for around 7GHz could be the beginning of 6GR.

- Proposal 2: RAN4 can discuss the parameters, assumptions and necessary propagation models for the around 7GHz frequency band for the coexistence study.

- Proposal 3: RAN4 can consider Macro Urban, Macro Suburban, Macro Rural, Micro urban, Indoor hotspot and Indoor factory as the scenarios of around 7GHz.

- Proposal 4: RAN4 should not precluded other possible scenarios for the 6GR during the coexistence study.

- Observation 3: Coexistence study for around 4GHz and 30GHz have significant differences from antenna element configuration with 5G.

- Proposal 5: RAN4 can discuss the parameters and coexistence study necessity of frequency band around 4GHz and 30GHz.

- Proposal 6: RAN4 can wait more information about the 700MHz, 2GHz and 15GHz from other working group and discuss from RAN4’s perspective whether there are significate differences for coexistence study for these bands.

- Proposal 7: RAN4 can analysis whether the coexistence study is necessary between 6GR TN 14.75-15.35GHz and NTN Ku-band 14GHz – 14.5GHz uplink.

- Proposal 8: RAN4 should consider the necessity of redo the coexistence study between 5G NTN and 6G TN in the bands around 30GHz.

- Proposal 9: RAN4 can consider the necessity of the coexistence study between 5G NTN and 6G NTN in the existing bands of 3GPP.