3GPP TSG-RAN WG4 Meeting #116	R4-2520532

Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Source:	Nokia

Title:	Performance requirements using SU SCM

Agenda item:	7.5.3

Document for:	Discussion

Introduction

The Performance requirements for FR1 SU-MIMO with spatial channel model has kicked-off in the last meeting and many agreements have been captured in the first WF [4].

Much progress has been made on issues that impact the agreed introduction PDSCH performance requirements [1], while PMI only relevant discussions are expected to be prominent in RAN4#117.

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

As such our main targets for this contribution are as follows:

- Define PDSCH performance requirements

  - Agree the configuration for 2CWs and the KPIs.

- Discuss PMI requirements

  - Agree PMI distribution way forward

In this first meeting we do not have a companion simulation contribution; however, the prior study simulations remain valid, including the study simulation summary [3].

Potential updates to rCDL channel model

The last meeting has resulted in agreements [4] that answer all PDSCH requirement relevant notes from the WI objective:

Frequency generalization for rCDL channel modeling - Agreement: - Define the requirements based on the same channel model for all different FR1 frequencies for this WI - The maximum Doppler fD should be fixed as X Hz - As baseline for Rel-20 demod requirement, X =10 - FFS if other fD can be considered - On the note to Maximum Doppler: - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler). Angle scaling for rCDL - Agreement: - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details. - Agreement on the details: - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies. - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model - Additional rCDL Channel Models based on different AS are not precluded Duplex mode and SCS - Agreement: - Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

However, these agreements should be captured in the Rel-19 SI TR, if possible.

UE antenna modelling

In RAN4#116b it was raised, whether RAN4 demod should consider more complex UE antenna modelling [4]:

UE antenna modelling - <For INFORMATION> It is RAN4 common understanding that the following were added in Rel-19 version of TR 38.901: - #1: Candiadate location for UE antenna placements - #2: Directional antenna assumption for UE - Further discuss the two options in the next meeting: - Option 1: Update the UE antenna placement and assumption of rCDL based on v19.0.0 of 38.901. - Option 2: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline - The moderator requests proponents of option 1 to provide detailed simulation assumptions in the next meeting.

The Rel-19 study discussed the question of UE antenna modelling in RAN4#112, and it was raised online (and in adhocs) that a specific UE antenna model is not of high importance to capture the spatial properties of a typical deployment in the CDL model. The results of the study, especially 2CW performance and SINR distributions, have shown that no complex UE antenna model is needed to achieve typical spatial effects in the CDL model.

From the spatial effects and measurements presented in the Rel-19 study we conclude that complex UE antenna models are not needed to capture typical spatial effects in the CDL model.

The Rel-19 TR 38.901 has introduced additional UE antenna models that distinguish themselves from legacy by having categories for antenna placement on the DUT as well as directional antenna assumptions, which correspond mostly to blocking/antenna imbalance scenarios.

Rel-19 TR 38.901 antenna (blockage) models are only needed for system level analysis. In link level, the antenna placement would introduce frequency dependence and device categories, and the directional antenna assumptions don’t benefit the spatial typicality and are most relevant for system level effects, thus hurting test feasibility.

Finally, we caution that adding UE antenna directivity/array factors will reduce the diversity of the channel model. We already shrunk the rCDL model to 12 clusters, which significantly reduced diversity, but this was acceptable as the simulated and measure spatial effects we still matching in a “typical” sense. If we introduce a new more restrictive UE antenna modelling, the balance of “typical spatial properties” that was achieved in Rel-19 rCDL will be changed, and the study would no long

The rCDL model was already diversity reduced to 12 clusters, but with the Rel-19 rCDL UE antenna model “typical spatial properties” and channel conditions was still observed. Changing the antenna model would necessitate increasing cluster diversity again to match typical conditions.

We caution against adding complexity in the pursuit of modelling reality exactly. Channel models are not intended to match any one instance of reality, but to capture required aspects of a typical deployment: In our case the typical spatial properties of a MIMO channel. Reality itself is unachievable and not required.

The current UE antenna assumptions are sufficient to model typical spatial channel modelling MIMO effects. RAN4 shall neither complicate the UE antenna model further, nor introduce device categories, nor further reduce channel diversity. RAN4 shall, thus, stay with the current “cross Pol (1, n/2, 2), where n is number of Rx and half lambda spacing, with omni antenna element radiation pattern” approach.

Cluster selection simplification

Following the end of the FS_NR_demod_SCM study, we think it is permissible to evaluate if the rCDL model could be simplified, with little impact to the performance scaling that has been developed and aligned over the last 1.5 years.

In particular, the AAV filtered cluster reduction method has been observed to bring little benefit over more simple methods. During the studies, it was observed that spatially filtering (the power) of clusters before selecting the 12 strongest ones, only resulted in one of the weakest clusters to be differently selected form simply choosing the first 12 clusters.	
The performance impact between the current spatially filtered cluster selection and a simple “select first 12 cluster, renormalize powers, rescale RMS delay spread” approach, are captured in the following figure:

Figure 1:Performance impact between the current spatially filtered cluster selection and a simple “select first 12, renormalize powers, rescale RMS delay spread” in the 8T8R 2CW scenario.

We observe that the performance difference between the 875 rCDL and simplified cluster selection is marginal at best.

The is almost no difference is performance and spatial properties between the current 38.753 rCDL cluster selection procedure and a simplified approach (“select first 12, renormalize powers, rescale RMS delay spread”).

Thus, we propose:

Simplify cluster selection procedure for rCDL included conversion to 12 clusters, by adopting “select first 12 clusters, renormalize powers, rescale RMS delay spread” approach.

Or stated more simply: Remove AAV weighted filtering for cluster selection.

C2 rCDL variant

At RAN4#116b it was unanimously agreed to update the AS of rCDL-C1 with new candidate values from Rel-19 38.901. The resulting model was tentatively called rCDL-C2, with its parameter table to be discussed between meetings.

The pre-RAN4#117 email discussion has resulted in the following candidate:

Table 1: rCDL-C2 candidate pre-RAN4#117.

| CLID | Delay, nsec (post-reduction scaling) | Power, dB | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.3 | -117.2 | 95.5 | 101.1 |
| 2 | 132 | -1.2 | -22.9 | 123.5 | 98 | 70.9 |
| 3 | 138 | -5.5 | -34.5 | -140 | 101.6 | 67 |
| 4 | 402 | 0 | -7.8 | 166.7 | 99.1 | 77.1 |
| 5 | 416 | -10.4 | 39.6 | 68 | 110 | 61.7 |
| 6 | 502 | -10.1 | -50 | 77.5 | 92 | 54.7 |
| 7 | 518 | -13.7 | 44.2 | -71.8 | 111.7 | 69.5 |
| 8 | 776 | -8.1 | -44 | 78.9 | 107.3 | 106.9 |
| 9 | 826 | -9.8 | -49.9 | -89.5 | 108.2 | 47.4 |
| 10 | 1372 | -11.7 | -59.1 | 90.4 | 87.8 | 49.2 |
| 11 | 1712 | -16.2 | 58.9 | 46.8 | 108.2 | 126.6 |
| 12 | 2906 | -16.9 | 49.8 | 23.7 | 88 | 60.3 |

We note that under the updated Rel-19 901 based AS scaling approach, the previously used “c” values are no longer applicable and they are replaced with “s” scaling values:

| Scale ‘s’ in 38.901 V19.1.0 |  |  |  |
| --- | --- | --- | --- |
| AoD | AoA | ZoD | ZoA |
| 0.65121 | 0.85833 | 1.82164 | 1.95018 |

While companies continue to confirm the details of the models, we propose to use this candidate for the time being.

While rCDL simplification/modification is further discussed, RAN4 to proceed with requirement derivation under the assumption of the above rCDL-C2 candidate, with post reduction RMS delay spread scaling.

Specification of PDSCH requirements

The preceding Rel-19 study (FS_NR_demod_SCM) has carried out all the technical work to define normative requirements for the 8T8R 2CW case. Most details are present in TR 38.753.

Requirement configuration

To re-use the aligned simulation results from TR 38.753, we propose to reuse the configuration [2] from there:

Re-use the TR 38.753 2CW configuration to define SU 8T8R 2CW/8-layer requirements. I.e., reuse configuration from [R4-2508622], including rCDL with 10 Hz Doppler (in its AS updated “C2” variant), and use random wideband type I precoding.

For PDSCH, no further modification of rCDL (outside of already agreed AS scaling update) is necessary. PMI preferences are not a concern for PDSCH with random precoding and a single set of requirements is traditionally used for all bands. Update to match recent RAN1 angular changes for Uma are straightforward given the table derivation framework in TR 38.753.

For PDSCH, no modification of the rCDL channel model framework (outside of already agreed AS scaling update) is necessary.

Requirement definition

As found in the study, the SNR operating point for a fixed relative throughput for each CW varies between each CW. Looking at a per CW TPUT curve from the study, this is visible as follows:

Figure 1: TPUT curves for SU 8T8R 8layers (2CWs) at low speed.

Hence, when using the customary “SNR@70% relTPUT” metric with a fixed MCS for both CWs, we end up with two SNR operating points. This is not feasible from a test setup point of view, unless the test is run twice, once per CW at each operating point. 	
However, it is straightforward and following real deployment to choose two different MCS that approximate the spectral efficiency of each CW at the given SNR operating point.

Only a single operating point/SNR can be chosen for each run/CW demodulation, in any real test setup. This is aligned with deployment reality, where different MCSes must be chosen per CW that approximate the spectral efficiency of each CW at the given SNR operating point.

As in real deployments, there exists no perfect MCS to absolutely match the channel capacity at the given SNR, and the next closest decodable MCS must be chosen. I.e., in our test setup we shall aim to choose two CW/MCS pairs, whose 70%TPUT points are very close together, but the more stringent CW sets the final SNR requirement.

For 8T8R 8L test define single SNR operating point based on per CW throughput with different MCS per CW. The CW/MCS pairing is chosen to be reasonably close in their respective 70%TPUT points, and the more stringent CW sets the ultimate SNR requirement.

This may be captures in specification as follows:

Table 1: Proposal for 2CW demodulation requirements

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) per CW | SNR (dB) |
| 1-x | R.PDSCH.XXX | 40 / 30 | CW1: 64QAM, 0.50 CW2: 16QAM, 0.33 | FR1.30-XX | rCDL-C2-10 | 70 | XXX |

Note that using 70% of sum (of both CW) throughput, is a metric that can be abused.	
The overlapping curves in terms of relative per CW throughput are hiding, that the absolute throughput is rather different for each CW. The worse CW will have lower MCS than the better one, thus, a higher throughput at the better CW can easily compensate the absence of the worse CW in a sum throughput requirement.

Using a “70% of sum (of both CW) throughput” metric can be abused, both intentionally and unintentionally.

Optionality of requirements

Following the WID [1], the PDSCH requirements are to be optional. To achieve this, we propose to use optional UE capability and applicability rules. For example:

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SCM compatible MIMO receiver | FR1 TDD | PDSCH | Clause X.X.X Test X-X |  |

This is similar to the current requirement status of the “Simplified SU-MIMO 8Rx receiver” UE capability and applicability rule. Or in proposal form:

RAN4 to make SU PDSCH requirement optional, by following the simplified SU-MIMO 8Rx example, i.e., via optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules.

Specification of PMI reporting requirements

Alignment reached

The RAN#109 WID [1] is also quite clear in defining PMI reporting requirements when alignment is reached. The Rel-19 study is quite clear in that eType II alignment is already reached. This can be directly seen from the report TR 38.753 in section 7.1, as well as the full alignment collection results, and the alignment collection subset (both in [3]).

Observation for rCDL-C1: - In eType-II codebook case, three clusters of results can be observed: - Cluster 1: include source #3, #7 are in 70% normalized throughput and source #7 in 90% normalized throughput. - Cluster 2: include source #4, #5, #6, #9 in 70% normalized throughput and source #3, #4, #5, #6, #9 in 90% normalized throughput. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput. - Cluster 3: include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput.

Figure 2: TR 38.753 observations for rCDL-C1 eType-II follow PMI alignment.

Hence, we propose:

RAN4 to agree alignment of PMI performance results with 4T4R and 8T4R given eTypeII precoding. Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

Direct reuse of rCDL

The WID [1] is requesting to consider potential modifications of the rCDL model and requests to update the rCDL-C model parameter table, such as to take recent Rel-19 RAN1 modifications of the UMa use case into account. The latter is easy to achieve using the table derivation framework in TR 38.753, the former is unneeded, as we will discuss in the following.

Thus, we propose already at the beginning of this section:

For PMI reporting, no modification of the rCDL channel model is necessary, outside of update of parameter for “C1” variant due to recent RAN1 changes.

PMI preference in SCMs

It is the nature of a spatial channel model (SCM), that precoders and receivers that match the spatial preferences of the channel, result in better throughput/BLER/spectral efficiency performance than those who do not. Capturing the performance impact of the receiver/precoder algorithms adapting themselves to the channel, is one of the major advantages of SCMs over legacy channel models, as discussed in FS_NR_demod_SCM and captured in TR 38.753.

It is the nature of a spatial channel model (SCM), that precoders and receivers that match the spatial preferences of the channel perform better than those who do not.

Certain AIML use cases may have a need for random, but partially (and controllably) predictable, time-wise evolution of the spatial characteristics of the used SCM, in order to appropriately test features that involve, e.g., PMI prediction performance. 	
Such time-wise spatial properties evolution may be included in rCDL via application of the legacy beamsteering framework, or via mu_desired table evolution, or via direct DoA/DoD angle evolution. However, such extensions shall be handled by the features that use the channel model for AIML prediction use cases, and not by this demod enhancement work item. Each WI is responsible for the advancement of the requirement framework as is needed by the WI.

If random, but partially predictable, time-wise evolution of the spatial characteristics of a SCM is needed, this can be achieved via, .e.g., the direct application of the legacy beamsteering framework, or via mu_desired CDL table evolution, or via direct DoA/DoD angle evolution.

The legacy beamsteering framework (e.g., LTE TS36.101, Appendix B.2.3A.4) is arguably the least effort approach to include time-wise spatial properties evolution, as it simply rotates the effective precoder selected by the PMI feedback. This low effort approach is sufficient, if PMI selection distributions are to be evolving over time, i.e., if RAN4 intends to consider adversarial DUTs in conformance testing. 	
This rotation/“bending” of the selected PMI to precoder mapping is done in RE-domain directly on top of the precoder formulation and thus independent of the chosen propagation environment. See Figure 3 for a graphical representation of this.

Figure 3: Function of legacy beamsteering approach to change PMI selection over time.

Or put as an observation:

The legacy beamsteering framework rotates the effective precoder selected by the PMI. This rotation of  the selected PMI to precoder mapping is done in RE-domain directly on top of the precoder formulation and thus independent of the chosen propagation environment. This low effort approach is sufficient, if PMI selection distributions are to be evolving over time.

However, the PMI reporting requirements from this SU SCM objective do not need to consider time-wise evolution of the spatial characteristics of a SCM, as this only matters if we consider dishonest devices. For follow PMI measures to be preferring certain PMIs in SCM is a feature of deployments, as well as SCMs, and do not impact minimum performance requirements targeting the PMI selection algorithm implementation performance of a commercial device.

PMI reporting requirements from this SU SCM objective do not need to consider time-wise evolution of the spatial characteristics of a SCM.

eTypeII performance metric

Concerning whether “gamma” KPIs are used (as in legacy), or whether SNR@70%or90%TPUT is used (as done in the study), we have a slight preference for the absolute performance numbers used in the study.	
The background, why LTE used “the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding” at the 70% relative TPUT point obtained at SNR_follow, and why 5GNR changed the TPUT to 90%, is unclear at this point. Though this likely pertained to an effort to show >1 relative TPUT improvement with new code book-based precoding schemes. As the previous Rel-19 study indicated, the legacy channel models were not adapted to let interference cancelation intended multi-beam and combination codebooks show their strengths. While these fundamental limitations of the legacy channels are addressed via SCMs, the enhanced 5GNR codebooks still have limited performance gains in the baseline SU MIMO use case with assumed limited numbers of antenna elements (i.e., “AAV3”/direct port mapping without array abstraction from TR 38.753).

As such we think it is not useful to chase gain numbers via non-relevant KPIs. RAN4 should return to absolute performance KPIs in SCM for PMI reporting, to allow insights in the performance scaling differences between the two codebook types.

RAN4 to define PMI reporting performance requirements with the metric of SNR@70% follow PMI throughput.

eTypeII requirement configuration

Concerning requirement configuration, we shall also reuse the setup from the study:

RAN4 to define PMI reporting performance requirements with 4T4R and 8T4R given eTypeII follow precoding re-using the configuration from the Rel-19 stud, i.e., Table 6.2-1 and Table 6.2-2 in TR38.753. Consider both rank 4 and 2, with priority for 4.

We don’t see a reason to deviate from the configuration of the study, which has been aligned and checked for feasibility many times.

Conclusion

The following observations and proposals were made:

=== Potential updates to rCDL channel model

== UE antenna modelling

- From the spatial effects and measurements presented in the Rel-19 study we conclude that complex UE antenna models are not needed to capture typical spatial effects in the CDL model.

Rel-19 TR 38.901 antenna (blockage) models are only needed for system level analysis. In link level, the antenna placement would introduce frequency dependence and device categories, and the directional antenna assumptions don’t benefit the spatial typicality and are most relevant for system level effects, thus hurting test feasibility.

The rCDL model was already diversity reduced to 12 clusters, but with the Rel-19 rCDL UE antenna model “typical spatial properties” and channel conditions was still observed. Changing the antenna model would necessitate increasing cluster diversity again to match typical conditions.

- The current UE antenna assumptions are sufficient to model typical spatial channel modelling MIMO effects. RAN4 shall neither complicate the UE antenna model further, nor introduce device categories, nor further reduce channel diversity. RAN4 shall, thus, stay with the current “cross Pol (1, n/2, 2), where n is number of Rx and half lambda spacing, with omni antenna element radiation pattern” approach.

== Cluster selection simplification

The is almost no difference is performance and spatial properties between the current 38.753 rCDL cluster selection procedure and a simplified approach (“select first 12, renormalize powers, rescale RMS delay spread”).

Simplify cluster selection procedure for rCDL included conversion to 12 clusters, by adopting “select first 12 clusters, renormalize powers, rescale RMS delay spread” approach.

== C2 rCDL variant

While rCDL simplification/modification is further discussed, RAN4 to proceed with requirement derivation under the assumption of the above rCDL-C2 candidate, with post reduction RMS delay spread scaling.

=== Specification of PDSCH requirements

== Requirement configuration

Re-use the TR 38.753 2CW configuration to define SU 8T8R 2CW/8-layer requirements. I.e., reuse configuration from [R4-2508622], including rCDL with 10 Hz Doppler (in its AS updated “C2” variant), and use random wideband type I precoding.

For PDSCH, no modification of the rCDL channel model framework (outside of already agreed AS scaling update) is necessary.

== Requirement definition

Only a single operating point/SNR can be chosen for each run/CW demodulation, in any real test setup. This is aligned with deployment reality, where different MCSes must be chosen per CW that approximate the spectral efficiency of each CW at the given SNR operating point.

For 8T8R 8L test define single SNR operating point based on per CW throughput with different MCS per CW. The CW/MCS pairing is chosen to be reasonably close in their respective 70%TPUT points, and the more stringent CW sets the ultimate SNR requirement.

Using a “70% of sum (of both CW) throughput” metric can be abused, both intentionally and unintentionally.

== Optionality of requirements

RAN4 to make SU PDSCH requirement optional, by following the simplified SU-MIMO 8Rx example, i.e., via optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules.

=== Specification of PMI reporting requirements

== Alignment reached

RAN4 to agree alignment of PMI performance results with 4T4R and 8T4R given eTypeII precoding. Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

== Direct reuse of rCDL

For PMI reporting, no modification of the rCDL channel model is necessary, outside of update of parameter for “C1” variant due to recent RAN1 changes.

== PMI preference in SCMs

It is the nature of a spatial channel model (SCM), that precoders and receivers that match the spatial preferences of the channel perform better than those who do not.

If random, but partially predictable, time-wise evolution of the spatial characteristics of a SCM is needed, this can be achieved via, .e.g., the direct application of the legacy beamsteering framework, or via mu_desired CDL table evolution, or via direct DoA/DoD angle evolution.

The legacy beamsteering framework rotates the effective precoder selected by the PMI. This rotation of  the selected PMI to precoder mapping is done in RE-domain directly on top of the precoder formulation and thus independent of the chosen propagation environment. This low effort approach is sufficient, if PMI selection distributions are to be evolving over time.

PMI reporting requirements from this SU SCM objective do not need to consider time-wise evolution of the spatial characteristics of a SCM.

== eTypeII performance metric

RAN4 to define PMI reporting performance requirements with the metric of SNR@70% follow PMI throughput.

== eTypeII requirement configuration

RAN4 to define PMI reporting performance requirements with 4T4R and 8T4R given eTypeII follow precoding re-using the configuration from the Rel-19 stud, i.e., Table 6.2-1 and Table 6.2-2 in TR38.753. Consider both rank 4 and 2, with priority for 4.

- References

- RP-252957, WID: NR demodulation performance: Phase 6, RAN#109, September 2025.

- R4-2508622, Simulation assumptions for SCM, Huawei, HiSilicon, MediaTek, RAN4#115.

- R4-2509413, Results collection table for NR SCM, BT plc, RAN4#116.

- R4-2514823, WF on NR_demod_Ph6_Ph5, China Telecom, RAN4#116-bis.