3GPP TSG RAN4 Meeting #117							R4-2520554

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	6GR General RF and UE RF

Agenda item:	8.3

Document for:	Approval

Introduction

Under 6G system General RF and UE RF topic, the scope includes non-spectrum and non-AI UE RF, spectrum aggregation framework, UE RF related coverage and energy efficiency, and other joint UE-BS RF issues. According to the feature lead guidance, during RAN4#117 discussion is expected on the following topics only [1].

- General aspects

- Single carrier output power

- CA output power

- Tx requirements

- Rx requirements

- Frequency range between FR1 and FR2-1

- Spectrum aggregation

- Joint UE and BS RF

- Testing and verification

- Others

# Topic #1: High level principles for 6G UE RF study - Agreements - High level principles during 6G UE RF study - Strive towards minimum performance requirements applicable for all regions to allow global circulation of terminals, accommodate local regulatory requirements as necessary - Strive towards only one set of requirements for each feature and avoid multiple sets of requirements from multiple implementation assumptions, considering e.g. different form factors and target use cases as well as trade-off between UE performance and specification complexity. - Strive towards requirement framework where dependencies between features are minimized and understood. Strive to identify dependencies as early as possible. - Strive towards requirements which are testable and meaningful - Final requirements are to be defined in the work item. - For all topics in the way forward it is possible to contribute new aspects to consider and study in future meeting(s). NOTE: No further discussion is expected on this topic in next meeting. # Topic #2: UE reference architecture - Agreements in online session - Strive to agree UE reference architecture on case-by-case manner for features being studied. Discuss reference architecture together with the feature itself as necessary.(first 6G agreement in RAN4) - Base evaluations on agreed reference architecture, while resulting requirements should apply for any feasible implementation architecture as much as possible - If no reference architecture is agreed, recommend companies to mention assumed architecture together with evaluation results for easier comparisons between inputs NOTE: No further discussion is expected on this topic in next meeting. # Topic #3: Single carrier output power requirements - Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR # Topic #4: Power class framework for CA - Agreements - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study how to maximize the UL transmission power capability for CA - Prioritize discussion for single carrier in RAN4#117 and RAN4#118 # Topic #5: Tx requirements - Agreements - PA modelling is discussed in system parameter thread - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear - Spurious emissions requirements, SEM and ACLR will be defined for 6GR - FFS for - OOB boundary - potential accommodations for different frequency sub-ranges - Encourage inputs on regulatory survey being conducted in spectrum agenda - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5) - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework # Topic #6: Rx requirements - Agreements - Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s - Study concept for RX requirements for new frequencies, especially those above 7.125GHz - Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases - Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well - Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference - Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements - Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable # Topic #7: Frequency range between FR1 and FR2-1 - Agreements - Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz. # Topic #8: Spectrum aggregation - Agreements - Below spectrum aggregation topics can be discussed as starting point. - RF switch-time performance - Need for intra-band support of non-contiguous UL CA and RB allocations - Study benefits-drawbacks of using CA vs. using wider single carrier BW - Study how to account for simultaneous TX/RX between the bands properly from the beginning - Discuss band combination simplification in spectrum agenda # Topic #9: Joint UE and BS RF - Agreements - Encourage companies to contribute whether there are concrete study points for low power receiver. - Coexistence study including need for coex study will be discussed in agenda for BS RF and co-existence. - Prioritize work for the baseline single carrier output power requirement framework before detailed studies on ACLR and SEM relaxation. - Discuss PAPR reduction topics in system parameter agenda - For Tx EVM relaxation - Discuss non-AI based UL demod in UE RF thread - The impact on BS receiver could be considered. - Do not discuss AI based solution(s) in UE RF thread, except the potential common part. - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible. - Strive to study TX non-linearity sources, including PA, in this thread - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G. - Strive to avoid overlapping work between different threads - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g. - Which aspects need to be considered to conclude on feasibility - Need for network control # Topic #10: Others - Agreements - Encourage companies to contribute on UE energy efficiency

Discussion

Single carrier output power requirements

UE Power Class

In recent years PC2 has become de-facto default power class for 5G NR TDD bands. Introduction of PC2 power amplifiers PA’s made possible to do PC1.5 with two PC2 PAs. Therefore, it is natural that 6G baseline is PC2 for TDD bands and perhaps also FDD bands in lower frequencies if PC2 low band PA’s are in market.

When we say that PC2 is default power class we mean that it is the only power class that is mandatory to be defined for new 6G band and it naturally is the first power class to be defined. Additional HPUE power classes can be defined of course also but for eMBB PC3 may not be defined at all.

For 6GR bands the default power class is PC2 (i.e. 26 dBm)

For 6GR bands PC2 is the first power class to be defined.

For 5G interband UL CA, 3GPP specified UL CA power classes in a table manner for PC3 and PC2. After multi-PA designs became common and UEs had combinations of PC3 and PC2 PAs it was decided to introduce higherPowerLimit-r17 capability where instead of defining new UL CA power class in a table the UE output power capabilities are determined from single band operation power class indication by calculating 10 log10 ∑ pPowerClass,c. This way for inter-band UL CA/DC UE is allowed for each band to transmit up-to single band power class capabilities.

higherPowerLimit-r17

Indicates whether UE supports increase in maximum output power above the power class indication for inter-band UL CA and NR-DC band combinations as defined in clause 6.2A of TS 38.101-1

In 6G RAN4 do not define interband UL CA power classes. UL CA output power capabilities are determined by constituent single bands power classes. Approach is similar as NR higherPowerLimit feature.

MPR for eMBB UE types

Here with eMBB we mean “basic UEs” which have requirements specified without a clause suffix in 36.101 or 38.101-1. In 4G and 5G different MPRs have been specified for different power classes but MPRs are quite similar between different power classes.

In 6GR we think that there is a need to define MPR separately to battery operated mobile UEs and non-battery-operated UEs i.e. mains supply operated UEs such as FWA and CPE. Reason being that for battery operated UEs current consumption is the key parameter that needs to be managed and that causes a trade of between PA linearity and UE current consumption where suitable balance always needs to be found. In mains supply operated UE current consumption is not as critical, and PA linearity and other analogue component characteristics such as LNA noise figure and IQ-modulator performance can be improved. This means that need from MPR and A-MPR is reduced for mains supply operated UEs compared to battery-operated UEs and REFSENS is improved.

All analogue component performance improvements in mains supply operated UE compared to battery operated UE needs to be evaluated

Dedicated MPRs are defined for battery-operated UEs and mains supply operated UEs.

Dedicated A-MPRs are defined for battery-operated UEs and mains supply operated UEs.

UE type indication is needed to be able to separate battery-operated UEs and mains supply operated UEs.

Pcmax

Upon studying the technical specification 38.101-1, one can observe that the configured transmitted power for 5G is fairly complex which is understandable as there is legacy carried from 4G and new requirements added by different releases of 5G. We think that for 6G there are opportunities to optimize the equations and parameters for PCMAX,f,c

The configured maximum output power PCMAX,f,c is set within the following bounds for single carrier 5G in 38.101-1:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

Regarding 6G, we think that the same framework can still be utilized, albeit in a simplified and optimized form. From our view the necessity of incorporating ΔTIB,c or ΔPPowerBoost into the equation needs reconsideration. The parameter ∆TC,c, may be updated to align with technological advancements available today or within 6G timeframe. The upper limit of PCMAX_H,f,c, should be constrained only by term PEMAX,c,  to allow UE to transmit its full capability. Furthermore, although it does not influence the equation, the application of ΔPPowerClass  in SRS scenarios could be re-evaluated.

Investigate the simplification of PCMAX,f,c equations and the optimization of associated parameters

Tx requirements

SEM, ACLR and Spurious emissions

Traditionally General SEM is defined by 3GPP and additional SEM’s have been added based for regional regulations. ALCR (and ACS) comes from co-existence simulations between 3GPP systems and simulations are performed by RAN4. Spurious emissions are coming from ITU recommendations. This framework can be used also for 6G.

In previous RAN4 meeting following was agreed

  - Spurious emissions requirements, SEM and ACLR will be defined for 6GR

    - FFS for

      - OOB boundary

In section 2.8.1 of this paper we are promoting a scheme called UL scheduling range where Foob is determined by BS channel bandwidth which is figure below is called flexible channel bandwidth. Flexible channel bandwidth is another improvement we seek for 6G and is discussed in system parameters thread but in simplest form flexible CH BW = BS CH BW = UE CH BW.

Figure 2.4.1-1: OOB boundary

Best UE performance would be achieved if Foob is determined by BS channel bandwidth in case UE CH BW is smaller

PA dimensioning

In 6G we are proposing a modest PA linearity update for eMBB battery operated UE compared to NR. More specifically we propose to use in MPR simulations a PA dimensioning: MPR = 0 for CP-OFDM QPSK Inner. This is the PA calibration point which is fixed and determines MPRs for rest of the modulations. Some of the modulations and waveforms would have negative MPR which could be seen as built in boosting capability as output power would exceed Ppowerclass.

PA dimensioning for battery operated eMBB UEs is MPR = 0 for CP-OFDM QPSK Inner region.

Furthermore we propose to have different PA dimensioning point for UEs that are operated from mains power source such as FWA. Reason being that there are less restriction on power consumption thus PA can be operated in more linear region.

PA dimensioning for mains power operated eMBB will produce more UE Tx output power than PA dimensioning for battery operated eMBB UEs

Modulator impairments

Modulator impairments should be based on what is actually needed for most demanding modulation order. In 5G it is impossible to meet UL 256-QAM EVM requirement with the nominal 28 dBc LO leakage and IQ-image requirements, instead much more stringent modulator impairment values are budgeted for EVM of 256-QAM compared to lower order modulations. This is not very technical approach. Hence baseline for 6G modulator requirements should be at least what is needed for 256-QAM and if higher modulation orders are considered then modulator requirements need to be derived from those. Improvement of modulator impairments will directly impact to MPR and A-MPR numbers and increase UE Tx power.

Modulator impairments should be based on what is actually needed for most demanding modulation order to meet EVM requirement.

Rx requirements

REFSENS

In the current specification, REFSENS is defined in absolute numbers for two antenna ports, except for n7, n38, n41, n48, n77, n78, n79, and n104 for which four RX antenna ports is assumed, in Table 7.3.2-1a. This is for PC3. Then in Table 7.3.2-1b, REFSENS is defined for TDD, SDL, and FDD bands with variable duplex operation as an equation. Then there are two delta tables Table 7.3.2-1c and Table 7.3.2-1d, which specifies RSD for PC3 to PC2, with and without TX diversity. The RSD with TX diversity is more severe compared to without TX diversity. Then there are two more delta tables Table 7.3.2-2a and Table 7.3.2-2a for four RX (ΔRIB,4R) and eight RX (ΔRIB,8R). Regarding 1 RX (e)RedCap devices, there is a flat delta ΔR1R defined in Table 7.3I.2-1, which some companies think is a mistake.

The uplink configuration for validation of the REFSENS requirement is another big table with barely any justification of why and how those numbers came into existence. There are bands for which due to reasons varying from small TX-RX separation, harmonics, etc., smaller uplink configuration is utilized instead of full channel allocation. However, all these reasoning are hidden behind absolute numbers, thus making the specification very unintuitive.

Current specification with only absolute numbers is unintuitive.

For many bands, due to a myriad of reasons, neither the delta values nor the absolute REFSENS values scale with channel bandwidth. As a result, specifying only absolute numbers obscures the underlying complexity and rationale behind how the specification was defined. This presents a significant challenge for future-proofing the specification, as it embeds hidden assumptions and impairments and for any change required, one must first reverse engineer the original impairment calculations, then recalculate the values using new assumptions. It would aid in future proofing and technical correctness of the specification if the band specific values are agreed and documented.

Current way of specifying REFSENS is not future proof.

To make specifications future proof document the underlying assumptions, band specific impairment factors, and calculation methods explicitly in the normative specifications.

Another issue with 5G REFSENS specification is related to the values assumed for the NF and IM. The NF number of 9.0 dB was first proposed back in 1999 in meeting#1[6]. Since then, there has been leaps and bound improvement in the device technology, external LNAs are the norm providing a significant improvement in the noise performance of the UE. However, these improvements are not visible in the UE RF specification.

UE NF of 9.0 dB was first proposed in 1999.

Implementation margin (IM) of 2.5 dB for REFSENS calculation is another parameter that has been stuck in the past. Further, it is a black box of margin as there is no consensus on what exactly it covers, why it is same for TDD and FDD bands. This unfortunately is another ambiguous number with no clear description.

Implementation margin of 2.5 dB as a catch all margin is ambiguous and contributes to the intuitiveness of the specification.

A unified equation covering various cases such as different duplexing, multiple receivers, etc., shall be utilized for defining the REFSENS requirements.

A better NF assumption shall be agreed for 6GR keeping the technology advancement of last two decades.

Investigate if different REFSENS needs to be defined for battery operated and mains supplu operated UEs

Study the need and possible values for implementation margin.

ACS and Blocking

3GPP has defined several UE blocking requirements. These are in band blocking (IBB), out of band blocking (OOB) and narrow band blocking (NBB) which have been almost unchanged for a quite a long time, in fact there are virtually no or just small differences between 4G and 5G requirements.

These blocking requirements have worked well and there may not be any need to make big (or any) adjustments to levels of blockers or ranges of interferers. Obviously blocking requirements for 6G need discussion and changes are possible but there would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker levels and interferer offsets.

There would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker level and interferer offsets.

As was discussed we do not necessarily see a need for changing the blocker levels and interferer offsets, but we see a need to restructure how we write the spec and especially how we do the tables. In our view especially how the IBB requirement is defined needs improvements. In our view major reason why IBB blocking requirement is obscure and hard to decipher from tables is that IBB1 is with fixed offset, similarly as ACS and IBB2 is a range similarly as OOB. It would make quite a lot of sense to re-organize ACS and IBB requirements such that ACS and IBB1 are combined together as ACS1 and ACS2, levels are kept the same. IIB2 would become the only IBB requirement see Fig 2.1.8-1.

Figure 2.1.8-1: Possible 6G UE ACS and Blocking requirements

How the UE blocking requirements are defined/organized should be discussed and not just copied from NR.

Self-interference due to spectrum aggregation

Self-interference has been a major problem since carrier aggregation was introduced. The whole setup surrounding requirements for self-interference has been challenged by considerations of corner test cases, minimum or coincidental channel combinations ending up at harmonic or intermodulation issues, untestable since antenna isolation impacts performance, and test points for conducted operation allow plenty of PA power back-off that non-linear products vanish in conducted testing.

Marking a CA combination for MSD is easily misinterpreted as a CA combination that should be avoided, but this is often not the case, when used by operators, since they do not have, or they may avoid the spectrum that may cause the MSD type of interference.

Marking a CA combination for MSD is easily misinterpreted as a CA combination that should be avoided, but this is often not the case.

A simplification is needed to:

- Make self-interference (and MSD) a minor effect

- Reduced or No requirements on performance, but mark the CA combination as MSD risk

- Mitigate self-interference when it is present

Learning from Low-band carrier aggregation switching

In the study of Low-band carrier aggregation switching (LBCA switching, or LBCAsw), it was desired to make better use of spectrum in the downlink. Operators showed use cases where DL spectrum was not supported, although it would serve for downlink load balancing. The aspect of switching the downlink carrier operation between two downlink bands was asked, but the answer was more comprehensive. In some cases, the UE antenna would prevent CA operation and in other presented cases the downlink switching would cause self-interference to the UE for cases like CA_n12A-n29, since the supplementary downlink (SDL) would be next to the uplink carrier in the frequency domain. The study concluded not only with a downlink switching scheme, but more importantly an uplink-downlink switching scheme that would eliminate the self-interference issue and likewise apply to the challenge in CA_n5A-29A of large-distanced low band frequencies on same antenna. What may not have been realized is that this study showed that self-interference may be mitigated with Tx-Rx switching.

Self-interference may be mitigated with Tx-Rx switching

As a further development on the study of MSD and UE self-interference, the switching method designed through the LBCAsw study should be considered for 6GR as a standardized configuration for band combinations that have MSD

The switching method designed through the low-band carrier aggregation switching study should be considered for 6GR as a standardized configuration for band combinations that has MSD.

With this approach there is no self-interference, at the expense of load-balancing between UL and DL. Everything is NW managed but based on UE/RAN4 analysis, ensuring the UE will never be subject to self-interference in CA configurations again. In case the self-interference is found in the Rx operation of a TDD band, it would be an option to apply TDD operations at the uplink of the FDD carrier as a simple way to make the relation in the Tx-Rx switching. Similar if the uplink is TDD and the downlink is FDD, the downlink switching of the FDD carrier may follow the TDD switching scheme. For FDD-FDD cases the Rx-Tx switching scheme may be any that are not allowing simultaneous uplink and downlink at the different bands.

These specific features for handling carrier aggregation, as here with switching, needs to be either a part of the fundamental framework of carrier aggregation in 6G or there needs to be explicit indications when a specified carrier aggregation configuration is dependent on these. The notation framework for band combinations is discussed further under the 6G spectrum agenda (8.5).

Simplification of MSD requirements

Even if the considerations to address MSD through switching, there will be CA combinations and UE implementations that do not support the switched operation, so MSD requirements as identified in 5G NR, may remain in 6G, but in a new and simpler format. Nokia showed in [4] how combining CA requirements and relate the requirements to band groups, would create a simpler and smaller list of affected combinations.

Many companies have now raised exactly the aspect of band groups in 6G, and we therefore present again the work from 5G NR that the MSD requirements can be simplified in tables that define:

- General relaxation per MSD type (UL Harmonic, Harmonic mixing, Cross-band interference and IMD)

- Band groups to add granularity in relaxation numbers

- CA combination list with aggressor, victim and co-existence issue identifiers including text equation

To simplify the specification a suggestion is to list relaxation per UE issue in a single table and then list all the types of issues for the specific band combinations in list of supported band combinations. By grouping a list of issues per band combination would provide an overview of the individual band combinations combined need for UE relaxation. An extract from [4] shows a few band combination examples of the simpler table.

Table 2.5.5-1: Extract of simplified table of [4].

| NR CA band combination (CA_nX-nY) | NR Rx band (nZ) | Uplink Harmonic | Harmonic Mixing | Cross Band Interf. | Order | NC ULCA | IMD test point equation UL Band(x,y) to DL Band(z) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 |  |  |  | IMD3 same |  | I2*nX-nYI=nZ |
| CA_n1-n3 | n3 |  |  | 1,>2 |  |  |  |
| CA_n1-n8 | n1 |  |  |  | IMD4 |  | I2*nX-2*nYI=nZ |
| CA_n1-n28 | n1 | UL3 |  |  |  |  |  |
| CA_n1-n38 | n1 |  |  | >2 |  |  |  |

The changes proposed in [4] were not done during NR times as the specification impact was deemed too large but for 6G the change principle can be used from the beginning.

Then the above-mentioned overview table could point to MSD LUTs.

More preferably of course if there is a way to characterize band combinations without a specific overview table and just define LUTs then that would reduce a lot of RAN4 workload.

RAN4 to discuss whether to define for 6GR a single table of supported band combinations together with their impairments requiring relaxation and pointer to MSD LUTs or as an alternative define rules for band combinations which point to MSD LUTs

Advanced self-interference determination

Regardless of switching mechanisms or simplified tables, the critical issue of self-interference persists—what is the tangible impact of the actual UE radio configuration? Every MSD value of the specification is based on assumptions of an architectural implementation reaching back to LTE and every MSD value up for agreement in RAN4 has been based on common agreement. This eliminates any opportunity for implementation advantages, as the MSD compliance testing is disconnected from real-world sources and coupling. Even if a UE design excels in performance, an operator may reject the band combination due to concerns or ambiguities in the specifications. Where is the motivation to make a good UE design?

We see a need to study advances in UE self-interference determination as a further assistance information for; first the network radio link information; later operators to utilize spectrum in CA combinations that may have been rejected for use.

There is a need to study advances in UE self-interference determination how that information is provided for NW as a further assistance information.

Frequency range between FR1 and FR2-1

Since the general distribution on the frequency range between FR1 and FR2-1 takes place under the Spectrum agenda (8.5) this section only addresses some of the aspects related to UE requirements which may be different in the bands around 7GHz, 12GHz and 15GHz which is currently mentioned as potential 6G bands.

Discussion on general aspects of the frequency range between FR1 and FR2-1 takes place under the Spectrum agenda (8.5). Specific for UE requirements is included under this agenda.

Expected changes to UE architecture in the range between FR1 and FR2-1

Adding new spectrum will most likely result in more antennas included into the design of the UEs. This may create enhanced penalties to existing known UE issues, such as antenna gain and loss in multi antenna designs, isolation between the modes of operation, and multiplication in front-end components to expand MIMO capabilities in UE uplink and downlink.

Adding additional hardware, as antennas, for the new frequency range between FR1 and FR2-1 may result in degradation in overall performance of the UE across all supported frequency bands.

6GR implies use of new spectrum and potentially more MIMO streams and thus potentially much more complex systems than current smart phone UE design. It would be advantageous if 6GR requirements would be written such that the impact of the design for the new spectrum ranges and operation does not simply converge to relaxation requirements.

RAN4 shall ensure that including the frequency range between FR1 and FR2-1 shall not negatively impact requirements for operation in other frequency bands.

Spectrum aggregation

UL Switching

The initial value proposition of UL Tx switching in 5G was aimed at improving data rates and reducing latency by utilizing one of the UE transmitter chains for a new carrier when UE is operating in 2TX TDD mode during the DL phase. However, practical challenges, such as switching gaps, complicated PCell control plane processes and the outcome was a reduction in anticipated benefits. Furthermore, network operation is dynamic, hence the switching event is triggered by DCI. This will make both UE and network operation ambiguous. It would be desired to use a semi-static switching pattern same way as for switched LBCA feature. UEs indicate switching period capabilities of {35, 140, 210} microseconds, which the network uses to schedule temporal gaps by blanking symbols, leading to UL throughput penalties of 1, 2, or 3 symbols depending on the UE's capability. By the time switching period capabilities were established for Rel-16, durations were rooted in legacy implementation. Considering modern RF chip technology and leveraging insights from 5G, from our perspective it is feasible to avoid utilizing at least the longest capability of 210us. Even today there is a special case of 0 microseconds switching time for SUL (TS 38.101-1, 5.2C, Note 1), applicable to UEs configured with both NR UL and SUL carriers requiring simultaneous Rx/Tx operations. Currently 5G specifications mandates DL interruptions when UL switches from one carrier to another which is having detrimental impact on throughput. The need for the interruptions in modern RF chips may be reevaluated.

UL switching should be based on semi-static switching pattern

Re-evaluation of switching period lengths to match modern RF chip technology

Re-evaluation of the need for DL interruptions considering modern RF chip technology

DL-UL Switching

This is known also as LBCA switching, or LBCAsw. In WID for the LB CA switching for release 19, there was a recommendation to utilize the Rel-16 TX switching framework as much as possible. This can be seen problematic as the reasons why UL switching framework ended up to the form it is having now are because of the dependencies that are unique for UL implementation.

For DL, there are more HW resources available that can be activated at specific time instances without necessitating the use of the same HW for two carriers during the switch event. The factors influencing the required time for DL carrier switching differ from those for UL. RFFE reconfiguration for DL switching is more straightforward than for UL and is typically performed concurrently with the transceiver. Unlike dynamic UL switching, DL CA switching operates based on a semi-static RRC pattern which makes planning for timing predictable.

The switching periods for DL CA are specified as {35,70,140}us. The network is tasked with scheduling resources for a UE, taking into account the signalled capability and total timing advance (TTA). UL transmission at the UE side precedes DL reception by the TTA amount. If a UE shares an antenna with the two carriers to be switched, or if the UL band is close to the DL band, simultaneous UL/DL operation is prohibited. At late phase of Rel19 a new requirement emerged that UL transient periods cannot overlap with the switching periods in time.

Figure 1 FDD to SDL UL time mask options with TTA =0us. The number of blanked UL symbols is dependent on the position of the UL transient period with respect to the switching period.

Figure 2 SDL to FDD UL time mask options with TTA =0us. The number of blanked UL symbols is dependent on how much the UE can tolerate the UL and switching gap overlap.

As the network schedules the switching gap based on the number of symbols, the longest switching period results in the blanking of one “extra” symbol for UL when TTA=0.

Release 19 operation is such that the two DL carriers operate one at the time based on the switching pattern. Release 20 intend to update the feature such that it allows the two carriers operate simultaneously during other cycle of the switching pattern

UL switching framework is ill-suited for DL.

Transient periods and timing advance are adversely affecting the DL throughput.

DL Switching

No NR framework exists for DL Rx switching, but its use-case originates from fragmented CA, as a way to switch to another fragmentation or band when adjacent channel leakage is too high, and for load balancing. As there is no UL involved the required switching time from one carrier to another may be seamless if different LOs and RX chains are used. In case of using shared RX chain and doing an LO retune the required switching times are still shorter than in other two switching features.

RAN4 shall aim for unified framework covering all three switching methods

UE channel bandwidth adaptation

Despite its initial motivations to support diverse UE bandwidth capabilities and enhance power saving, the 5G NR BWP design presents several significant challenges. These include considerable signalling overhead and complex configuration changes, primarily because BWP switching involves unnecessarily heavy Radio Resource Control (RRC) configuration, designed to support impractical use cases such as different Subcarrier Spacings (SCSs) per BWP.

BWP switching framework 5G NR has considerable problems. The lack of robustness is one problem. In the related error case, gNB and UE have different understanding on the active BWP, which may result in radio link failure with significant service interruption. Another problem relates to relatively long BWP switching delays during which the UE is not required to transmit any UL signals or receive any DL signals. There was also a notable lack of RAN4 involvement in NR BWP, which particularly affects uplink coverage. This is because many UL BWP configurations allow unnecessarily large Maximum Power Reduction (MPR) or Additional MPR (A-MPR), thereby lowering the minimum requirement of UE transmit power. Finally, the coupled Uplink (UL) and Downlink (DL) BWP in TDD is problematic. 5G TDD mandates that UL and DL BWPs share the same center frequency, a restriction that results in configuration and scheduling limitations, suboptimal UL coverage, and prolonged switching times.

The 5G NR BWP design, despite aiming for UE power saving and diverse bandwidth support, is hampered by excessive signalling overhead, poor switching robustness and latency, a lack of RAN4 involvement leading to large MPR/A-MPR, and restrictive coupled UL/DL BWPs in TDD.

For 6G Bandwidth Adaptation, while addressing 5G's shortcomings, 6G must continue to support varying UE bandwidth configurations and capabilities. It is crucial to define the necessity of bandwidth adaptation without reintroducing complexity. The bandwidth switching framework needs to be reconsidered to make it robust by design, and to minimize both switching delay and the service interruption involved.

In downlink, bandwidth adaptation is useful for UE power saving, supporting UEs with limited BW capabilities. A part of the needed BW adaptation solution is an unified solution to facilitate SSB measurements for the cases when cell-defining SSB is outside the UEs (active) DL BW. Whether UE power saving in DL is achieved with BW adaptation, PDCCH adaptation, or both may need further investigation. In uplink, BW adaptation is critical from a coverage perspective and necessitates active RAN4 involvement. For example, proper placement of LO along with BW adaptation needs attention to minimize A-MPR.  In addition to dynamic bandwidth adaptation supporting also load balancing scenario, 6GR needs to support initial BWP functionality, which can be seen as a key enabler for less capable UEs. RAN4 should consider the RF switching time performance, and target to minimize the delays with considerations on LO location.

Since the 6G target is to enable leaner BW adaptation by solving the lessons learned from 5G, 6G could reconsider the terminology for BWP. One option could be to call it as a scheduling range.

Study 6GR bandwidth scalability by early cooperation with other working groups with the following aspects:

- Simplify Bandwidth Adaptation:

  - Reduced configuration overhead

  - Reduced number of bandwidth adaptation options compared to NR.

  - Support only a single Subcarrier Spacing (SCS) per carrier.

  - Minimize the number of BW adaptation types and BW-specific parameters.

- Separate DL and UL BW Adaptation:

  - Consider DL and UL bandwidth adaptation independently to optimize each direction.

  - Assume separate LO for DL and UL BW adaptation in TDD

- Improve Robustness, Reduce Latency and minimize interruptions in BW adaptation:

  - Enhance the reliability and speed of bandwidth adaptation – the procedure must support robust failure recovery.

- Study to optimize RF/RRM switching delays and MPR/A-MPR with better assumptions related to LO placement in bandwidth adaptation

- Streamline UE Capabilities:

  - Define concise set of UE capabilities for BW adaptation and identify the essential operation to ensure good baseline feature support for BW adaptation operation.

Another issue is that 6G may support channel bandwidths up to 200MHz or 400MHz. While NR supports intra-band contiguous carrier aggregation (CA) to utilize such bandwidths, in 6GR primary focus should be in optimization of single wideband carrier to handle contiguous spectrum allocations at least for channel BWs up to 200MHz. Single wideband carrier maximizes UL power, minimizes overhead e.g., in terms of common channels and avoidance of guard bands, and enables easier BW adaptation. Whether there is a need for intra-band UL CA should be carefully discussed, as there may be no need to support multiple solutions for the cases where aggregated BW is not larger than minimum requirement for maximum (DL/UL) UE CBW.

Wide single carrier bandwidth brings clear benefits over CA, and there may be no need to support multiple solutions for the cases where aggregated BW is not larger than minimum requirement for maximum (DL/UL) UE CBW.

Flexible UL & DL pairing

From RAN1#122 several companies presented benefits for flexible pairing of uplink and downlink carriers of different bands. Flexible pairing of DL and UL carriers should be possible for all the types of bands like TDD, FDD and SDL bands. We see number of benefits and use cases for flexible pairing of DL and UL carriers from different frequency bands without having operations on more than one DL and one UL carrier at the time. To make flexible pairing of DL/UL practical we see that it is best suited for co-sited deployments, which allows UE to use the DL reference signal of the DL carrier for accurate synchronization and pathloss measurements to ensure proper timing alignment for all subsequent uplink physical layer procedures.

Flexible pairing of DL and UL carriers could be used to improve the UL coverage for cell-edge UEs without negative throughput and capacity impacts on DL operations. For cell-edge UEs the network could configure UL carrier on a lower frequency band like the 700 MHz frequency band while DL carrier could be maintained on a higher frequency band like 3.5 GHz or upper 6GHz band with wider channel bandwidth. In this way we would not need to compromise DL performance due to UL coverage reasons.

. For cell-edge UEs the network could configure UL carrier on a lower frequency band like the 700 MHz frequency band while DL carrier could be maintained on a higher frequency band like 3.5 GHz or upper 6GHz band with wider channel bandwidth.

Flexible pairing of DL and UL will be also beneficial from the UE power saving perspective as the UE can receive and transmit signal only on one DL and UL carrier respectively. This will also enable resource utilization and capacity benefits as the network does not need to reserve resources for a given UE on more than one DL and UL carrier.

Flexible pairing of DL and UL will be also beneficial from the UE power saving perspective

Furthermore, load balancing in practical deployments will perform better. In today’s networks significant amount of traffic during CA operations is in the primary cell, which is not good from the load balancing and system performance perspective.

Flexible DL/UL pairing would benefit load balancing in practical deployments

Key considerations for flexible DL and UL pairing:

- Example scenarios for flexible pairing

  - Mid-band TDD + Low-band FDD (DL/UL available on both carriers)

  - A new TDD band at ~7 GHz + Low-band FDD.

  - A new TDD band at ~7 GHz + 3.5 GHz TDD.

  - Low-band FDD + SDL

- Initial access may be allowed via low-band or mid-band.

- Low band UL carriers may be shared across multiple DL carriers but UE may be using only one DL and UL carrier at the time.

- Within a certain band combination, there could be possibilities of flexible DL CA and UL combinations without actual CA operations

- For timing requirements across the carriers co-sited deployment can be assumed for flexible DL and UL carrier pairing.

Figure 2.2.3-1: Example of different flexible UL and DL pairing configurations

RAN1 and RAN2 are leading efforts to address flexible uplink and downlink pairing but securing RAN4's involvement is essential to ensure comprehensive coverage under the spectrum aggregation agenda. Awareness must be created within RAN4 to align its contributions with the ongoing developments.

As the topic is still in its early stages of development, RAN4's perspective is critical for generating inputs that align with RF specifications, deployment assumptions and ensure a viable solution. Uplink (UL) and downlink (DL) serve distinct purposes and have different requirements. UL focuses on coverage, which is optimized at lower frequencies, while DL emphasizes throughput, achieved through larger channel bandwidths typically unavailable at lower frequencies. Pairing one UL carrier with one DL carrier would create an improved "single band" state, but this approach introduces challenges related to synchronization and timing for uplink control, which requires specific deployment constraints like co-sited deployments so that UE can use the given DL carrier for UL timing and pathloss estimation like discussed earlier in this section.

In RAN4, RF requirements will need to address new combination-sets of supported UL and DL bands. Some combinations, such as n8 UL and n3 DL, may introduce implications like UL harmonics, necessitating the use of MSD. RAN4 must study requirements for these combinations and assess the implications of either introducing new bands or covering combination-sets across existing bands, along with operational requirements.

Additional requirements which the flexible combining of the DL band with any of the related UL bands creates may be captured in specification separately. See an example tables below, where a downlink band (x) supports pairing with uplink carriers different to (x), as y, z and q. When pairing these, there could be a need to define new requirements to such operation as indicated in the column “Requirement” if the 2 CA case is not defined for x+y, x+z or x+q, otherwise reference may be made to the CA requirements.

| DL carrier on 6G band | UL carrier on 6G band | “Requirement” Tx and Rx |
| --- | --- | --- |
| x | y | x and y |
|  | z | x and z |
|  | q | x and q |
| y | z | y and z |
|  | q | y and q |
|  | x | y and x |

Table 2.5.5-1: Additional requirements from flexible combining

For higher order combinations the tables could be implemented in the specifications as:

| DL carriers on 6G band | UL carrier on 6G band | “Requirement” Rx |
| --- | --- | --- |
| x-y | z | x and y |
|  | q | x and y |

Table 2.5.5-1: Additional requirements for DL CA from flexible combining

Joint UE and BS RF

UL scheduling range

For 6G, sone of the goals are to increase the UL Tx power and lower the UE power consumption. These were also the goals on 5G BWP but did not materialize in RAN4 specs. In Rel-19 RAN4 worked on a single carrier MPR reduction scheme which allows in some limited cases convert outer allocations into inner allocations, but benefits of this feature are rather limited especially as it does not address A-MPR cases.

In 6G we would like to take Rel-19 work on single carrier MPR enhancement further and make it more flexible and also applicable for A-MPR. Basic principle is shown in figure 3.2.1-1.

Figure 3.2.1-1: UL scheduling range

The UL scheduling range would have the following properties

- UL scheduling range is configured to UE, semi-statically.

- UL scheduling range bandwidth is smaller or at maximum the same as the UE and/or BS channel bandwidth

- UE places the local oscillator (LO) in the middle of the UL scheduling range (bandwidth)

- UE can set UL baseband processing, LO and RF filters according to UL scheduling range bandwidth and location in frequency.

- Bandwidth of intermodulation distortion (IMD) interference generated in the power amplifier (PA) due to allocation, LO leakage and IQ-image are directionally proportional to the bandwidth of the UL scheduling range i.e. the smaller the UL scheduling range the less interference

- UE Tx current consumption decreases when UL scheduling range bandwidth decreases as baseband and D/A-conversion clock rates can be reduced.

Impact to MPR reduction is achieved by proper LO placement and reduction of UL scheduling range bandwidth into what is needed instead of what is UE channel bandwidth. In R19 RAN4 WI this approach was already partially defined but the flexibility will be constrained, and hence there is room for improvement.

Impact to MPR reduction is achieved by proper LO placement and reduction of UL scheduling range bandwidth into what is needed instead of what is UE channel bandwidth.

Figure 2.3.1-1: UL scheduling range impact to MPR

UL scheduling range impact to A-MPR reduction will be significant as in some cases A-MPR can be fully avoided. This is because the IMD created in power amplifier by the IQ-Image, LO leakage impairments and the actual transmission are not spread as far in out-of-band region comparing the case when UL bandwidth is same as UE channel bandwidth.

UL scheduling range impact to A-MPR reduction will be significant as in some cases A-MPR can be fully avoided.

Figure 2.3.1-2: UL scheduling range impact to A-MPR

Proposal 13: RAN4 needs to consider bandwidth adaptation (UL scheduling range) from the beginning, to minimize MPR and A-MPR.

UL scheduling range is configured by the network semi-statically for each UE individually. When UL scheduling range is configured to the UE, then UE set’s LO in the middle of the scheduling range and adapts filters as necessary. LO and filter are unchanged until scheduling range is reconfigured. UE UL scheduling instances happen within the UL scheduling range bandwidth.

UL scheduling range is configured by the network semi-statically for each UE individually.

Tx EVM relaxation

Following agreements have been made as part of way forward R4-2514642:

- Agreements … - For Tx EVM relaxation - Discuss non-AI based UL demod in UE RF thread - The impact on BS receiver could be considered. - Do not discuss AI based solution(s) in UE RF thread, except the potential common part. - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible. - Strive to study TX non-linearity sources, including PA, in this thread - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G. - Strive to avoid overlapping work between different threads - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g. - Which aspects need to be considered to conclude on feasibility - Need for network control

RAN #109 approved new WI for NR in [5]. Tx EVM relaxation was discussed in RAN as part of the WI, however there was a decision to continue the work in RAN4 6G study context, hence there is a note in WID:

Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

According to the note above RAN4 should discuss the scope and other details including whether there is network control for the feature. In our view there should definitely be network control/configuration, it is difficult to understand how could the feature be specified without. Network should have possibility to operate also without the relaxed UE Tx EVM. Also scope of the work and all assumptions should be clarified. Scope and assumptions should include at least following:

- Scenarios including waveform and frequency range(s)

- Receiver assumptions

- Evaluation assumptions

- RAN4 specification impact

Network control would be needed in case 3GPP agrees to introduce UE Tx EVM feature

RAN4 to discuss the scope and assumptions for the relaxed UE Tx EVM feature

EVM is intended as a general measure of the transmitter accuracy. Anomalies such as IQ imbalance, phase noise, nonlinear degradation, filter passband ripple and group delay, and thermal noise are all factors that contribute to EVM. The total EVM attributed to the various degradation effects is the sum of the square of EVMs due to each source of degradation.

The below table [TS38.101-1, Clause 6.4.2.1] for the UE transmitter lists the 5G EVM requirements after correcting for sample timing offset, RF frequency offset and carrier leakage:

Table 6.4.2.1-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

3GPP defines the MPR requirements in uplink as maximum allowed power reduction such that UE transmit power shall satisfy the minimum RF requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

With PA non-linearity (or other impairments) compensation, the receiver can basically estimate or learn the impairments of the PA and compensate those in the receiver. General architecture of the digital post distortion (DPoD) model is shown in figure 2.3.3-1.

Figure 2.3.3-1: UL / DL general architecture including DPoD block.

As shown in figure 2.3.3-2, for 256QAM EVM is always the bottle neck, while for lower orders (e.g., 64QAM) it is mainly either IBE (in the inner region) or ACLR (in the outer region). Also for 256QAM, if EVM was relaxed, IBE would become the gating factor. For this reason, relaxed TX EVM could be suitable mainly for FR1 in the case of a high modulation scheme, e.g. >=256 QAM where the EVM is the gating factor. In other words, gain potential only for 256QAM or higher unless also IBE/ACLR is relaxed. The result below shows only the gating factor for each modulation but does not show the level where the next possible RF requirement would become a gating factor in case the requirement was changed for the current gating factor. Hence it does not show anything about the possible gain potential for each modulation. Gain potential should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

There would be gain potential mainly for 256QAM where EVM is always the bottle neck, while for lower orders (e.g., 64QAM) it is mainly either IBE (in the inner region) or ACLR (in the outer region)

Gain potential for Tx EVM relaxation should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

It would be beneficial if PA model discussed in this agenda item will also be used for the system parameter – waveform discussion, both in RAN1 and RAN4. However, if PA model targeted to this feature is not readily available then especially RAN1 cannot stop work on waveform evaluation because of that. RAN4 should check what is the situation in PA model situation in Q1 2026 and discuss if PA model for Tx EVM relaxation feature is ready to be shared with RAN1, if it is not ready RAN4 still needs to respond RAN1 as they sent an LS to RAN4 and asked PA model for their analysis purposes.

Aim to use the same PA model for RAN1 waveform evaluation and RAN4 TxEVM relaxation under advanced receiver assumption study.

Among the PA models studied in TR 38.803 (V14.4.0), we see the Hammerstein N-L model (unfortunately called “Wiener model” in the report) as the most appropriate for our studies. The Hammerstein N-L model is linear-in-parameters, it is invertible (when nonlinearity is invertible, and linear block is invertible), and its inverse (with L-N structure) can be used in conformance tests definition, to compare the upper bound of possible gain and linear RX performance.	
The performance of Hammerstein N-L is similar to memory polynomial (MP) models [8], i.e., it does not seem worth it to specify an MP model and its many more parameters.	
Similarly, Volterra series models have too many parameters that can lead to overfitting and poor generalization, which can make the use of such models debatable and even controversial. In the case of conformance test definition, generalization is crucial.	
Simple memoryless AM-AM/PM-AM tables may be feasible after study, but the approach is expected to overestimate the performance of DPoD schemes, as they are exceedingly simple to invert and learn.

Having a memoryless PA model gives unrealistic advantage for Digital post distortion evaluation

Use a Hammerstein (N-L) model for modelling of the PA as an example.

Based on our gating factor analysis presented in Figure 2.3.3-2 Tx EVM relaxation is only useful for 256-QAM as stated earlier but for 64-QAM and lower where IBE is the gating factor another methods could be considered.

Figure 2.3.3-2: Gating factors, 100MHz, 30 KHz

IBE is already limiting the UL maximum power in 5G NR in many cases such as for all waveforms, power classes, most modulation orders and RB allocations, etc.

5G NR Rel-16 allowed Network to configure UEs in FR2 serving cells to omit IBE requirements completely and boost UL power since UEs are clearly spatially separated. However, this solution is not fully applicable in lower frequency ranges due to wider beam and/or UE beamforming architecture. Thus partial IBE relaxation, as depicted in Figure 2.3.3-3, by shifting IBE edge from active RBs can be considered in lower frequency ranges. The partial IBE relaxation allows to achieve up to 2.5 dB power gain even without Tx EVM relaxation for modulation orders below 256 QAM and assist Tx EVM relaxation to achieve higher gains for high modulation order such as 256QAM.

- Figure 2.3.3-3: Partial IBE relaxation for higher UL maximum power

Partial and full IBE relaxation can increase UL power for all waveforms and most modulation orders in all frequency ranges and can increase Tx EVM relaxation gain for high modulation orders such as 256 QAM.

RAN4 to consider NW controlled partial and full IBE relaxation without Tx EVM relaxation for modulation order up to 64QAM, and with Tx EVM relaxation for higher modulation order.

Testing and verification

RAN4 should not define requirements that cannot be verified, this is main thing we want to say, but the related testing requirements and test & verification methods should be discussed in OTA session.

The definition of conductive and OTA tests for new 6G bands around 7, 12 and 15 GHz will depend on the UE implementation and complexity of the test setup in the TE chamber.

For example, in a 4Tx conductive test, the test will be complicated because 4 cables are required to connect to the UE PCB. So, the OTA test is preferred. Another example is that the connector on the PCB can significantly impact RF radiation performance in the FR2 frequency range; therefore, only OTA tests are defined, no conductive tests. So, if the connector also has significant impact to RF radiation performance in 6g new frequency range, then only OTA tests will be defined. However, the detailed discussion should be moved to 6G OTA session.

RAN4 should not define requirements that cannot be verified

Detailed testing and verification discussions is done in testing/ota agenda.

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations and proposals have been made:

Proposal 1: For 6GR bands the default power class is PC2 (i.e. 26 dBm)

Proposal 2: For 6GR bands PC2 is the first power class to be defined.

Proposal 3: In 6G RAN4 do not define interband UL CA power classes. UL CA output power capabilities are determined by constituent single bands power classes. Approach is similar as NR higherPowerLimit feature.

Observation 1: All analogue component performance improvements in mains supply operated UE compared to battery operated UE needs to be evaluated

Proposal 4: Dedicated MPRs are defined for battery-operated UEs and mains supply operated UEs.

Proposal 5: Dedicated A-MPRs are defined for battery-operated UEs and mains supply operated UEs.

Proposal 6: UE type indication is needed to be able to separate battery-operated UEs and mains supply operated UEs.

Proposal 7: Investigate the simplification of PCMAX,f,c equations and the optimization of associated parameters

Observation 2: Best UE performance would be achieved if Foob is determined by BS channel bandwidth in case UE CH BW is smaller

Proposal 8: PA dimensioning for battery operated eMBB UEs is MPR = 0 for CP-OFDM QPSK Inner region.

Proposal 9: PA dimensioning for mains power operated eMBB will produce more UE Tx output power than PA dimensioning for battery operated eMBB UEs

Proposal 10: Modulator impairments should be based on what is actually needed for most demanding modulation order to meet EVM requirement.

Observation 3: Current specification with only absolute numbers is unintuitive.

Observation 4: Current way of specifying REFSENS is not future proof.

Proposal 11: To make specifications future proof document the underlying assumptions, band specific impairment factors, and calculation methods explicitly in the normative specifications.

Observation 5: UE NF of 9.0 dB was first proposed in 1999.

Observation 6: Implementation margin of 2.5 dB as a catch all margin is ambiguous and contributes to the intuitiveness of the specification.

Proposal 12: A unified equation covering various cases such as different duplexing, multiple receivers, etc., shall be utilized for defining the REFSENS requirements.

Proposal 13: A better NF assumption shall be agreed for 6GR keeping the technology advancement of last two decades.

Proposal 14: Investigate if different REFSENS needs to be defined for battery operated and mains supplu operated UEs

Proposal 15: Study the need and possible values for implementation margin.

Observation 7: There would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker level and interferer offsets.

Observation 8: How the UE blocking requirements are defined/organized should be discussed and not just copied from NR.

Observation 9: Marking a CA combination for MSD is easily misinterpreted as a CA combination that should be avoided, but this is often not the case.

Observation 10: Self-interference may be mitigated with Tx-Rx switching

Proposal 16: The switching method designed through the low-band carrier aggregation switching study should be considered for 6GR as a standardized configuration for band combinations that has MSD.

Proposal 17: RAN4 to discuss whether to define for 6GR a single table of supported band combinations together with their impairments requiring relaxation and pointer to MSD LUTs or as an alternative define rules for band combinations which point to MSD LUTs

Observation 11: There is a need to study advances in UE self-interference determination how that information is provided for NW as a further assistance information.

Observation 12: Discussion on general aspects of the frequency range between FR1 and FR2-1 takes place under the Spectrum agenda (8.5). Specific for UE requirements is included under this agenda.

Observation 13: Adding additional hardware, as antennas, for the new frequency range between FR1 and FR2-1 may result in degradation in overall performance of the UE across all supported frequency bands.

Proposal 18: RAN4 shall ensure that including the frequency range between FR1 and FR2-1 shall not negatively impact requirements for operation in other frequency bands.

Proposal 19: UL switching should be based on semi-static switching pattern

Proposal 20: Re-evaluation of switching period lengths to match modern RF chip technology

Proposal 21: Re-evaluation of the need for DL interruptions considering modern RF chip technology

Observation 14: UL switching framework is ill-suited for DL.

Observation 15: Transient periods and timing advance are adversely affecting the DL throughput.

Proposal 22: RAN4 shall aim for unified framework covering all three switching methods

Observation 16: The 5G NR BWP design, despite aiming for UE power saving and diverse bandwidth support, is hampered by excessive signalling overhead, poor switching robustness and latency, a lack of RAN4 involvement leading to large MPR/A-MPR, and restrictive coupled UL/DL BWPs in TDD.

Proposal 23: Study 6GR bandwidth scalability by early cooperation with other working groups with the following aspects:

Observation 17: Wide single carrier bandwidth brings clear benefits over CA, and there may be no need to support multiple solutions for the cases where aggregated BW is not larger than minimum requirement for maximum (DL/UL) UE CBW.

Observation 18: . For cell-edge UEs the network could configure UL carrier on a lower frequency band like the 700 MHz frequency band while DL carrier could be maintained on a higher frequency band like 3.5 GHz or upper 6GHz band with wider channel bandwidth.

Observation 19: Flexible pairing of DL and UL will be also beneficial from the UE power saving perspective

Observation 20: Flexible DL/UL pairing would benefit load balancing in practical deployments

Observation 21: Impact to MPR reduction is achieved by proper LO placement and reduction of UL scheduling range bandwidth into what is needed instead of what is UE channel bandwidth.

Observation 22: UL scheduling range impact to A-MPR reduction will be significant as in some cases A-MPR can be fully avoided.

Proposal 24: Proposal 13: RAN4 needs to consider bandwidth adaptation (UL scheduling range) from the beginning, to minimize MPR and A-MPR.

Observation 23: UL scheduling range is configured by the network semi-statically for each UE individually.

Proposal 25: Network control would be needed in case 3GPP agrees to introduce UE Tx EVM feature

Proposal 26: RAN4 to discuss the scope and assumptions for the relaxed UE Tx EVM feature

Observation 24: There would be gain potential mainly for 256QAM where EVM is always the bottle neck, while for lower orders (e.g., 64QAM) it is mainly either IBE (in the inner region) or ACLR (in the outer region)

Proposal 27: Gain potential for Tx EVM relaxation should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

Proposal 28: Aim to use the same PA model for RAN1 waveform evaluation and RAN4 TxEVM relaxation under advanced receiver assumption study.

Observation 25: Having a memoryless PA model gives unrealistic advantage for Digital post distortion evaluation

Proposal 29: Use a Hammerstein (N-L) model for modelling of the PA as an example.

Observation 26: IBE is already limiting the UL maximum power in 5G NR in many cases such as for all waveforms, power classes, most modulation orders and RB allocations, etc.

Observation 27: Partial and full IBE relaxation can increase UL power for all waveforms and most modulation orders in all frequency ranges and can increase Tx EVM relaxation gain for high modulation orders such as 256 QAM.

Proposal 30: RAN4 to consider NW controlled partial and full IBE relaxation without Tx EVM relaxation for modulation order up to 64QAM, and with Tx EVM relaxation for higher modulation order.

Observation 28: RAN4 should not define requirements that cannot be verified

Observation 29: Detailed testing and verification discussions is done in testing/ota agenda.

- References

R4-2514642, WF for [116bis][102] 6G general RF and UE RF, RAN4#116bis, Feature lead (Qualcomm)

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

R4-2514626, WF on 6G Spectrum

R4-2413338, On UE RF specifications table improvement, Nokia

RP-230864, “Uplink coverage enhancements”, Nokia, Nokia Shanghai Bell

RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5”

TSGW4#1(99)005, MS receiver sensitivity in UTRA FDD mode, Nokia, RAN4#1, Espoo, Finland, 21st – 22nd January, 1999.

A. Melendez-Cano et al., "Behavioral Modeling for Power Amplifiers Comparing MPM, Wiener and Hammerstein with FPGA-Based Implementation," 2016 International Conference on Mechatronics, Electronics and Automotive Engineering (ICMEAE), Cuernavaca, Mexico, 2016, pp. 119-124, doi: 10.1109/ICMEAE.2016.030.