3GPP TSG-RAN WG4 Meeting #117			R4-2520556

Dallas, TX, USA, Nov 17th – 21st, 2025

Agenda item:	8.10

Title:	Discussion on sensing for 6GR

Source:	Samsung

Document for:	Approval

- Introduction

In RAN#108, the Integrated Sensing and Communication (ISAC) on 6G Radio (6GR) has been approved as a new study item [1,2], where RAN4 working group is expected to study and discuss the physical functions and procedures, RF, coexistence, and testability for radio sensing.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In RAN#109, the key performance indicators and use cases of ISAC have been discussed, and the core technical performance requirements and basic use cases have correspondingly added to TR38.914. [3]

## 5.1 Key performance indicators - Sensing-related capabilities are measured in terms of the following technical performance requirements: - Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. - Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. - Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. - [Sensing Resolution: TBD] ## 5.4 Requirements of New and Existing Services - The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

In this contribution, we discuss the potential RAN4 scopes for ISAC on 6GR and develop the basic working plan for RAN4 6GR ISAC topic before receiving any results from RAN1.

- Discussion

Integrated Sensing and Communication have been envisioned as one of the most promising vertical service in 6GR, which is expected to enable the dedicated communication system with addition environmental perception capability, and thus provide better mobile user experience in the newly emerging scenarios, such as low-altitude economy, automotive, and digital twins, etc. To capture the cross-domain service capability, the framework of physical interfaces, signal processing procedures and device behaviors needs to be updated to match the performance requirements for both sensing service and communication service for 6GR. However, a brunch of RAN4 works are dependent on the decision of other working groups. Thus, before sufficient progress has been made by RAN1/RAN2/RAN3, RAN4 will first discuss the regulations and potential RAN4 dependent scopes in the ISAC area.

2.1 Regulatory Status and Requirements

As one of the emerging topics in mobile communication field, there has hardly been any specific discussions or regulations on the ISAC topic. Concerning that radar technologies have developed and evolved for a long time, the regulations of radar systems from different institutions would be beneficial for the further RAN4 6G sensing discussions. In the following subsection, the report of ISAC and regulations on the radar system from the Ministry of Industry and Information Technology (MITT) of China and FCC are listed to provide useful information for further RAN4 6GR ISAC discussions.

2.1.1 Radar Regulations of MIIT

First of all, there are various kind of radar system in reality, each of them designed for different application scenarios in corresponding professional areas, for example, air traffic control radar, weather radar, maritime radar, automotive radar, speed radar, etc. Generally, different type of radars operates in different frequency range and have different RF requirement/regulations. For RAN4 6G Sensing topic, it is necessary to identify the which kind of radar is mostly relevant to the sensing in ISAC from either application scenario or operation frequency range perspective.

From the application scenario point of view, there have been two kind of radars that are closely related to the sensing use cases defined in RAN plenary meeting, which are: micro-target detection radar and short-range detection radar. For these two types of radars, the radio frequency range regulations are defined as follows

Section 6 Micro Target Detection Radar Article 23 [Micro Target Detection Radar] According to the primary service classification of radiolocation, based on the target size of detected objects, the frequency ranges used for micro target detection radar (including insect detection, bird detection, drone detection radar, etc.) are as follows: (1) 2900 – 3100MHz frequency band. (2) 9000 – 10000MHz frequency band. (3) 15.7 – 17.2GHz frequency band. Section 7 Other Radars Article 24 [Short-Range Detection and Unmanned Aerial Vehicle Radar] According to the primary service classification of radiolocation, the frequency range used for short-range detection radar and unmanned aerial vehicle radar is the 24 – 24.25GHz frequency band. Relevant requirements shall be implemented in accordance with Announcement No. 52 of 2019 by the Ministry of Industry and Information Technology.

In specific, the Micro Target Detection Radar is defined as

Micro Target Detection Radar: A ground-based radar used to detect small targets such as insects, birds, or drones, obtaining information about their quantity, position, speed, and performing inversion analysis of their intrinsic characteristics.

which corresponds to the UAV use cases discussed in RAN#109 and TR38.914[3].

As the ISAC is an emerging technology, there is no specific regulations on the ISAC-supported devices. However, there is indeed a regulation clause related to the new type of radar, which is quoted below

Chapter 3 Management of Radar Frequencies, Stations, and Transmission Equipment Article 26 [Experimental Authorization] To encourage and support scientific research and the application of advanced technologies, and to meet the needs of economic and social development for new types of radar, the use of radar that complies with frequency allocation but does not conform to the frequency ranges and types specified in the above provisions shall, upon completion of necessary electromagnetic compatibility analysis and demonstration, apply to the national radio regulatory authority for temporary or experimental frequency usage authorization. Relevant requirements shall be implemented in accordance with the "Radio Frequency Usage Licensing Management Measures."

The above regulations provide the definitions and frequency range of some types of radar. From the RAN4 perspective, the RF technical characteristics, coexistences of ISAC system, etc. are of critical interest. Although, the ISAC device possesses both communication and sensing functions and may have different behavior compared with dedicated radar system, based on the RAN1 and RAN plenary discussions, the multiplexing method for the communication and sensing counter parts in 6GR ISAC technology is most likely to be orthodox multiplexing access, e.g., TDM, FDM. Therefore, the RF technical requirement of radar system would be valuable for the future discussion of RF requirements for the 6GR ISAC systems. For the convenience of discussion, the general RF requirements for the radar devices is attached below

Annex 2 Radio Frequency Technical Requirements for Radar Equipment 1. General RF Technical Requirements for Radar 1.1 Frequency Tolerance Operating Frequency Range; Limit. 9 – 1606.5 kHz; 100×10-6. 1606.5 – 4000 kHz; For the power less or equal to 200W: 20×10-6. For the power above 200W: 10×10-6. 4 – 29.7 MHz; 10×10-6. 29.7 – 100 MHz; 50×10-6. 100 – 470 MHz; 50×10-6. 470 – 2450 MHz; 500×10-6. 2450 – 10500 MHz; 1250×10-6. 10.5 – 40 GHz; 5000×10-6. Above 40 GHz; 5000×10-6. 1.3 Spurious Emission Power The measurement frequency range for spurious emissions is shown in the following table: Equipment Operating Frequency Range; Spurious Emission Frequency Range. Minimum Frequency; Maximum Frequency. 9 kHz – 100 MHz; 9 kHz; 1 GHz. 100 MHz – 300 MHz; 9 kHz; 10th Harmonic Frequency. 300 MHz – 5.2 GHz; 30 MHz; 5th Harmonic Frequency. 5.2 GHz – 13 GHz; 30 MHz; 26 GHz. 13 GHz – 150 GHz; 30 MHz; 2rd Harmonic Frequency. 150 GHz – 300 GHz; 30 MHz; 300 GHz. The spurious emission power limits are as shown in the following table: Spurious Emission Frequency Range; Limit; Measurement Bandwidth; Detection Method. 9 kHz – 150 kHz; -30 dBm or -100 dBc; 1 kHz; RMS. 150 kHz – 30 MHz; 10 kHz; RMS. 30 MHz – 1 GHz; 100 kHz; RMS. 1 GHz – 200 GHz; 1 MHz; RMS. 1.4 Receiver Selectivity Radar equipment shall operate normally when subjected to interference signals as specified in the following table. Additionally, the leakage power of interference signals measured within the 40dB bandwidth of the radar receiver shall not exceed 12dB of the minimum detectable signal level. Frequency Offset Related to Operating Center Frequency; Interference Signal Level*; Roll-off Factor of Interference Signal. 0.5 to 5 times the -40dB bandwidth; 40 – 70 dB or -30 dBm; -30 dB/dec. 5 to 10.8 times the -40dB bandwidth; 70 – 90 dB or -30 dBm; -60 dB/dec. Above 10.8 times the -40dB bandwidth; -30 dBm; 0 dB/dec. * Note: Interference signal level relative to the minimum detectable level of the radar equipment..

Specifically, for the Micro Target Detection Radar, the RF requirements are defined as follows:

7. Small Target Detection Radar 7.1 RF Technical Requirements for Small Target Detection Radar in the 2900-3100MHz Frequency Band (1) Operating Frequency: 2900 – 3100MHz. (2) Peak Power: No exceeding 60kW. (3) Occupied Bandwidth: No exceeding 50MHz. (4) Spectrum Emission Mask: Comply with the relevant provisions of the general technical requirements for radar in this Annex. (5) Frequency Tolerance: No exceeding 1250 × 10-6. (6) Spurious Emission Power: Spurious Emission Frequency Range; Limit; Measurement Bandwidth; Detection Method. 30 MHz – 1 GHz; -30 dBm or -100 dBc; 100 kHz; RMS. 1 GHz – 26 GHz; -30 dBm or -100 dBc; 1 MHz; RMS. Note: The spurious domain is defined as the region outside 2.5 times the necessary bandwidth from the center frequency.. (7) Noise Figure: No exceeding 3dB. (8) Receiver Selectivity: Comply with the relevant provisions of the general RF technical requirements for radar in this Annex. 7.2 RF Technical Requirements for Small Target Detection Radar in the 9000-10000MHz Frequency Band (1) Operating Frequency: 9000 – 10000MHz. (2) Peak Power: No exceeding 60kW. (3) Occupied Bandwidth: No exceeding 50MHz. (4) Spectrum Emission Mask: Comply with the relevant provisions of the general technical requirements for radar in this Annex. (5) Frequency Tolerance: No exceeding 1250 × 10-6. (6) Spurious Emission Power: Spurious Emission Frequency Range; Limit; Measurement Bandwidth; Detection Method. 30 MHz – 1 GHz; -30 dBm or -100 dBc; 100 kHz; RMS. 1 GHz – 26 GHz; -30 dBm or -100 dBc; 1 MHz; RMS. Note: The spurious domain is defined as the region outside 2.5 times the necessary bandwidth from the center frequency.. (7) Noise Figure: No exceeding 5dB. (8) Receiver Selectivity: Comply with the relevant provisions of the general RF technical requirements for radar in this Annex. 7.3 RF Technical Requirements for Small Target Detection Radar in the15.7-17.2GHz Frequency Band (1) Operating Frequency: 15.7 – 17.2GHz. (2) Peak Power: No exceeding 60kW. (3) Occupied Bandwidth: No exceeding 50MHz. (4) Spectrum Emission Mask: Comply with the relevant provisions of the general technical requirements for radar in this Annex. (5) Frequency Tolerance: No exceeding 5000 × 10-6. (6) Spurious Emission Power: Spurious Emission Frequency Range; Limit; Measurement Bandwidth; Detection Method. 30 MHz – 1 GHz; -30 dBm or -100 dBc; 100 kHz; RMS. 1 GHz –2rd Harmonic Frequency; -30 dBm or -100 dBc; 1 MHz; RMS. Note: The spurious domain is defined as the region outside 2.5 times the necessary bandwidth from the center frequency.. (7) Noise Figure: No exceeding 8dB. (8) Receiver Selectivity: Comply with the relevant provisions of the general RF technical requirements for radar in this Annex.

2.1.2 Report and Potential Regulations of FCC

Similarly, since ISAC is a novel concept, FCC does not have dedicated rule or regulations on this technology. But FCC does pay attention to this newly emerging technology and is currently studying 6GR ISAC policy issues. There is also working-group report to the FCC Technology advisory council containing the information of 6GR ISAC. From the report, we can take a glimpse of the concerns of FCC on this topic and potential future regulations. The ISAC related content in this report is summarized below for the discussion convenience.

Integrated Sensing and Communication (ISAC) In 3GPP, ISAC stands for Integrated Sensing and Communication, a technology that combines wireless communication with sensing capabilities, such as radar-like detection, localization, and environmental sensing within the same system. The 3GPP Service requirements for Integrated Sensing and Communication provides a robust framework for integrating sensing capabilities into 5G networks, enabling advanced applications such as object detection, environmental monitoring, and human activity tracking. It emphasizes security, privacy, regulatory compliance, and energy efficiency while addressing the technical challenges of implementing sensing services. The outlined performance requirements and operational guidelines ensure that wireless sensing services can meet diverse application needs while maintaining high standards of accuracy, reliability, and user protection. Integrating sensing capabilities into 5G networks requires addressing several key considerations to ensure performance, security, privacy, and regulatory compliance: Performance - Accuracy and Resolution: High precision in detecting and tracking objects, including horizontal and vertical positioning, velocity estimation, and range resolution, is essential. - Latency and Refresh Rate: Low latency and high refresh rates are critical for real-time applications like motion tracking and collision avoidance. - Error Metrics: Minimizing missed detection and false alarm probabilities is vital for reliable sensing results. - Energy Efficiency: Adjusting sensing parameters and temporarily disabling unused transmitters/receivers can optimize energy consumption. Regulatory Compliance - Spectrum Usage: Sensing operations must adhere to spectrum regulations, including the use of licensed and unlicensed bands, and ensure operator control in network coverage areas. - Operator Policies: Configuration, activation, and deactivation of sensing services must align with operator policies and local regulations. - Consent and Contextual Information: User consent is required for certain operations, such as combining 3GPP and non-3GPP sensing data or linking results to subscriber identities. 6G Integrated Sensing and Communication (ISAC) brings exciting possibilities but also faces several technical and policy/regulatory challenges. Policy and Regulatory Challenges include: - Spectrum Allocation and Licensing Regulators must allocate and license spectrum bands that support both high-bandwidth communication and precise sensing without causing harmful interference. This may involve revisiting current spectrum policies and creating new frameworks for dynamic and shared spectrum use. - Privacy Regulations Sensing capabilities can capture detailed environmental and personal data, raising privacy concerns. Compliance with data protection requires clear policies on data collection, storage, consent, and usage. Security Standards and Compliance Ensuring that ISAC systems adhere to cybersecurity standards to prevent unauthorized access and tampering. - Cross-Border Coordination Wireless signals and sensing data may cross national borders, necessitating international coordination on spectrum use and data sharing. Harmonizing regulations among countries is vital to avoid conflicts and enable global interoperability. - Liability and Ethical Considerations Determining liability in cases where sensing failures lead to accidents or privacy breaches. Policies must clarify responsibilities among manufacturers, service providers, and users, and address ethical issues related to pervasive sensing.

Advisements & Recommendations Integrated Sensing and Communication (ISAC) 1. ISAC business cases of network as a sensor, sensing-as-a-service will bring new architectural and policy questions to address sensing, positioning and localization of people, objects, cars and UAS/ airspace security. 2. ISAC will bring to fore security and privacy considerations of how, where and to whom the data collected by Mobile network operators will be accessible. 3. There will be considerations of how much spectral bandwidth will be allocated for sensing v/s communication tasks and how monetization frameworks will be developed. 4. ISAC and radar sensing, radio location communication rules and policy 5. The rise of 3GPP Integrated Sensing and Communication heralds transformative wireless capabilities but challenges the FCC’s traditional regulatory framework based on function-specific spectrum and device rules. To fully enable ISAC, it is recommended the FCC address issues such as: a. Adaptation of spectrum allocation and sharing policies. b. Development of new interference management and device certification rules. c. Coordination of privacy and safety considerations. d. Leading forward-looking rulemaking to enable coexistence and innovation. Security, Privacy, and Data Governance 5. Consideration of spectrum allocation between sensing and communication functions must factor into policy development. Spectrum Considerations and Policy Alignment 4. The FCC should identify spectrum needs across various radio services, balancing allocations between communication and sensing tasks to support emerging 6G use cases.

Based on the above regulation status survey, it can be found that ISAC is a relatively new technology area, and currently, there have not been any established regulations or RF requirements on this technology yet.

Currently, ISAC is a still relatively new technology area, and there have not been any established regulations or RF requirements on this technology yet.

Although there is no established regulation for reference, 6GR ISAC discussion may excavate valuable insight from the existing regulations on the communication systems and similar radar systems, because operation principles behind the ISAC and the traditional dedicated communication and radar systems may share some similarities. The existing RF requirements defined for communication systems and radar systems can serve as valuable references and starting points for the study of ISAC. Some of these legacy requirements may remain applicable, either directly or with suitable adaptation, to ISAC systems. Thus, RAN4 could adopt the existing requirements in regulations as starting point or reference for 6GR ISAC discussions.

RAN4 could adopt the existing requirements in regulations as starting point or reference for 6GR ISAC discussions.

2.2 Potential Architecture of ISAC Systems

Before discussion of potential architecture of ISAC system, it is worth noting that ISAC is a very general concept which refers to the systems that can integrate sensing capability and data transmission capability in a single hardware platform. However, the concept of sensing is not as clear as data transmission / communication. Sensing could refer to the radar-like detection, localization, trajectory tracking, etc. Due to this vague concept definition, sensing devices as well as ISAC systems could vary significantly, and their corresponding architecture could also be greatly disjoint. Although the scope of the architecture discussion is limited to the RAN plenary use cases, the detail use cases and sensing mode of ISAC system is not agreed on the last RAN plenary meeting. Since the architecture of ISAC system highly depends on the sensing mode selection, RAN4 have to decide which sensing mode to be focused on before make any decision on architectures.

2.2.1 Sensing Mode Discussion

Generally, multitudinous sensing scenarios places various performance requirements for sensing system. For instance, the UAV or vehicle detection and tracking scenarios demand ISAC system to possess large sensing coverage in order to detect the remote object in sky. The human gesture detection scenarios require ISAC system to have sufficient high sensing resolution in order to serve for indoor human activities identification task. A single ISAC system may not support such diverse performance requirements from different use cases. To cope with these requirements, ISAC systems can be classified to different types based on six sensing modes, which includes: 1) Transmit and Receive Point (TRP) monostatic sensing mode, 2) UE monostatic sensing mode, 3) TRP-to-TRP bistatic sensing mode, 4) UE-to-UE bistatic sensing mode, 5) TRP-to-UE bistatic sensing model, 6) UE-to-TRP bistatic sensing model. Each sensing mode has its own advantages and disadvantages. On the one hand, for the monostatic sensing modes, the sensing task is generally performed employing full-duplex method. In these cases, the node is fully in charge of sensing task and is with no necessity of sharing information with other nodes using extra reference signals, which is more implementation efficient. On the other hand, TRPs have more computation resources and less power supply limitation than UEs and thus the TRP related sensing system may have larger sensing coverage and better sensing capability. In addition, a UE’s location is more flexible than that of a TRP, which may support flexible environment sensing/target detection tasks. Based on these advantages, RAN4 could consider TRP monostatic sensing mode, TRP-to-TRP bistatic sensing mode, TRP-to-UE sensing mode, and UE-to-TRP sensing mode in ISAC technology study at first.

RAN4 considers TRP monostatic sensing mode, TRP-to-TRP bistatic sensing mode, TRP-to-UE sensing mode, and UE-to-TRP sensing mode in ISAC technology study at first.

However, considering the potential commercial deployment scenarios, Proposal 2 does not indicate that the other sensing modes are completely excluded from the future study. Furthermore, RAN4 may select one possible sensing mode as the warm-up option to push forward the in-depth technical discussions, then align the topic with RAN1 after RAN1 6G ISAC study begins.

RAN4 may select one possible sensing mode as the warm-up option to push forward the in-depth technical discussions, then align the topic with RAN1 after RAN1 6G ISAC study begins.

2.2.2 Hardware Architecture

In general, the hardware architecture of an ISAC system is also dependent on the sensing mode for the selected ISAC system. For convenience of analysis, the architectures can be categorized into to two groups: monostatic sensing architecture, and bistatic sensing architecture. Taking the monostatic sensing as an example, the sensing waveform is generated from the baseband digital signal processor, then fed to transmit antennas through up-converting, DAC, power amplifying and RF filtering operations. After reflection of the target, the echo signal is received by the receive antenna, and down-convert to baseband signal through RF chains. During the signal receiving process, interference cancellation techniques may be adopted, depending on the multiplexing type and detailed sensing implementation of transceiver.

Fig. 1 Block diagram of an ISAC system working in monostatic sensing mode

For the bi-static sensing ISAC system, the hardware architecture can be modeled in a similar way. But the sensing operation involves multiple ISAC devices as follows:

Fig. 2 Block diagram of an ISAC system working in bistatic sensing mode

However, the interference cancellation components may also exist in the bistatic sensing ISAC devices, because there may be data transmission need for the device. If the sensing task occupies the entire time and frequency resource at the given time period, the architecture can be further simplified to the following one.

Fig. 3 Simplified block diagram of an ISAC system working in bistatic sensing mode

Based on the above discussions, it can be discovered that the architecture of an ISAC system depends on the sensing mode of the corresponding system. The discussion of potential ISAC architecture requires RAN4 identify which sensing mode to be studied.

The architecture of an ISAC system depends on the sensing mode of the corresponding system. The discussion of potential ISAC architecture may require RAN4 identify the sensing modes to be studied.

2.2.3 Antenna configuration

For ISAC systems, the configuration of antenna pattern can influence the final performance of the ISAC system. Depending on their respective locations and sensing entity, antenna configuration may hold settings corresponding to those sensing entities’ antennas. Meanwhile, in monostatic sensing modes, transmit and receive antennas could exist at the same location. Under such circumstances, sensing would typically employ full-duplex methods unless distance become excessively large, necessitating consideration of antenna panel separations designed to mitigate self-interference. Based on this consideration, the below options for antenna configuration with physical separation can be considered as shown in Fig. 4:

- Option #1: The total number of antenna elements in monostatic equal to that of communication. That means, if the total number of antennas in communication is L, then the number of transmitting antennas in monostatic is L/2, and the number of receiving antennas is also L/2.

- Option #2: The total number of antenna elements in monostatic double that of each direction in communication. That means, if the total number of antennas for downlink in communication is L, then the number of transmitting antennas in monostatic is L, and the number of receiving antennas is also L.

Due to differences in antenna configurations affecting antenna gains in channel impulse response, common assumptions will be necessary for efficient calibration work.

| Option 1: The total number of AEs in monostatic equal to communication | Option 2: The total number of AEs in monostatic double to communication |
| --- | --- |

Fig. 4 Antenna configuration for TRP monostatic sensing

In monostatic sensing mode, Tx and Rx antennas could exist at the same location. The monostatic sensing would typically employ full-duplex methods unless a distance to the target become excessively large, necessitating consideration of antenna panel separations designed to mitigate self-interference.

Different antenna configurations affect antenna gains in channel impulse response.

RAN4 to identify the antenna configuration for monostatic sensing mode before further performance discussions.

2.2.4 Multiplexing Method for ISAC

The selection of multiplexing method would decide the interference cancellation of an ISAC system and thus influence the architecture of an ISAC system. For multiplexing sensing and communication, three options can be considered, i.e., time-division multiplexing (TDM), frequency-division multiplexing (FDM) and spatial-division multiplexing (SDM).

For TDM method, sensing can be allocated with separate time domain resources with entire bandwidth, which is beneficial for achieving good range estimation performance. In this case, switching time may need to be considered when transitioning between sensing and communication. Note that for the monotonic sensing, simultaneous DL and UL (full-duplex) is needed.

For TDM, dedicated time resources (with full exploitation of the entire allocated bandwidth) are beneficial for range estimation, and the switching time between communication and sensing needs to be considered.

Fig. 5 Example of multiplexing methods for ISAC system

For FDM method, the assumptions of resource allocation between sensing and communication need clarification. Two primary scenarios are typically considered: 1) where frequency resources for sensing are fully separated from those for communication, i.e., sensing and communication occupy mutually exclusive portions of frequency ranges in a band, and 2) where sensing RS mapping occupies a subset of subcarriers of a given frequency range and the remaining subcarriers are multiplexed for DL communication signals.

However, it is worth noting that for the FDM method, only a part of bandwidth is allocated to the sensing service. Thus, the sensing performance (e.g., sensing resolution) may be affected according to this frequency bandwidth configuration.

For FDM, the multiplex method can be classified to two scenarios: 1) multiplexing in a sub-band level, which divides the whole channel bandwidth into two mutually exclusive frequency ranges and 2) multiplexing in a subcarrier-level while occupying the same entire channel bandwidth. The clarification is able to be made based on RAN1’s further investigations.

Although technically feasible, SDM can be deprioritized at least for performance evaluation purposes due to (1) the increased receiver processing complexity and (2) precoding restrictions given the objective of separating the spatial streams.

For TRP monostatic sensing, RAN4 should prioritize TDM and FDM method, while deprioritizing SDM due to its processing complexity and restrictions on precoding selection.

2.3 Potential RAN4 Scopes

Based on the RAN#108 agreements, the physical function, interfaces, signal processing procedure, and performance evaluation should be discussed by RAN1, and the related RAN4 topics have to wait for the conclusion from RAN1. However, the RF, co-existence, and testability topics are mainly dependent on RAN4. Therefore, RAN4 could begin the 6GR ISAC discussions by selecting the RAN4 dependent (mostly) topics in these areas.

2.3.1 Frequency Band Discussion

Frequency band is an important topic in RF related scopes and is also a critical factor for an ISAC system to achieve good performance in either sensing task or communication task. In particular, in the sensing, selecting the appropriate frequency band is important taking into account trade-off such as sensing coverage, sensing performance (range resolution availability for bandwidth and angular resolution availability for antenna array). Traditionally, the frequency range/band is RAN4 dependent topic. Therefore, RAN4 could study the characteristics of each frequency bands, such as FR1 (sub-6 GHz), FR2 (from 24 to 100 GHz) and new FR (from 7 to 24 GHz), and regulations on these frequency band, then discuss which frequency band/bands should be adopted for the 6GR ISAC system.

RAN4 could study the characteristics and regulations of different frequency band, then discuss which frequency band/bands should be adopted for the 6GR ISAC system.

The characteristic of different frequency band can be summarized as follows:

- FR1: FR1 operates in the lower frequency range, offering better propagation through wall and other obstacles. As a result, it provides wider coverage compared to higher frequency band. However, from sensing precision perspective, it may lead to low range resolution because of its limited bandwidth. Additionally, beamforming are less effective

- FR2: FR2 operates in the higher frequency range compared to other bands (e.g., above 24 GHz), offering fast data transmission, but suffering from propagation through obstacles. Its high frequency means that it is highly susceptible to interference from physical objects in its path, leading to reduced signal quality and weaker connections over long distance. On the other hand, from sensing system perspective, high frequency allows for not only using of large bandwidth which can enhance high range resolution than FR1 and but also advanced beamforming performance which enable high angular resolution.

- New FR: it operates in the mid-band frequency range, providing balance between coverage and capacity. While it does not propagate as well through obstacles as FR1, it offers larger bandwidth and better beamforming performance than FR1.

Given that advantages and disadvantages associated with each frequency band, it seems that no single option can meet all potential performance across different scenarios. Therefore, RAN4 need to discuss whether frequency bands in this study will be prioritized or open to all of availability.

RAN4 to discuss whether frequency bands in this study will be prioritized or open to all of availability.

2.3.2 Coexistence Discussion

ISAC combines communication and sensing functionalities within the same radio system, which introduces new RF characteristics compared to traditional communication-only base stations. These new characteristics may include different RF properties, transmission patterns, and signal periodicity associated with sensing functions and its RF radiation behavior may differ from existing NR transmission, potentially affecting coexistence with other systems operating in the same or adjacent frequency bands.

Generally, there are three typical coexistence scenarios for the 6GR ISAC systems:

- ISAC BS – TDD BS adjacent channel coexistence

- ISAC BS – FDD BS adjacent channel coexistence

- ISAC BS – ISAC BS adjacent channel coexistence

Once the target frequency band for ISAC operation is identified, such coexistence discussion can be conducted and the system configuration assumptions, e.g., waveform, could refer to the conclusion made by the RAN1 NR ISAC study item.

RAN4 could discuss the potential types of the adjacent channel coexistence problem for the ISAC system and decide which type of the coexistence problem to be prioritize.

2.3.3 Testability Discussion

ISAC systems introduce sensing functionality into the communication system. However, the sensing operation and the corresponding performance are entirely different from the traditional communication counterpart. The 3GPP standard working groups should develop new system requirements and a new testing framework for the sensing functionality. From the RAN4’s perspective, RAN4 could study the testability related topic for 6GR ISAC if RAN plenary meeting makes any positive decision on the UE related sensing modes.

RAN4 could begin the testability related topics study for 6GR ISAC once there is any decision on the sensing modes.

- Conclusion

- Currently, ISAC is a still relatively new technology area, and there have not been any established regulations or RF requirements on this technology yet.

- RAN4 could adopt the existing requirements in regulations as starting point or reference for 6GR ISAC discussions.

RAN4 considers TRP monostatic sensing mode, TRP-to-TRP bistatic sensing mode, TRP-to-UE sensing mode, and UE-to-TRP sensing mode in ISAC technology study at first.

RAN4 may select one possible sensing mode as the warm-up option to push forward the in-depth technical discussions, then align the topic with RAN1 after RAN1 6G ISAC study begins.

The architecture of an ISAC system depends on the sensing mode of the corresponding system. The discussion of potential ISAC architecture may require RAN4 identify the sensing modes to be studied.

In monostatic sensing mode, Tx and Rx antennas could exist at the same location. The monostatic sensing would typically employ full-duplex methods unless a distance to the target become excessively large, necessitating consideration of antenna panel separations designed to mitigate self-interference.

Different antenna configurations affect antenna gains in channel impulse response.

RAN4 to identify the antenna configuration for monostatic sensing mode before further performance discussions.

For TDM, dedicated time resources (with full exploitation of the entire allocated bandwidth) are beneficial for range estimation, and the switching time between communication and sensing needs to be considered.

For FDM, the multiplex method can be classified to two scenarios: 1) multiplexing in a sub-band level, which divides the whole channel bandwidth into two mutually exclusive frequency ranges and 2) multiplexing in a subcarrier-level while occupying the same entire channel bandwidth. The clarification is able to be made based on RAN1’s further investigations.

For TRP monostatic sensing, RAN4 should prioritize TDM and FDM method, while deprioritizing SDM due to its processing complexity and restrictions on precoding selection.

RAN4 could study the characteristics and regulations of different frequency band, then discuss which frequency band/bands should be adopted for the 6GR ISAC system.

RAN4 to discuss whether frequency bands in this study will be prioritized or open to all of availability.

RAN4 could discuss the potential types of the adjacent channel coexistence problem for the ISAC system and decide which type of the coexistence problem to be prioritize.

RAN4 could begin the testability related topics study for 6GR ISAC once there is any decision on the sensing modes.

References

RP-251861 “New SID: Study on Integrated Sensing and Communication (ISAC) for NR,” RAN#108

RP-251881, “New SID: Study on 6G Radio,” NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

RP-252882 “TR38.914 ‘Study on 6G Scenarios and Requirements (Release 20)’”

Regulations on Radar Radio Management (Trial), Ministry of Industry and Information Technology

A. Gosain, B. Daly, M. Doczkat, K. Etemad, FCC Technology Advisory Council 6G Working Group Report, August 5, 2025

X. Luo, Q. Lin, R. Zhang, et al. ISAC–A Survey on Its Layered Architecture, Technologies, Standardizations, Prototypes and Testbeds. IEEE Communications Surveys & Tutorials, 2025

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)”

TR38.858 “Study on evolution of NR duplex operation (Release 18)”

R1-2506548 “On evaluation assumptions for 6GR air interface”, Moderator (Huawei), 3GPP TSG RAN1 Meeting #122

R1-2505580 “Discussion on ISAC Evaluation assumptions and performance evaluation”, Samsung, 3GPP TSG RAN1 Meeting #122

R1-2505582 “Design of 6GR air interface”, Samsung, 3GPP TSG RAN1 Meeting #122

R1-2505583 “Evaluation assumptions for 6GR”, Samsung, 3GPP TSG RAN1 Meeting #122

R1-2507250 “Discussion on ISAC evaluation assumptions and performance evaluation”, Samsung, 3GPP TSG RAN1 Meeting #122b

Steven M. Kay. Fundamentals of statistical signal processing: Estimation theory, Pearson, 1995

Steven M. Kay. Fundamentals of statistical signal processing: Detection theory, Pearson, 1995