3GPP TSG-RAN WG4 Meeting #117	R4-2520581

Dallas, USA,  17th – 21st November 2025

Agenda item:	7.6.3

Source:	Apple

Title:	On NB-IoT PC1.5 A-MPR

WI/SI:	NR_IoT_NTN_req_Ph2-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

3GPP Rel-17 specifications added support for the NR/5G-based non-terrestrial network (NTN) communication, which was followed by the Rel-17 and Rel-18 activities to enable NTN also for the NB-IOT based solutions. While initially both NR and NB-IOT based solutions had only PC3 power class, Rel-19 enabled support for higher power classes such as PC2.

As a follow-up of the Rel-19 high-power enhancements for NTN, 3GPP Rel-20 approved a new WI, one of the objectives of which is to enable PC1.5, i.e. 29dBm, power class for NB-IOT [1] for the 1Tx UE RF architecture   using bands 256 and 255 as exemplary bands. One of the UE RF Tx requirements discussed during the RAN4#116bis meeting was MPR/A-MPR, for which obviously MPR first needs to be defined.

In this discussion paper we focus on MPR values  based on the simulation results taken for NTN PC1.5 single Tx.

# 2  Discussion

The WF [1] from the RAN4#116bis meeting established  NB-IoT NTN UE Power Class 1.5 MPR tables as starting point for further discussion. Companies were also invited to provide additional evaluation results using one of two specified calibration options, whereupon Option 1 is based on a common RF with NTN PC2.

In this discussion paper we use Option 1 with the following assumptions and requirements from the WF [1]:

- Interested companies can provide further evaluation in the next RAN4 meeting based on following assumptions to check whether modifications on the above two tables are needed - Option 1 - Calibration points for PC1.5 (based on NR PC2 PA): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss /or [5] dB - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc / or [31] dBc - CIM3: 60 dBc - NR ACLR: 31dBc

Simulation results for NB-IoT SEM are provided below:

| ##### 1-Tone ##### (at lower edge) | ##### 3-Tones ##### (at lower edge) | ##### 6-Tones ##### (at lower edge) | ##### 12-Tones |
| --- | --- | --- | --- |

Figure 2-1: NB-IOT MPR simulation results for 29dBm

Observation 1:	1-tone allocation at the edge exceeds the NB-IOT mask limits and should get 0.5dB allowance. However, once 1-Tone is shifted one position into the channel has 0dB power back-off.

Observation 2:	3-tone allocation does not require any power back-off.

Observation 3:	6-tone allocation needs 0.5dB back-off (which is within the existing PC2 MPR limits).

Observation 4:	12-tone alloction requires 2dB (which is within the existing PC2 MPR limits).

Based on the simulation results we are proposing the Table 2-1 for single-tone allocations and Table 2-2 for multi-tone allocations. The only difference compared to the PC2 MPR is addition of 0.5dB power back-off for the 1-tone allocation at the edge.

Table 2-1: Single-Tone Allocations

| Modulation | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- |
| SCS | 15 kHz |  |  |
| Tone positions for 1 Tone allocation | 0 | 1-10 | 11 |
| MPR | ≤ 0.5 dB | 0 dB | ≤ 0.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |

Table 2-2: Multi-Tone Allocations

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 0 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 0.5dB |  | ≤ 0.5 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal: Consider Table 2-1 and Table 2-2 when defining MPR allowance for PC1.5 NTN NB-IoT.

# 4	Conclusions

This contribution provides simulation results for PC1.5 NTN NB-IoT and has the following observation and conclusion:

Observation 1:	1-tone allocation at the edge exceeds the NB-IOT mask limits and should get 0.5dB allowance. However, once 1-Tone is shifted one position into the channel has 0dB power back-off.

Observation 2:	3-tone allocation does not require any power back-off.

Observation 3:	6-tone allocation needs 0.5dB back-off (which is within the existing PC2 MPR limits).

Observation 4:	12-tone alloction requires 2dB (which is within the existing PC2 MPR limits).

Proposal:	Consider Table 1 and Table 2 when defining MPR allowance for PC1.5 NTN NB-IoT.

Table 1: Single-Tone Allocations

| Modulation | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- |
| SCS | 15 kHz |  |  |
| Tone positions for 1 Tone allocation | 0 | 1-10 | 11 |
| MPR | ≤ 0.5 dB | 0 dB | ≤ 0.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |

Table 2: Multi-Tone Allocations

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 0 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 0.5dB |  | ≤ 0.5 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

# 4	References

- RP-252962, “New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2”, Qualcomm, 3GPP TSG RAN Meeting #109

- R4-2515157, “WF for [116bis][318] NR_IoT_NTN_Ph2”, CHTTL, 3GPP TSG-RAN WG4 Meeting #116bis