3GPP RAN4 Meeting #117		R4-2520604

Dallas, USA, November 17-21, 2025

Agenda item:	5.11.4

Source:	Apple

Title:	Views on UE RF requirements for the US 4.9 GHz band

WI/SI:	NR_TDD_band_4900MHz_US-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

3GPP has initiated a work item to specify requirements for the US 4940-4990 MHz band [1].  During the RAN4 #116bis meeting the following agreements related to UE RF were reached [2]:

## <Sub-topic 4-1> Power limits <Agreement>: Maximum output power - Only FCC “high power device” is considered for 3GPP UE, and two requirements below need to be satisfied. - Peak power spectral density: 21 dBm per one MHz. - High power maximum conducted output power: Channel bandwidth (MHz); High power maximum conducted output power (dBm). 10; 30.0. 15; 31.8. 20; 33.0. 25; 34.0. 30; 34.8. 40; 36.0. 50; 37.0. <Way forward>: Maximum output power tolerance - FFS impact of maximum output power tolerance on maximum output power requirements above. ## <Sub-topic 4-2> Additional requirement for FCC mask M applicable to FCC “high power device” <Agreement>: Resolution bandwidth - Adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth. - Spec wording is FFS. <Agreement>: Zero dB reference for applicable FCC Emission mask M - The zero dB reference for FCC Emission mask M is calculated based on UE power class maximum output power and the configured UE channel bandwidth. - The zero dB reference is used for mask location and for A-MPR analysis. . <Agreement>: New NS, Impacted frequency range for new NS, Other requirements for new NS - Specify a new NS for the US 4.9GHz band with FCC part 90 including: - The Mask M requirement (note this covers the 5% channel bandwidth edge (inside), the SEM domain and the spurious domain). - Maximum output power limitation in Sub-topic 4-1. <Agreement>: For the next meeting, companies are invited to provide the simulation assumption in order to have a unified understanding of assumption when the simulation campaign starts. <Note>: It is encouraged companies to study the impact of FCC part 90 for A-MPR simulation.

This contribution provides our views on emission limits, UE to UE coexistence, and Rx aspects.

# 2	Discussion

##### 2.1 Emission limits

The FCC has specified emission limits for the UE in two sections of CFR Part 90: §90.1215 [3] and §90.210 [4].  In our understanding, the UE shall comply with all requirements in both clauses.  The excerpt of the power limits is shown below [3]:

The RAN4 interpretation of these limits is contained in the agreements from [2], and this understanding is ready to be used in the A-MPR simulation assumptions.

Observation 1:	RAN4 has captured the requirements related to the power limits in CFR §90.1215 adequately in R4-2515103.

The excerpt of the emission mask requirement from §90.210 is given below [4]:

The regulation applies to "high power transmitters (greater than 20 dBm)" which suggests that the output power of the transmitter (also denoted as “P”) could represent either the nominal power capability of the transmitter (i.e. power class) or the actual power being transmitted during measurement. This distinction is critical as it will result into different RF requirements and A-MPR allowances.

Interpretation 1: “Output power of the transmitter” as Maximum Output Power (MOP)

Under this interpretation, the output power represents the maximum output power of the transmitter hardware, regardless of current operating conditions and the applied MPR/A-MPR. This approach aligns with traditional spurious emissions handling where limits are based on transmitter capability rather than instantaneous conditions. This interpretation simplifies appliance of requirements and the compliance testing. For example, a PC3 UE would always apply the same spurious limits regardless of whether it is transmitting at full power or backing off due to MPR, A-MPR or power control. In this instance, the calculation of a 0dB reference would be similarly expressed as done in [5].

0dBref=PPC nominal-10log10designated channel BWRBW

The unintuitive aspect of this approach is that a PC2 UE operating at a 3dB power back-off (transmit power: 23dBm) has relaxed performance requirements compared to a PC3 UE operating at a 0dB power back-off (transmit power: 23dBm).

Observation 2:	One interpretation option is to base spurious emissions limits on the transmitter's maximum output power. This simplifies compliance testing but creates the counterintuitive situation where a PC2 device operating at reduced power has relaxed requirements compared to a PC3 device operating at the same actual power level.

Interpretation 2: “Output power of the transmitter” as Actual Transmit Power

Interpreting the output power as the instantaneous or configured transmit power during measurement makes the spurious limits dynamic based on actual operating conditions. The spurious emission limits are directly related to the actual power being transmitted. When a transmitter operates at reduced power then the spurious emissions limits are scaled proportionally. This understanding seems more natural as it does not require knowledge about power class, power tolerance and power boost configuration. The emission test is solely considering the transmit power at the time of measurement and scales the emission mask accordingly. Then the emissions are checked for pass/fail. However, this understanding creates measurement challenges since the emission limits would need to be determined for the instantaneous transmitted power.

In this case the calculation of 0dB reference would be dependent on the configured transmitted power:

0dBref=Ptransmit power-10log10designated channel BWRBW

The scaling approach has the advantage that it appears more natural when requirements are compared between power classes. A PC2 UE operating at a 3dB power back-off (transmit power: 23dBm) has the exact same performance requirements as a PC3 UE operating at a 0dB power back-off (transmit power: 23dBm) as the mask scales with respect to the transmit power.

Observation 3:	Another interpretation is to make emissions limits dynamic by scaling them proportionally to the actual transmit power. This creates a more intuitive approach where devices operating at the same power level have identical requirements regardless of power class. However, it introduces measurement complexity due to the scaling of requirements with respect to transmit power.

With laying out both interpretations, we would like to recommend adopting Interpretation 2. It provides the strictest implementation of requirements and offers the highest assurance of meeting the FCC's intended emission limits.

Proposal 1:	We recommend adopting Interpretation 2, where emissions limits are scaled proportionally to the actual transmit power. It provides the strictest implementation of requirements and offers the highest assurance of meeting the FCC's intended emission limits.

##### 2.2 Spurious emissions for UE co-existence

When specifying requirements on spurious emissions for UE co-existence, it is useful to consider the regional deployment scenario associated with the new band.  Considering that this band’s operation is anticipated to be restricted to the US, the protection requirements should only consider US bands.

Proposal 2:	Requirements on spurious emissions for UE co-existence for the new 4.9 GHz band need to protect US bands only.

We have provided a possible structure of this requirement for the new band in the table below, where the new content is highlighted in yellow, and existing requirements for n77, n78, and n79 are shown for reference:

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 41, 53, 54, 65, 66, 70, 71, 74, 85, 103 NR Band n100, n101, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n78 | E-UTRA Band 1, 2, 3, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 28, 32, 34, 38, 39, 40, 41, 50, 65, 66, 67, 70, 71, 74, 75, 76 NR Band n100, n101, n105, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 34, 38, 39, 40, 41, 42, 65, 74 NR Band n105 | FDL_low | - | FDL_high | -50 | 1 |  |
| [US 4.9 GHz band] | E-UTRA band 2, 4, 5, 12, 14, 25, 26, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 | [NEW NOTE] |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. … NEW NOTE: [US 4.9 GHz band] operation is restricted to the US region |  |  |  |  |  |  |  |

##### 2.3 Rx aspects

An important commercial consideration related to the new 4.9 GHz band is captured in the WID as follows [1]:  “Reuse NR band n79 UE hardware assumption if possible.”  Band n79 hardware assumptions play a significant role in the definition of Rx requirements for the new band.

Considering REFSENS, it is straightforward to anticipate that 6Rx support will be of interest to US operators.  Toward that end, the ΔRIB,6R parameter already specified for band n79 should be reused.

Proposal 3:	The ΔRIB,6R parameter already specified for band n79 should be reused for the new US 4.9 GHz band.

The band n79 hardware assumption has an impact on the definition of the out of band blocking requirement for the new band.  Although the US 4.9 GHz band plan defines a narrower frequency range than the full band n79, the front end architecture in existing UEs which support band n79 is designed for the full range.  To avoid impacting the front end complexity of these UEs, and to avoid introducing the corresponding time to market delay, the out of band blocking requirements for the US 4.9 GHz band need to consider the full span of n79 in the definition.

Proposal 4:	Out of band blocking requirements for the new US 4.9 GHz band need to consider the full span of band n79.

We have provided a suggested approach to implementing Proposal 4 in the out of band blocking requirement definition table below.

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3*BWChannel or 3*BWChannel ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel) or FDL_high + MAX(200,3*BWChannel) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| n104 (NOTE 5) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| [US 4.9 GHz band] (NEW NOTE) | Finterferer (CW) | MHz | N/A | -150 < f – 4400 ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – 5000 < 150 | 1 ≤ f ≤ 4400 – MAX(150,3*BWChannel) or 5000 + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel from the band edge. For BWChannel larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 5175 MHz. For BWChannel > 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. The power level of the interferer (PInterferer) for Range 2 shall be modified to -33 dBm for the range 5925 – MAX(60,3*CBW) ≤ f < FDL_low - MAX(60,3*CBW). NEW NOTE: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 5175 MHz. The power level of the interferer (PInterferer) for Range 2 shall be modified to -33 dBm for the range 5925 – MAX(60,3*CBW) ≤ f < 4400 - MAX(60,3*CBW). |  |  |  |  |  |

# 3	Conclusions

This contribution provides our views on emission limits, UE to UE coexistence, and Rx aspects for then new 4.9GHz band. The following observations and proposals are made:

Observation 1:	RAN4 has captured the requirements related to the power limits in CFR §90.1215 adequately in R4-2515103.

Observation 2:	One interpretation option is to base spurious emissions limits on the transmitter's maximum output power. This simplifies compliance testing but creates the counterintuitive situation where a PC2 device operating at reduced power has relaxed requirements compared to a PC3 device operating at the same actual power level.

Observation 3:	Another interpretation is to make emissions limits dynamic by scaling them proportionally to the actual transmit power. This creates a more intuitive approach where devices operating at the same power level have identical requirements regardless of power class. However, it introduces measurement complexity due to the scaling of requirements with respect to transmit power.

Proposal 1:	We recommend adopting Interpretation 2, where emissions limits are scaled proportionally to the actual transmit power. It provides the strictest implementation of requirements and offers the highest assurance of meeting the FCC's intended emission limits.

Proposal 2:	Requirements on spurious emissions for UE co-existence for the new 4.9 GHz band need to protect US bands only.

Proposal 3:	The ΔRIB,6R parameter already specified for band n79 should be reused for the new US 4.9 GHz band.

Proposal 4:	Out of band blocking requirements for the new US 4.9 GHz band need to consider the full span of band n79.

# 4	References

RP-252929, “Revised WID on Introduction of NR TDD 4.9GHz band for US operation,” Fujitsu, 3GPP RAN #109, September 2025

R4-2515103, “Way Forward for [116bis][322] NR_TDD_4.9GHz_band,” Fujitsu, 3GPP RAN4 #116bis, October 2025

ECFR, “§90.1215 Power limits”, https://www.ecfr.gov/current/title-47/section-90.1215

ECFR, “§90.210 Emission masks”, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I/section-90.210

R4-2514419, “On US 4.9 GHz UE RF requirements”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis