# 3GPP TSG-RAN WG4 Meeting #117	                                                           R4-2520614

# Dallas, USA, Nov. 17-21, 2025

Source:	Apple

Title:	RRM core requirements and testing framework Beam Management

Agenda item:	6.11.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Delay requirements for applicable functionality reporting

- Testing framework based beam management

Discussion

Measurement delay requirements

We are discussing if legacy 5G NR L1-RSRP measurement-delay requirements (TS 38.133 §9.5.4.2) can or can’t be reused when AI/ML-based beam-management (BM) prediction is introduced in RAN4 work. In particular, we are focusing on the values M,N,P defined in legacy measurement delay requirements.

Firstly, let’s have a conceptual recap of the values M, N, P:

- M (temporal-freshness / averaging factor)
Counts how many consecutive CSI-RS periods (after we have already made measurements with every Rx-beam once) the UE must keep gathering fresh samples before it can hand an L1-RSRP value to higher layers. The spec sets M = 1 when a higher-layer timeRestrictionForChannelMeasurement is present or for aperiodic CSI-RS; otherwise M = 3 for periodic / semi-persistent CSI-RS.

- N (spatial-sweep factor)
With repetition = ON the same Tx beam-set is sent in several CSI-RS symbols/ SSB bursts; the UE measures its Rx beam each time.  If repetition = ON for periodic CSI-RS in that resource set N = ceil( maxNumberRxBeam / N_res_per_set ), where: maxNumberRxBeam is a UE capability / handled-beam upper bound signalled and N_res_per_set is the number of CSI-RS resources in the configured resource set (i.e., how many distinct spatial “shots” we already get per period).  In many FR2 RRM test configurations and lab discussions, UEs are assumed to support up to 8 Rx beams (a practical test-capability level reflected in UE capability signalling work and used in various RAN4 RRM setups). When: maxNumberRxBeam = 8, and the network configures one CSI-RS resource in the resource set (N_res_per_set = 1), and repetition = ON (meaning the UE must get enough CSI-RS opportunities to cover all beams), then the formula gives N = ceil(8 / 1) = 8. That’s the scenario that led to  “legacy N = 8”.  Therefore,  “N = 8” is a configuration-derived value, not a spec constant.

- P (availability-loss factor)
Inflates the measurement window when some CSI-RS occasions collide with SMTC windows or measurement gaps.  P = 1 when there is no overlap; it rises according to the overlap formulas in §9.5.4.2 when collisions occur.

N tells us how many CSI-RS periods/SSB bursts in total the UE must span to complete its Rx-beam sweep.

- If N = 1, the UE measures what it needs in the very next CSI-RS occasion and we pay only one period of delay.

- If N = 4, it means we wait through four CSI-RS periods before every Rx-beam has been measured once.

When the CSI-RS resource-set is configured with repetition = OFF, each CSI-RS resource may carry a different Tx beam, but the UE only measures one Rx-beam per resource instance so by definition it never needs to cycle through its other Rx-beams to cover spatial directions. In that case the spatial-sweep factor N in the 38.133 delay formula collapses to 1.

When repetition = ON, we re trading off how many CSI-RS periods the UE spends sweeping its Rx-beams (that’s N) against how many CSI-RS symbols we load into each period (that’s N₍res_per_set₎).  Here are the tradeoffs involved:

Pros of a smaller N

- Lower latency: Fewer full CSI-RS periods means the first complete Rx-beam sweep, and thus the multi-beam measurement vector arrives more quickly, shaving milliseconds off the end-to-end prediction.

- Reduced UE power draw: The RF front end and digital beam-steering logic only turns on for a handful of symbols before reporting, which conserves battery.

- Faster beam-failure recovery: Should we need to fall back to measured beams, having a small-N sweep means we can detect link degradation sooner.

Cons of a smaller N

- Higher DL overhead: To hit a small N we must pack more CSI-RS symbols per period (larger N₍res_per_set₎), which could consume more time-frequency resources on PDSCH or special CSI-RS slots.

- Scheduler pressure: Denser CSI-RS placements can collide with data or other measurements, forcing tighter resource-planning and potentially more PDCCH/DCI juggling.

Pros of a larger N

- Lower CSI-RS footprint: With fewer symbols per period, the network can serve more UEs or carry more data, since CSI-RS “steals” less air-time.

- Simpler scheduling: Sparsely spaced CSI-RS patterns could ease the burden on scheduling

Cons of a larger N

- Increased measurement delay: Every extra period we wait, it multiplies the sweep delay by T_CSI-RS ot T_SBB risking stale inputs to your AI/ML model.

- Higher UE energy cost: The RF and beam-steering logic keep running across many periods, drawing more power and potentially delaying sleep in DRX.

- Slower reaction to channel changes: In fast-moving or deep-fade scenarios, a large N can let the serving beam degrade significantly before the next full measurement arrives.

During RAN4#116 the following agreement was reached:

Agreement:

observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report)

For case 1 reuse M,N,P from legacy measurement requirements

For case 2, use T*M, N P  (M,N,P same as legacy measurement requirement)

FFS on T value, T can also be 1. T can also be based on capability

This observation period is to be used for the prediction delay requirement

BM-Case 2 (temporal-domain prediction, observation window)

In the legacy system, the UE measures reference signals such as SSBs or CSI-RS directly and computes the RSRP using standard filtering and averaging methods. The reporting delay is therefore only due to measurement and basic signal processing. This delay is already well defined in the specifications through fixed parameters like Z3 and Z3’, which describe how much time must pass between receiving a trigger or reference signal and sending the report. Because the process is straightforward and deterministic, these timing values are uniform across UEs and guarantee interoperability.

RAN1 has already agreed to extend legacy timing parameters (Z3 and Z3’) by introducing d and d’ to capture AI/ML inference delay.These extensions ensure that additional processing time required for spatial and spatio-temporal beam prediction is reflected in the reporting timeline.RAN4 shall therefore align its specification of prediction delay with RAN1’s d’ to maintain consistency across standards.

In the AI/ML-based system, the UE still starts with the same reference signal measurements, but instead of reporting them directly, it must first run them through an AI/ML model that predicts the best beams or their RSRP values. This step introduces an additional inference delay, which depends on the model’s complexity and the UE’s processing capability. As a result, the reporting timeline must be extended beyond the legacy Z3 and Z3’ by a new parameter (d or d’). The main difference, therefore, is that legacy reporting is purely measurement-driven with fixed timing, while AI/ML reporting adds an inference stage, requiring extra time to ensure the prediction can be completed before the report is sent.

During RAN4#116bis the following agreements were reached :

Issue 2-1: Measurement period for inference – case 2

Agreement:

Measurement period for inference – case 2

K(number of samples)*[M]*N*P

K as defined by RAN1

RAN4 to use M=1 for BM Case 2

Issue 2-2: Reporting delay

Agreement for reporting delay:

Reuse d’ as to be agreed by RAN1

d’ to be added to the measurement period to reflect the extra time needed by the UE to perform inference

Depending on the RAN1 specifications, d’ shall not be double counted in the overall delay for periodic, aperiodic or semi-persistent reporting

Final specifications to be agreed after RAN1 includes d’ in the specs\

LCM and performance monitoring related requirements

During RAN4#116 the following agreements were discussed:

Issue 2-4: Activation delay

Agreement:

- UE has to be ready to start measurements for inference after sending RRC reconfiguration complete

- Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay

- This applies to all reporting schemes

Issue 2-2: Prediction report delay

Agreement:

  - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay

    - The reporting delay includes the inference delay

  - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

Activation delay: This is the one-time interval from when the UE sends RRC Reconfiguration Complete until it is ready to start the first measurements for inference. It is basically a readiness check and does not itself include model loading or inference execution.

Inference delay: This is the time the UE takes to run the AI/ML model and produce a prediction once measurement data are available.
Activation delay ends before inference delay begins, but any model-loading work that the UE chooses to perform at inference time rather than before activation is counted inside the inference delay. So activation delay defines the start of the measurement phase, while inference delay covers the model’s run-time processing within each prediction report.

Aperiodic Reporting Timing (RAN4#116 Agreement)
For aperiodic CSI prediction, the RAN4#116 agreement clarifies that the DCI trigger defines the measurement start but not the report slot. The DCI message sent by the gNB explicitly indicates which reference-signal resources the UE shall measure, thereby marking the start of the measurement delay and ensuring both UE and network know when and what is being measured. After those measurements, the UE runs its AI/ML inference and prepares the report. Because the DCI does not specify the exact transmission slot for the report, the UE sends its prediction as soon as measurement and inference are complete, using available uplink resources. The total prediction reporting delay therefore equals the measurement delay plus the reporting delay, with the reporting delay explicitly including the inference delay (model execution and related processing) This framework ensures that for aperiodic reporting the network knows precisely when measurement begins, while allowing flexible, readiness-based timing for the final report.

For periodic reporting, the gNB provides a fixed reporting periodicity and uplink resources directly in the RRC configuration. For semi-persistent (SP) reporting, the network provides the same information in RRC and later activates it with a MAC-CE command. In both cases, the reporting schedule is fully known to both UE and network once configuration (and MAC-CE activation, if needed) is complete.

However, when a UE finishes its measurements and AI/ML inference, the next scheduled uplink slot may not yet have started. To capture this gap, RAN4 specifies that the prediction reporting delay must include the time until the first available reporting occasion, the wait from processing completion to the earliest configured report slot.

Consequently, for SP and periodic schemes the prediction reporting delay is defined as the sum of three elements: measurement delay, inference delay, and the waiting time until the first scheduled reporting opportunity. This ensures the delay KPI reflects the true end-to-end latency from measurement start to the moment the first report is actually transmitted, even though the reporting times themselves are predetermined by RRC or MAC-CE configuration

    - Performance monitoring and LCM related requirements

The following agreement was reached in RAN4#115 meeting:

Agreement from RAN4#115: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |  |
| SP report | Not support | Support | Support |  |
| P report | Support | Support | Support |  |

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

RAN1 achieved following agreement for performance monitoring mechanism and metric:

RAN1#121 Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N)

- Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is log2(N+1)

RAN1 has introduced RS-PAI (Reference-Signal Prediction Accuracy Indicator) as a new report quantity for AI/ML beam management. RS-PAI counts the number of accurate prediction instances N_p over N valid monitoring instances and therefore requires the UE to accumulate multiple reference-signal transmissions, compute the metric, and then report it. This workflow differs from legacy beam-prediction accuracy metrics, which simply compared one measured prediction with one ground truth, and it brings new timing uncertainty: the network cannot easily know when the UE will finish collecting N samples, complete computation, and send the first RS-PAI report.

Without a standardized end-to-end delay requirement, the gNB cannot confidently schedule uplink resources, set timers for fallback, or verify conformance. Similarly, accuracy requirements must be defined so that RS-PAI truly reflects prediction quality and can be reproducibly tested, at least in static radio conditions, across different UE implementations. In short, RS-PAI introduces window accumulation + computation steps that legacy “measure once → report once” rules never covered, creating a gap in RAN4’s performance-monitoring framework.

RS-PAI depends on internal UE model behavior, but the Test Equipment (TE) cannot directly access ground truth prediction accuracy. RS-PAI (Reference Signal – Prediction Accuracy Indicator) is intended to quantify how accurately the UE’s internal AI/ML model predicts beam-related events or signal strengths. However, the test equipment (TE) does not have access to the UE’s internal prediction ground truth or the actual input-label mapping used during inference. This means that verifying the absolute correctness of the reported RS-PAI value is infeasible: the UE could theoretically report any ratio (Nₚ/N), and the TE could not confirm it against true prediction outcomes.

To make RS-PAI testable in practice, we can instead verify its relative behavior under controlled channel evolution. The idea is to test whether RS-PAI tracks the expected trend as the channel changes from highly random (unpredictable) to highly deterministic (easily predictable).

In this trend-based test, the TE gradually varies the channel conditions in a reproducible sequence.

- Phase 1 – Randomized channel: frequent beam changes, low spatial correlation → RS-PAI expected to be low.

- Phase 2 – Moderate correlation: partial stability in dominant beams → RS-PAI expected to increase.

- Phase 3 – Stable channel: slowly varying single dominant beam → RS-PAI expected to approach high ratio (near 1).

If the UE’s reported RS-PAI follows this increasing trend as the environment becomes more predictable, the TE can confirm that RS-PAI reflects real model behavior, even without knowing the internal ground truth. Conversely, if RS-PAI remains constant or fluctuates erratically despite deterministic channel conditions, the UE fails the test.

This approach allows RAN4 to define reproducible verification procedures for RS-PAI as a performance-monitoring KPI and to establish corresponding Life-Cycle Management (LCM) requirements. Once the RS-PAI reliability is verified, thresholds for model deactivation, switching, or fallback can be meaningfully tested, ensuring that RS-PAI-based triggers produce consistent LCM actions under both BM-Case 1 (static) and BM-Case 2 (temporal) conditions.

Because current 5G test equipment does not support controlled, time-varying channel predictability, a trend-based RS-PAI validation using dynamic fading conditions is not feasible without introducing new TE functionality. To stay consistent with existing conducted test methods, RS-PAI verification should instead rely on a static or quasi-static setup, where the stability of the reported metric is evaluated rather than its evolution over changing channels.

Static Consistency Validation for RS-PAI: RAN4 is invited to define a conformance test for RS-PAI based on repeated reporting under static radio conditions. The UE shall report RS-PAI across multiple iterations in an unchanged environment, and the TE shall verify that at least 90% of the reported values remain within a defined tolerance band. This ensures that RS-PAI is stable, repeatable, and suitable for use in LCM decisions, without requiring dynamic channel manipulation or new TE capabilities.

RAN4 Requirement on RS-PAI Thresholds for LCM Triggering: RAN4 shall define explicit RS-PAI-based threshold requirements to govern LCM behavior for AI/ML beam-management models. RS-PAI (Nₚ / N) shall be used as the primary KPI for monitoring live prediction performance, and minimum acceptable RS-PAI levels, together with required persistence windows, shall be standardized. LCM actions (e.g., model switch or deactivation with fallback) shall be triggered when the RS-PAI metric remains below the specified threshold for a defined number of consecutive reports. This ensures that the UE-reported RS-PAI is not only measured but mapped to standardized trigger criteria, allowing consistent, testable, and interoperable LCM behavior across vendors and deployments.

During RAN4#116bis the following agreement was reached:

Issue 2-3: Reporting delay requirements for monitoring

Agreement:

Do not define any explicit requirements for reporting delay of RS-PAI performance monitoring metric.

Testing framework for AI/ML based Beam Management

The testability issues for AI/ML based BM use case were highlighted by multiple companies during previous few RAN4 meetings but the discussions continued without any concrete conclusion on the topic. Below are the agreements/summary of the progress on this topic during the past RAN4 meetings which are documented in the corresponding WFs.

Agreements from RAN4#110 meeting Issue 2-3: Test setup feasibility for FR2 Companies are invited to provide further analysis on what the test setup should enable in terms of test environment Agreements from RAN4#113 meeting 2.3.2 Agreements in main session: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Agreements from RAN4#114 meeting 2.4 Testing for one sided models Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Agreements from RAN4#114-bis meeting Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

The following agreements were reached in RAN4#116:

Issue 2-6: Test system setup

Agreement:

Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing)

Single AoA tests can also be performed in the multi AoA based RRM test setup

Further analyze what spatial channel models can be emulated with this test setup

  - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup

Single AoA tests are not precluded

Issue 2-7: Test system channel model

Agreement:

Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber.

Take CDL-C Umi as one example

Other channel models can be further discussed

The Rohde & Schwarz paper  proposes a multi-AoA over-the-air chamber (feIFF) with four fixed probes arranged around the DUT. Each probe is independently driven in amplitude and phase so that their combined far-field matches the 3GPP CDL-C channel for any selected gNB Tx beam in Set B. For every Tx beam, the controller computes probe weights so that the superposed field at the UE reproduces the cluster delays, powers and AoAs defined by the CDL model. The received RSRP at the UE is therefore a joint outcome of (i) the chosen Tx beam’s intended AoD, (ii) the CDL cluster powers and phases, and (iii) the probe geometry and weights that synthesize those arrivals.

In a multi-AoA OTA test chamber such as the feIFF system described by Rohde & Schwarz, the AoA (Angle of Arrival) values that matter are those specified by the CDL channel model, and the probes are the hardware used to reproduce those AoAs. Here is how they relate and what is actually emulated:

- CDL channel AoA (from 3GPP TR 38.901/38.827)
Each cluster in a CDL channel is defined by its mean AoA (and spread). These are the target arrival directions that a real UE would experience in the field for a given gNB Tx beam.

- Probe AoA (physical chamber geometry)
The chamber contains a finite number of probes (four in feIFF) placed at fixed azimuth angles around the UE. Each probe has a known physical direction, its own “probe AoA.”

- Relationship and emulation
To make the UE “see” the CDL-specified AoAs, the test controller computes complex weights (amplitude and phase) for each probe so that the superposition of their fields creates an effective wavefront arriving from the desired CDL AoA.

  - If a CDL AoA coincides with a probe’s physical direction, most power is sent through that probe.

  - If a CDL AoA lies between probes, several probes transmit simultaneously with calibrated weights so that their combined field angle equals the target AoA.

- Which angles are truly emulated
The CDL AoAs are the angles being emulated; the probes are simply the “actuators” used to synthesize those angles in space. The AoD (Angle of Departure) of the Tx beam is part of the logical CDL model and influences how cluster powers are assigned, but physically it is not emulated as a separate outgoing direction, only the arrival geometry at the UE is reproduced

The channel model governs how much of each Tx beam’s energy reaches the UE from each direction. Clusters with higher power or favorable angles contribute more strongly to the synthesized field. As the chamber steps through Tx beams in Set B, the UE measures RSRP per beam; differences in those readings directly reflect how the CDL-defined angular spreads and fades couple with each specific Tx beam pattern.

Because several probes radiate simultaneously and can represent multiple arrival directions at once, the UE is exposed to a spatially rich scene and can freely sweep or select its own Rx beams exactly as it would in the field. This dynamic is not possible in single-AoA setups, where the UE effectively “sees” only one direction at a time.

The paper stresses that the Rx-beam index used for each Tx-beam measurement should be provided as input to the AI/ML model. Even if the UE sweeps all Rx beams and reports only the best RSRP per Tx beam, the corresponding Rx-beam ID preserves the spatial context of each measurement. Including these indices allows the AI/ML model to learn the true Tx/Rx geometry and improves prediction of future Tx beams under mobility.

In R4-2511597, Rohde & Schwarz demonstrate that a model trained on the full 24-cluster CDL-C channel can be reliably evaluated on a 6-cluster simplified channel suited for the feIFF chamber: in their preliminary system-level simulation the ground truth and training used the full CDL model, while prediction/testing ran on the simplified model, yielding ≤ 1 % loss in top-1/top-3 beam prediction accuracy, which shows that training on the full channel generalizes well to simplified test conditions.

From Anritsu’s paper: the Anritsu paper stresses that a conformance test setup must cover both AI/ML beam-management use cases—BM-Case 1, which predicts the best Set A downlink beams from current Set B spatial measurements, and BM-Case 2, which predicts future Set A beams from past Set B measurements, and also allow mixed sequences of these two modes, since in real deployments a UE may dynamically shift between spatial and temporal prediction, or chain them in series, to maintain robust beam tracking and mobility performance

Anritsu emphasizes channel-model realism, noting that field performance of AI/ML beam management depends heavily on the propagation model. They caution that it is not yet proven whether a single simplified CDL channel can capture real-world effects and argue that multiple CDL-type channel models (e.g., CDL-A/B/C) may be required to represent the full range of angular spreads, cluster dynamics, and mobility variations encountered in practice.

Anritsu underscores the need for flexibility in test hardware, comparing single-Angle-of-Arrival (1AoA) setups with multi-AoA chambers. A 1AoA system places the UE in front of a single CATR or “wireless-cable” feed and drives it with a fading simulator capable of synthesizing numerous virtual clusters, Doppler shifts, and delay taps. This makes it possible to reproduce very detailed CDL or other channel models, including complex spatially consistent mobility, without being constrained by the number or positioning of physical probes. In short, 1AoA provides exceptionally rich channel modelling and is well suited for evaluating propagation effects and mobility patterns defined in standards.

However, 1AoA lacks realistic Rx-beam sweep behaviour. The signal effectively arrives from only one physical direction at a time, meaning every receive beam is illuminated almost identically. As a result, a UE performing beam sweeping sees no genuine spatial diversity, and its beam-selection timing and decision processes cannot be evaluated accurately. In contrast, a multi-AoA chamber uses several fixed probes transmitting simultaneously at different azimuths, so the UE experiences true multi-directional arrivals and can sweep or select Rx beams naturally, but such setups cannot perfectly emulate spatially consistent mobility and are harder to adapt when multiple CDL channel models must be applied

Instead they argue for keeping both 1AoA and multi-AoA options open, because each has complementary strengths and limitations:

- 1AoA – flexible for many channel models and spatially consistent mobility but limited for UE Rx-beam sweeping.

- Multi-AoA – good for Rx-beam sweeping but limited for spatially consistent mobility and for supporting many different channel models.

Multi-AoA chambers such as eIFF or feIFF can reproduce rich instantaneous spatial conditions for Rx-beam sweeping, but they are not well suited for spatially consistent mobility as defined in 3GPP TR 38.901 §7.6.3.2. Their probes are fixed in position, so while signal amplitude and phase can be varied to mimic fading, the chamber cannot continuously shift cluster angles and delays as a UE moves. This means the smooth, geometry-driven evolution of multipath components that real mobility requires can only be approximated by power or phase changes, falling short of true spatial consistency and limiting accurate testing of temporal beam-prediction scenarios such as BM-Case 2.

Anritsu R4-2509134 highlights that how the UE’s receive-beam (Rx-beam) information is provided to the AI/ML model is still not harmonized, and that this choice directly shapes the required test setup. RAN1’s earlier agreements (referenced in the paper) list several options: (1) the UE sweeps all Rx beams for every measurement sample and reports only the best Rx beam as input to the AI model; (2) it provides measurements of specific Rx beams—either per sample (2a) or for the entire measurement set (2b); or (3) it uses random Rx beams per sample. Each option results in a different AI input structure, from a single best-beam RSRP to a richer set of {Tx beam, multiple Rx beams, RSRP} tuples, and therefore dictates how many measurements, probes, and reporting fields must be emulated in the test chamber.

Because the conformance procedure must reflect the actual inference path of the AI/ML model, RAN4 needs to reconfirm which option (or combination) is most representative of real UE behavior. For example, if the UE always provides only the best Rx beam, the test setup can focus on accurately emulating peak RSRP values; but if multiple or random Rx beams are included, the test must present genuine multi-directional arrivals and capture Rx-beam indices for each Tx-beam measurement. Anritsu therefore proposes that companies clarify and align the latest assumptions before finalizing the test environment, since the selected approach will drive measurement complexity, signaling requirements, and ultimately the validity of AI-beam prediction performance results

Anritsu proposes a two-step fallback method: If no single chamber setup can handle all requirements, allow a two-phase evaluation, first the AI model’s prediction accuracy without Rx-beam sweeping, then a separate assessment of UE Rx-beam sweeping behaviour.

However, from a system realism and performance-validation perspective, this approach raises significant concerns, especially for BM-Case 1, where the AI/ML model must learn and generalize spatial correlations between SSB beams, neighbor cells, and frequency layers under stochastic propagation.

BM-Case 1 represents spatial beam management  learning relationships between observed SSB measurements (Set B) and the best candidate beams or cells (Set A).The reliability of any AI/ML model in this case critically depends on:

- Angular diversity: distribution of cluster mean AoAs/AoDs across many UEs and environments.

- Statistical randomness: variations in cluster powers, delays, and angular spreads.

- Environment evolution: transitions between LOS/NLOS, partial blocking, and multipath re-composition.

The simplified 6-cluster CDL used in current simulations does not reproduce any of these dynamics.
All clusters have fixed AoAs/AoDs, fixed relative powers, and identical geometry across realizations.
As a result, every sample is essentially a re-randomization of per-ray phase and amplitude around a static spatial skeleton.

Under such a static channel:

- The AI/ML model can easily memorize the deterministic mapping between Set B and Set A beams instead of learning general propagation relationships.

- The reported high Top-1/Top-3 accuracies (~90 %) only confirm the model’s internal consistency within this fixed geometry, not its robustness.

- When the UE faces field environments with different cluster directions, power ratios, the trained model’s inference may deviate sharply from expected performance.

Therefore, passing the simplified CDL test cannot be interpreted as field readiness; it merely verifies the UE’s ability to run the AI/ML pipeline within the chamber.

In a real SLS or field deployment, cluster mean AoAs vary randomly, power imbalances fluctuate, a model that performs well on the deterministic feIFF dataset might fail under those conditions. Essentially, passing this test only shows the model is numerically compatible with the simplified channel, not that it will perform robustly in the field.

(BM-Case1 Channel Evaluation Scope): RAN4 to distinguish between testability validation and performance validation when evaluating AI/ML BM models.

- Use the simplified CDL (6 clusters, fixed AoAs) only for chamber feasibility testing and OTA controllability verification

- Introduce field-representative CDL variants with randomized cluster AoAs/AoDs and power distributions to ensure performance validation.

- Establish clear criteria and accuracy margins separately for each validation category.

In the feIFF chamber, the physical geometry of the probes, their azimuth angles and positions is fixed. Each probe illuminates the DUT from a specific direction (for example, −75°, −15°, +45°, +105°). However, what the UE actually perceives is not each probe individually but the combined electromagnetic field created by all probes transmitting simultaneously with certain relative powers and phases.

Each probe, at its own fixed angle, samples a particular point on the Tx beam radiation pattern. The gain of a specific Tx beam toward that probe’s direction corresponds to how much energy that beam would radiate toward that spatial sector in the field. When all probes transmit together, their individual contributions add up at the UE to form a composite angular illumination pattern a synthetic representation of how the Tx beam interacts with multiple scatterers originating and arriving from different angles.

Now, if we change only the relative power levels (and possibly the phases) of these probes, we modify how much each angular component contributes to the total received field. Increasing power on one probe while reducing it on another shifts the angular centroid of the illumination, essentially rotating the effective AoD of the signal without physically moving any hardware. The field arriving at the UE now emphasizes a different region of the Tx beam pattern, just as if the multipath geometry in the real world had rotated or new scatterers had appeared.

By repeating this process, using several different power distributions across the same four probes we can create multiple channel realizations, each corresponding to a different effective AoD configuration. For each realization, we transmit one Tx beam at a time  and measure the received power across all UE Rx beams. This gives us a complete set of RSRP values that represent how that beam’s pattern interacts with the rotated angular illumination.

Through this mechanism, the test setup can sweep through the Tx beam pattern of every beam and capture its spatial gain characteristics under many synthetic angular conditions. Across these realizations, the UE experiences a variety of power distributions, resembling the random AoD variability that would naturally occur in a field environment as the UE moves, the channel scatters change, or nearby reflectors shift.

Thus, by re-weighting the powers of fixed probes, we can emulate statistical angular diversity, effectively “rotating” the AoD distribution seen by the UE, without physically rotating the probes or the DUT. This preserves test repeatability and chamber calibration while introducing the spatial randomness necessary to make the measurements more representative of real-world beam management conditions.

(AoD Randomization for Performance Validation): RAN4 to introduce AoD randomization within the feIFF test setup by varying only the probe power distributions to emulate rotated composite illumination angles (AoD) for each realization.

- Each realization applies a new probe-power weighting set to “rotate” the effective AoD seen by the UE, creating field-like angular diversity.

- Sequentially transmit all Tx beams under each configuration to generate multiple RSRP datasets.

- Use these randomized AoD realizations specifically for AI/ML performance validation, while retaining the fixed 6-cluster CDL geometry for testability validation.

In the feIFF chamber, the AoA (Angle of Arrival) at the UE corresponds to the directions from which the probe signals arrive relative to the UE’s antenna array. When the UE (or DUT) remains fixed, each probe’s AoA with respect to the UE array is constant, meaning the Rx beams “see” the incoming waves always from the same directions. Consequently, the Rx beam patterns are sampled at the same angular points across all tests, which makes the dataset too static.

To emulate field-like conditions, we can introduce AoA variability by rotating the UE.This changes how each probe’s wavefront aligns with the UE’s antenna array and thus modifies the effective coupling between each incoming plane wave and the Rx beam patterns.

(AoD Randomization for AoA Randomization for Performance Validation): Introduce AoA randomization through controlled UE azimuthal rotation within the feIFF setup to emulate field-representative spatial perception for BM-Case 1.

- Conduct measurements over multiple DUT orientations (e.g., 6–12 azimuth angles).

- Each orientation alters the probe AoAs relative to the UE Rx beam patterns, producing unique Rx-beam gain distributions.

- Combine AoA rotation with AoD probe-power randomization to form a joint angular-diversity dataset for AI/ML performance validation, while maintaining fixed geometry cases for testability validation.

BM-Case 2 focuses on time-varying beam behavior, where the optimal Tx/Rx beam pair evolves as the radio channel changes. In a real environment, each multipath cluster  representing a dominant scatterer drifts in angle, fluctuates in power. Its parameters, such as AoD(t), AoA(t), and P(t), vary smoothly over time, creating the temporal dynamics that an AI/ML beam-tracking algorithm must learn to predict.

In the feIFF setup, this evolution can be emulated by treating each probe as a radiator representing one or more clusters. The probes are fixed in position, but their power weights (and phases) are continuously adjusted to match the time-varying angular power distribution of the virtual clusters. As clusters drift in azimuth or change strength, energy is gradually re-distributed among the probes. At each time step, the chamber applies the updated probe-power vector, transmits one Tx beam at a time, and measures the RSRP across all Rx beams. The sequence of measurements over time reproduces the temporal evolution of beam quality that defines BM-Case 2, allowing evaluation of how accurately the UE predicts upcoming beam changes or maintains stable tracking.

(Cluster-Driven Temporal Evolution for BM-Case 2): RAN4 to emulate BM-Case 2 temporal evolution by driving the feIFF probe powers according to time-varying virtual cluster parameters

- Define a baseline set of clusters (AoD/AoA/power), continuously evolve these cluster parameters (drift, shadowing, blockage).

- At each time step, update the probe-power vector to produce time-correlated RSRP sequences for evaluating AI/ML beam-tracking performance.

Conclusion

In the paper, the following Observations and Proposals were made:

- RAN4 to use M=1 for BM Case 2

- Static Consistency Validation for RS-PAI: RAN4 is invited to define a conformance test for RS-PAI based on repeated reporting under static radio conditions. The UE shall report RS-PAI across multiple iterations in an unchanged environment, and the TE shall verify that at least 90% of the reported values remain within a defined tolerance band. This ensures that RS-PAI is stable, repeatable, and suitable for use in LCM decisions, without requiring dynamic channel manipulation or new TE capabilities.

RAN4 Requirement on RS-PAI Thresholds for LCM Triggering: RAN4 shall define explicit RS-PAI-based threshold requirements to govern LCM behavior for AI/ML beam-management models. RS-PAI (Nₚ / N) shall be used as the primary KPI for monitoring live prediction performance, and minimum acceptable RS-PAI levels, together with required persistence windows, shall be standardized. LCM actions (e.g., model switch or deactivation with fallback) shall be triggered when the RS-PAI metric remains below the specified threshold for a defined number of consecutive reports. This ensures that the UE-reported RS-PAI is not only measured but mapped to standardized trigger criteria, allowing consistent, testable, and interoperable LCM behavior across vendors and deployments.

(BM-Case1 Channel Evaluation Scope): RAN4 to distinguish between testability validation and performance validation when evaluating AI/ML BM models.

- Use the simplified CDL (6 clusters, fixed AoAs) only for chamber feasibility testing

- Introduce field-representative CDL variants with randomized cluster AoAs/AoDs and power distributions to ensure performance validation.

- Establish clear criteria and accuracy margins separately for each validation category.

(AoD Randomization for Performance Validation): RAN4 to introduce AoD randomization within the feIFF test setup by varying only the probe power distributions to emulate rotated composite illumination angles (AoD) for each realization.

- Each realization applies a new probe-power weighting set to “rotate” the effective AoD seen by the UE, creating field-like angular diversity.

- Sequentially transmit all Tx beams under each configuration to generate multiple RSRP datasets.

- vUse these randomized AoD realizations specifically for AI/ML performance validation, while retaining the fixed 6-cluster CDL geometry for testability validation.

(AoD Randomization for AoA Randomization for Performance Validation): Introduce AoA randomization through controlled UE azimuthal rotation within the feIFF setup to emulate field-representative spatial perception for BM-Case 1.

- Conduct measurements over multiple DUT orientations (e.g., 6–12 azimuth angles).

- Each orientation alters the probe AoAs relative to the UE Rx beam patterns, producing unique Rx-beam gain distributions.

- Combine AoA rotation with AoD probe-power randomization to form a joint angular-diversity dataset for AI/ML performance validation, while maintaining fixed geometry cases for testability validation.

(Cluster-Driven Temporal Evolution for BM-Case 2): RAN4 to emulate BM-Case 2 temporal evolution by driving the feIFF probe powers according to time-varying virtual cluster parameters

- Define a baseline set of clusters (AoD/AoA/power), continuously evolve these cluster parameters (drift, shadowing, blockage).

- At each time step, update the probe-power vector to produce time-correlated RSRP sequences for evaluating AI/ML beam-tracking performance.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.

- R4-2511597, “Discussion on test setup for AI/ML-based beam management”, Rohde & Schwarz, RAN4 #116 meeting, 25th – 29th August 2025, Bengaluru, India.

- R4-2509134, “FR2 test setup for AI/ML beam management evaluation”, Anritsu Corporation, RAN4 #116 meeting, 25th – 29th August 2025, Bengaluru, India.