# 3GPP TSG-RAN WG4 Meeting #117	                          R4-2520620                                                     Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.4

Source:	Apple

Title:	Discussion on RRM Core requirement for 2-sided models

Document for:	Discussion

Introduction

RAN#108 has approved a new WI on AI/ML for NR air interface enhancements in Rel-20 [1], where the objective includes CSI feedback enhancement with two-sided model and inter-vendor training collaboration for two-sided model.

Performance Monitoring and LCM

The following objectives were captured in WID RP-252445:

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Data collection [RAN2]:

- Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

- NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In Rel-19 RAN1 discussions, the following agreements were made related performance monitoring and error cause identification for CSI compression use case:

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches. Agreement For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field. Conclusion For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following • Data distribution mismatch that is identified by either NW side or UE side • Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part. • Note that this conclusion does not preclude other methods. • Note that triggering of root cause detection may be after detecting performance degradation

NW side monitoring for 2-sided models

Below we discuss NW side monitoring and LCM related procedures based on RAN1 discussions:

Network-side (NW-side) performance monitoring is essential for AI/ML-based CSI compression, because only the gNB can consistently observe end-to-end system behaviour. The core problem is to reliably detect when an on-air AI/ML encoder–decoder pair starts to underperform or drift, so that the network can trigger lifecycle-management actions (model fallback, re-training, or deactivation) and diagnose the cause. The simplest and lowest-overhead first line of defense is system/link or eventual KPI monitoring: throughput, SNR, ACK/NACK ratio, or BLER. These metrics require no extra signaling and can run continuously, even capturing MU-MIMO effects. However, they only indicate that something is wrong, not whether the AI/ML CSI compression itself is to blame.

To pinpoint root causes, a two-step NW-side process is proposed. After an eventual-KPI trigger, the gNB configures intermediate KPI monitoring that compares the UE’s reported ground-truth CSI with the reconstructed CSI. This requires UE reports of high-resolution target CSI, typically based on Rel-16 eType-II or enhanced eType-II-like codebooks, so that metrics such as averaged SGCS or NMSE can be computed accurately. Some proposals suggest infrequent or on-demand reporting to limit uplink overhead, while others explore lower-overhead mechanisms such as predefined vector sets or small redundancy elements to let the network estimate reconstruction quality without full high-resolution payloads.

The most widely supported framework is: (1) frequent, zero-overhead NW-side monitoring of eventual KPIs to flag potential failures; (2) targeted, low-rate intermediate-KPI checks using high-resolution target CSI to verify whether the AI/ML CSI compression model is the cause and to guide model lifecycle actions.  (LCM) Proposals differ mainly in how to reduce UE complexity and uplink cost: e.g., subband-specific basis selection to simplify SVD/EVD, long periodicities, or hybrid schemes where UE-side detection triggers NW-side checks, but they converge on keeping NW-side monitoring as the baseline for reliable, testable supervision of AI/ML CSI-compression performance.

Monitoring metrics and their role
For network-side (NW-side) performance monitoring of AI/ML CSI-compression, two layers of metrics are used in sequence.

System/link or eventual KPIs: such as throughput, SNR, ACK/NACK ratio, and BLER, are derived from normal traffic or measurement reports and incur essentially no extra signaling. They give continuous, low-complexity visibility of link health, including MU-MIMO interactions. When these KPIs show sustained degradation, the gNB activates:

Intermediate KPI monitoring, typically based on SGCS or NMSE, by requesting high-resolution target CSI (enhanced Rel-16 eType-II codebook) together with the UE’s AI-CSI report. The gNB reconstructs and compares CSI to compute averaged SGCS/NMSE over multiple samples, filtering out fast fading and isolating true model errors.

Criteria for switching, activation, and deactivation
Model lifecycle decisions: switch, activation, deactivation are directly tied to these metrics. A new model is activated only if its intermediate KPI meets or exceeds a configured minimum over a defined observation window (e.g., average SGCS ≥ P_min). Persistent KPI degradation below a lower bound, or significant throughput/BLER loss, triggers model fallback or deactivation. To prevent oscillations, companies propose two-level thresholds: an early-warning band that triggers closer monitoring and a hard-failure threshold that forces fallback. Both per-sample and window-averaged statistics are studied, but only averaged values are used for binding decisions.

Delay, reaction time, and monitoring frequency
Because model changes affect RRC signaling and MU-MIMO scheduling, NW-side monitoring incorporates explicit time buffers. Lead-time windows and counter-based smoothing ensure that switch or deactivation occurs only after sustained KPI deviation, typically over tens of CSI reporting periods (hundreds of milliseconds), rather than instantly. In steady operation, eventual KPI checks run continuously at no cost, while intermediate KPI checks are scheduled on demand or with long periodicities, when the network wants to check model health. This staggered, reference-documented approach balances early detection with low overhead and smooth, standards-compliant model lifecycle management.

UE side monitoring for 2-sided models

UE-side performance monitoring is meant to give the terminal itself the ability to sense when its AI/ML CSI-compression encoder may be drifting or malfunctioning, without waiting for the network to detect a problem. The challenge is to design a method that (i) reflects true model health, (ii) does not incur heavy power or signaling cost, and (iii) avoids false triggers that would lead to unnecessary model switching or fallback.

The Release-19 studies divide solutions into two broad categories. Category 1 uses UE-side intermediate KPI estimation: either by running a small proxy decoder (Case 2-1), directly estimating an intermediate KPI such as SGCS or NMSE (Case 2-2), or directly outputting a range indicator of that KPI. Category 2 relies on precoded reference signals (CSI-RS or DMRS) sent from the gNB based on its reconstructed CSI so the UE can measure link KPIs like CQI or BLER. Key requirements are to define suitable performance metrics, reporting formats, and triggers (time- or event-based) that distinguish genuine AI/ML model issues from transient radio fluctuations.

Monitoring metrics and their role
For UE-side performance monitoring of AI/ML CSI-compression, the most common metrics are intermediate KPIs, for example SGCS or NMSE, either as precise values or as value-range indicator. These indicators measure how closely the UE’s locally reconstructed or estimated CSI matches the true channel, giving a model-specific view that is more direct than system KPIs like throughput or BLER. Some proposals also explore out-of-distribution statistics (e.g., energy score or Wasserstein distance) and precoding-gain comparisons between AI/ML and legacy codebooks when precoded reference signals are used. By running these checks internally and, when needed, reporting SGCS or NMSE together with target CSI and the AI-CSI report, the UE can help the network verify or complement NW-side monitoring.

Criteria for switching, activation, and deactivation
Decisions to switch, activate, or deactivate a model at the UE are driven by sustained degradation of these metrics. The common proposal is to use averaging windows or counter-based triggers, similar to beam-failure detection, so that short-term fading does not cause false alarms. A model switch or fallback is initiated only when the averaged SGCS or NMSE falls below a configured minimum, or when repeated comparisons show AI-based precoding gain consistently worse than the legacy eType-II reference. Activation of a new or updated model requires the reverse, meeting or exceeding the minimum KPI threshold across the configured window. These criteria can be combined with event-based triggers such as large Doppler change, beam/sector change, or PDSCH ACK/NACK degradation.

Delay, reaction time, and monitoring frequency
Because unnecessary toggling could waste power or destabilize scheduling, UE-side proposals build in delay and hysteresis. Reports are typically event-driven or use long periodicities (e.g., hundreds of milliseconds to minutes) so that model changes only occur after sustained evidence of under-performance. Precoded-RS-based methods are especially sensitive to channel aging and therefore require careful timing to avoid mis-detection. In practice, most companies see UE-side monitoring as a complement or trigger for network checks: light, capability-dependent, and less frequent than continuous NW-side eventual KPI monitoring, with well-defined thresholds and reaction windows to ensure that switch, activation, or deactivation is both accurate and stable.

Discussion on RAN4 Requirements

In this section we discuss how RAN4 should define requirements from LCM point of view:

To verify lifecycle management (LCM) actions such as activation, switching, or fallback, the same monitoring process used in the field must be reproducible in a controlled test environment. RAN4 should therefore define a clear, testable set of normative KPIs and measurement methods that can serve as the formal trigger signals for LCM in both deployment and conformance testing.

- NW-side: Specify how eventual KPIs (throughput, SNR/CQI, ACK/NACK, BLER) are continuously collected as a low-overhead first check, and how intermediate KPIs (e.g., SGCS, NMSE) are computed by comparing the UE’s compressed CSI with high-resolution target CSI (legacy or enhanced eType-II). The standard should include averaging windows (e.g., mean over N≥5 CSI occasions) and their mapping to reference channel models and Doppler profiles so that test equipment can replicate field monitoring exactly.

- UE-side: Define how capable UEs report KPIs that can trigger or support LCM decisions, such as SGCS or NMSE range indicators, precoding-gain KPIs (difference between AI/ML-based and legacy precoder gain measured via precoded CSI-RS/DMRS), and out-of-distribution metrics (e.g., energy score, Wasserstein distance) to capture channel distribution drift.

By formally defining these KPIs, their averaging methods, and verification procedures, RAN4 ensures that LCM triggering logic can be faithfully exercised and validated in the lab, giving operators confidence that monitoring and model lifecycle actions will behave consistently in live networks.

(2) Criteria for switching / activate / deactivate (UE-side & NW-side)
RAN4’s main role is to translate high-level lifecycle logic into concrete, testable requirements so that vendors can prove in conformance testing when and how an AI/ML CSI-compression model must switch, activate, or fall back. This means specifying what KPIs drive the decision, the thresholds, and the observation windows, and ensuring that these can be reproduced with controlled channel models and fading profiles in a RAN4 test environment.

- NW-side: RAN4 should define how the gNB decides to activate or deactivate a model based on window-averaged KPI values. Typical rules are:
– Activation when averaged intermediate KPI (e.g., SGCS or NMSE) stays above a configured minimum P_min over at least N CSI occasions.
– Deactivation/Fallback when averaged KPI falls below a lower limit P_fail for N occasions, or when eventual KPIs (throughput, BLER) show sustained degradation beyond an operator-set percentage.
RAN4 needs to specify the KPI calculation method, the minimum number of occasions N, and the test sequences that verify both activation and fallback thresholds under repeatable fading conditions.

- UE-side: If the UE itself can trigger a model switch or fallback, it should base that on the same class of averaged KPIs, and may include event-based triggers such as rapid Doppler increase, or persistent ACK/NACK problems. RAN4 should discuss how these triggers are signaled to the network, how the KPIs are reported (periodic or on-demand), and how the correctness of UE-initiated decisions is checked in lab testing.

- Potential requirement: Model activation or deactivation shall be based on window-averaged KPI statistics (e.g., SGCS, NMSE, optional precoding-gain KPI) and a two-threshold scheme (warning and failure). The procedure: including averaging window size, trigger conditions, and signalling shall be fully specified so that conformance test equipment can reproduce the required activation and fallback behaviour under standardized fading profiles.

By making these decision criteria explicit and testable, RAN4 ensures interoperability and predictable model lifecycle management across vendors, and provides operators with a consistent safety net when AI/ML CSI-compression performance drifts in the field.

(3) Delay / reaction-time requirements (UE-side & NW-side)
RAN4 should convert delay and reaction-time concepts into measurable, testable requirements so that model activation, switching, or fallback does not destabilize live networks. This means defining timers, KPI averaging windows, and counters that vendors must implement and that can be reproduced in RAN4 conformance tests.

- Minimum observation and hold times: Any model switch, activation, or deactivation, whether network- or UE-triggered, shall occur only after the relevant KPI threshold is continuously violated for at least T_decide and shall remain in the new state for at least T_hold. T_decide confirms a persistent issue; T_hold prevents rapid toggling (flapping).

- NW-side: gNB decisions must meet these T_decide and T_hold intervals, verifiable with RAN4 channel emulators

- UE-side: UEs capable of triggering fallback or switch must apply the same observation and hold times, signaling these timers in capability information for lab validation.

- Test reproducibility: RAN4 will specify reference channel models, Doppler profiles, and KPI sampling intervals so test equipment can reproduce real fading and mobility and verify compliance.

These requirements ensure that no model change is instantaneous and that every lifecycle action is deliberate, stable, and interoperable across vendors.

(4) Frequency of monitoring (UE-side & NW-side)
RAN4 should turn monitoring frequency into clear, testable requirements so AI/ML CSI-compression models are checked often enough to detect drift without adding unnecessary signaling. The standard should set default periodicities, allowed on-demand triggers, and verification methods so labs can reproduce field behaviour. For the network side, eventual KPI monitoring (throughput, SNR/CQI, ACK/NACK, BLER) runs continuously, while intermediate KPI checks (e.g., SGCS, NMSE with high-resolution target CSI) are configurable for long periodic monitoring or triggered when warning thresholds are crossed. For the UE side, any supported time- or event-based reporting (e.g., every few seconds or on rapid Doppler/KPI changes) must be signaled in UE capability information and validated in test cases. By formalizing these periodicities, triggers, and verification methods, RAN4 ensures predictable, low-overhead monitoring across vendors and reliable detection of model drift.

- Define LCM Threshold Requirements for AI/ML-Based CSI Compression: RAN4 should specify the KPI thresholds and persistence rules that determine when LCM actions (model switch, activation, fallback) shall be triggered, Thresholds must be based on window-averaged KPI statistics with required observation and hold times to avoid oscillation and ensure stable behavior across vendors.

- This allows RAN4 to standardize when a model is considered compliant or degraded, while leaving the execution of LCM actions to implementation, ensuring predictable and interoperable lifecycle control without mandating LCM test procedures.

# Conclusion

In the previous sections we made the following proposal:

- Define LCM Threshold Requirements for AI/ML-Based CSI Compression: RAN4 should specify the KPI thresholds and persistence rules that determine when LCM actions (model switch, activation, fallback) shall be triggered, Thresholds must be based on window-averaged KPI statistics with required observation and hold times to avoid oscillation and ensure stable behavior across vendors.

- This allows RAN4 to standardize when a model is considered compliant or degraded, while leaving the execution of LCM actions to implementation, ensuring predictable and interoperable lifecycle control without mandating LCM test procedures.

# References

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm.

- RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2