# 3GPP TSG-RAN WG4 Meeting #117                                 R4-2520643

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	6.16.3

Source:	Apple

Title:	On Demodulation Performance Requirements for LP-WUS

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

# Background

RAN4 RRM has concluded the Core Part of the analysis and has moved into the Performance Part. RAN4 RF session continued working on their technical report. On RAN1 side, signal design has been completed on UE operation on both Idle/Inactive and Connected Mode.

For the LP-WUR, there are two types of receivers considered: an OOK receiver and a OFDM receiver. From LP-WUS design point of view, RAN1 has specified OOK (both OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK on-duration symbols, and with the possibility of this OFDM sequence to carry information the information as well.

The OOK receiver type performs envelop detection to decode the informaton carried over the OOK ON and OFF symbols. This receiver has lower power consumption but also lower link-level performance. It’s not sensitive to frequency error and it’s moderately sensitive to timing errors. See Figure 1.

Figure 1: OOK Receiver - homodyne/zero-IF architecture with baseband envelope-detection based LP-WUR

The OFDM receiver has higher complexity since it may perform time-domain or frequency-domain correlation with known sequences, decoding the information carried in the overlay sequences. This receiver has higher power consumption but also a better link-level performance. It can be sensitive to both time and frequency errors, but may reuse PSS/SSS for synchronization. See Figure 2.

Figure 2: OFDM Receiver - IQ based LP-WUR

In terms of the OOK waveform, RAN1 has designed based on OOK with Manchester Encoding as the baseline waveform for LP-WUS and LP-SS. The OOK-based receiver can be used to detect the OOK symbols, while the OFDM receiver can detect also the OFDM overlaid sequence. DFT-s-OFDM is used to generate OOK waveform to allow the reuse of the gNB hardware for generating this waveform while multiplexing this newly generated signal with legacy NR signals.

In terms of modulation, the parameter M can be 1, 2, or 4, representing the number of OOK symbols contained within a single OFDM symbol. Regarding the information payload, an LP-WUS can carry up to 5 information bits. These bits are mapped to one of 32 possible codepoints, each corresponding to a group of UEs. These codepoints indicate which UEs should begin PDCCH or Paging monitoring at a specific Paging Occasion (PO). This codepoint-based wake-up signaling mechanism is used for LP-WUS in both idle/inactive and connected modes.

In terms of coding, for # of information bits >=3 bits, Reed-Muller (RM) code is used with rate matching to match the number of OFDM symbols available. For more details, see TS 38.212 Section 5.3.3 where RM code is specified for Uplink Control Information (UCI). For 1 and 2 information bits, the procedure is given in the same Section 5.3.3.

In terms of the so-called Overlaid OFDM Sequences, these are specified to be on top of the OOK ON symbols, i.e. the ON duration of the OOK pattern transmitted for the LP-WUS. These overlaid sequences may or may not carry actual information. The receiver can perform sequence correlation in either time or frequency domain, and it is expected that an OFDM receiver can detect early the LP-WUS compared to an OOK receiver, depending on the number of candidate overlay sequences for each OOK ON symbol.

When using overlaid OFDM sequences, several details are important. First, the length of the overlaid OFDM sequences is given by 12 * X / M for all M values, where X = 11 is the number of PRBs for the LP-WUS, meaning the sequence length is 132, 66 and 33 for M = 1, 2 and 4, respectively. In addition, overlaid OFDM sequences use Zaddoff-Chu (ZC) sequences with cyclic extension. BZC is given by the largest prime number such that BZC < LZC, with the corresponding allowed root values and cyclic extensions, which are in turn obtained from RRC signalling. In addition, the maximum number of candidate overlaid sequences per OOK ON chip for one cell is 16 for M=1; 8 for M=2; and 4 for M=4. If the overlaid sequence does not carry information, a single overlaid sequence is on each OOK ‘ON’ symbol. Otherwise, the raw information bits are used to determine the overlaid OFDM sequences for each OOK ON chip. This is

- Assume L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

- Assume N is the number of raw information bits. If N / log2 L is not an integer, bit 0 as MSB is used for padding.

- The raw information bits are divided into K segments from MSB to LSB, where K = ceil(N / log2 L).

In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially. See Figure 3 below.

Figure 3: Overlay sequence to bit mapping in OOK waveform

In general, it can be observed that several new aspects are introduced with this new LP-WUS waveform. Hence, RAN4 needs to discuss if and how to determine the relevant demodulation requirements based on the multiple novel aspects discussed above.

# Discussion

From the latest RAN4#116bis, we have a number of open items still to discuss.

With respect to whether to define separate test cases for idle/inactive mode and connected mode, the latest agreement is shown below.

In our view, the performance of LP-WUS detection can we determined by a proper test configuration, regardless of whether the UE is operating in IDLE/INACTIVE or CONNECTED modes. In the end, what is being tested is the signal processing capabilities of the UE, not procedures behind. For the entire test setup, it’s up to RAN5 to define the testability and the specific test configuration.

Observation 1: The UE signal processing capabilities and the underlying SNR performance of LP-WUS detection is rather transparent to whether the UE is operating in IDLE/INACTIVE or CONNECTED mode.

Proposal 1: Defer to RAN5 the decision and details of how to test LP-WUS miss-detection performance.

Another important aspect is what to assume about the OFDM-based LP-WUR and whether to consider one-shot or not as a decoding assumption.

As discussed during the meeting, but not agreed, we believe that there should be no assumption on UE behavior if it’s not a direct consequence of a capability or a specific UE declaration. In this case, we believe that LP-WUS detection should be left up to UE implementation. Whether a UE chooses in the field to use one-shot or full-repetition for sequence detection in OFDM LR will depend on specific algorithm specification.

Observation 2: It’s up to UE implementation how to detect and demodulate a signal if there is no specific capability signalling associated, i.e. one-shot vs full-repetition decoding.

Proposal 2: Do not make any specific UE behavior assumptions when it comes to LP-WUS detection via a sequence detection (OFDM) based LP-WUR.

In addition to this, another open item is related to whether to define a single requirement for OFDM and OOK receivers, as per the extract below:

Since an OOK and a OFDM receivers are fundamentally different, we do not believe that there’s any advantage in collapsing these two requirements into one. As explained before, we usually determine performance based on capabilities if those capabilities will clearly affect performance and behavior.

Observation 3: The OOK and OFDM based LP-WUR are fundamentally different in terms of signal processing.

Proposal 3: Define two set of requirements, one for each receiver type: OOK and OFDM.

Finally, there is still an open issue related to applicability rules for OFDM and OOK receivers, which boils down to applicability rules. See below:

On this, we believe that this is a lower priority topic that can be better handled during CR discussions. In addition, some companies may still prefer insisting in define a single requirement for both receiver types, that part should be clarified before drafting applicability rules.

Proposal 4: Defer the discussion of applicability rules until simulation results have been analyzed for both receivers, OOK and OFDM.

Finally, the most important part during this meeting is to downselect test cases and parameters that have been discussed. In the last way forward we can observe a long list of test cases, which we strongly believe is quite bloated. See below:

In the first case, we believe that M=4 is sufficient for FR1, since M=1 needs to be tested in FR2. Moreover we don’t see a strong need to test both M factors in FR1.

Proposal 5: For FR1, consider M=4 only for both the OOK and the OFDM implementations of LP-WUR.

In terms of propagation, both TDLA and TDLC channels could be taken into consideration since there are valid operating points. However, the expected mobility of a UE operating with this feature is assumed to be low. Hence, a good approach would be to define requirements based only on TDLA30-10 in FR1.

Observation 3: For best LP-WUS/WUR operation, expectation is that UE should operate in rather low mobility conditions.

Proposal 6: For FR1, downselect the propagation channel to TDLA30-10 only.

Finally, in terms of information/coded bits, we believe that it’s reasonable to test both pairs, since both receivers should be tested in their decoding capabilities.

Proposal 7: In terms of information bits and coded bits after rate-matching, consider both {2,12} and {5,32} pairs for the OOK receiver.

Finally, it is worth noting that the test cases associated to the sequence detector (OFDM) LP-WUR are not correctly specified, as per the table from the previous WF. For sequence detection, the bits are transmitted uncoded. Hence, it is worth clarifying that the notion of information bits (2 or 5) or coded bits after rate-matching (L=12,32) does not apply here.

Observation 4: The notion of information bits vs coded bits after rate-matching do not apply for the OFDM LP-WUR since bits are transmitted uncoded, but rather only in the pattern of OOK symbols that are used for the sequence to be detected.

Proposal 8: For sequence detection test cases, consider only a payload of 5 bits, plus the corresponding 1 bit for padding.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: The UE signal processing capabilities and the underlying SNR performance of LP-WUS detection is rather transparent to whether the UE is operating in IDLE/INACTIVE or CONNECTED mode.

Proposal 1: Defer to RAN5 the decision and details of how to test LP-WUS miss-detection performance.

Observation 2: It’s up to UE implementation how to detect and demodulate a signal if there is no specific capability signalling associated.

Proposal 2: Do not make any specific UE behavior assumptions when it comes to LP-WUS detection via a sequence detection (OFDM) based LP-WUR.

Observation 3: The OOK and OFDM based LP-WUR are fundamentally different in terms of signal processing.

Proposal 3: Define two set of requirements, one for each receiver type: OOK and OFDM.

Proposal 4: Defer the discussion of applicability rules until simulation results have been analyzed for both receivers, OOK and OFDM.

Proposal 5: For FR1, consider M=4 only for both the OOK and the OFDM implementations of LP-WUR.

Observation 4: For best LP-WUS/WUR operation, expectation is that UE should operate in rather low mobility conditions.

Proposal 6: For FR1, downselect the propagation channel to TDLA30-10 only.

Proposal 7: In terms of information bits and coded bits after rate-matching, consider both {2,12} and {5,32} pairs for the OOK receiver.

Observation 5: The notion of information bits vs coded bits after rate-matching do not apply for the OFDM LP-WUR since bits are transmitted uncoded.

Proposal 8: For sequence detection test cases, consider only a payload of 5 bits, plus the corresponding 1 bit for padding.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024