3GPP TSG-RAN WG4 Meeting #117                                      	                        R4-2520648

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	7.5.4

Source:	Apple Inc.

Title:	On Demodulation Performance Requirements for 6Rx UEs with Inter-Cell and Intra-Cell Inter-User Interference

Release:	Rel-20

Document for:	Discussion

# Introduction

A new WID on NR demodulation performance has been created for 5G Advanced in Release 20. The WID covers two main areas: First, specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1, and second, to define performance requirements for 6Rx UE (both handheld and FWA) in interference scenarios. In this contribution we focus on the second objective, and the relevant objectives in the WID are captured below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference

  - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

    - Use the Rel-17 interference model for 2/4Rx UE as a baseline

    - Up to 4 MIMO layers for target cell

  - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Use the existing interference model for 2/4Rx UE as a baseline

    - Up to 2 MIMO layers for each user

# General Aspects

From the previous RAN4#116bis, we have the following topics that need to be decided:

First, in terms of applicability rules and other general aspects, we believe that since there are many meetings ahead for this WI, to prioritize technical discussion until sufficient progress has been made.

Observation #1: RAN4#117 is the second meeting to treat this work item.

Proposal #1: Postpone general topics and applicability rules discussions until sufficient progress has been made with test definition and parameters.

Second, with respect to UE types, it is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. We believe that since in Rel-19 discussion it has been agreed to define a single set of requirements for both types, we propose to continue the same approach.

Observation #2: During Rel-19 discussion of 6Rx UEs, it has been agreed to define a single set of requirements for FWA and handheld UEs for the case of 4 MIMO layers.

Observation #3: PDSCH requirements with Inter-Cell and Intra-Cell Inter-User interference in previous releases have been defined using the ULA Low model since these are conducted requirements.

Proposal #2: Define a single set of requirements for FWA and Handheld UEs considering and antenna correlation model defined by parameters α = 0 and β = 0.

# PDSCH Requirements with Inter-Cell Interference

In this section we focus on the aspects specific to PDSCH requirements with inter-cell interference. From RAN4#117, the following open issues remain:

In addition, the rank decision needs to be also agreed. From the previous meeting companies agreed that for initial evaluation purposes, RAN4 would rank 1, rank 2 and rank 4 and make further selection based on simulation.

Observation #4: At the serving cell, existing Release 17 2Rx/4Rx requirements for inter-cell interference consider Rank 1 at the serving cell, while Release 19 8Rx requirements for inter-cell interference consider Rank 2 and Rank 4. At the interference cells, both Release 17 and Release 19 consider random rank with 70% and 30% probability for Rank 1 and Rank 2.

During Release 20, we will define requirements for both FWA and Handheld UEs. During Release 17. requirements were defined for any kind of UE, while during Release 19 the requirements focused on FWA devices only.

Proposal #3: For 6Rx UEs, consider Rank 1 and Rank 2 transmission at the serving cell.

Finally, with respect to propagation conditions, we believe that both TDLC300-100 and TDLA30-10 can be considered as a starting point for handheld devices. For FWA devices, TDLA30-10 is sufficient.

Observation #5: Typical propagation conditions for FWA and handheld devices differ in both frequency and time selectivity.

Proposal #4: For inter-cell interference requirements for 6Rx handheld UEs, consider TDLC300-100 and TDLA30-10 as a starting point. For FWA devices, consider TDLA30-10 only.

However, if the same requirements will be defined for both FWA and handheld UEs, we believe that we may also need to consider this fact when selecting propagation conditions. In our view, the common denominator between both UE types is TDLA30-10.

Observation #6: It has been discuss to define only a single requirement for both handheld and FWA devices, so if this is agreed, the propagation condition needs to be unique.

Proposal #5: For inter-cell interference requirements, TDLA30-10 only if a single set of requirements is agreed for both FWA and handheld devices.

# PDSCH Requirements with Intra-Cell Inter-User Interference

In this section we focus on the aspects specific to PDSCH requirements with intra-cell inter-user interference. During the past RAN4#116bis meeting, the following open issues remain.

During Rel-19 discussion we proposed that for having a more consistent TS 38.101-4, we should define 6Rx CQI requirements. In the same light, since CQI requirements with interference are already defined for 8Rx, we believe that those should be defined for 6Rx as well.

Proposal 6: For intra-cell inter-user interference, consider TDLA30-10 channel, based on defining the same requirements for FWA and handheld UEs.

Proposal 7: For intra-cell inter-user interference, cover MCS13 and MCS17 for evaluation.

# CQI Reporting with Inter-Cell Interference

In this section we focus on the aspects specific to CQI reporting. From the previous meeting, the open issues to be discussed are the following:

During Rel-19 discussion we proposed that for having a more consistent TS 38.101-4, we should define 6Rx CQI requirements. In the same light, since CQI requirements with interference are already defined for 8Rx, we believe that those should be defined for 6Rx as well.

Observation #7: Rel-19 has defined CQI reporting requirements for 8Rx UEs with inter-cell interference.

Proposal #8: RAN4 to define CQI reporting requirements for 6Rx UEs with inter-cell interference.

Proposal #9: RAN4 to reuse Rel-17 interference models for 6Rx devices.

Proposal #10: RAN4 to reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: RAN4#117 is the second meeting to treat this work item.

Proposal #1: Postpone general topics and applicability rules discussions until sufficient progress has been made with test definition and parameters.

Observation #2: During Rel-19 discussion of 6Rx UEs, it has been agreed to define a single set of requirements for FWA and handheld UEs for the case of 4 MIMO layers.

Observation #3: PDSCH requirements with Inter-Cell and Intra-Cell Inter-User interference in previous releases have been defined using the ULA Low model since these are conducted requirements.

Proposal #2: Define a single set of requirements for FWA and Handheld UEs considering and antenna correlation model defined by parameters α = 0 and β = 0.

Observation #4: At the serving cell, existing Release 17 2Rx/4Rx requirements for inter-cell interference consider Rank 1 at the serving cell, while Release 19 8Rx requirements for inter-cell interference consider Rank 2 and Rank 4. At the interference cells, both Release 17 and Release 19 consider random rank with 70% and 30% probability for Rank 1 and Rank 2.

Proposal #3: For 6Rx UEs, consider Rank 1 and Rank 2 transmission at the serving cell.

Observation #5: Typical propagation conditions for FWA and handheld devices differ in both frequency and time selectivity.

Proposal #4: For inter-cell interference requirements for 6Rx handheld UEs, consider TDLC300-100 and TDLA30-10 as a starting point. For FWA devices, consider TDLA30-10 only.

Observation #6: It has been discuss to define only a single requirement for both handheld and FWA devices, so if this is agreed, the propagation condition needs to be unique.

Proposal #5: For inter-cell interference requirements, TDLA30-10 only if a single set of requirements is agreed for both FWA and handheld devices.

Proposal #6: For intra-cell inter-user interference, consider TDLA30-10 channel, based on defining the same requirements for FWA and handheld UEs.

Proposal #7: For intra-cell inter-user interference, cover MCS13 and MCS17 for evaluation.

Observation #7: Rel-19 has defined CQI reporting requirements for 8Rx UEs with inter-cell interference.

Proposal #8: RAN4 to define CQI reporting requirements for 6Rx UEs with inter-cell interference.

Proposal #9: RAN4 to reuse Rel-17 interference models for 6Rx devices.

Proposal #10: RAN4 to reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx.

# References

- RP-252957 “New WID: NR Demodulation Performance, Phase 6”, by RAN4 chair (Apple), 3GPP TSG-RAN Meeting #109, Beijing, China, September 15-18, 2025