3GPP TSG-RAN WG4 Meeting #117                                                                       R4-2520649

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	6.2.3

Source:	Apple Inc.

Title:	PDSCH Demodulation Requirements for Type-4 UEs for Non-collocated Carrier Aggregation

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The initial Intra-band Non-collocated EN-DC/NR-CA WI was approved during RAN#95 meeting. The focus of that WI is to study the feasibility of supporting non-collocated scenarios, including the minimum performance requirements for PDSCH. The work has been delimited to TDD in bands n77/n78. This is, band combinations DC_42_n77/n78 for EN-DC, and CA_n77(2A) and CA_n78(2A) for NR-CA. About minimum PDSCH performance requirements, they should be obtained based on the agreed conditions of applicable maximum received time difference (MRTD) requirement of 33us and the maximum power imbalance between component carriers of 25dB. In this paper, we resume this work for Type-4 UEs during Release 19.

# 2. Discussion

During the last RAN4#116bis there was substantial progress since Type-4 UE requirements are expecting to take the exact same setup than those requirements defined in Rel-18 for Type-2 UEs. The only difference is that each RF chain now can support up to 4 MIMO layers. The previous agreements are presented below:

Hence, at this point the only remaining technical topic is to provide our simulation results based on these agreements, and the fact that the performance requirement for the weaker carrier in Rel-18 will be reused in Rel-19.

Observation 1: Performance requirements for Type-2 UE are -0.9dB for QPSK 0.30 Rank 2 for the weaker carrier, and 23.3dB for 256QAM 0.69 Rank 2 for the stronger carrier. Performance requirement for the weaker carrier will be reused.

Using these considerations and the MCS levels agreed during RAN4-108bis, our results are as follows:

| Table | MCS | Rank | SNR |
| --- | --- | --- | --- |
| 64QAM | 4 | 2 | -0.9 |
| 256QAM | 21 | 4 | 21.5 |
| 256QAM | 22 | 4 | 23.2 |
| 256QAM | 23 | 4 | 23.7 |
| 256QAM | 24 | 4 | 24.9 |

Proposal 1: Choose MCS22 as the operating point for the stronger carrier in Type-4 UEs.

# 4. Conclusion

Our observations and proposals about the testing criteria for Type-2 UEs in Intra-band Non-collocated NR-CA scenarios are summarized below:

Observation 1: Performance requirements for Type-2 UE are -0.9dB for QPSK 0.30 Rank 2 for the weaker carrier, and 23.3dB for 256QAM 0.69 Rank 2 for the stronger carrier. Performance requirement for the weaker carrier will be reused.

Proposal 1: Choose MCS22 as the operating point for the stronger carrier in Type-4 UEs.

# References

- R4-2316917	, “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson.

- R4-2311306, “On frequency separation for intra-band non-collocated CA”, Apple Inc.