3GPP TSG-RAN WG4 Meeting #117                                       	                       R4-2520650

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	7.10.3

Source:	Apple Inc.

Title:	On the RRM Impact of MIMO Phase 6 Enhancements

Release:	Rel-20

Document for:	Discussion

# Scope

A new WID on NR MIMO Phase 6 has been finished. First, to improve the performance of TDD systems, especially for cell edge user’s experience, SRS capacity and coverage need to be further enhanced by introducing the utilization of more contiguous symbols for SRS configuration via introducing SRS transmission across two adjacent slots (S and U slots). Second, as a UE is transitioned from IDLE/INACTIVE to CONNECTED mode, from SCell deactivation to activation, or from SCell dormancy state to active state, due to the lack of priori measurement/reporting the user perceived throughput (UPT) is low, so early CSI is introduced. Finally, larger number of antenna ports (up to 128) has been introduced in Rel-19 NR, partly to cater for the emergence of higher frequency bands in FR1. However, as the number of antenna ports increases, the linear increase in RS overhead from the current CSI-RS design becomes a bottleneck, so it is beneficial to reduce the CSI-RS frequency-domain density for larger number of CSI-RS ports aggregated across multiple CSI resources to enhance throughput in larger-scale MIMO deployments

On the first topic, improvement of SRS capacity and coverage, we do not see obvious RRM impact. On the second topic on early CSI triggering there is clear RRM impact since the activation delay requirements should be modified. Finally, for CSI-RS overhead reduction we don’t see impact.

# SRS Enhancements for UL Capacity and Coverage

For UL capacity and coverage enhancement, there is potential impacts on SRS AS switching requirements considering both the total number of OFDM symbols used for SRS and because of the relaxation allowing cross-slot from S to U symbols. In Rel-17, in RAN4 we defined SRS AS interruption requirements based on per SRS resource, and it has been assumed that SRS symbols are allocated in the last 6 symbols of a slot. From RAN1 point of view, the fact that the SRS can be allocated to other symbols in addition to the last 6 symbols of a slow has been captured in the 3GPP specifications, but no new RAN4 RRM requirements have been defined.

Observation 1: Legacy SRS antenna port switching interruption requirements assume that SRS symbols are allocated in the last 6 symbols of a slot, even though RAN1 specifications have relaxed SRS allocation.

In addition, in this new WID, in terms of time domain enhancements we have introduced cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set. For further clarification, there can be different time domain resource mapping under this approach: First, different SRS resources are in two adjacent slots, and second, one SRS resource is split into two adjacent S+U slots.

Observation 2: Cross-slot SRS enhancement may consider both 1) different SRS resources in two adjacent slots and 2) one SRS resource split into two adjacent slots S+U.

In addition, during RAN1 discussion there has been proposals to extend the duration of SRS resources, such that the duration can extend beyond the 14 symbols defined today.

Observation 3: The length of SRS resources could be extended beyond 14 OFDM symbols as per RAN1 discussion but has not yet been agreed.

By checking previous RAN4 discussions, we can again assume that to calculate the interruption requirement, the components of the interruption time of SRS AS are the switching time before and after the SRS transmission, which is based on UE capability, and the SRS transmission time in terms of OFDM symbols, while the antenna switching time has been assumed in the past to be of 15us. For ease of implementation, the interruption time then is mapped to a certain number of slots, i.e. rounded up, based on the numerologies of the aggressor and victims. The interruption length is rounded up to number of slots to account fort to MRTD and TA since it would be quite hard to pinpoint the exact number of symbols impacted.

Observation 4: SRS AS interruption requirements are defined considering that the interruption is given by the switching time before and after the SRS transmission, plus the SRS resource duration.

Finally, we include here the two relevant agreements in the past RAN1 meeting relevant for enhanced time-domain operation of SRS.

The first agreement, it adjust the notation with respect to starting symbol and SRS resource length, removing the restriction on the offset. In addition, the second agreement clarifies that the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set.

Proposal 1: RAN4 to review the applicability of existing SRS carrier and antenna port switching requirements to the case of SRS resources are transmitted not only in last 6 symbols of a slot, and when SRS resources could occupy two adjacent slots.

The interruption requirements for SRS antenna switching are defined based on either symbol-level or slot-level, depending on the number of SRS resources. It has been discussed in the past whether to define interruption requirements in slot level or symbol level, but as discussed before, it’s better to maintain the slot level as much as possible as not to overcomplicate implementation on both gNodeB and UEs

Proposal 2: If we agree to revisit the interruption requirements for SRS, keep requirements defined on the slot granularity.

Finally, specifically speaking on the scenario where SRS resources span more than 14 OFDM symbols, we believe we should not assume this yet.

Proposal 3: RAN4 wait for more RAN1 progress regarding the scenario of SRS resources spanning more than 14 OFDM symbols.

# Early CSI for Transition from IDLE/INACTIVE to CONNECTED

For this topic, RAN1 has agreed the support of aperiodic CSI-RS for tracking triggered via MSG4 of 4-Step RACH. The general 4-step procedure is as follows:

Step 1: The UE receives resource/reporting configuration in System Information (SI) before Msg3.

Step 2: The UE reports its CSI capability for the feature via Msg3.

Step 3: The UE receives a Msg4 that triggers the early SRS/CSI/CSI-RS.

Step 4: The UE performs the triggered aperiodic transmission/reception/reporting.

During this procedure, the IDLE/INACTIVE UE needs to refine its time/frequency synchronisation. It can then acquire precise tracking based on A-TRS transmitted within the CSI-RS transmission bandwidth so the UE can receive the CSI-RS for CSI on the appropriate location.

During the last RAN4 meeting in Prague the following agreements were made:

In general we believe that since no delay requirements exist in RAN4 RRM, we should not prioritize this topic since no direct impact is foreseen in TS 38.133. During the last RAN1 meeting we can observe that most agreements were made on specific CSI reportings applicable, but this has no impact on RRM specification. For example:

Hence, we believe that RAN4 should deprioritize this topic and wait for further RAN1 progress. If after further RAN1 progress we can identify any relevant RRM impact, we are open to discuss this issue again.

Observation 5: RAN4 does not define delay requirements for IDLE/INACTIVE to CONNECTED mode transitions, so there is no RRM impact even if early CSI triggering is configured in this specific scenario.

Proposal 4: Do not define any new RRM requirements for early SRS/CSI-RS/CSI triggering when UE transitions from IDLE/INACTIVE to CONNECTED mode. Deprioritize this topic unless clear RRM impact is identified after waiting for further RAN1 progress.

# Early CSI Acquisition for SCell Transition from Dormant to non-Dormant State

The benefits for early CSI acquisition for switching out of SCell dormancy include allowing MIMO CSI availability immediately after the UE switches out of the SCell dormancy and allowing the network to enable SCell dormancy more often to save energy. This is, SCell dormancy can be used more frequently if the network can rely that the added throughput is high without delay.

In SCell dormancy, both periodic and semi-persistent CSI reporting for the SCell transmitted on another serving cell is already supported. However, periodic and semi-persistent SRS transmission during SCell dormancy is not supported, and aperiodic SRS/CSI/CSI-RS transmission triggered by the same signaling of the BWP switching DCI to switch out of dormancy is also not supported. Therefore, in this feature when the UE switches out of the dormant BWP to the non-dormant BWP, early SRS/CSI-RS/CSI can be triggered for faster CSI acquisition simultaneously.

The latest agreements from RAN1 point of view are the following, for which the timeline is still an open item that need to be agreed. On the other hand, even though the exact triggering mechanisms are still FFS, it’s largely understood that the DCI indicating switching out of SCell dormancy will be used.

On the other hand, from RAN4 point of view, the latest agreement on the topics is below:

Observation 6: RAN1 progress on SCell transition out of dormancy has agreed on DCI triggered switch, but timeline of the aperiodic CSI reporting and aperiodic CSI-RS for CSI has not been discussed yet.

Proposal 5: Postpone the discussion of SCell transition from dormant to non-dormant state until further progress has been achieved in RAN1.

At the moment the only applicable requirement in TS 38.133 is the BWP delay. The Active BWP switch delay is directly related to the SCell Dormancy feature, as dormancy for an SCell is implemented through switching the active BWP to a dormant BWP. However, no CSI reporting component is part of the requirements so no impact is foreseen here yet.

Proposal 6: For the applicability of existing BWP delay requirement in TS 38.133 Section 8.6, leave the topic up to company contribution.

# Early CSI Acquisition for SCell Transition from Deactivated to Activated State

Existing 3GPP TS 38.321 specifications explicitly prohibit UEs from transmitting SRS, reporting CSI feedback, and monitoring PDCCH on a still deactivated SCell. This effectively creates a situation where the CSI stage cannot start unless the SCell has completely transitioned to activated state. Hence, one way to improve the total delay between the UE receiving the SCell activation command and the point in which the UE sends the first valid CSI report is by intertwining the activation phase and the CSI phase. As opposed to the previous cases, here the requirement has a clear CSI component, so there is potential RAN4 that needs to be studied.

One way to improve the total delay between the UE receiving the SCell activation command and the point in which the UE sends the first valid CSI report is by intertwining the activation phase and the CSI reporting phase.

As already discussed before, the SCell activation timeline can be long, so the benefits of reducing processing time due to CSI acquisition may be limited.

Observation 7: Current framework for SCell Activation containes a variety of cases depending on knowledge of the SCell, UE measurement capability, and other aspects.

At the very beginning of the analysis, we believe we should restrict ourselves to the case of known cells only. We are still open to consider unknown cell scenarios as well, and also to analyze on which of these scenarios the feature should create meaningful impact. However, we believe RAN4 should start with known cells only, for the context of Rel-17 Fast SCell activation as a first priority, and then the slower Rel-15 Scell activation scenarion.

Observation 8: For SCell activation delay with early triggering, is it expected that the most gain can be achieved in the known cell scenarios.

Proposal 7: For SCell activation delay with early triggering, prioritize known cells with Rel-17 and Rel-15 mechanisms.

At the moment, the latest RAN4 agreements are the following:

In this regard, then we believe it’s appropriate to start discussing the timeline of events from RAN4 points of view, and at the same time monitor RAN1 progress. The latest relevant agreement in RAN1 is shown below:

Proposal 8: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after SCell activation command is received, based on Rel-17 Fast SCell framework and Rel-15 legacy framework for known cells.

# Conclusions

Our observations and proposals are summarized below:

Observation 1: Legacy SRS antenna port switching interruption requirements assume that SRS symbols are allocated in the last 6 symbols of a slot, even though RAN1 specifications have relaxed SRS allocation.

Observation 2: Cross-slot SRS enhancement may consider both 1) different SRS resources in two adjacent slots and 2) one SRS resource split into two adjacent slots S+U.

Observation 3: The length of SRS resources could be extended beyond 14 OFDM symbols as per RAN1 discussion but has not yet been agreed.

Observation 4: SRS AS interruption requirements are defined considering that the interruption is given by the switching time before and after the SRS transmission, plus the SRS resource duration.

Proposal 1: RAN4 to review the applicability of existing SRS carrier and antenna port switching requirements to the case of SRS resources are transmitted not only in last 6 symbols of a slot, and when SRS resources could occupy two adjacent slots.

Proposal 2: If we agree to revisit the interruption requirements for SRS, keep requirements defined on the slot granularity.

Proposal 3: RAN4 wait for more RAN1 progress regarding the scenario of SRS resources spanning more than 14 OFDM symbols.

Observation 5: RAN4 does not define delay requirements for IDLE/INACTIVE to CONNECTED mode transitions, so there is no RRM impact even if early CSI triggering is configured in this specific scenario.

Proposal 4: Do not define any new RRM requirements for early SRS/CSI-RS/CSI triggering when UE transitions from IDLE/INACTIVE to CONNECTED mode. Deprioritize this topic unless clear RRM impact is identified after waiting for further RAN1 progress.

Observation 6: RAN1 progress on SCell transition out of dormancy has agreed on DCI triggered switch, but timeline of the aperiodic CSI reporting and aperiodic CSI-RS for CSI has not been discussed yet.

Proposal 5: Postpone the discussion of SCell transition from dormant to non-dormant state until further progress has been achieved in RAN1.

Proposal 6: For the applicability of existing BWP delay requirement in TS 38.133 Section 8.6, leave the topic up to company contribution.

Observation 7: Current framework for SCell Activation containes a variety of cases depending on knowledge of the SCell, UE measurement capability, and other aspects.

Observation 8: For SCell activation delay with early triggering, is it expected that the most gain can be achieved in the known cell scenarios.

Proposal 7: For SCell activation delay with early triggering, prioritize known cells with Rel-17 and Rel-15 mechanisms.

Proposal 8: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after SCell activation command is received, based on Rel-17 Fast SCell framework and Rel-15 legacy framework for known cells.

# References

- RP-252957 “New WID: NR Demodulation Performance, Phase 6”, by RAN4 chair (Apple), 3GPP TSG-RAN Meeting #109, Beijing, China, September 15-18, 2025