3GPP TSG-RAN4 Meeting #117	R4-2520682

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.1

Source:	Apple

Title:	On 6G system parameters - Waveform

Document for:	Discussion

# 1	Introduction

At the last meeting, RAN4 started 6G discussion. A WF on 6G system parameters was agreed [1], including the following aspects on waveform:

# Topic #1: Waveform NOTE: Main proposals in this document are based on inputs for this RAN4 meeting, which are served as reference for information and could be referred for some agreements in the WF. ## Framework study for waveform - Main proposals - Close collaboration with RAN1 - RAN4 must work closely with RAN1 to define evaluation methodologies, assumptions, and a common PA model for simulations. - Some proposals suggest postponing detailed RAN4 waveform discussions until 2026, after RAN1 has made sufficient progress. - Key Performance Indicators (KPIs) - Waveform evaluation should include KPIs like PAPR, Cubic Metric (CM), SEM, ACLR, OBE, EVM, implementation complexity, and compatibility with 5G band combinations. - Evaluation metric - Propose using "Net Gain" as a starting point for evaluating low-PAPR schemes. - Specific studies include: - Study Low-PAPR techniques: Investigate both transparent and non-transparent techniques (like CFR-SE) to reduce PAPR and assess their impact on RF requirements. - Explore new waveforms for new use cases: Study the feasibility and impact of supporting DFT-s-OFDM in the DL (e.g., for massive IoT) and new waveforms for Integrated Sensing and Communications (ISAC). - Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement ## PA model - Main proposals - Proposal: Study improved PA modelling. Further discuss considering - PA dimensioning (MPR-0) - Pre-distortion - Memory effects - APT PA model - Doherty PA - PA model being based on each company’s own choice. - Different PA models e.g. for IoT or FWA - Better alignment of simulations and measurements - Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

In this contribution, we continue the discussion on 6G waveform based on the WF above.

# 2	Discussion

## 2.1 Evaluation methodology

Good progress was made at the last meeting in the sense that the evaluation methodology/framework was largely agreed upon, as captured in the above WF. In the following, we share our views on the open issues.

First, we agree that at this moment, RAN4 should focus on the evaluation of low PAPR waveforms for UL. This is because UL coverage enhancement is one key target of 6G radio design.

Second, since any realistic assessment of the gain would involve the consideration of PA models and compliance with RAN4 requirements, it is likely the PAPR gain of the waveform may not translate dB to dB into the power gain.  Therefore, we think the net coverage gain is a good evaluation metric to use in comparing the performance of different waveforms.

The net coverage gain can be defined as follows:

G = SNR + MPR,

Where SNR is the potential SNR degradation derived from link-level simulation given a BLER, and MPR is the power backoff gain due to reduced PAPR.

There were also questions raised at the last meeting on which WG should be responsible for evaluating or deriving the two parameters, i.e., SNR and MPR. In our understanding, RAN1 can be responsible for SNR and RAN4 for MPR.

Proposal 1: It is proposed to agree on the use of net coverage gain as the evaluation metric in the UL low PAPR waveform evaluation. More specifically, RAN4 evaluation should focus on deriving MPR based on simulation.

In addition, given the various waveform proposals, some with bandwidth extension to further reduce PAPR and some without, it is proposed to keep the same spectrum efficiency. To this end, there are two cases:

- Case 1: The RBs in the bandwidth extension can be scheduled for other UEs. In this case, RAN4 needs to investigate the impact on IBE requirement, i.e., will the same IBE requirement as in the case of no bandwidth extension apply or there will be different IBE requirement?

- Case 2: The RBs in the bandwidth extension cannot be scheduled for other UEs. In this case, the same spectrum efficiency can be kept by adjusting the coding rate while keeping the same modulation, in the evaluation.

In either case, we believe the out-of-band emission requirement should be the same, as this requirement is used to ensure coexistence with adjacent channels.

Proposal 2: It is proposed to keep the same spectrum efficiency in the waveform evaluation for fair comparison.

We also note that RAN1 has discussed those aspects and made agreements along the lines of Proposals 1&2. To have a clear understanding in RAN4 evaluation, we think it would be good that RAN4 makes formal agreements too.

## 2.2 Related RF requirements to be considered

To evaluate the power backoff gain MPR, the waveform passing the TX chain including PA model needs to meet the RAN4 requirements including ACLR, SEM, IBE, EVM. In addition, the spectrum utilization or minimum guard band requirement needs to be met. As this is a 6G SI that is not going to specify the 6G requirement, it is reasonable to consider using 5G requirements as a starting point.

That said, ACLR requirement warrants further consideration. This is because the amount of potential power boosting by the low PAPR waveforms may mean that a UE with a certain power class is able to transmit power that amounts to the power of a higher power class. For example, for PC2, the max. power is 26dBm, if power boosting of 5dB or more is possible, which would lead to a TX power of 31dBm or higher, would it mean in this case the ACLR of PC1, i.e., 37dB should apply instead of 31dB for PC2, as shown in the NR ACLR requirement below? It is better to have a common understanding on this.

Proposal 3: In the evaluation, it is proposed to consider reusing the 5G requirements including ACLR, SEM, IBE, EVM and spectrum utilization. In addition, FFS on which ACLR requirement to use depending on the amount of power boosting rendered by the low PAPR waveform.

## 2.3 PA modelling

While it is preferred to use a common PA model in the waveform evaluation to facilitate performance comparison, it may be difficult to align PA models from different UE/chipset vendors because of their proprietary implementation choices, e.g., the use of DPD and other implementation considerations such as PA power efficiency. Therefore, it appears more practical to allow each company to choose a PA model that suits their implementation and state clearly how the PA is calibrated. It is noted that although performance difference over the common PA model between a candidate waveform and baseline waveform (i.e., Pi/2 BPSK over DFT-S-OFDM) is not attainable, each company can still show the relative difference over the PA model they choose to use.

Proposal 4: In the evaluation, companies can choose their own PA model and state clearly how the PA is calibrated to facilitate comparison. Use of supply voltage adaptations in case of high power back-off scenarios shall not be precluded.

It is known that memory effects become more pronounced when the channel bandwidth increases. This is because the envelope of the signal changes faster in time over a large channel bandwidth, compared to a narrowband signal. On the other hand, a given PA has fixed time constant for its bias networks or matching networks. As a result, if the PA cannot follow these rapid changes, there will be strong memory effects.

That said, adaptive DPD algorithms can be used to mitigate such memory effects over large channel bandwidth. As discussed above, each company may have its proprietary DPD algorithms to address the memory effects. Therefore, it is OK not to explicitly consider memory effects.

Proposal 5: In the evaluation, it is up to companies if memory effects can explicitly modeled/simulated.

Furthermore, we need to discuss the PA calibration conditions. In 5G, the calibration conditions are given below:

| Waveform | DFT-S-OFDM |
| --- | --- |
| Modulation | QPSK |
| Channel bandwidth | 20MHz |
| RB allocation | 100RB |
| PC2/PC3 ACLR | 31/30dB |
| MPR | 1dB |

Besides the ACLR value that was raised in Section 2.2, we can discuss if a larger channel bandwidth than 20MHz needs to be considered.

Proposal 6: In the evaluation, it is FFS if a larger than 20MHz CBW should be used.

## 2.4 Initial results following the methodology

We share some initial evaluation results below based on the discussions above. In the evaluation, a high-efficiency PC2 PA is used with the calibration conditions given in Section 2.3.

| Transparent vs non-Transparent; with BW 20% extension or without BW extension | Modulation (with DFT-s-OFDM and Coding Rate=0,3066) | Δ𝑆𝑁𝑅 @10% BLER (dB) | 99.99% ΔPAPR (dB) | ΔPout (dBm) @ 31dBc ACLR | ΔPout (dBm) @ 37 dBc ACLR | Net Gain (dB), ΔPAPR-based | Net Gain (dB), ΔPout-based @ 31 dBc ACLR | Net Gain (dB), ΔPout-based @ 37 dBc ACLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| non-Transparent, No BW Extension | π/2-BPSK | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | π/2-BPSK + FDSS [0.28 1 0.28] | -0,58 | 3,56 | 4,54 | 4,88 | 2,98 | 3,96 | 4,3 |
|  | π/2-BPSK + FDSS [0.175 1 0.175] | -0,23 | 2,07 | 2,8 | 2,88 | 1,84 | 2,57 | 2,65 |
|  | GMSK 1-pulse (BT=0.5) | -0,32 | 3,76 | 4,12 | 4,38 | 3,44 | 3,8 | 4,06 |
|  | GMSK 1-pulse (BT=0.35) | -0,42 | 4,26 | 4,75 | 5,12 | 3,84 | 4,33 | 4,7 |
|  | GMSK 1-pulse (BT=0.25) | -0,51 | 4,51 | 4,93 | 6,1 | 4 | 4,42 | 5,59 |
|  | GMSK 2-pulse (BT=0.25) | -0,61 | 4,89 | 5,1 | 7,81 | 4,28 | 4,49 | 7,2 |
|  | GMSK 1-pulse (BT=0.18) | -0,58 | 4,27 | 4,95 | 7,15 | 3,69 | 4,37 | 6,57 |
|  | GMSK 2-pulse (BT=0.18) | -0,81 | 5,11 | 5,11 | 8,13 | 4,3 | 4,3 | 7,32 |

# 3	Conclusions

In this contribution, we discuss 6G system parameters and make the following proposals.

Proposal 1: It is proposed to agree on the use of net coverage gain as the evaluation metric in the UL low PAPR waveform evaluation. More specifically, RAN4 evaluation should focus on deriving MPR based on simulation.

Proposal 2: It is proposed to keep the same spectrum efficiency in the waveform evaluation for fair comparison.

Proposal 3: In the evaluation, it is proposed to consider reusing the 5G requirements including ACLR, SEM, IBE, EVM and spectrum utilization. In addition, FFS on which ACLR requirement to use depending on the amount of power boosting rendered by the low PAPR waveform.

Proposal 4: In the evaluation, companies can choose their own PA model and state clearly how the PA is calibrated to facilitate comparison. Use of supply voltage adaptations in case of high power back-off scenarios shall not be precluded.

Proposal 5: In the evaluation, it is up to companies if memory effects can explicitly modeled/simulated.

Proposal 6: In the evaluation, it is FFS if a larger than 20MHz CBW should be used.

# 4	References

R4-2514641, "WF for 6GR system parameter," Feature lead (Huawei, HiSilicon)