3GPP TSG-RAN4 Meeting  #117	R4-2520683

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.3

Source:	Apple

Title:	On 6G system parameters – channel bandwidth

Document for:	Discussion

# 1	Introduction

At the last meeting, RAN4 started 6G discussion. A WF on 6G system parameters was agreed [1], including the following aspects on channel bandwidth:

- Max Channel Bandwidth

- Min Channel Bandwidth

- FFT size

- Numerology

- Spectrum utilization

- Asymmetric channel bandwidths

In this contribution, we continue the discussion on 6G channel bandwidth based on the WF above.

# 2	Discussion

## 2.1 Max. CBW

The following agreements were captured in the WF:

### Sub-topic 3-1: Max Channel Bandwidth - Main proposals - Frequency-range specific max CBW proposals: - Sub-6 GHz (FDD): 50 MHz or 100 MHz (using 15 kHz/[30kHz] SCS and 4K/8K FFT). - Sub-6 GHz (TDD): 200 MHz (using 30 kHz SCS and 8K FFT). - Upper mid-bands (approx. 6-24 GHz): 200 MHz or 400 MHz (using 30/60 kHz SCS and 8K/16K FFT). - Several proposals further split above frequency range with different sub-frequency ranges for consideration on max CBW and numerology selection e.g. - 6425MHz ~ 7125MHz - 7125MHz ~ 8.4GHz - ~15GHz around - FR2-1 (24-52 GHz): 800 MHz (using 120 kHz SCS and 8K FFT). - Implementation and feasibility studies:

- Study wideband RF impact: Propose that RAN4 studies the RF implications of wide CBWs (200 MHz, 400 MHz, 800 MHz) on requirements like EVM, ACLR, SEM and receiver sensitivity. - Study UE implementation paths: For a 400 MHz CBW, study different UE baseband architectures (e.g., single 16K FFT vs. two 8K FFTs vs. two-component carrier aggregation). - Flexible and asymmetric support: - Different UL/DL CBW: Propose supporting different maximum channel bandwidths for Uplink and Downlink. - BS vs. UE asymmetry: Propose introducing wide CBWs (e.g., 400 MHz) only for base stations, not for UEs, to manage UE complexity. - Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

There were proposals of both 200MHz and 400MHz to be considered or studied from the last meeting. In the following, we share our views.

First, given the increased complexity of supporting 400MHz compared to 200MHz, it is necessary to consider spectrum allocation or availability. In our understanding, 400MHz contiguous spectrum is not expected to available in many regions. Even in the regions where the spectrum allocation may become possible, the corresponding rules are yet to be published, leading to the uncertainty regarding how much contiguous spectrum would eventually be allocated for IMT.

Observation 1: 400MHz contiguous spectrum is not expected to become available in many regions.

Furthermore, 6G design from day 1 should include a mechanism that ensures future larger max. CBW can be smoothly introduced if needed. In fact, 5G design has enabled such mechanism already. Therefore, we see no strong argument that 400MHz must be supported by UEs from day 1.

Proposal 1: 6G design from day 1 should include a mechanism that ensures future larger max. CBW can be smoothly introduced if needed.

Second, we discuss the UE implementation complexity. Following the RAN1 agreement in Oct. meeting (copied below), it can be seen that if a UE supports 400MHz CBW, the required FFT size is 16K; if the UE supports 200MHz CBW, the required FFT size is 8K.

Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

The implementation complexity for 8K FFT vs. 16K FFT is shown below.

- Computational complexity: The number of complex multiplications and additions required for an FFT of size N is on the order of O(N*log N). For 8K FFT, N = 8192, complexity is proportional to 8192 * log₂(8192) = 106496. For 16K FFT, N =16384, complexity is proportional to 16384 * log₂(16384) = 229376, meaning 2.15 times increase over 8K FFT.

- Memory requirements: The memory required includes input/output buffers, memory needed to store twiddle factor tables, and memory for intermediate storage. As memory scales approximately linearly with N, 16K FFT doubles the memory required for 8K FFT.

- Power consumption: The consumed power includes two aspects, dynamic power required for operation and Static/leakage power needed to keep the transistors on. Since both compute and memory access roughly double, the power consumption doubles for 16K FFT.

- Latency: Assuming the same clock speed at UE, 16K FFT leads to longer latency (~2X) than 8K FFT. If UE implements some hardware pipelines by using more parallel butterfly units to maintain the same processing timeline, silicon area, power consumption and cost would increase.

- Silicon area and cost: If area scales roughly linearly with FFT size, then 16K FFT would double the die area compared to 8K FFT. Additionally, the chipset cost would increase too.

Observation 2: 16K FFT would significantly increase implementation complexity compared to 8K FFT in terms of computational complexity, memory requirement, power consumption, and silicon area and cost.

From RF perspective, the wider the CBW is, the higher the requirements on RF design. It is more challenging for a PA to maintain linearity and gain flatness over the entire channel bandwidth. To maintain linearity over a wider bandwidth, the PA may become less power-efficient, which generates more heat and consumes more power. It is also harder to address the PA memory effects with adaptive DPD algorithms. On the filter side, the larger the passband is, the harder to design a filter with high selectivity or narrow transition band. It also complicates the antenna design, as usually antennas bandwidth is limited. More specifically, wider channel bandwidth demands more broadband antenna tuning networks, which adds complexity and loss.

Furthermore, supporting wider CBW requires ADCs/DACs with higher sampling rate, which becomes more complex and consumes more power.

Observation 3: Compared to 200MHz, 400MHz CBW would put higher requirements on RF design including PA, filters, antennas, etc. Furthermore, it requires ADCs/DACs with higher sampling rate, which becomes more complex and consumes more power.

Third, it remains extremely unlikely that a single UE will be scheduled with more than 200MHz bandwidth. For example, given 200MHz bandwidth, modulation of 256QAM, coding rate of 0.8, four MIMO layers, the achieved data rate is 200*8*0.8*4 = 5.12Gbps. It is unclear what service would require such high data rate.

Observation 4: It is unlikely a single UE will be scheduled with more than 200MHz bandwidth. For example, given 200MHz bandwidth, modulation of 256QAM, coding rate of 0.8, and four MIMO layers, the achieved data rate is 200*8*0.8*4 = 5.12Gbps, which is more than most services would demand.

Fourth, if the network is configured with 400MHz contiguous spectrum, the UE can use 200 + 200 MHz CA to support it. In our observation, there is no significant performance difference between 400MHz CBW vs. 200+200MHz intra-band contiguous CA.

- Spectrum utilization (SU): Since SU of 100MHz in 5G is already very high at 98.28%, and it is likely the SU of 200MHz and 400MHz is not going to be much higher than 98.28%, the GB difference between 400MHz and 2x200MHz is going to be very small (<1%).

- Assuming all CA enhancements such as SSB-less operation, cross-carrier scheduling, and multi-cell PDSCH/PUSCH scheduling with a single DCI, etc. are deployed in the network and supported by UE, the overhead/complexity associated with CA is not much higher than that of single carrier

Proposal 2: In case where 400MHz contiguous spectrum is available, 200 + 200 MHz CA can be used.

Considering the above reasoning, we believe UE max. CBW of 200MHz strikes a good balance between performance needs and implementation complexity. Note that compared to 5G where the max. CBW of 100MHz is specified for FR1, this means the max. CBW is doubled, which is aligned with the expectation that a new G may increase CBW to meet the ever-increasing data rate.

Proposal 3: For 6G, maximum CBW of 200MHz can be considered, which both BS and UE should support. UE can use 200+200MHz CA to support 400MHz at the network side.

## 2.2 FFT size and numerology

FFT size

To support larger CBW such as 200MHz in 6G, it is natural to consider a larger FFT size than 5G. At the same time, reasonable implementation complexity and cost needs to be considered. Given the above analysis on complexity comparison between 8K FFT and 16K FFT, it is proposed to increase FFT size from 4K in 5G to 8K in 6G.

Proposal 4: It is proposed to support 8K FFT in 6G.

Numerology

As stated in the SID, 6G is aimed to “avoiding multiple numerologies for the same band / sub-range”. Based on the 5G commercial deployments where only single numerology is used for a band, it is sensible to specify single numerology on a band.

Furthermore, although ECP is specified in 5G for 60kHz SCS, it has not been deployed. As a result, the support of ECP should be removed in 6G.

Specifically, we propose to have the following SCS and CP length for 6G in Table 1:

Table 1: Numerology for different frequency ranges

| Frequency range | Numerology |
| --- | --- |
| FR1 (up to 7.125GHz) and new spectrum in ~7GHz (FDD and TDD) | FDD bands: 15kHz SCS, NCP TDD bands: 30kHz SCS, NCP |
| 12 – 15GHz (TDD) | Deprioritized as the spectrum availability is unclear |
| FR2-1 (TDD) | 120kHz SCS, NCP |

Proposal 5: It is proposed to have the SCS and CP length for 6G as shown in Table 1.

As a result, 8K FFT and 30kHz SCS can be used to support 200MHz CBW for spectrum up to 8.4GHz.

## 2.3 Spectrum utilization

The WF captures the following agreements:

### Sub-topic 3-5: Spectrum utilization - Main proposals - Foundational principles and framework: - Propose to adopt the 5G NR design philosophy of a unified spectral emission mask as a baseline for SU evaluation, but evolve it for 6G. - Propose that 6G SU shall not be smaller than 5G NR. Specifically, for the same channel bandwidth, lower SCS should have higher SU than higher SCS. - Propose that the guard band for a given SCS and channel bandwidth should not be larger than that of all larger channel bandwidths for the same SCS. - Specific studies and evaluations: - Study SU enhancement techniques: Study the benefits and RF trade-offs of techniques to enhance SU, such as using larger NRB and advanced spectral confinement. - Evaluate SU systematically: Conduct a comprehensive evaluation of SU, considering the trade-offs between improved SU, RF performance (OOBE, EVM, ACLR), and UE/BS complexity under detailed parameter assumptions (PA model, I/Q imbalance, phase noise, etc.). - For existing channel bandwidths (e.g.,≤100 MHz), reusing 5G SU as starting point. - Study enhanced SU for the existing CBWs, and prioritize enhancing SU for smaller channel bandwidths where it is currently low (<95%). - For newly introduced larger channel bandwidths (e.g., 200MHz), a new SU evaluation is necessary. - Investigate requirement relaxation: Study the relaxation of ACLR, SEM, and EVM requirements as a method to improve SU. - Agreement - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

In our view, RAN4 can defer the discussion on many aspects of the spectrum utilization because the study of many contributing factors just started. For example, UE TX and RX architecture is being discussed in UE RF, waveform discussion is still ongoing, and RF requirements need more time to settle. Without such key factors being stabilized, any effort to study if 6G spectrum utilization can be improved compared to 5G is not efficient.

That said, we think RAN4 can and should study if any new/advanced spectrum confinement technique compared to existing frequency-domain filtering or time-domain windowing can be considered for 6G, from the angle of maturity and implementation complexity.

Proposal 6: It is proposed to defer the spectrum utilization as the discussion on many contributing factors just started. The only exception is RAN4 can study new/advanced spectrum confinement technique if there is such a proposal.

# 3	Conclusions

In this contribution, we discuss 6G system parameters and make the following proposals.

Observation 1: 400MHz contiguous spectrum is not expected to become available in many regions.

Proposal 1: 6G design from day 1 should include a mechanism that ensures future larger max. CBW can be smoothly introduced if needed.

Observation 2: 16K FFT would significantly increase implementation complexity compared to 8K FFT in terms of computational complexity, memory requirement, power consumption, and silicon area and cost.

Observation 3: Compared to 200MHz, 400MHz CBW would put higher requirements on RF design including PA, filters, antennas, etc. Furthermore, it requires ADCs/DACs with higher sampling rate, which becomes more complex and consumes more power.

Observation 4: It is unlikely a single UE will be scheduled with more than 200MHz bandwidth. For example, given 200MHz bandwidth, modulation of 256QAM, coding rate of 0.8, and four MIMO layers, the achieved data rate is 200*8*0.8*4 = 5.12Gbps, which is more than most services would demand.

Proposal 2: In case where 400MHz contiguous spectrum is available, 200 + 200 MHz CA can be used.

Proposal 3: For 6G, maximum CBW of 200MHz can be considered, which both BS and UE should support. UE can use 200+200MHz CA to support 400MHz at the network side.

Proposal 4: It is proposed to support 8K FFT in 6G.

Proposal 5: It is proposed to have the SCS and CP length for 6G as shown in Table 1.

Proposal 6: It is proposed to defer the spectrum utilization as the discussion on many contributing factors just started. The only exception is RAN4 can study new/advanced spectrum confinement technique if there is such a proposal.

# 4	References

R4-2514641, "WF for 6GR system parameter," Feature lead (Huawei, HiSilicon)