3GPP TSG-RAN WG4 Meeting #117	R4- 2520718

# Dallas, USA, 17th – 21st November, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for RRM measurement prediction

Agenda item:	7.12.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1].

Discussion

- Sub-topic 2-1: Core requirements for RRM measurement prediction

### Issue 2-1-1: What kind of core requirements to be defined

Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the follow requirements need to be defined: - Measurement delay/period requirements (OPPO, CMCC, ZTE, Samsung, QC) - Measurement and scheduling restrictions in OW/PW (QC, Samsung) - Prediction delay requirements (OPPO) - Prediction reporting delay requirements (OPPO, HW, QC) - OW/PW length (HW, LGE, Samsung) - Data collection delay requirements (Ericsson) - SCell related requirements (Samsung, QC) - UE mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements (QC)

According to TR38.744, the RRM measurement prediction use case have two basic parameters as following figure. OW(Observation Window) and PW(Prediction Window). RAN4 need to discuss how to decide the length of OW and PW. If necessary the length of OW and PW can be decied based on the simulation results.

The configuration of OW and PW may affect the measurement delay and period requirements. Therefore, a new definition of the measurement delay and period requirements may be necessary for AI/ML mobility. Additionally, we believe that prediction reporting delay requirements also need to be defined. Therefore, the requirements that we think should be defined are as follows:

- Measurement delay/period requirements

- Prediction reporting delay requirements

- OW/PW length

Proposal 1: RAN4 need to define requirements at least for

- Measurement delay/period requirements

- Prediction reporting delay requirements

- OW/PW length

### Issue 2-1-2: Measurement period requirements

- Proposal 1 (OPPO): - For temporal domain prediction case A, the legacy measurement period requirements can be reused. - For temporal domain prediction case B, define the measurement period requirements considering the assumptions on MRRT, skipping patterns and measurement configurations. - For frequency domain prediction, the legacy measurement requirements can be reused by clarifying in CSSF that the frequency layers configured for prediction are not counted. - Proposal 2 (CATT): - RAN4 to discuss the impacts on RRM core requirements for case A and case B separately with more RAN2 progress on framework design, e.g., supported OW/PW length. - Proposal 3 (Xiaomi): - For case B, Observation window length will have impact on measurement duration and report delay - For intra-frequency intra-cell prediction, RAN4 to select a set of typical OW/PW patterns and configurations as the basis for defining new measurement time requirements. - For inter-frequency inter-cell RSRP prediction, RAN4 should study the impact on CSSF reduction in measurement period and define new requirements accordingly. - Proposal 4 (CMCC): - For intra-frequency temporal domain prediction, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy - For inter-frequency inter-cell frequency domain prediction, only the actually measured frequency layers are considered in the measurement delay requirements - Proposal 5 (vivo): - RAN4 needs to consider both the OW window and the sliding/non-sliding window factors. - For Case B, it depends on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission. - For inter-frequency cell prediction, RAN4 should identify the potential RRM impacts on e.g., cell known condition and CSSF. - Proposal 6 (Samsung): - For temporal domain case B, RAN4 to discuss the measurement relaxation. - The relationship of measurement relaxation factor and the skipping pattern/MRRT parameter should be involved - For frequency domain prediction, RAN4 to specify the inter-frequency measurement delay requirement considering CSSF (CSSFinter) reduction, and - to specify the inter-frequency with gap measurement delay requirement and scheduling restriction requirement considering MG reduction

The measurement period requirement should be discussed separately depending on whether it is for intra-frequency intra-cell prediction or inter-frequency inter-cell prediction. For intra-frequency intra-cell cases, it may vary depending on the configuration of OW and PW or whether case A or case B, whereas for inter-frequency inter-cell prediction, only the frequency layer that is actually measured needs to be considered, as in Proposal 4.

Proposal 2: RAN4 to discuss separately depending on whether it is for intra-frequency intra-cell prediction or inter-frequency inter-cell prediction.

- For intra-frequency intra-cell prediction: Depends on the configuration of OW and PW or whether case A or case B

- For inter-frequency inter-cell prediction: Only the frequency layer that is actually measured needs to be considered, as in Proposal 4.

### Issue 2-1-3: Prediction delay requirements

- Proposal 1 (OPPO): - Prediction delay is the time to get prediction report which includes measurement period and inference delay. - For details of inference delay, wait for the progress of beam management in Rel-19 NR_AIML_air.

We do not have a strong view, but we think the prediction delay requirement can be discussed under the prediction reporting delay requirement.

Proposal 3: Prediction delay requirement can be discussed under the prediction reporting delay requirement.

### Issue 2-1-4: Prediction reporting delay requirements

- Proposal 1 (OPPO): - For the prediction reporting delay requirements to be defined, the time for the first available reporting resource and time for L3 filtering are not included. - Proposal 2 (CATT): - RAN4 to define requirements for both case A and case B with a unified framework, e.g., the total delay = measurement delay + inference delay + other needed components. - The requirements defined for Case B can be used for scenario 3 as a baseline. RAN4 need to discuss applicability requirements for scenario 3 considering the aspects introduced by inter-frequency prediction.

While we don't have a strong view, it would be beneficial for the prediction reporting delay to include the time until the first available reporting resource, even if the time for L3 filtering is excluded.

This is because if the time until the first available reporting resource is not included in the requirement for the prediction reporting delay test, the UE might have an issue where it would have to inform the test equipment of the actual point in time it obtained the inference result.

Proposal 4: We propose that the prediction reporting delay requirement should include the time after inference until the first available reporting resource.

### Issue 2-1-5: LCM related requirements

- Proposal 1 (OPPO): - Do not define deactivation/switching/fallback delay requirements for AI functionality. - For AI functionality activation delay and performance monitoring, wait for RAN2 progress on LCM framework. - Proposal 2 (CATT): - For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. - RAN4 to discuss how to define LCM requirements for performance monitoring and fallback procedures after RAN2 have more concrete progress. - Proposal 3 (Tejas Networks): - RAN4 shall specify the handling of LCM for all agreed scenarios and/or sub-cases. - Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values. - RAN4 to specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference. - Proposal 4 (HW): - For RRM measurement prediction, whether and how to define activation delay can wait for more progress in R19 AI/ML air interface beam management. - Proposal 5 (vivo): - RAN4 further discusses whether related LCM requirements are needed, depending on RAN2's design. - For RRM measurement prediction, it is within RAN4's scope of discussion to define the relevant metrics for performance monitoring (if any). - Proposal 6 (Ericsson): - RAN4 to identify performance metric to be reported for performance monitoring at network side. - RAN4 to define delay requirement for performance metric reporting for performance monitoring of UE sided model at the network node. - Proposal 7 (Samsung): - RAN4 to specify requirements for model monitoring including monitoring related conditions (e.g., criterion and the corresponding threshold) definition

Although performance monitoring requirements may be influenced by RAN2’s decisions, we believe that RAN4 can start discussions at this stage. Therefore, it would be beneficial for RAN4 to initiate discussions on performance monitoring requirements. We support Proposal 5 (vivo) and Proposal 7 (Samsung) among the proposals listed above.

Proposal 5: RAN4 to initiate discussions on performance monitoring requirements. We support Proposal 5 (vivo) and Proposal 7 (Samsung) among the proposals listed above.

- Sub-topic 2-2: Performance requirements for RRM measurement prediction

### Issue 2-2-1: Performance metrics definition

- Proposal 1 (Xiaomi): - RAN4 shall discuss on the definition of prediction accuracy for multi-instance reports for temporal domain Case A. - Proposal 2 (HW): - For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss whether only RSRP prediction accuracy under certain channel correlation coefficient is to be specified. - Proposal 3 (Samsung): - RAN4 to discuss the accuracy metrics/KPIs for cell prediction requirements, the following two options of the accuracy KPI can be taken as the candidate - Cell prediction accuracy: Top-1(%), Top-K(%) - Maximum RSRP among Top-K predicted cells is larger than the RSRP of the strongest cell – x dB - Proposal 4 (Apple): - RAN4 is invited to adopt a clear time-indexed definition of relative RSRP accuracy for intra-frequency temporal prediction. - At time t, cell 1 predicts its Layer-3 RSRP at t + Δt1, while cell 2 provides a reported (measured or predicted) RSRP at t + Δt2. The metric compares the predicted power gap to the true power gap at those future times and is expressed as ( P_pred1(t + Δt1) - P_rep2(t + Δt2)) -( P_true1(t + Δt1) - P_true2(t + Δt2)) - RAN4 shall define relative inter-frequency accuracy through two complementary cases to ensure consistent evaluation of cross-frequency prediction. - Case A – Time-domain, cross-frequency: Cell 1 on f₁ predicts its L3-RSRP at t + Δt₁ while Cell 2 on f₂ (same FR) provides a reported value at t + Δt₂ (measured or predicted); the KPI compares predicted vs. true inter-cell power gaps to assess future-time signal differences. - Case B – Frequency-domain, cross-cell: Cell 1 on f₁ is predicted across frequency using measurements from other cells, while Cell 2 on f₂ provides a reported value (predicted or measured); the KPI compares predicted vs. true inter-cell power gaps for cross-frequency inference. - Explicitly state the mathematical KPI as (Pred f1 – Pred/Reported f2) – (GT f1 – GT f2) for both cases.

We support Proposal 2 (HW). However, I’m not sure whether this proposal should be categorized under performance metrics definition.

Regarding Proposal 3, Unlike beam management, for cell-level prediction, absolute accuracy of the cell’s L3-RSRP value or its relative accuracy is more important than ordering. Therefore, we believe that evaluating the ratio of cells included in the Top-K or the proportion within a certain offset from the strongest cell is not appropriate.

Regarding Proposal 4, rather than enforcing a clear time index, it seems more practical to restrict the prediction or measurement values of the two metrics to fall within a certain time range. Furthermore, we prefer applying a margin to the accuracy requirement to account for the time difference between the two metrics.

Proposal 6: Support proposal 2(HW). For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss whether only RSRP prediction accuracy under certain channel correlation coefficient is to be specified.

### Issue 2-2-2: Prediction accuracy requirements for measurement prediction

Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the prediction accuracy has the same or different accuracy requirements as non-AI/ML RRM measurement. - Discuss whether the following parameters need to be the side condition of the accuracy requirements: - OW/PW length - MRRT - Skipping pattern - UE speed - L1 filtering approach and sample number

Regarding AI prediction accuracy requirements, we believe they should guarantee the same level as non-AI RRM measurement accuracy requirements. This is because parameters in the HO procedure are configured based on non-AI RRM measurement accuracy, and if AI prediction accuracy requirements are set differently, it could lead to many variables being affected.

In the discussion on prediction accuracy requirements, we think the following items need to be included with respect to side conditions:

- OW/PW length

- Skipping pattern

- UE speed

- SINR

Proposal 7: Prediction accuracy requirement should guarantee the same level as non-AI RRM measurement accuracy requirements. And at least following parameters are need to be the side conditions of the accuracy requirements:

- OW/PW length

- Skipping pattern

- UE speed

- SINR

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: RAN4 need to define requirements at least for

- Measurement delay/period requirements

- Prediction reporting delay requirements

- OW/PW length

Proposal 2: RAN4 to discuss separately depending on whether it is for intra-frequency intra-cell prediction or inter-frequency inter-cell prediction.

- For intra-frequency intra-cell prediction: Depends on the configuration of OW and PW or whether case A or case B

- For inter-frequency inter-cell prediction: Only the frequency layer that is actually measured needs to be considered, as in Proposal 4.

Proposal 3: Prediction delay requirement can be discussed under the prediction reporting delay requirement.

Proposal 4: We propose that the prediction reporting delay requirement should include the time after inference until the first available reporting resource.

Proposal 5: RAN4 to initiate discussions on performance monitoring requirements. We support Proposal 5 (vivo) and Proposal 7 (Samsung) among the proposals listed above.

Proposal 6: Support proposal 2(HW). For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss whether only RSRP prediction accuracy under certain channel correlation coefficient is to be specified.

Proposal 7: Prediction accuracy requirement should guarantee the same level as non-AI RRM measurement accuracy requirements. And at least following parameters are need to be the side conditions of the accuracy requirements:

- OW/PW length

- Skipping pattern

- UE speed

- SINR

Reference

[1] R4-2514827, “WF on NR_AIML_Mob_RRM”, Moderator(OPPO)

[2] R4-2513532, “Topic summary for [116bis][226] NR_AIML_Mob_RRM”, Moderator(OPPO)