3GPP TSG-RAN WG4 Meeting # 117	R4- 2520720

# Dallas, USA, 17th – 21st November, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for measurement event prediction

Agenda item:	7.12.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary[2] and WF [1].

Discussion

- Sub-topic 3-1: Core requirements for indirect measurement event prediction

### Issue 3-1-1: Core requirements for indirect measurement event prediction

- Proposal 1: (HW, OPPO, LGE, Ericsson, Samsung) - The core requirements for RRM measurement prediction can be reused for indirect measurement event prediction, unless specific issues are identified for indirect measurement event prediction. - Proposal 2: (HW) - If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay. - RAN4 to study the requirement of event prediction delay which is the time between the instance (at t0) where measurement event is predicted and reported, and the instance of predicted measurement event (at t1). - Proposal 3: (CATT, vivo) - RAN4 to discuss the detailed impacts of measurement event prediction on RRM core requirements after RAN2 make enough progress. - RAN4 to discuss the requirements for RRM measurement prediction first and try to reuse the requirements defined for RRM measurement prediction as much as possible. - Proposal 4: (Tejas Networks) - Based on future event prediction, RAN4 shall consider shortlisting potential target cells for handover. - Proposal 5: (Xiaomi) - RAN4 to define predicted event triggered L3 reporting delay. - Proposal 6: (ZTE) - For indirect event prediction, the impact on event prediction report delay should be studied.

If a reporting procedure for RRM measurement prediction results is defined in the case of indirect measurement event prediction, then the core requirements for RRM measurement prediction can be reused.

Proposal 1: The core requirements for RRM measurement prediction can be reused for indirect measurement event prediction, unless specific issues are identified for indirect measurement event prediction.

- Sub-topic 3-2: Performance requirements for indirect measurement event prediction

### Issue 3-2-1: Performance metrics for indirect measurement event prediction

Further discuss based on the open issues in [1] and the following way forward: - Define the accuracy requirements for predicted L3-RSRP for indirect measurement event prediction. - Discuss whether the following performance metrics need to be defined and align the understanding on the definitions: - Option 1: Required window length under certain precision/accuracy - Option 1-1: define ETD of event prediction as a requirement, under the conditions of certain fixed precision and recall or F1 score. - Option 1-2: define ETD of event prediction as a requirement, under the condition of certain event prediction accuracy (e.g., true event prediction or false event prediction). - Option 2: Required precision/accuracy under certain window length - Option 2-1: define precision and recall or F1 score of event prediction as a requirement, under the condition of certain fixed ETD. - Option 2-2: define event prediction accuracy (e.g., true event prediction or false event prediction) of event prediction as a requirement, under the condition of certain fixed ETD. - Option 2-3: Windowed precision ≥ P_min - Precision is the ratio of correctly predicted A3 events to all predicted events, counting only matches within the allowed lead-time window. - Option 3: Lead-time of event prediction - Option 3-1: Lead-time window [L_min, L_max] - Each accepted prediction must arrive at least L_min but no more than L_max before the actual A3 trigger. - Option 3-2: Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted): - It is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event.

Regarding the performance metric, L3 RSRP accuracy requirement and F1 score can be baseline. F1 score is defined at TR38.744 as following:

F1 score=2×Precision ×Recall Precision+Recall

Where:

Precision= n3n1+n3

Recall= n3n2+n3

For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following:

- Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa;

- Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD];

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD].

The ETD, i.e. timing difference between ground-truth event and predicted event, is illustrated in Figure 5.3.1-1:

We believe it is more reasonable to evaluate accuracy metrics within a limited window length. If accuracy is fixed first, determining the window length required to meet that condition could be very complex and make testing overly complicated. Therefore, it seems more practical to first set a fixed window length and then measure accuracy based on the F1 score definition for performance evaluation. So, we support option 2-1.

Proposal 2: Our preference is to first set a fixed window length and then measure accuracy based on the F1 score definition for performance evaluation. So, we support option 2-1.

### Issue 3-2-2: Predicted RSRP accuracy requirements

Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. - Discuss whether TTT configuration should be the side condition of the Predicted L3-RSRP accuracy requirements for indirect measurement event prediction.

We believe that the Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. However, we consider that TTT configuration is largely unrelated to the Predicted L3-RSRP accuracy requirement. So, the TTT configuration does not need to be considered as the side condition of the Predicted L3-RSRP accuracy requirements for indirect measurement event prediction.

Proposal 3: The Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. However, the TTT configuration does not need to be considered as the side condition.

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: The core requirements for RRM measurement prediction can be reused for indirect measurement event prediction, unless specific issues are identified for indirect measurement event prediction.

Proposal 2: Our preference is to first set a fixed window length and then measure accuracy based on the F1 score definition for performance evaluation. So, we support option 2-1.

Proposal 3: The Predicted L3-RSRP accuracy requirements defined for RRM measurement prediction can be reused for indirect measurement event prediction. However, the TTT configuration does not need to be considered as the side condition.

Reference

[1] R4-2514827, “WF on NR_AIML_Mob_RRM”, Moderator(OPPO)

[2] R4-2513532, “Topic summary for [116bis][226] NR_AIML_Mob_RRM”, Moderator(OPPO)